3GPP TSG RAN WG1 Meeting #92	R1-1801468

Athens, Greece, February 26th – March 2nd, 2018

Agenda Item:	7.1.4.3

Source:	Huawei, HiSilicon

Title:	Evaluation of channel coding schemes

Document for:	Discussion and Decision

# Introduction

This contribution is a revision of R1-1800105.

As discussed in [1], URLLC scenario has different requirements from eMBB scenario. URLLC scenario targets applications with lower code rate and smaller block size payloads than eMBB. For eMBB scenario, two LDPC base graphs, BG1 and BG2, were specified for data channels. LDPC BG2 is used for block lengths up to 3840 bits and BG1 for larger blocks. In this contribution, we further evaluate the LDPC BG2 performance at BLERs on the order of 1e-5 and lower which are the target BLERs for URLLC scenarios.

# Discussion

## BLER performance

We set up the evaluation similar to [1] to survey one example of a code length of 1920 and modulation of QPSK, a typical 4RB allocation units. Herein, we further compare the BLER performance of LDPC BG2 with Polar code in Figure 1, where K is the information block length and N is the code length. LDPC BG2 code has error floor in the high reliable zone (BLERs around 1e-5 and lower) whereas Polar code does not.

Figure 1.  Error floor of LDPC BG2

To further investigate the LDPC BG2 performance, we evaluate more cases: agreed eMBB TB sizes, listed in Table 2 (typical small payload size) in the Appendix with six coding rates: 1/12, 1/8, 1/6, 1/3, 1/2 and 2/3. The required SNR for target BLER at 1e-4,1e-5 and 1e-6 are shown in Figures 2, 3 and 4, respectively.

- BLER = 1e-4

Figure 2.  Required SNR of target BLER at 1e-4

- BLER = 1e-5

Figure 3.  Required SNR of target BLER at 1e-5

- BLER = 1e-6

Figure 4.  Required SNR of target BLER at 1e-6

Observation-1: Performance of LDPC BG2 code becomes increasingly inconsistent for the granularity of NR block sizes as the target BLER decreases to values of 1e-5 or 1e-6.

## Results for LDPC BG2 error floor cases

To further investigate the error floor cases, we consider different values of a threshold related to the changing slope of the BLER vs. SNR curves at different target BLER values.

Figure 5. Threshold of the BLER curves slope

In Figure 5,

- K1 is the BLER vs. SNR slope for target BLER 10-2 ~ 10-3

- K2 is the BLER vs. SNR slope for target BLER 10-5 ~ 10-6

The ratio K1/K2 indicates the change in slope of the BLER vs. SNR curves and is used to assess whether there is error floor or not:

- If K1K2>threshold, there is error floor around target BLER at 1e-5

Different values can be considered for the threshold value. For the cases listed in Table 2 in the Appendix with six different coding rates: 1/12, 1/8, 1/6, 1/3, 1/2 and 2/3, and using the BLER curves in Figures 2, 3 and 4, the percentage of error floor cases at different thresholds is summarized in Table 1.

Table 1.  Percentage of error floor cases in LDPC BG2

| Threshold | Percentage of error floor cases |
| --- | --- |
| 1.2 | 64.34% |
| 1.3 | 60.57% |
| 1.4 | 56.81% |
| 1.5 | 52.69% |

As it can be seen from the Table above, there is a high probability (greater than 50%) that for information block lengths corresponding to NR TB sizes the error floor issue exists.

Observation-2: The eMBB LDPC BG2 performance has a high probability of showing error floor for target BLER at 1e-5.

## Lifting size and QC-LDPC codes

One important feature of QC-LDPC is that the parity check matrix shares the same properties within one lifting size. This is because within one lifting size the parity check matrix has the same minimum girth and the trapping set properties. According to the evaluation results, the parity check matrix shares the same error floor tendency under two scenarios:

- same information length (i.e. same K), different N but within the same lifting size;

- same encoded bit length (i.e. same N), different K but within the same lifting size;

Figure 6.  Error floor appears with the same parity check matrix

From the Figure above, we can see that when the information length (K) and encoded bits length (N) fluctuate and the lifting size remains fixed, the BLER-SNR curves share the same error-floor trend.

Observation-3: For QC-LDPC codes, for different combinations of K and N, and a certain fixed lifting size, the BLER-SNR curves share the same error-floor trend.

# Conclusion

From the analysis of the LDPC BG2 performance, it is clear that there exists performance degradation as the target BLER is on the order of 1e-5 and lower. Given the reliability requirements and the information block lengths and code rates targeted by the URLLC scenario, the adoption of LDPC BG2 is not a robust solution because of the presence of an error floor and the inferior performance respect to polar codes as shown in Figure 1.

Based on the performance analysis above, we have the following observations:

Observation-1: Performance of LDPC BG2 code becomes increasingly inconsistent for the granularity of NR block sizes as the target BLER decreases to values of 1e-5 or 1e-6.

Observation-2: The eMBB LDPC BG2 performance has a high probability of showing error floor for target BLER at 1e-5.

Observation-3: For QC-LDPC codes, for different combinations of K and N, and a certain fixed lifting size, the BLER-SNR curves share the same error-floor trend.

# References

[1] R1-1801467, Channel coding for URLLC, Huawei, HiSilicon

[2] Chairman’s notes for RAN1 #91.

# Appendix

Table 2. TBS table [2]