3GPP TSG RAN WG1 Meeting #92                                       	R1-1802945

Athens, Greece, February 26th –March 2nd, 2018

Agenda Item:	7.1.1.4.1

Source:	Ericsson

Title:	Remaining details of RACH formats

Document for:	Discussion, decision

# Introduction

RACH agreements and working assumptions in RAN1 AH 1801 include the following:

Agreements:

Confirm the following working assumptions with the following update:

- Number of time domain RACH occasions within a RACH slot

  - For preamble format (working assumption), at least the following # of occasions:

    - A1: 6

    - A2: 3

    - A3: 2

    - B4: 1

    - B1: 6 or 7

    - C0: 4 6 or 7

    - C2: 2

    - A1/B1: 6 or 7

    - A2/B2: 3

    - A3/B3: 2

  - The value is not applicable (N/A) for format 0-3

Agreements:

- To adopt the following for section 6.3.3.2 of 38.211

------------- text proposal, 38.211 section 6.3.3.2 -----------------------------------------------

Table 6.3.3.2-2: Random access configurations for FR1 and paired spectrum/SUL (short sequence)

| PRACH Configuration Index | Preamble format |  |  | Subframe number | Starting symbol | Number of PRACH slots within a subframe | Number of PRACH occasions within a RACH slot |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | A1 | 16 | 0 | 4,9 | 0 | 1 | 6 |
|  | A1 | 16 | 1 | 4 | 0 | 2 | 6 |
|  | A1 | 8 | 0 | 4,9 | 0 | 1 | 6 |

<parts of table elided>

|  | A3/B3 | 1 | 0 | 0,2,4,6,8 | 0 | 2 | 2 |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | A3/B3 | 1 | 0 | 0,1,2,3,4,5,6,7,8,9 | 0 | 2 | 2 |

------------- end text proposal, 38.211 section 6.3.3.2 -----------------------------------------------

In this contribution, we focus on PRACH format details, including RACH configuration table.

# Discussion

## Agreed formats

Agreed formats are for convenience summarized in Table 1 (long sequence) and Table 2 (short sequence). With short sequence, multiple preambles can be time-multiplexed in one slot as illustrated in Figure 1 (start symbol 0) and Figure 2 (start symbol 2).

Table 1. PRACH preamble formats for long sequence (L = 839)

| Preamble format | SCS (kHz) | BW (MHz) | N_OS | N_RP | T_SEQ (Ts) | T_CP (Ts) | T_GP (Ts) | Use case |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 1.25 | 1.08 | 1 | 1 | 24576 | 3168 | 2975 | LTE refarming |
| 1 | 1.25 | 1.08 | 2 | 1 | 2*24576 | 21024 | 21904 | Large cell, up to 100 km |
| 2 | 1.25 | 1.08 | 4 | 1 | 4*24576 | 4688 | 4528 | Coverage enhancement |
| 3 | 5 | 4.32 | 4 | 1 | 4*6144 | 3168 | 2976 | High speed case |

Table 2. PRACH preamble formats for short sequence (L = 139) *

| Preamble format |  | # of seq. | TCP (Ts) | TSEQ (Ts) | TGP (Ts) | Path profile (Ts) | Path profile (s) | Maximum cell radius (m) | Use case |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| A | 1 | 2 | 288 | 4096 | 0 | 96 | 3.13 | 938 | Small cell |
|  | 2 | 4 | 576 | 8192 | 0 | 144 | 4.69 | 2,109 | Normal cell |
|  | 3 | 6 | 864 | 12288 | 0 | 144 | 4.69 | 3,516 | Normal cell |
| B | 1 | 2 | 216 | 4096 | 72 | 96 | 3.13 | 352 | Small cell |
|  | 2 | 4 | 360 | 8192 | 216 | 144 | 4.69 | 1,055 | Normal cell |
|  | 3 | 6 | 504 | 12288 | 360 | 144 | 4.69 | 1,758 | Normal cell |
|  | 4 | 12 | 936 | 24576 | 792 | 144 | 4.69 | 3,867 | Normal cell |
| C | 0 | 1 | 1240 | 2048 | 1096 | 144 | 4.69 | 5300 | Normal cell |
|  | 2 | 4 | 2048 | 8192 | 2916 | 144 | 4.69 | 9200 | Normal cell |

* Example for 15 kHz SCS, other SCSs obtained by time scaling

Figure 1. PRACH preamble formats for short sequence, with start symbol 0, with SCS of 15 kHz

Figure 2. PRACH preamble formats for short sequence, with start symbol 2, with SCS of 15 kHz

## Interference from PRACH into PDCCH in TDD systems

A significant concern is TRP-to-TRP DL-to-UL interference between PRACH and a preceding PDCCH transmitted in a gap at the beginning of the slot. Such TRP-to-TRP interference is not limited to the nearest cell, but can reach many cells away, and may hence require larger guard. Although this may not be an issue in all scenarios, it is important to have flexibility to avoid the problem in any future deployments where it might occur.

As the issue is about capacity rather than coverage, it is needed also for short sequences, not only for long sequence formats.

The concern can be accounted for by supporting a RACH start symbol value up to 6 or 7 also for some short sequence formats. For explicit table proposals, see Section 2.3.

Support also a RACH start symbol value of 6 or 7 also for some short sequences, to allow mitigation of TRP-to-TRP interference in TDD systems.

Start symbol 6 or 7 may be needed only in certain scenarios, and it is hence enough to have that value for a limited number of PRACH configuration indices. On the other hand, start symbol 2 may be important in a wide range of scenarios, to avoid TRP-to-TRP DL-to-UL interference and leave more room for DL-to-UL switching times. Hence, start symbol 2 should have ample support in the PRACH configuration tables for unpaired spectrum.

Support RACH start symbol value of 2 for a wide range of configurations in the PRACH configuration tables for unpaired spectrum.

## RACH configuration table for unpaired spectrum

RACH configuration tables for short sequence formats in unpaired spectrum have not yet been agreed. To a large extent they can be similar to the tables for paired spectrum, with a few important differences:

The RACH configuration tables for unpaired spectrum can be based on the table for paired spectrum, but it should have ample support for starting symbol 2 and some support for starting symbol 6 or 7.

An example table (example for format A3 only) is provided in Table 3, which is directly based on the table for FR1 paired spectrum in Chairman’s notes [1], but with the FR1 subframe indices translated to the FR2 60 kHz slot (0.25 ms) indices, and with some other entries updated in line with Proposal 3 and the discussions in Section 2.2. All updates (except the subframe-to-slot number translations) are highlighted with colors (red for modifications, blue for additions). The details of the table contents need to be coordinated with the decisions on, e.g., mappings from SS blocks to RACH occasions, see [2].

Base RACH configuration tables for unpaired spectrum on Table 3.

Table 3. PRACH configuration table for unpaired FR2, example for format A3

| PRACH Config. Index | Preamble Format | nSFN mod x= y |  | 60 kHz slot (0.25 ms) index within system frame | Start symbol Index | Number of RACH slots within 0.25 ms | Number of time domain RACH occasions within a RACH slot |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | x | y |  |  |  |  |
| X+1 | A3 | 16 | 1 | 16–19,36–39 | 2 | 1 | 2 |
| X+2 | A3 | 8 | 1 | 16–19,36–39 | 0 | 1 | 2 |
| X+3 | A3 | 4 | 0 | 16–19,36–39 | 2 | 1 | 2 |
| X+4 | A3 | 16 | 1 | 16–19 | 0 | 2 | 2 |
| X+5 | A3 | 8 | 1 | 16–19 | 2 | 2 | 2 |
| X+6 | A3 | 4 | 0 | 16–19 | 0 | 2 | 2 |
| X+7 | A3 | 2 | 1 | 8–11,24–27, 36–39 | 2 | 2 | 2 |
| X+8 | A3 | 2 | 0 | 4–7 | 0 | 2 | 2 |
| X+9 | A3 | 2 | 0 | 16–19 | 2 | 2 | 2 |
| X+10 | A3 | 2 | 0 | 28–31 | 2 | 2 | 2 |
| X+11 | A3 | 2 | 0 | 28–31 | 6 | 2 | 2 |
| X+12 | A3 | 1 | 0 | 16–19 | 2 | 2 | 2 |
| X+13 | A3 | 1 | 0 | 16–19 | 0 | 1 | 2 |
| X+14 | A3 | 1 | 0 | 28–31 | 2 | 2 | 2 |
| X+15 | A3 | 1 | 0 | 28–31 | 6 | 2 | 2 |
| X+16 | A3 | 1 | 0 | 4–7,24–27 | 0 | 1 | 2 |
| X+17 | A3 | 1 | 0 | 8–11,28–31 | 2 | 2 | 2 |
| X+18 | A3 | 1 | 0 | 16–19,36–39 | 2 | 1 | 2 |
| X+19 | A3 | 1 | 0 | 4–7,16–19,28–31 | 0 | 2 | 2 |
| X+20 | A3 | 1 | 0 | 0–3,8–11,16–19,24–27,32–35 | 2 | 2 | 2 |
| X+21 | A3 | 1 | 0 | 0–39 | 0 | 2 | 2 |
| X+22 | A3 | 1 | 0 | 4–7,12–15,20–23,28–31,36–39 | 0 | 2 | 2 |

## Valid RACH resources

The UE should only be allowed to transmit PRACH preambles at time instances which are not indicated in RMSI as used for actually transmitted SS blocks. This is in order to avoid the need for full duplex in the TRP. In other words, the UE should not be allowed to transmit a preamble if any part of it would overlap with any OFDM symbol indicated as used for an actually transmitted SS block.

The UE should only be allowed to transmit PRACH preambles at time instances (OFDM symbols) not indicated as used for actually transmitted SS blocks.

It is up to the network to ensure that there are no collisions between other transmission and RACH. This minimizes unnecessary signaling in the network.

In order to keep RMSI signaling to a minimum, it should be up to the network to avoid collisions between RACH and any other transmissions besides SS blocks.

# Conclusion

Based on the discussion in Section 2 we propose the following:

Proposal 1	Support also a RACH start symbol value of 6 or 7 also for some short sequences, to allow mitigation of TRP-to-TRP interference in TDD systems.

Proposal 2	Support RACH start symbol value of 2 for a wide range of configurations in the PRACH configuration tables for unpaired spectrum.

Proposal 3	The RACH configuration tables for unpaired spectrum can be based on the table for paired spectrum, but it should have ample support for starting symbol 2 and some support for starting symbol 6 or 7.

Proposal 4	Base RACH configuration tables for unpaired spectrum on Table 3.

Proposal 5	The UE should only be allowed to transmit PRACH preambles at time instances (OFDM symbols) not indicated as used for actually transmitted SS blocks.

Proposal 6	In order to keep RMSI signaling to a minimum, it should be up to the network to avoid collisions between RACH and any other transmissions besides SS blocks.

# References

Chairman’s notes, 3GPP TSG-RAN WG1#NR1801, Vancouver, BC, Canada, 22nd – 26th January 2018

R1-1802946, “Remaining details of RACH procedure”, Ericsson, 3GPP TSG RAN WG1 Meeting #92, Athens, Greece, 26th February – 2nd March 2018