3GPP TSG RAN WG1 Meeting #92bis								    R1-1804884

Sanya, China, April 16th – 20th, 2018

Agenda Item:	7.6.5

Source:	InterDigital Inc.

Title:	On Potential Waveform Solutions for NR-U

Document for:	Discussion

# Introduction

In RAN#75 meeting, a new SID was proposed to study NR-based access to unlicensed spectrum [1]. One justification for such proposal is based on the vast unlicensed spectrum available worldwide. This was also a justification to initiate LTE-based LAA. Another justification is the flexible frame structure and numerology that NR has, which could also be brought to unlicensed access.

The WI description [1] highlights the following objectives:

- “Study NR-based operation in unlicensed spectrum (RAN1, RAN2, RAN4) including

  - Physical channels inheriting the choices of duplex mode, waveform, carrier bandwidth, subcarrier spacing, frame structure, and physical layer design made as part of the NR study and avoiding unnecessary divergence with decisions made in the NR WI

    - Consider unlicensed bands both below and above 6GHz, up to 52.6GHz

    - Consider unlicensed bands above 52.6GHz to the extent that waveform design principles remain unchanged with respect to below 52.6GHz bands

    - Consider similar forward compatibility principles made in the NR WI

  - Initial access, channel access. Scheduling/HARQ, and mobility including connected/inactive/idle mode operation and radio-link monitoring/failure

  - Coexistence methods within NR-based and between NR-based operation in unlicensed and LTE-based LAA and with other incumbent RATs in accordance with regulatory requirements in e.g., 5GHz, 37GHz, 60GHz bands

    - Coexistence methods already defined for 5GHz band in LTE-based LAA context should be assumed as the baseline for 5GHz operation. Enhancements in 5GHz over these methods should not be precluded. NR-based operation in unlicensed spectrum should not impact deployed Wi-Fi services (data, video and voice services) more than an additional Wi-Fi network on the same carrier;”

In RAN meeting #79 [2], two new Study Items on NR waveform beyond 52.6GHz were proposed [3][4]. Among the potential use cases, V2X, IAB, NR-licensed, NR-unlicensed, Non-terrestrial, etc. are identified. Per decision at this RAN meeting, an email discussion is to be kicked off on “NR design beyond 52.6 GHz” to consolidate the scope for this SI proposal [5].

This contribution discusses the channelization, waveform, and numerology aspects for NR operation in unlicensed spectrum for high frequency bands, i.e., 60GHz.

# Waveform and Numerology for High-Frequency Unlicensed Bands

For 60 GHz unlicensed bands, RAN1 should study if the existing waveforms and numerologies in NR are applicable, and if new waveforms need to be explored. In addition to regulatory requirements, the study of waveforms and corresponding numerologies in high frequency bands should consider other factors including narrow beamforming, hardware limitations (such as phase noise and RF non-linearity), and incumbent’s waveforms (e.g., SC-QAM in IEEE 802.11ad/ay and OFDM in IEEE 802.11ay).

A waveform that has high peak-to-average power ratio (PAPR) requires a large amount of power back-off at the input to the PA to avoid non-linear signal distortion, which in turn reduces PA efficiency and the coverage area. Therefore, we think that single carrier-like schemes such as SC-QAM and DFT-s-OFDM, which can introduce a PAPR advantage over OFDM, are some of the most appealing waveforms for NR-U at 60 GHz.

## SC-QAM

The SC-QAM waveform by up-sampling, filtering and down-sampling QAM symbols directly, is a candidate waveform for 60 GHz band for NR-U. SC-QAM introduces low hardware complexity and yields low PAPR for low modulation orders and it has been adopted in IEEE 802.11ad [6] and to be in IEEE 802.11ay [7][8]. The illustrations for the corresponding transmitter and reference receiver diagrams for the incumbents at 60 GHz are given in Figure 2-1. At the transmitter side, a data block of length of Ncb×512 can be up-sampled by 3, filtered with shaping filter, and down-sampled by 2, and the resulting samples are transmitted with the rate of fs=Ncb×2.64 GHz, where Ncb={1,2,3,4}. The up-sample rate of 3 and the down-sample rate of 2 are chosen since these settings allow both OFDM and SC-QAM to operate at the same sample rate of 2.64 GHz.

Figure 2-1: Basic SC-QAM transmitter and receiver block diagrams at 60 GHz (Incumbent)

The spectrum utilization examples due to the incumbents are provided in Figure 2-2. For example, the spectrum can be occupied by four SC-QAM waveforms operating on each channel or an SC-QAM waveform operating on the entire 4×1.76 GHz.

Figure 2-2: Spectrum utilization examples with SC-QAM at 60 GHz (Incumbent)

For SC-QAM, low-complexity frequency domain processing techniques at the receiver are typically enabled by the methods that convert the linear convolution of the channel to a circular convolution. For example, extending the data symbol block with a sequence or cyclically extending the data blocks before pulse shaping are the traditional approaches that convert the impact of channel on data blocks to a circular convolution at the receiver for SC systems. Among these methods, it has been shown that extending the data block with a sequence can be more beneficial than the cyclic extension as it enables various techniques for non-linear frequency domain equalization (FDE), time-frequency synchronization, channel estimation, phase tracking (similar to the principles of PTRS), and noise estimation. IEEE 802.11ad/ay have adopted the sequence-based approach and the data blocks are extended with a set of Golay sequences to enable single-tap MMSE-FDE.

Albeit the benefits of the SC-QAM, the main concerns on SC-QAM waveform for NR-U may be listed as follows:

- Standardization effort: The numerology for SC-QAM may need to be different than the agreed numerologies for below 52.6 GHz, which may result in extra standardization effort for NR-U.

- Performance: The down-sampling at the SC receiver before FDE as shown in Figure 2-1 leads to a suboptimal performance in fading channel. This is mainly because the down-sampling in time corresponds to overlapping in frequency domain as in Figure 2-3, which leads to incoherent combinations under a fading channel as the channel coefficients are arbitrary complex numbers in frequency. It has been shown that the down-sampling before FDE causes degradation at the receiver around 1 dB in peak-throughput as compared to a DFT-s-OFDM waveform [9].

Figure 2-3 Equivalent representation of down-sampling with DFT and IDFT operations

- Spectral efficiency: SC-QAM waveform handles the interference among the channels by utilizing a shaping filter. The filter leads to roll-off bands and additional guard bands may be needed to decrease the interference. However, this bands cause a less spectrally-efficient system.

## Wide-band DFT-s-OFDM

Another option for the waveform in 60GHz band is to use a wide-band DFT-s-OFDM with large DFT precoder while reusing the existing waveform structure defined for the licensed bands below 52.6 GHz for NR-U. One can show that the data symbols are convolved with a Dirichlet sinc function circularly in time with DFT-s-OFDM. Hence, its time-domain characteristics is similar to SC-QAM. The PAPR is slightly larger than that of SC-QAM due to the side lobes of Dirichlet sinc in time. However, this difference can be addressed by introducing frequency domain windowing as shown in Figure 2-4 [9]. Since DFT-s-OFDM is also a precoded OFDM symbol, it can enable efficient FDMA by eliminating the guard bands required for the one with SC-QAM waveform and orthogonalize the waveform on different channels. Another advantage is that it can admit phase tracking reference symbols (PTRS) which was agreed in the previous meetings for the licensed bands, which is essential for the phase noise compensation at high frequencies.

Figure 2-4 :Comparison of SC-QAM and DFT-s-OFDM in terms of PAPR

Considering the channelization at 60 GHz, the OCB requirement, and the incumbents’ waveforms, NR-U can support similar bandwidth to that of incumbents at 60 GHz. To this end, additional numerology options to the existing numerologies can be introduced for NR-U at 60 GHz as in Table 1. For example, Option 2, Option 4, or Option 6, which leads to maximum bandwidth of 1.584 GHz, may be considered. As compared to other options, Option 2 uses a smaller IDFT size of 1024 (i.e., less-hardware complexity) and a larger subcarrier spacing of 1920 kHz (i.e., more immunity against phase noise) comparing to other options. Option 4 (Δf=960 kHz) and Option 6 (Δf=480 kHz) is similar to the existing numerology with Δf=240 kHz and Δf=120 kHz, respectively, but the sample rate is quadrupled. Considering the numerology options in Table 1, the DFT precoder size for NR-U at 60 GHz may be set to maximum available size, i.e., M=max12×2α23α35α5≤12×#of RBs.

Table 1 Possible numerology options for NR-U at 60 GHz band

|  | Δf [kHz] | M | N | fs [kHz] | # of guard tones on edge | Bandwidth | # of RBs |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Existing numerology (R15) | 15 | 12×2α23α35α5 | 4096 | 30720×2=61440 | 398 | 49500 | 275 |
|  | 30 | 12×2α23α35α5 | 4096 | 30720×4=122880 | 398 | 99000 | 275 |
|  | 60 | 12×2α23α35α5 | 4096 | 30720×8=245760 | 398 | 198000 | 275 |
|  | 120 | 12×2α23α35α5 | 4096 | 30720×16=491520 | 398 | 396000 | 275 |
|  | 240 | 12×2α23α35α5 | 2048 | 30720×16=491520 | 196 | 397440 | 138 |
| Option 1 | 960 | 12×64 | 1024 | 30720×32=983040 | 98 | 737280 | 69 |
| Option 2 | 1920 | 12×64 | 1024 | 30720×64=1966080 | 98 | 1474560 | 69 |
| Option 3 | 480 | 12×135 | 2048 | 30720×32=983040 | 196 | 777600 | 138 |
| Option 4 | 960 | 12×135 | 2048 | 30720×64=1966080 | 196 | 1555200 | 138 |
| Option 5 | 240 | 12×270 | 4096 | 30720×32=983040 | 398 | 777600 | 275 |
| Option 6 | 480 | 12×270 | 4096 | 30720×64=1966080 | 398 | 1555200 | 275 |

Note that these numerology options maintain the orthogonality between the tones on different channels. The channelization in 60 GHz band is based on 2.16 GHz. Since 2.16 GHz is integer multiple of 15×2n kHz for n≤7 (e.g., 2160e6/1920e3=1125 for n=7), it is possible to orthogonalize the subcarriers at different channels up to Δf=15×2n≤1920 kHz. For example, if the subcarrier spacing is chosen as 480 kHz, it is possible to place 4500 orthogonal subcarriers at the bandwidth of 2.16 GHz. Hence, one can use 4096 of 4500 orthogonal tones (i.e., the IDFT size is assumed to 4096) to achieve the orthogonality among the tones located at other channels as illustrated in Figure 2-5.

Figure 2-5: An example utilization of the 60 GHz channels

The waveform design should enable the coexistence with incumbents. Hence, the requirements such as ACI should also be considered. Since the incumbents’ waveform in 60 GHz is SC-QAM, they can lead to low adjacent channel interference to NR-U. Windowing or filtering should also be applied for DFT-s-OFDM. In addition, since large subcarrier spacing with existing NR numerology reduces to cyclic prefix duration, the inter-symbol interference may be re-evaluated for 60 GHz. Similar approaches to the flexible guard intervals in IEEE 802.11ay may be utilized to address inter-symbol interference [10][11] and low out-of-band emission.

## B-IFDM

The interlaced resource allocation method and corresponding multiple access scheme, called block-interleaved frequency division multiple access (B-IFDMA) along with DFT or identity precoder, is another candidate for 60 GHz.

Figure 2-6: B-IFDM transmitter block diagram

In B-IFDM, the input of IDFT can be portioned into B clusters of size of A×12, and the cluster are separated by C×12 tones in frequency domain. The number of interlaces can be calculated as CA+1.

In B-IFDM, the precoding matrix size M is equal to A×B. If the precoding matrix is DFT, this waveform does not exactly have the characteristics of SC-waveform as the DFT-s-OFDM structure is distorted by adding C×12 zeros between the clusters. Hence, the PAPR of this waveform is worse than DFT-s-OFDM. If the precoding is an identity matrix, this structure is similar to OFDM and decreases the complexity at the receiver and the transmitter. On the other hand, the PAPR of the corresponding signal is high without any extra mitigation method.

We do not recommend B-IFDM for the operation at 60 GHz for NR-U because of the following reasons:

- Immunity against impairments: While it is trivial to address the phase noise impact with SC-QAM and wide band DFT-s-OFDM along with the existing NR’s PTRS design, B-IFDM may not get benefit from the agreed PTRS structure for licensed bands. In addition, B-IFDM with DFT precoding is not exactly a SC waveform as the mapping in frequency is not localized. Hence, the PAPR performance is worse than that of SC-QAM and DFT-s-OFDM.

- Standardization effort: Since interlace structure in 60 GHz may require new reference signal designs, the effort for new RS design may cause extra standardization effort for NR-U. As the channel bandwidth is 2.16 GHz in 60 GHz, a completely new interlace design (i.e., A,B, and C in Figure 2-6) is needed. This may cause extra standardization effort for NR-U.

- Time-domain utilization: In 60 GHz, time domain duplexing (TDD) may be more beneficial than frequency domain duplexing (FDD) to reduce the beam training procedure as the network can exploit the channel reciprocity with TDD. As the chance of having interference is low with narrow beam transmission, a simple LBT protocol may be desired in 60 GHz band. Hence, the interlace structure in frequency may not be beneficial since the channel selectivity reduces in 60 GHz band along with narrow beams.

Based on the above discussion, we have the following proposal for NR-U waveform at 60 GHz.

Proposal 1: For 60 GHz band, OFDM and DFT-s OFDM in both UL and DL should be studied considering incumbent waveforms, hardware impairments, and regulatory requirements.

# Conclusions

In this contribution, we reviewed the waveform technologies for NR operation in unlicensed spectrum. We propose the following:

Proposal 1: For 60 GHz band, OFDM and DFT-s OFDM in both UL and DL should be studied considering incumbent waveforms, hardware impairments, and regulatory requirements.

# References

RP-170828, New SID on NR-based Access to Unlicensed Spectrum, RAN 75, Dubrovnik, Croatia, March 6 - 9, 2017

3GPP TSG RAN Meeting #79 Chennai, India, March 19 - 22, 2018

RP-180320 New SID: NR design above 52.6GHz, Intel Corporation, RAN Meeting #79 Chennai, India

RP-180453 New SID on NR waveform beyond 52.6GHz, Qualcomm Europe Inc.(France), RAN Meeting #79

RP-180594 List of additional email discussions on Rel-16 proposals, RAN Meeting #79

"Part 11: Wireless LAN Medium Access Control (MAC) and Physical Layer (PHY) Specifications," 2016.

IEEE 802.11 Working Group, “Enhanced throughput for operation in license-exempt bands above 45 GHz,” IEEE P802.11ay/D0.3, Mar. 2017.

Y. Ghasempour, C. R. C. M. da Silva, C. Cordeiro, and E. W. Knightly, "IEEE 802.11ay: Next-Generation 60 GHz Communication for 100 Gb/s Wi-Fi," IEEE Communications Magazine, vol. 55, no. 12, pp. 186-192, Dec. 2017.

A. Sahin, R. Yang, F. LaSita, and R. Olesen, "A Comparison of SC-FDE and UW DFT-s-OFDM for Millimeter Wave Communications", IEEE International Conference on Communications (ICC), Kansas City, May 2018, https://arxiv.org/abs/1801.09177

R1-1609599, “Way forward waveform for carrier frequencies beyond 40 GHz,” Nokia, Alcatel-Lucent Shanghai Bell, Mitsubishi Electric, InterDigital Communications

R1-1609889 Waveform design considerations for carrier frequencies above 40 GHz, InterDigital Communications, Oct 2016.