3GPP TSG RAN WG1 Meeting #92bis		R1-1805035

Sanya, China, April 16th – 20th, 2018

Source:	NTT DOCOMO, INC.

Title:	Remaining issues on PRACH formats

Agenda Item:	7.1.1.4.1

Document for: 	Discussion and Decision

- Introduction

At the RAN1#92 meeting, remaining issues regarding RACH configuration table and others were discussed and RAN1 made agreements on the RACH configuration table for FR1, unpaired spectrum and short sequences, and a part of the configuration table for FR2 [1]. In this contribution, we discuss on remaining issues regarding RACH configuration table.

- Relationship between RACH occasion and TDD configuration/SSB location

  - Collision between RACH occasion and DL part/SSB

During discussion on RACH configuration table for FR2, the problem of collision between RO and DL part/SSB was raised, and RAN1 agreed that NR introduces handling of valid/invalid RO when such collision occurs as following.

Agreements: - For the previous working assumption: - If the Semi-static UL/DL configuration is in RMSI, only PRACH occasions within the UL part is transmitted - UE assumes that RACH occasions configured in RMSI are not collided with DL transmission - Introducing start symbol(s) larger than 2 for a limited number of entries in the Configuration table. - For FR2, it is replaced with the following working assumption (no change of the previous working assumption regarding FR1) - For the Semi-static UL/DL configuration in RMSI, only PRACH occasions within the UL part and X part are valid as long as it does not precede or collide with an SSB in the RACH slot - UE is not expected to receive DL signals during any valid RACH occasion - Note: In the UL/DL semi-static configuration, 0 UL slots and 0 UL symbols can be configured - FFS how to handle the gap necessary in between SS/PBCH block or DL part and PRACH tx

However, the handling of valid/invalid RO is introduced only for FR2. If the handling is not introduced for FR1, i.e., current working assumption is kept for FR1, it is just error case if the collision between RO and DL part/SSB occurs. On the other hand, based on the handling introduced for FR2, since RO collided with DL part/SSB is invalid, gNB is allowed to configure such RACH configuration, SSB indication and DL/UL configuration. It is beneficial in terms of gNB configuration flexibility. Also, since the handling is already introduced for FR2, applying the same handling rule to both FR1 and FR2 can simplify UE behaviour on valid/invalid RO determination. Therefore, the above new working assumption should not be limited for FR2, and should be applied for both FR1 and FR2 in order to avoid different specification and different UE behaviour between FR1 and FR2.

Proposal 1: In both FR1 and FR2, the same handling rule for determination of valid RO should be applied.

  - Gap between SSB/DL part and RO

As for how to handle the gap between SSB/DL part and RO, following alternatives can be considered.

Alt1: If a RO collides with the SSB/DL part, the whole RACH slot including the RO is invalid.

Alt2: If a RO collides with 2 symbols after the end of SSB/DL part, the RO is invalid.

Alt3: If a RO collides with DL part or X part, the RO is invalid.

Alt1 can provide sufficient gap for cell size by gNB configuration, i.e., DL/UL configuration and actually transmitted SSB configuration. A few symbols after the last possible SSB within a slot is sufficient for the gap in most cases. Also in Alt1, ROs can be uniformly located within a RACH slot, i.e., same number of ROs is located within each RACH slot. In Alt2, although more ROs can be located than Alt1, the gap may not be enough depending on deployment scenario, and ROs may be non-uniformly located within the RACH slot containing SSB/DL part, e.g., there is 1 RO within a RACH slot containing SSB/DL part, and 7 ROs within another RACH slot. In Alt3, the RO collided with X part including the gap is also invalid. In that case, since some UL parts in RMSI indication are needed for RO, end of a DL/UL periodicity needs to be explicitly assigned as UL part in RMSI, and then 64 SSBs cannot be transmitted for some DL/UL periodicities. Based on the above discussion, Alt1 is generally better than others, but Alt1 cannot provide sufficient gap only in the case where the last symbol in the slot before a RACH slot is DL part and first 2 symbols in the RACH slot should be the gap. Therefore, we propose combination of Alt1 and Alt2, where Alt2 is used only in the case where the last symbol in the slot before a RACH slot is DL part, otherwise Alt1 is used.

Proposal 2: If a RO collides with the SSB/DL part, the whole RACH slot including the RO is invalid.

- In case that the last symbol in the slot before a RACH slot is DL part, if a RO collides with first 2 symbols in the RACH slot, the RO is invalid.

  - Invalid RO in different RACH configuration period

Based on above working assumption for FR2, when SS burst set periodicity is longer than RACH configuration period, relative position and number of valid ROs in different RACH configuration period may be different, which may be resulting in increasing UE implementation complexity and gNB scheduling complexity. Thus, if a RO becomes invalid in any RACH configuration period due to collision with SSB, corresponding RO in every RACH configuration period should become invalid, which is similar as the proposal discussed during the last meeting.

Proposal 3: If a RO becomes invalid in any RACH configuration period due to collision with SSB, corresponding RO in every RACH configuration period becomes invalid.

- RACH configuration table for FR2

  - The number of PRACH slots within a 60 kHz slot

The number of PRACH slots within a 60 kHz slot in RACH configuration table for FR2 should be 1 for most entries so that the entries can be applied for DL/UL configuration where only one consecutive UL slot is located within a DL/UL periodicity. It is beneficial to locate each UL slot apart considering HARQ ACK timing. We can consider realizing such DL/UL configuration for some cases by the handling rule of invalid RO in Section 2, but there is still issue for following cases even based on the handling rule. Fig.1 shows possible DL/UL configurations with short PUSCH/PUCCH in middle of the periodicity, i.e., slot#1, for short delay of UL/ACK, which is based on SCS = 120 kHz.

Figure.1: Example of confliction case between RO and actual DL/UL configuration which UE can know after dedicated signalling

In case of Ex1 in Fig.1, semi-static DL/UL configuration in RMSI has to indicate symbol#10, 11, 12, 13 in slot#1, all symbols in slot#2 and slot#3 as X(flexible) part, i.e., no explicit DL/UL indication for these symbols/slots, since semi-static DL/UL configuration in RMSI can have only one DL/UL switching point within a DL/UL periodicity. In other words, slot#2 and #3 need to be indicated as X part in RMSI even though the slots is actually DL part which UE can know after dedicated signalling. Based on current working assumption for FR2, RO can be located in X part, and then if number of PRACH slots within 60 kHz slot = 2, slot#3 has valid ROs. Also in case of Ex2, there is same issue that slot#2 has valid ROs. Then, DL/UL configurations shown in Fig.1 cannot be supported, and that is strong restriction on DL/UL configuration. That's why we propose number of PRACH slots within 60 kHz slot = 1 for most entries since it is safer in order to allow DL/UL configurations such as Fig.1. Although some entries, e.g., with index 1, 9, 11 for format A1 in working assumption of RACH configuration table, can be applied for DL/UL configurations shown in Fig.1, these entries don't have enough number of RACH occasions within a slot due to large starting symbol index. Also, in cases of 0.625 ms and 0.5 ms DL/UL periodicity for e.g. low latency, we cannot use bi-period pattern, i.e., concatenated two DL/UL configuration, since minimum value of DL/UL periodicity is 0.5 ms. Thus, the number of PRACH slots within 60 kHz slot should be 1 for most entries.

Proposal 4: The number of PRACH slots within 60 kHz slot should be 1 for most entries for all formats in RACH configuration table for FR2.

If there is clear motivation for support of the number of PRACH slots within 60 kHz slot = 2, we should utilize remaining entries in working assumption of RACH configuration table for FR2, so that both of the number of PRACH slots within 60 kHz slot = 1 and 2 can be supported.

Proposal 5: The remaining entries in working assumption of RACH configuration table for FR2 should be utilized so that both of the number of PRACH slots within 60 kHz slot = 1 and 2 can be supported.

If proposal 4 and 5 are not accepted based on some valid reason, our compromised suggestion is that different PRACH formats have different values of number of PRACH slots within 60 kHz slot. We already have some formats with same time duration, e.g., A1, B1, A1/B1 and C0 have 2 symbol duration, and then number of PRACH slots within 60 kHz slot = 1 can be mainly applied for one format among multiple formats with same time duration in order to avoid strong restriction of DL/UL configuration. Our proposal is number of PRACH slots within 60 kHz slot = 1 should be mainly applied at least for format C0, A2/B2, C2.

Proposal 6: If proposal 4 and 5 are not accepted based on some valid reason, the number of PRACH slots within 60 kHz slot should be 1 for most entries at least for format C0, A2/B2, C2 in RACH configuration table for FR2.

  - Starting symbol index

As for selecting starting symbol index 0 or 2, starting symbol index for B1, C0 should be 0 so that many ROs can be located within a RACH configuration period. Table I shows the number of RO within a RACH configuration period for some major entries for format B1, C0. For configuration index #0, i.e., slot number {4,9,14,19,24,29,34,39}, there are 8 RACH slots within a RACH configuration period. If it is required to map ROs with all actually transmitted SSBs within a RACH configuration period, gNB can configure up to 56 SSBs in case of starting symbol index 0. But in case of starting symbol index 2, gNB can configure only up to 48 SSBs. Also, for configuration index #8 and 13, i.e., slot number {23,27,31,35,39} and slot number {7,15,23,31,39}, there are 5 RACH slots within a RACH configuration period. In such cases, if gNB configures 64 SSBs, 2 RACH configuration periods are enough for RO mapping with all 64 SSBs in case of starting symbol index 0. On the other hand, in case of starting symbol index 2, 4 RACH configuration periods are needed for the RO mapping, which is resulting in RACH delay. For configuration index #10, i.e., slot number {3,7,11,15,19,23,27,31,35,39}, there are 10 RACH slots within a RACH configuration period. In that case, similarly as configuration index #8 and 13, starting symbol index 0 is efficient since a RACH configuration period is enough for the RO mapping with 64 SSBs, and 2 RACH configuration period are needed for the RO mapping in case of starting symbol index 2. Configuration indices #0, 8, 13 and 10 are just example, and the same set of slot number in these configuration indices would be applied for other entries with x = 2, 4, 8 and 16. Then, the same issue on the number of RACH configuration periods for the RO mapping would be raised in many entries. Therefore, we propose starting symbol index for B1, C0 should be 0 so that SSB to RO mapping period can be smaller number of RACH configuration periods as much as possible.

Proposal 7: Starting symbol index should be 0 at least for format B1 and C0.

Table I: Relationship between starting symbol index within a RACH slot and the number of ROs within a RACH slot or a RACH configuration period

| Preamble Format | Starting symbol index within a RACH slot | The number of ROs within a RACH slot | The number of RO within a RACH config period |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Config index #0 | Config index #8,13 | Config index #10 |
| B1, C0 | 0 | 7 | 56 | 35 | 70 |
|  | 2 | 6 | 48 | 30 | 60 |

  - Summary

In summary, proposals on RACH configuration table including discussion and proposal above are shown in Appendix. Table A1 is RACH configuration table for FR2.

Proposal 8: NR supports RACH configuration table for FR2 in Table A1 in Appendix.

- Conclusion

In this contribution, we discussed and proposed our view on RACH configuration table. We made the following proposals.

Proposal 1: In both FR1 and FR2, the same handling rule for determination of valid RO should be applied.

Proposal 2: If a RO collides with the SSB/DL part, the whole RACH slot including the RO is invalid.

- In case that the last symbol in the slot before a RACH slot is DL part, if a RO collides with first 2 symbols in the RACH slot, the RO is invalid.

Proposal 3: If a RO becomes invalid in any RACH configuration period due to collision with SSB, corresponding RO in every RACH configuration period becomes invalid.

Proposal 4: The number of PRACH slots within 60 kHz slot should be 1 for most entries for all formats in RACH configuration table for FR2.

Proposal 5: The remaining entries in working assumption of RACH configuration table for FR2 should be utilized so that both of the number of PRACH slots within 60 kHz slot = 1 and 2 can be supported.

Proposal 6: If proposal 4 and 5 are not accepted based on some valid reason, the number of PRACH slots within 60 kHz slot should be 1 for most entries at least for format C0, A2/B2, C2 in RACH configuration table for FR2.

Proposal 7: Starting symbol index should be 0 at least for format B1 and C0.

Proposal 8: NR supports RACH configuration table for FR2 in Table A1 in Appendix.

References

- 3GPP RAN1#92, Chairman’s note, Mar. 2018.

Appendix: Proposal on RACH configuration table

Table A1: Random access configurations for FR2 and unpaired spectrum.

| PRACH Configuration Index | Preamble format |  |  | Slot number with 60 kHz SCS | Starting symbol | Number of PRACH slots within a 60 kHz SCS slot | Number of time-domain PRACH occasions within a RACH slot |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | A1 | 1 | 0 | 4,9,14,19,24,29,34,39 | 0 | 1 | 6 |
| 1 | A1 | 1 | 0 | 4,9,14,19,24,29,34,39 | 7 | 1 | 3 |
| 2 | A1 | 1 | 0 | 24,29,34,39 | 0 | 2 | 6 |
| 3 | A1 | 1 | 0 | 24,29,34,39 | 7 | 1 | 3 |
| 4 | A1 | 1 | 0 | 18,19,38,39 | 0 | 2 | 6 |
| 5 | A1 | 1 | 0 | 18,19,38,39 | 7 | 1 | 3 |
| 6 | A1 | 1 | 0 | 0,1,2,…,39 | 0 | 2 | 6 |
| 7 | A1 | 1 | 0 | 0,1,2,…,39 | 7 | 1 | 3 |
| 8 | A1 | 1 | 0 | 23,27,31,35,39 | 0 | 1 | 6 |
| 9 | A1 | 1 | 0 | 23,27,31,35,39 | 7 | 1 | 3 |
| 10 | A1 | 1 | 0 | 3,7,11,15,19,23,27,31,35,39 | 0 | 1 | 6 |
| 11 | A1 | 1 | 0 | 3,7,11,15,19,23,27,31,35,39 | 7 | 1 | 3 |
| 12 | A1 | 1 | 0 | 1,3,5,7,…,37,39 | 0 | 2 | 6 |
| 13 | A1 | 1 | 0 | 7,15,23,31,39 | 0 | 1 | 6 |
| 14 | A1 | 1 | 0 | 7,15,23,31,39 | 7 | 1 | 3 |
| 15 | A1 | 16 | 1 | 4,9,14,19,24,29,34,39 | 0 | 1 | 6 |
| 16 | A1 | 16 | 1 | 3,7,11,15,19,23,27,31,35,39 | 0 | 1 | 6 |
| 17 | A1 | 8 | 1 | 4,9,14,19,24,29,34,39 | 0 | 1 | 6 |
| 18 | A1 | 8 | 1 | 3,7,11,15,19,23,27,31,35,39 | 0 | 1 | 6 |
| 19 | A1 | 4 | 1 | 4,9,14,19,24,29,34,39 | 0 | 1 | 6 |
| 20 | A1 | 4 | 1 | 3,7,11,15,19,23,27,31,35,39 | 0 | 1 | 6 |
| 21 | A1 | 2 | 1 | 4,9,14,19,24,29,34,39 | 0 | 1 | 6 |
| 22 | A1 | 2 | 1 | 3,7,11,15,19,23,27,31,35,39 | 0 | 1 | 6 |
| 23 | A1 | 1 | 0 | 23,31,39 | 0 | 1 | 6 |
| 24 | A1 | 1 | 0 | 1,3,5,7,…,37,39 | 7 | 1 | 3 |
| 25 | A1 | 1 | 0 | 4,9,14,19,24,29,34,39 | 0 | 2 | 6 |
| 26 | A1 | 2 | 0 | 4,9,14,19,24,29,34,39 | 0 | 2 | 6 |
| 27 | A1 | 4 | 0 | 4,9,14,19,24,29,34,39 | 0 | 2 | 6 |
| 28 | A1 | 1 | 0 | 23,27,31,35,39 | 0 | 2 | 6 |
| 29 | A2 | 1 | 0 | 4,9,14,19,24,29,34,39 | 0 | 1 | 3 |
| 30 | A2 | 1 | 0 | 4,9,14,19,24,29,34,39 | 9 | 1 | 1 |
| 31 | A2 | 1 | 0 | 24,29,34,39 | 0 | 2 | 3 |
| 32 | A2 | 1 | 0 | 24,29,34,39 | 9 | 1 | 1 |
| 33 | A2 | 1 | 0 | 18,19,38,39 | 0 | 2 | 3 |
| 34 | A2 | 1 | 0 | 18,19,38,39 | 9 | 1 | 1 |
| 35 | A2 | 1 | 0 | 0,1,2,…,39 | 0 | 2 | 3 |
| 36 | A2 | 1 | 0 | 0,1,2,…,39 | 9 | 1 | 1 |
| 37 | A2 | 1 | 0 | 23,27,31,35,39 | 0 | 1 | 3 |
| 38 | A2 | 1 | 0 | 23,27,31,35,39 | 9 | 1 | 1 |
| 39 | A2 | 1 | 0 | 3,7,11,15,19,23,27,31,35,39 | 0 | 1 | 3 |
| 40 | A2 | 1 | 0 | 3,7,11,15,19,23,27,31,35,39 | 9 | 1 | 1 |
| 41 | A2 | 1 | 0 | 1,3,5,7,…,37,39 | 0 | 2 | 3 |
| 42 | A2 | 1 | 0 | 7,15,23,31,39 | 0 | 1 | 3 |
| 43 | A2 | 1 | 0 | 7,15,23,31,39 | 9 | 1 | 1 |
| 44 | A2 | 16 | 1 | 4,9,14,19,24,29,34,39 | 0 | 1 | 3 |
| 45 | A2 | 16 | 1 | 3,7,11,15,19,23,27,31,35,39 | 0 | 1 | 3 |
| 46 | A2 | 8 | 1 | 4,9,14,19,24,29,34,39 | 0 | 1 | 3 |
| 47 | A2 | 8 | 1 | 3,7,11,15,19,23,27,31,35,39 | 0 | 1 | 3 |
| 48 | A2 | 4 | 1 | 4,9,14,19,24,29,34,39 | 0 | 1 | 3 |
| 49 | A2 | 4 | 1 | 3,7,11,15,19,23,27,31,35,39 | 0 | 1 | 3 |
| 50 | A2 | 2 | 1 | 4,9,14,19,24,29,34,39 | 0 | 1 | 3 |
| 51 | A2 | 2 | 1 | 3,7,11,15,19,23,27,31,35,39 | 0 | 1 | 3 |
| 52 | A2 | 1 | 0 | 23,31,39 | 0 | 1 | 3 |
| 53 | A2 | 1 | 0 | 1,3,5,7,…,37,39 | 9 | 1 | 1 |
| 54 | A2 | 1 | 0 | 4,9,14,19,24,29,34,39 | 0 | 2 | 3 |
| 55 | A2 | 2 | 0 | 4,9,14,19,24,29,34,39 | 0 | 2 | 3 |
| 56 | A2 | 4 | 0 | 4,9,14,19,24,29,34,39 | 0 | 2 | 3 |
| 57 | A2 | 1 | 0 | 23,27,31,35,39 | 0 | 2 | 3 |
| 58 | A3 | 1 | 0 | 4,9,14,19,24,29,34,39 | 0 | 1 | 2 |
| 59 | A3 | 1 | 0 | 4,9,14,19,24,29,34,39 | 7 | 1 | 1 |
| 60 | A3 | 1 | 0 | 24,29,34,39 | 0 | 2 | 2 |
| 61 | A3 | 1 | 0 | 24,29,34,39 | 7 | 1 | 1 |
| 62 | A3 | 1 | 0 | 18,19,38,39 | 0 | 2 | 2 |
| 63 | A3 | 1 | 0 | 18,19,38,39 | 7 | 1 | 1 |
| 64 | A3 | 1 | 0 | 0,1,2,…,39 | 0 | 2 | 2 |
| 65 | A3 | 1 | 0 | 0,1,2,…,39 | 7 | 1 | 1 |
| 66 | A3 | 1 | 0 | 23,27,31,35,39 | 0 | 1 | 2 |
| 67 | A3 | 1 | 0 | 23,27,31,35,39 | 7 | 1 | 1 |
| 68 | A3 | 1 | 0 | 3,7,11,15,19,23,27,31,35,39 | 0 | 1 | 2 |
| 69 | A3 | 1 | 0 | 3,7,11,15,19,23,27,31,35,39 | 7 | 1 | 1 |
| 70 | A3 | 1 | 0 | 1,3,5,7,…,37,39 | 0 | 2 | 2 |
| 71 | A3 | 1 | 0 | 7,15,23,31,39 | 0 | 1 | 2 |
| 72 | A3 | 1 | 0 | 7,15,23,31,39 | 7 | 1 | 1 |
| 73 | A3 | 16 | 1 | 4,9,14,19,24,29,34,39 | 0 | 1 | 2 |
| 74 | A3 | 16 | 1 | 3,7,11,15,19,23,27,31,35,39 | 0 | 1 | 2 |
| 75 | A3 | 8 | 1 | 4,9,14,19,24,29,34,39 | 0 | 1 | 2 |
| 76 | A3 | 8 | 1 | 3,7,11,15,19,23,27,31,35,39 | 0 | 1 | 2 |
| 77 | A3 | 4 | 1 | 4,9,14,19,24,29,34,39 | 0 | 1 | 2 |
| 78 | A3 | 4 | 1 | 3,7,11,15,19,23,27,31,35,39 | 0 | 1 | 2 |
| 79 | A3 | 2 | 1 | 4,9,14,19,24,29,34,39 | 0 | 1 | 2 |
| 80 | A3 | 2 | 1 | 3,7,11,15,19,23,27,31,35,39 | 0 | 1 | 2 |
| 81 | A3 | 1 | 0 | 23,31,39 | 0 | 1 | 2 |
| 82 | A3 | 1 | 0 | 1,3,5,7,…,37,39 | 7 | 1 | 1 |
| 83 | A3 | 1 | 0 | 4,9,14,19,24,29,34,39 | 0 | 2 | 2 |
| 84 | A3 | 2 | 0 | 4,9,14,19,24,29,34,39 | 0 | 2 | 2 |
| 85 | A3 | 4 | 0 | 4,9,14,19,24,29,34,39 | 0 | 2 | 2 |
| 86 | A3 | 1 | 0 | 23,27,31,35,39 | 0 | 2 | 2 |
| 87 | B1 | 1 | 0 | 4,9,14,19,24,29,34,39 | 0 | 1 | 7 |
| 88 | B1 | 1 | 0 | 4,9,14,19,24,29,34,39 | 8 | 1 | 3 |
| 89 | B1 | 1 | 0 | 24,29,34,39 | 0 | 2 | 7 |
| 90 | B1 | 1 | 0 | 24,29,34,39 | 8 | 1 | 3 |
| 91 | B1 | 1 | 0 | 18,19,38,39 | 0 | 2 | 7 |
| 92 | B1 | 1 | 0 | 18,19,38,39 | 8 | 1 | 3 |
| 93 | B1 | 1 | 0 | 0,1,2,…,39 | 0 | 2 | 7 |
| 94 | B1 | 1 | 0 | 0,1,2,…,39 | 8 | 1 | 3 |
| 95 | B1 | 1 | 0 | 23,27,31,35,39 | 0 | 1 | 7 |
| 96 | B1 | 1 | 0 | 23,27,31,35,39 | 8 | 1 | 3 |
| 97 | B1 | 1 | 0 | 3,7,11,15,19,23,27,31,35,39 | 0 | 1 | 7 |
| 98 | B1 | 1 | 0 | 3,7,11,15,19,23,27,31,35,39 | 8 | 1 | 3 |
| 99 | B1 | 1 | 0 | 1,3,5,7,…,37,39 | 0 | 2 | 7 |
| 100 | B1 | 1 | 0 | 7,15,23,31,39 | 0 | 1 | 7 |
| 101 | B1 | 1 | 0 | 7,15,23,31,39 | 8 | 1 | 3 |
| 102 | B1 | 16 | 1 | 4,9,14,19,24,29,34,39 | 0 | 1 | 7 |
| 103 | B1 | 16 | 1 | 3,7,11,15,19,23,27,31,35,39 | 0 | 1 | 7 |
| 104 | B1 | 8 | 1 | 4,9,14,19,24,29,34,39 | 0 | 1 | 7 |
| 105 | B1 | 8 | 1 | 3,7,11,15,19,23,27,31,35,39 | 0 | 1 | 7 |
| 106 | B1 | 4 | 1 | 4,9,14,19,24,29,34,39 | 0 | 1 | 7 |
| 107 | B1 | 4 | 1 | 3,7,11,15,19,23,27,31,35,39 | 0 | 1 | 7 |
| 108 | B1 | 2 | 1 | 4,9,14,19,24,29,34,39 | 0 | 1 | 7 |
| 109 | B1 | 2 | 1 | 3,7,11,15,19,23,27,31,35,39 | 0 | 1 | 7 |
| 110 | B1 | 1 | 0 | 23,31,39 | 0 | 1 | 7 |
| 111 | B1 | 1 | 0 | 1,3,5,7,…,37,39 | 8 | 1 | 3 |
| 112 | B1 | 1 | 0 | 4,9,14,19,24,29,34,39 | 0 | 2 | 7 |
| 113 | B1 | 2 | 0 | 4,9,14,19,24,29,34,39 | 0 | 2 | 7 |
| 114 | B1 | 4 | 0 | 4,9,14,19,24,29,34,39 | 0 | 2 | 7 |
| 115 | B1 | 1 | 0 | 23,27,31,35,39 | 0 | 2 | 7 |
| 116 | B4 | 1 | 0 | 4,9,14,19,24,29,34,39 | 2 | 1 | 1 |
| 117 | B4 | 1 | 0 | 24,29,34,39 | 0 | 2 | 1 |
| 118 | B4 | 1 | 0 | 18,19,38,39 | 0 | 2 | 1 |
| 119 | B4 | 1 | 0 | 0,1,2,…,39 | 0 | 2 | 1 |
| 120 | B4 | 1 | 0 | 23,27,31,35,39 | 2 | 1 | 1 |
| 121 | B4 | 1 | 0 | 3,7,11,15,19,23,27,31,35,39 | 0 | 1 | 1 |
| 122 | B4 | 1 | 0 | 1,3,5,7,…,37,39 | 0 | 2 | 1 |
| 123 | B4 | 1 | 0 | 7,15,23,31,39 | 2 | 1 | 1 |
| 124 | B4 | 16 | 1 | 4,9,14,19,24,29,34,39 | 2 | 1 | 1 |
| 125 | B4 | 16 | 1 | 3,7,11,15,19,23,27,31,35,39 | 2 | 1 | 1 |
| 126 | B4 | 8 | 1 | 4,9,14,19,24,29,34,39 | 2 | 1 | 1 |
| 127 | B4 | 8 | 1 | 3,7,11,15,19,23,27,31,35,39 | 2 | 1 | 1 |
| 128 | B4 | 4 | 1 | 4,9,14,19,24,29,34,39 | 2 | 1 | 1 |
| 129 | B4 | 4 | 1 | 3,7,11,15,19,23,27,31,35,39 | 2 | 1 | 1 |
| 130 | B4 | 2 | 1 | 4,9,14,19,24,29,34,39 | 2 | 1 | 1 |
| 131 | B4 | 2 | 1 | 3,7,11,15,19,23,27,31,35,39 | 2 | 1 | 1 |
| 132 | B4 | 1 | 0 | 23,31,39 | 2 | 1 | 1 |
| 133 | B4 | 1 | 0 | 4,9,14,19,24,29,34,39 | 0 | 2 | 1 |
| 134 | B4 | 2 | 0 | 4,9,14,19,24,29,34,39 | 0 | 2 | 1 |
| 135 | B4 | 4 | 0 | 4,9,14,19,24,29,34,39 | 0 | 2 | 1 |
| 136 | B4 | 1 | 0 | 23,27,31,35,39 | 0 | 2 | 1 |
| 137 | A1/B1 | 1 | 0 | 4,9,14,19,24,29,34,39 | 2 | 1 | 6 |
| 138 | A1/B1 | 1 | 0 | 4,9,14,19,24,29,34,39 | 8 | 1 | 3 |
| 139 | A1/B1 | 1 | 0 | 18,19,38,39 | 2 | 2 | 6 |
| 140 | A1/B1 | 1 | 0 | 0,1,2,…,39 | 2 | 2 | 6 |
| 141 | A1/B1 | 1 | 0 | 0,1,2,…,39 | 8 | 1 | 3 |
| 142 | A1/B1 | 1 | 0 | 23,27,31,35,39 | 2 | 1 | 6 |
| 143 | A1/B1 | 1 | 0 | 23,27,31,35,39 | 8 | 1 | 3 |
| 144 | A1/B1 | 1 | 0 | 3,7,11,15,19,23,27,31,35,39 | 2 | 1 | 6 |
| 145 | A1/B1 | 1 | 0 | 3,7,11,15,19,23,27,31,35,39 | 8 | 1 | 3 |
| 146 | A1/B1 | 1 | 0 | 1,3,5,7,…,37,39 | 2 | 2 | 6 |
| 147 | A1/B1 | 1 | 0 | 7,15,23,31,39 | 2 | 1 | 6 |
| 148 | A1/B1 | 1 | 0 | 7,15,23,31,39 | 8 | 1 | 3 |
| 149 | A1/B1 | 16 | 1 | 4,9,14,19,24,29,34,39 | 2 | 1 | 6 |
| 150 | A1/B1 | 16 | 1 | 3,7,11,15,19,23,27,31,35,39 | 2 | 1 | 6 |
| 151 | A1/B1 | 8 | 1 | 4,9,14,19,24,29,34,39 | 2 | 1 | 6 |
| 152 | A1/B1 | 8 | 1 | 3,7,11,15,19,23,27,31,35,39 | 2 | 1 | 6 |
| 153 | A1/B1 | 4 | 1 | 4,9,14,19,24,29,34,39 | 2 | 1 | 6 |
| 154 | A1/B1 | 4 | 1 | 3,7,11,15,19,23,27,31,35,39 | 2 | 1 | 6 |
| 155 | A1/B1 | 2 | 1 | 4,9,14,19,24,29,34,39 | 2 | 1 | 6 |
| 156 | A1/B1 | 2 | 1 | 3,7,11,15,19,23,27,31,35,39 | 2 | 1 | 6 |
| 157 | A2/B2 | 1 | 0 | 4,9,14,19,24,29,34,39 | 2 | 1 | 3 |
| 158 | A2/B2 | 1 | 0 | 4,9,14,19,24,29,34,39 | 6 | 1 | 2 |
| 159 | A2/B2 | 1 | 0 | 18,19,38,39 | 2 | 2 | 3 |
| 160 | A2/B2 | 1 | 0 | 0,1,2,…,39 | 2 | 2 | 3 |
| 161 | A2/B2 | 1 | 0 | 0,1,2,…,39 | 6 | 1 | 2 |
| 162 | A2/B2 | 1 | 0 | 23,27,31,35,39 | 2 | 1 | 3 |
| 163 | A2/B2 | 1 | 0 | 23,27,31,35,39 | 6 | 1 | 2 |
| 164 | A2/B2 | 1 | 0 | 3,7,11,15,19,23,27,31,35,39 | 2 | 1 | 3 |
| 165 | A2/B2 | 1 | 0 | 3,7,11,15,19,23,27,31,35,39 | 6 | 1 | 2 |
| 166 | A2/B2 | 1 | 0 | 1,3,5,7,…,37,39 | 2 | 2 | 3 |
| 167 | A2/B2 | 1 | 0 | 7,15,23,31,39 | 2 | 1 | 3 |
| 168 | A2/B2 | 1 | 0 | 7,15,23,31,39 | 6 | 1 | 2 |
| 169 | A2/B2 | 16 | 1 | 4,9,14,19,24,29,34,39 | 2 | 1 | 3 |
| 170 | A2/B2 | 16 | 1 | 3,7,11,15,19,23,27,31,35,39 | 2 | 1 | 3 |
| 171 | A2/B2 | 8 | 1 | 4,9,14,19,24,29,34,39 | 2 | 1 | 3 |
| 172 | A2/B2 | 8 | 1 | 3,7,11,15,19,23,27,31,35,39 | 2 | 1 | 3 |
| 173 | A2/B2 | 4 | 1 | 4,9,14,19,24,29,34,39 | 2 | 1 | 3 |
| 174 | A2/B2 | 4 | 1 | 3,7,11,15,19,23,27,31,35,39 | 2 | 1 | 3 |
| 175 | A2/B2 | 2 | 1 | 4,9,14,19,24,29,34,39 | 2 | 1 | 3 |
| 176 | A2/B2 | 2 | 1 | 3,7,11,15,19,23,27,31,35,39 | 2 | 1 | 3 |
| 177 | A3/B3 | 1 | 0 | 4,9,14,19,24,29,34,39 | 2 | 1 | 2 |
| 178 | A3/B3 | 1 | 0 | 24,29,34,39 | 2 | 2 | 2 |
| 179 | A3/B3 | 1 | 0 | 18,19,38,39 | 2 | 2 | 2 |
| 180 | A3/B3 | 1 | 0 | 0,1,2,…,39 | 2 | 2 | 2 |
| 181 | A3/B3 | 1 | 0 | 23,27,31,35,39 | 2 | 1 | 2 |
| 182 | A3/B3 | 1 | 0 | 3,7,11,15,19,23,27,31,35,39 | 2 | 1 | 2 |
| 183 | A3/B3 | 1 | 0 | 1,3,5,7,…,37,39 | 2 | 2 | 2 |
| 184 | A3/B3 | 1 | 0 | 7,15,23,31,39 | 2 | 1 | 2 |
| 185 | A3/B3 | 16 | 1 | 4,9,14,19,24,29,34,39 | 2 | 1 | 2 |
| 186 | A3/B3 | 16 | 1 | 3,7,11,15,19,23,27,31,35,39 | 2 | 1 | 2 |
| 187 | A3/B3 | 8 | 1 | 4,9,14,19,24,29,34,39 | 2 | 1 | 2 |
| 188 | A3/B3 | 8 | 1 | 3,7,11,15,19,23,27,31,35,39 | 2 | 1 | 2 |
| 189 | A3/B3 | 4 | 1 | 4,9,14,19,24,29,34,39 | 2 | 1 | 2 |
| 190 | A3/B3 | 4 | 1 | 3,7,11,15,19,23,27,31,35,39 | 2 | 1 | 2 |
| 191 | A3/B3 | 2 | 1 | 4,9,14,19,24,29,34,39 | 2 | 1 | 2 |
| 192 | A3/B3 | 2 | 1 | 3,7,11,15,19,23,27,31,35,39 | 2 | 1 | 2 |
| 193 | A3/B3 | 1 | 0 | 23,31,39 | 2 | 1 | 2 |
| 194 | C0 | 1 | 0 | 4,9,14,19,24,29,34,39 | 0 | 1 | 7 |
| 195 | C0 | 1 | 0 | 4,9,14,19,24,29,34,39 | 8 | 1 | 3 |
| 196 | C0 | 1 | 0 | 24,29,34,39 | 0 | 2 | 7 |
| 197 | C0 | 1 | 0 | 24,29,34,39 | 8 | 1 | 3 |
| 198 | C0 | 1 | 0 | 18,19,38,39 | 0 | 2 | 7 |
| 199 | C0 | 1 | 0 | 18,19,38,39 | 8 | 1 | 3 |
| 200 | C0 | 1 | 0 | 0,1,2,…,39 | 0 | 2 | 7 |
| 201 | C0 | 1 | 0 | 0,1,2,…,39 | 8 | 1 | 3 |
| 202 | C0 | 1 | 0 | 23,27,31,35,39 | 0 | 1 | 7 |
| 203 | C0 | 1 | 0 | 23,27,31,35,39 | 8 | 1 | 3 |
| 204 | C0 | 1 | 0 | 3,7,11,15,19,23,27,31,35,39 | 0 | 1 | 7 |
| 205 | C0 | 1 | 0 | 3,7,11,15,19,23,27,31,35,39 | 8 | 1 | 3 |
| 206 | C0 | 1 | 0 | 1,3,5,7,…,37,39 | 0 | 2 | 7 |
| 207 | C0 | 1 | 0 | 7,15,23,31,39 | 0 | 1 | 7 |
| 208 | C0 | 1 | 0 | 7,15,23,31,39 | 8 | 1 | 3 |
| 209 | C0 | 16 | 1 | 4,9,14,19,24,29,34,39 | 0 | 1 | 7 |
| 210 | C0 | 16 | 1 | 3,7,11,15,19,23,27,31,35,39 | 0 | 1 | 7 |
| 211 | C0 | 8 | 1 | 4,9,14,19,24,29,34,39 | 0 | 1 | 7 |
| 212 | C0 | 8 | 1 | 3,7,11,15,19,23,27,31,35,39 | 0 | 1 | 7 |
| 213 | C0 | 4 | 1 | 4,9,14,19,24,29,34,39 | 0 | 1 | 7 |
| 214 | C0 | 4 | 1 | 3,7,11,15,19,23,27,31,35,39 | 0 | 1 | 7 |
| 215 | C0 | 2 | 1 | 4,9,14,19,24,29,34,39 | 0 | 1 | 7 |
| 216 | C0 | 2 | 1 | 3,7,11,15,19,23,27,31,35,39 | 0 | 1 | 7 |
| 217 | C0 | 1 | 0 | 23,31,39 | 0 | 1 | 7 |
| 218 | C0 | 1 | 0 | 1,3,5,7,…,37,39 | 8 | 1 | 3 |
| 219 | C0 | 1 | 0 | 4,9,14,19,24,29,34,39 | 0 | 2 | 7 |
| 220 | C0 | 2 | 0 | 4,9,14,19,24,29,34,39 | 0 | 2 | 7 |
| 221 | C0 | 4 | 0 | 4,9,14,19,24,29,34,39 | 0 | 2 | 7 |
| 222 | C0 | 1 | 0 | 23,27,31,35,39 | 0 | 2 | 7 |
| 223 | C2 | 1 | 0 | 4,9,14,19,24,29,34,39 | 2 | 1 | 2 |
| 224 | C2 | 1 | 0 | 4,9,14,19,24,29,34,39 | 8 | 1 | 1 |
| 225 | C2 | 1 | 0 | 24,29,34,39 | 2 | 2 | 2 |
| 226 | C2 | 1 | 0 | 24,29,34,39 | 8 | 1 | 1 |
| 227 | C2 | 1 | 0 | 18,19,38,39 | 2 | 2 | 2 |
| 228 | C2 | 1 | 0 | 18,19,38,39 | 8 | 1 | 1 |
| 229 | C2 | 1 | 0 | 0,1,2,…,39 | 2 | 2 | 2 |
| 230 | C2 | 1 | 0 | 0,1,2,…,39 | 8 | 1 | 1 |
| 231 | C2 | 1 | 0 | 23,27,31,35,39 | 2 | 1 | 2 |
| 232 | C2 | 1 | 0 | 23,27,31,35,39 | 8 | 1 | 1 |
| 233 | C2 | 1 | 0 | 3,7,11,15,19,23,27,31,35,39 | 2 | 1 | 2 |
| 234 | C2 | 1 | 0 | 3,7,11,15,19,23,27,31,35,39 | 8 | 1 | 1 |
| 235 | C2 | 1 | 0 | 1,3,5,7,…,37,39 | 2 | 2 | 2 |
| 236 | C2 | 1 | 0 | 7,15,23,31,39 | 2 | 1 | 2 |
| 237 | C2 | 1 | 0 | 7,15,23,31,39 | 8 | 1 | 1 |
| 238 | C2 | 16 | 1 | 4,9,14,19,24,29,34,39 | 2 | 1 | 2 |
| 239 | C2 | 16 | 1 | 3,7,11,15,19,23,27,31,35,39 | 2 | 1 | 2 |
| 240 | C2 | 8 | 1 | 4,9,14,19,24,29,34,39 | 2 | 1 | 2 |
| 241 | C2 | 8 | 1 | 3,7,11,15,19,23,27,31,35,39 | 2 | 1 | 2 |
| 242 | C2 | 4 | 1 | 4,9,14,19,24,29,34,39 | 2 | 1 | 2 |
| 243 | C2 | 4 | 1 | 3,7,11,15,19,23,27,31,35,39 | 2 | 1 | 2 |
| 244 | C2 | 2 | 1 | 4,9,14,19,24,29,34,39 | 2 | 1 | 2 |
| 245 | C2 | 2 | 1 | 3,7,11,15,19,23,27,31,35,39 | 2 | 1 | 2 |
| 246 | C2 | 1 | 0 | 23,31,39 | 2 | 1 | 2 |
| 247 | C2 | 1 | 0 | 1,3,5,7,…,37,39 | 8 | 1 | 1 |
| 248 | C2 | 1 | 0 | 4,9,14,19,24,29,34,39 | 2 | 2 | 2 |
| 249 | C2 | 2 | 0 | 4,9,14,19,24,29,34,39 | 2 | 2 | 2 |
| 250 | C2 | 4 | 0 | 4,9,14,19,24,29,34,39 | 2 | 2 | 2 |
| 251 | C2 | 1 | 0 | 23,27,31,35,39 | 2 | 2 | 2 |
| 252 |  |  |  |  |  |  |  |
| 253 |  |  |  |  |  |  |  |
| 254 |  |  |  |  |  |  |  |
| 255 |  |  |  |  |  |  |  |