3GPP TSG-RAN WG1 Meeting #92bis	Tdoc R1-1805596

Sanya, China, 16th – 20th April 2018	(R1-1805502)

Agenda Item:	7.1.1.2.1

Source:	Ericsson

Title:	Summary of 7.1.1.2.1 NR-PBCH

Document for:	Discussion, Decision

# Introduction

This is the summary for the 7.1.1.2.1 providing questions and proposals for agreement on the remaining details on PBCH based on the views expressed by companies in the contributions listed in the appendix.

# Raster shift value indication(M)

MediaTek, Samsung, Nokia, Huawei, HiSilicon, InterDigital and LGE pointed out that the RAN4 working agreement on cell search on raster means there is no need to indicate M in RMSI.

Proposal for agreement:

- From RAN1 perspective, the signalling of M in RMSI with parameter frequencyOffsetSSB		is not necessary in Rel-15

# Indication of cell-defining SS/PBCH block

## Signalling tables

The following companies want to confirm the working assumption on the signalling tables:

- Huawei, HiSilicon, Samsung, Qualcomm, InterDigital, Ericsson

OPPO wants to change the signalling to take advantage of that the second SS/PBCH block is at least one SS/PBCH block away.

Xioami suggest increasing the number of steps by using also value 14 from ssb-subcarrierOffset. InterDigital also made the same observation.

Proposal:

- Confirm working assumption on signalling tables

## Step size X

The following companies want to confirm the working assumption on step size X=1

- Huawei, HiSilicon, Samsung, Qualcomm, Ericsson,

ZTE wants to increase the step size to X=6. OPPO wants step size X=4. MediaTek wanted to change the value to X=3 for below 2.7GHz.

Proposal:

- Confirm working assumption on step size X=1

## Clarify that the second SS/PBCH block is the closest

Huawei suggested clarifying that the second SS/PBCH is the closest SS/PBCH block

Proposal:

- Adopt the following TP

---------------------------------------------- Start of Text Proposal 38.213 ------------------------------------------

13	UE procedure for monitoring Type0-PDCCH common search space

-------------------------------------------- Unchanged parts omitted -----------------------------------------

If a UE detects a first SS/PBCH block and determines that a control resource set for Type0-PDCCH common search space is not present, and for  for FR1 or for  for FR2, the UE determines the closest global synchronization channel number (GSCN) of a second SS/PBCH block having a control resource set for an associated Type0-PDCCH common search space as .  is the GSCN of the first SS/PBCH block and  is a GSCN offset provided by Table 13-16 for FR1 and Table 13-17 for FR2.

----------------------------------------------- End of Text Proposal 38.213------------------------------------------

## Search raster ambiguity

Vivo and LGE pointed out that with the working agreement in RAN4 on search raster, there is a potential ambiguity for bands that can use two search rasters, bands n38 and n41. They propose indicating which band that is being used is signalling in the MIB.

Before discussing further, RAN1 should await outcome of the RAN4 vote on the working agreement.

## Fraudulent use of mechanism

Intel commented that the agreement allows a fraudulent gNB to block for UE an entire frequency range from initial cell selection and questioned if the specification should such mechanisms They propose to allow UE to also search the ranges indicated if no valid cell is found outside the range. NTT DOCOMO raised the same issue.

Discuss further and consider if RAN2 should be consulted.

## Auxiliary information

Vivo suggests for SS/PBCH blocks without associated RMSI to consider that the UE should assume the indicated CDSSB and the reference non-CDSSB belong to the same cell.  Also, some of the bits in the PBCH intended for initial access can be used for delivering some auxiliary information of the closet cell defining SS/PBCH block.

Feature lead comment: These are more enhancements and not essential corrections.

# PBCH DMRS

Samsung proposed a text proposal to clarify that the half frame indicator is not carried by DMRS for when maximum number of SS/PBCH blocks is larger than 4.

Updated reason for the need for a clarification: The specification might give the impression that for Lmax=8 or Lmax=64, nhf=0 means that SSBs are only transmitted in the first half frame of a frame which is not the intention.

Proposal:

- Adopt the following TP

---------------------------------------------- Start of Text Proposal 38.211 ------------------------------------------

##### 7.4.1.4.1	Sequence generation

The UE shall assume the reference-signal sequence  for an SS/PBCH block is defined by

where  is given by clause 5.2. The scrambling sequence generator shall be initialized at the start of each SS/PBCH block occasion with

where

-	for, ,  is the number of the half-frame in which the PBCH is transmitted in a frame with  for the first half-frame in the frame and  for the second half-frame in the frame, and  is the two least significant bits of the SS/PBCH block index as defined in [5, TS 38.213]

-	for  or ,  and  is the three least significant bits of the SS/PBCH block index as defined in [5, TS 38.213]

with  being the maximum number of SS/PBCH beams in an SS/PBCH period for a particular band as given by [38.104].

----------------------------------------------- End of Text Proposal 38.211------------------------------------------

# Timing Alignment between Cells to Avoid Unnecessary Reading of PBCH

Intel stated that to avoid reading the PBCH, the UE can assume SFN and slot alignment among cells on the same carrier frequency in a TDD band for RRM measurement purposes and the parameter useServingCellTimingForSync in measurement object is set to TRUE for TDD bands. NTT DOCOMO discussed the same issue and presented three different options on how to handle the case.

Further discussion is needed.

# Appendix: Contributions used as basis for the summary

Additionally, some companies’ views were taken from contributions in AI 7.1.1.1(R1-1805501)..

| R1-1803607 | Remaining details of NR-PBCH Design | ZTE, Sanechips |
| --- | --- | --- |
| R1-1803626 | Remaining details of NR PBCH design | Huawei, HiSilicon |
| R1-1803807 | Remaining aspects on NR-PBCH | vivo |
| R1-1803997 | On synchronization raster indication | OPPO |
| R1-1804051 | Remaining issues on NR-PBCH | MediaTek Inc. |
| R1-1804089 | Discussion on power offset for PBCH and PDCCH | CMCC |
| R1-1804344 | Corrections on NR-PBCH | Samsung |
| R1-1804534 | Remaining Details on PBCH design and contents | LG Electronics |
| R1-1804648 | Remaining detail on the indication of cell-defining SS/PBCH block | Xiaomi Communications |
| R1-1804704 | Remaining details of NR-PBCH | Intel Corporation |
| R1-1804775 | Remaining details on NR-PBCH | Qualcomm Incorporated |
| R1-1804840 | Remaining Details for Physical Broadcast Channel | InterDigital, Inc. |
| R1-1805032 | Remaining issues on NR-PBCH | NTT DOCOMO, INC. |
| R1-1805135 | Remaining details related to SS/PBCH block | Nokia, Nokia Shanghai Bell |
| R1-1805216 | Remaining details of PBCH | Ericsson |