3GPP TSG-RAN WG1 #93	                                  		         R1-1805984

Busan, Korea, May 21-25th 2018

Source:	Mitsubishi Electric

Title:	Phase noise correction for non-terrestrial networks

Agenda Item:	7.3.2 Others (NTN)

Document for: 	Discussion/Decision

# Introduction

Non-terrestrial systems, either airborne or spaceborne, are expected to be an integral part of 5G deployments [1] and bring important benefits in areas such as ubiquitous coverage, high resilience, broadband delivery for high speed platforms, disaster/emergency relief etc. System design needs to take into account the potential impact of the NTN specificities, as already discussed in [5], [6], [7].

In this contribution we discuss phase noise tracking design principles, following the previous discussions in the last RAN1#92 meeting [3].This contribution is revised from R1-1804640 [8].

# Discussion

NTN systems may be deployed in wide range of frequency bands, ranging from 1-2GHz to mmWave (L/S/C/Ku/Ka/ Q/V bands). In [2], the following frequency bands were identified for evaluation purposes in a first approach:

-	For VSAT, Ka band: Downlink: 19.7 - 21.2 GHz, Uplink: 29.5 – 30.0 GHz

-	For UE, S band: Downlink: 2170 - 2200 MHz, Uplink: 1980 - 2010 MHz

Phase noise impact is important in mmWave. Phase noise is causing both a common phase error (CPE), resulting in constellation rotation, and inter-carrier interference (ICI), resulting in scattering of the constellation points.

In NR Rel.15 PTRS (phase tracking RS) were introduced to serve as basis for compensating the phase errors. Phase errors may come from phase noise, carrier frequency offset (CFO), Doppler shifts, etc.

## NR Rel.15 PTRS structure

NR Rel.15 PTRS structure [4] is very flexible.

For CP-OFDM, PTRS is inserted in the frequency domain, as it can be seen from an example depicted in Figure 1. Although the phase noise is a time domain process and the phase noise evolves between the beginning and the end of every OFDM symbol, given that PTRS is inserted in the frequency domain, only an average time domain estimation per OFDM symbol is available, allowing for CPE compensation only. CP-OFDM symbols carrying DMRS do not carry PTRS, since the common phase error is already absorbed in the channel estimation process and PTRS is not needed. The time domain density LPT-RS is either every OFDM symbol, or every other symbol, or every 4th symbol. The time domain density LPT-RS is dependent on the scheduled MCS as following:

Table 6.2.3-1 (TS 38.214): Time density of PT-RS as a function of scheduled MCS

| Scheduled MCS | Time density () |
| --- | --- |
| IMCS < ptrs-MCS1 | PT-RS is not present |
| ptrs-MCS1 IMCS < ptrs-MCS2 | 4 |
| ptrs-MCS2 IMCS < ptrs-MCS3 | 2 |
| ptrs-MCS3 IMCS < ptrs-MCS4 | 1 |

When the time domain density is lower than 1 (LPT-RS>1), the CP-OFDM symbol(s) after DMRS do not carry PTRS, since the CPE drift from the latest DMRS reference is low near DMRS symbols.

Figure 1 – PTRS mapping in the frequency domain (in red) for a given DMRS configuration (in blue) for CP-OFDM with LPT-RS=2, KPT-RS=2, krefRE=2, krefRB=1.

The frequency domain density KPT-RS is either every 2nd or every 4th RB, and the exact density depends on the scheduled bandwidth, as following:

Table 6.2.3-2 (TS 2=38.214): Frequency density of PT-RS as a function of scheduled bandwidth

| Scheduled bandwidth | Frequency density () |
| --- | --- |
| NRB < NRB0 | PT-RS is not present |
| NRB0 NRB < NRB1 | 2 |
| NRB1 NRB | 4 |

The thresholds giving the dependency on MCS (ptrs-MCSi) and scheduled bandwidth (NRBi) are user specific and configured through RRC. As depicted in Figure 1, PTRS (in red on the figure) appear on subcarriers k defined in TS 38.211 as:

where  is given by Table 6.4.1.2.2.1-1 (TS 38.211) for the DM-RS port associated with the PT-RS port according to clause 6.2.3 in [6, TS 38.214] and depends on the higher-layer parameter UL-PTRS-RE-offset.

For DFTsOFDM UL transmission, PTRS is inserted in the time domain, before DFT precoding. Since the phase noise is a time domain process, being able to track the phase noise in the time domain is more accurate than in the CP-OFDM case. Time-domain interpolation within the same DFTsOFDM symbol and/or across DFTsOFDM symbols is possible in this case. Only PUSCH DFTsOFDM symbols may carry PTRS, embedded within the PUSCH structure at the DFT precoder input.

The time domain density LPT-RS is either every DFTsOFDM symbol (LPT-RS=1), or every other symbol (LPT-RS=2). The time domain density is user specific and configured through RRC by the higher-layer parameter timeDensity. When the time domain density is every other symbol, the DFTsOFDM symbol right after DMRS does not contain PTRS.

Regarding the pre-DFT density for the DFTsOFDM symbols carrying PTRS, five different sequences are defined as depicted in the example in Figure 2. Each x sequence contains  groups of  PTRS samples. The five defined sequences are 2x2, 2x4, 4x2, 4x4 and 8x4 (see clause 6.4.1.2.2.2 of [4]), and the exact sequence to be used is chosen by comparing the allocated bandwidth to a set of user specific thresholds NRBi, configured through RRC as following:

Table 6.2.3-3 (TS 38.214): PT-RS group pattern as a function of scheduled bandwidth

| Scheduled bandwidth | Number of PT-RS groups | Number of samples per PT-RS group |
| --- | --- | --- |
| NRB0 NRB < NRB1 | 2 | 2 |
| NRB1 NRB < NRB2 | 2 | 4 |
| NRB2 NRB < NRB3 | 4 | 2 |
| NRB3 NRB < NRB4 | 4 | 4 |
| NRB4 NRB | 8 | 4 |

Any of the 5 predefined sequences can be enabled/disabled by configuring appropriate threshold values. The exact insertion position m in the pre-DFT space is given in TS 38.211 as:

Table 6.4.1.2.2.2-1 (TS 38.211): PT-RS symbol mapping.

| Number of PT-RS groups | Number of samples per PT-RS group | Index of PT-RS samples in OFDM symbol prior to transform precoding |
| --- | --- | --- |
| 2 | 2 | where |
| 2 | 4 | where |
| 4 | 2 | where |
| 4 | 4 | where |
| 8 | 4 | where |

Figure 2 – PTRS mapping in the pre-DFT space for DFTsOFDM: 2x2, 2x4, 4x2, 4x4, 8x4 sequences

For both CP-OFDM and DFTsOFDM, the implicit dependency on scheduled bandwidth (and/or MCS) were derived in NR after extensive simulation campaigns. The flexibility given by the possibility of user-specifically configuring the switching thresholds between patterns allows the system to adapt to the RF characteristics of each UE.

During the extensive simulation campaigns, it was noted that the performance of phase tracking algorithms is highly dependent on a large set of variables such as carrier frequency, subcarrier spacing, UE specific RF characteristics, configured MCS/bandwidth, frame length, DMRS configuration, Doppler, receiver implementation options, etc. The same pattern can have different behavior with different phase noise masks.

Observation: NR Rel.15 PTRS structure is very flexible and allows user-specific configuration.

Observation: The performance of phase tracking algorithms is highly dependent on a large set of variables such as carrier frequency, subcarrier spacing, UE specific RF characteristics, configured MCS/bandwidth, frame length, DMRS configuration, Doppler, receiver implementation options, etc.

## Initial evaluations in NTN context

As it can be seen in Figure 3, phase noise masks defined for NR and phase noise masks usually employed in Ka bands [8] may be significantly different. Also, in NTN, since the link budget may be significantly different from the terrestrial case, the typical MCS/bandwidths/numerology choices may be different from the NR eMBB simulation scenarios. Figure 3 depicts different PN masks around 30GHz for NR [9] and in a satellite context [10]. It can be easily seen that the phase masks usually employed in NTN contexts so far [10] may be rather different from the ones defined for NR. The impact of a realization of each phase noise mask onto a 16QAM constellation is depicted in Figure 4. DTH P2 mask is especially severe, displaying strong ICI levels.

Figure 3 - Different PN masks for NR (around 30GHz) and NTN

|  |  | ∙ TR 38.803 Ex2 mod1 (NR) ∙ DVB: DTH P1 ∙ DVB: DTH P2 ∙ DVB: VSAT P1 ∙ DVB: VSAT P2 |
| --- | --- | --- |

Figure 4 – Constellation impact of different phase noise masks

Figure 5 – Performance of 16QAM ¾ 50RB in the presence of different phase noise with or without compensation

Observation: The current phase noise masks defined in NR may be rather different from phase masks usually considered in NTN.

Observation: Typical NTN scenarios may be different from the typical NR simulation assumptions.

Figure 5 displays simulation results comparing the performance of a DFTsOFDM transmission of 16QAM ¾ over 50 allocated RBs under different phase noise masks, with and without phase noise compensation. Simulation parameters are given in the Annex. Several patterns were tested, and only the best performing one (spectral efficiency-wise) was plotted. With the current simulation assumptions, the 4x2 pattern was the best performing one for all phase noise masks. While the current patterns, designed under NR assumptions, are sufficient to compensate phase noise of types DTH P1 or VSAT, they are proven insufficient to deal with severe masks of type DTH P2. Denser patterns (e.g. 8x4) did not manage to compensate the severity of this phase noise mask at SCS=120kHz either. If DTH P2 is considered relevant in the context of NTN communications, higher SCS and/or PTRS redesign need to be considered.

As a consequence, although the NR PTRS patterns allow a very flexible configuration, in order to investigate whether the current patterns are enough or some redesign is necessary a set of NTN specific simulation assumptions need to be agreed, including at least assumptions on carrier frequency, subcarrier spacing, UE specific RF characteristics, configured MCS/bandwidth, frame length, DMRS configuration, Doppler, etc.

Observation: A set of NTN specific simulation assumptions need to be agreed in order to evaluate the performance of current NR PTRS patterns in an NTN environment.

# Conclusion

Observation: NR Rel.15 PTRS structure is very flexible and allows user-specific configuration.

Observation: The performance of phase tracking algorithms is highly dependent on a large set of variables such as carrier frequency, subcarrier spacing, UE specific RF characteristics, configured MCS/bandwidth, frame length, DMRS configuration, Doppler, receiver implementation options, etc.

Observation: The current phase noise masks defined in NR may be rather different from phase masks usually considered in NTN.

Observation: Typical NTN scenarios may be different from the typical NR simulation assumptions.

Observation: A set of NTN specific simulation assumptions need to be agreed in order to evaluate the performance of current NR PTRS patterns in an NTN environment.

# Annex: Simulation scenario

| Parameters | Value |
| --- | --- |
| Waveform | DFT-s-OFDM |
| Carrier Frequency | 30GHz |
| SCS | 120kHz |
| MCS | 16QAM ¾ |
| #Allocated PRBs | 50 |
| PTRS | In all DFT-s-OFDM symbols (except if DMRS) 4x2, 4x4, 8x4 patterns tested, only the one with the best spectral efficiency displayed |
| Channel model | CDL-A 50ns |
| Channel estimation | Realistic with one front-loaded DMRS (comb-type) |
| HPA | Polynomial, IBO = -8dB |
| CFO | Off |

# References

### RP-170717, “Study on NR to support Non-Terrestrial Networks”.

### TR 38.811 Study on New Radio (NR) to support non terrestrial networks.

### Chairman’s notes, RAN1#92, Athens, Greece, February 2018.

### TS 38.211 NR; Physical channels and modulation v15.1.0.

### R1-1802004, PTRS design consideration for NTN, Samsung, Athens, Greece, February 2018.

### R1- 1802797, NTN NR impacts due to Phase Noise, Fraunhofer IIS, Fraunhofer HHI, Athens, Greece, February 2018.

### RP-180177, NR-NTN: NR impact area identification, Phase Noise, and PTRS Design, Fraunhofer IIS, Chennai, India, 19-23rd March 2018.

### R1-1804640, Phase noise correction for non-terrestrial networks, Mitsubishi Electric, Sanya, China, April 2018.

### TR 38.803 Study on new radio access technology: Radio Frequency (RF) and co-existence aspects v14.2.0.

### DVB Document A171-2, Digital Video Broadcasting (DVB) Implementation guidelines for the second generation system for Broadcasting, Interactive Services, News Gathering and other broadband satellite applications; Part 2 - S2 Extensions (DVB-S2X), March 2015