3GPP TSG-RAN WG1 Meeting #93	 R1-1806011

Busan, Korea, May 21st – 25th, 2018 	Revision of R1-1803928

Agenda Item:	7.2.2

Source:	Ericsson

Title:	On Transmit Diversity for Ultra-high Reliability Use Cases

Document for:	Information

# Introduction

According to [1], RAN1 should identify techniques for supporting the ultra-reliable part of URLLC requirements set forth in [2] starting in RAN1 NR Ad-hoc#2 meeting in June 2017.

In this contribution, we discuss some transmit diversity schemes and provide evaluation results focusing on high reliability use cases/URLLC, e.g., cyclic delay diversity (CDD) for PDSCH and SFBC/CDD for short PUCCH.

# Discussion

URLLC reliability requirement is much stricter than eMBB, i.e., a small packet of size 32 byte should be transmitted within one-way latency of 1 ms with success probability of at least 1-10^(-5). Diversity is the key element to achieve such strict reliability requirements.

Traditional approaches to achieve diversity in time and frequency are to retransmit data multiple times and spread the data over a large bandwidth. However, these approaches are no longer feasible nor sufficient due to strict latency constraint and limited frequency resources. Other forms of diversity are therefore required. Transmit diversity is an approach to provide extra spatial diversity. It operates without the need of channel state information at the transmitter which can be relevant for URLLC when the reliability of CSI itself may be an issue.

As mention in [3], there are several transmit diversity schemes discussed in LTE and NR. In this contribution, we provide some evaluation results of transmit diversity for high reliability use cases/URLLC, e.g., CDD for PDSCH and SFBC/CDD for short PUCCH.

## Cyclic delay diversity (for PDSCH)

Cyclic delay diversity (CDD) is a simple transmit diversity scheme that involves creating extra copies of the transmit signal with some delay. For two transmit antennas, the main idea is to perform cyclic-shift of the original time-domain transmit signal or equivalently phase-shift of the original frequency-domain transmit signal to create another copy of the signal. The cyclic-shifted copy is then transmitted over another transmit antenna. This creates additional degree of frequency selectivity of the effective channel experienced by the two copies and seen by the receiver and thus provides additional frequency diversity. CDD can be straightforwardly extended to multiple transmit antennas using different cyclic shifts for each antenna. The scheme is of low complexity and is also transparent to the receiver. Depending on the cyclic delay value, there can be a tradeoff between frequency diversity gain and the channel estimation performance. In LTE, large cyclic delay diversity is specified for transmission mode 3.

Below we provide simulation results of CDD scheme for 2 TX antennas. Simulation assumptions relevant for URLLC are given below.

- LDPC code BG#2, layered sum-product decoder with 15 iterations

- QPSK

- R=1/5 and 1/10

- 4 and 7os mini-slot duration with 1 DMRS symbol (no FDM with data)

- SCS 30kHz

- Carrier frequency 700 MHz

- #PRBs corresponding to the payload (TBS) 280b is given in Table 1.

- Practical channel estimation

- 2 Rx antennas with MRC combining

Table 1: Number of PRBs corresponding to TBS of 280 bits

| Code rate/ mini-slot duration | 4os | 7os |
| --- | --- | --- |
| 1/5 | 18 | 9 |
| 1/10 | 36 | 18 |

We compare CDD scheme with the scheme that uses a simple 1/sqrt(2)*[1;1] precoder which duplicates symbols over the second transmit antenna with power scaling (denoted by “no CDD”) for different channel models, e.g., TDL-C 10ns, 30ns, 300ns with UE speed 3 kmph. Due to uncorrelated antenna assumption, the “no CDD” scheme is also equivalent to the scheme with 1 TX antenna.

Figure 1: Performance comparison between CDD and simple precoder schemes for TDLC-10ns.

Figure 2: Performance comparison between CDD and simple precoder schemes for TDLC-30ns.

Figure 3: Performance comparison between CDD and simple precoder schemes for TDLC-300ns.

From results in Figures 1-3, we see that CDD provides performance gain especially at low BLER. The gain is significant when the channel is relatively flat, e.g., at delay spread values of 10 or 30 ns. This is expected since for the frequency flat channel, CDD can create extra frequency selectivity and thus provide additional frequency diversity. The gain is less apparent for channel with high delay spread due to the abundance of frequency selectivity of the channel itself.

Transmit diversity such as CDD can provide performance gain at low BLER especially when used for relatively flat channel.

## SFBC or CDD (for PUCCH)

For short payload, up to 2bits, SORTD has been agreed for tx diversity. For UCI of more than 2 bits, natural candidates are SFBC, i.e., Alamouti in frequency, and CDD. SFBC is limited in two transmit antennas, given that for more antennas, the optimal designs have an effective rate that is less than 1 QAM symbol/RE. On the other hand, the intention with CSD, as mentioned previously, is to induce independent fading and a nice property is that the design is not restricted by the number of transmit antennas. Below we state the simulation assumptions for PUCCH.

| Carrier frequency | 4 GHz |
| --- | --- |
| Bandwidth | 20 MHz |
| PHY resources | 4 PRB |
| Channel model | TDL-C with delay spread |
| delay spread | 300ns |
| UE speed: | 3 km/h |
| antennas | 1 Tx and 2/4/8 Rx |
| SCS | 15 kHz |
| Channel estimation | practical |
| Noise estimation | ideal |

In following two figures, we see the performance comparison, of two candidate diversity techniques, namely SFBC and CDD. It can be seen that at low SNR the performance is the same for both diversity schemes. However, at higher SNR values, SFBC outperforms CDD in the whole SNR range. Based on these results we decide, in the following, to focus only on SFBC and compare it with single tx-antenna transmission.

SFBC outperforms CDD for medium payload transmission in short PUCCH.

Figure 1: Performance comparison of diversity techniques in 2-symbol PUCCH with 2 RX antennas, 4 PRBs and without frequency hopping

Figure 2: Performance comparison of diversity techniques in 2-symbol PUCCH with 2 RX antennas, 4 PRBs and with frequency hopping

### Repetition without frequency hopping

The baseline setup for comparison comprises the two-symbol PUCCH. In Figure 3, where we show results without frequency hopping, we observe that there is an SNR cross-over point beyond which SFBC is better and outperforms single-antenna setups at low BLER. This is because typically the diversity effects kick-in at the high SNR regime, where also the channel estimation is sufficiently good to avoid inter-stream interference which impairs the SFBC orthogonality and hence the diversity. The same observations hold for the case of 4RX antennas, illustrated in Figure 4, yet here the SNR cross-over points shift to higher SNR values and diversity is beneficial at a smaller SNR range. However, the SNR values where 10-5 BLER can achieved are too high to be applicable to URLLC. Based on these results, we can make the following observation:

SFBC can outperform no diversity for short PUCCH without frequency hopping, yet the performance is not sufficient for the purposes of URLLC.

Figure 3: Performance evaluation of tx diversity in 2-symbol PUCCH with 2 RX antennas, 4 PRBs and without frequency hopping

Figure 4: Performance evaluation of tx diversity in 2-symbol PUCCH with 4 RX antennas, 4 PRBs and without frequency hopping.

### Repetition with frequency hopping

For the 2-symbol PUCCH, frequency hopping is supported that can provide extra diversity in the system. In Figure 5, we have a similar behavior as in Figure 4, namely that SFBC outperforms single-antenna transmission for a small fraction of the SNR regime of interest. It is interesting to see Figure 6, where SFBC is outperformed by single-antenna transmitters; using frequency hopping when gNB is equipped with 4RX antennas is a better strategy in the whole SNR range. We can infer that for the latter setup, the system has enough inherent diversity and does not benefit from SFBC. In practice, though, 4RX antennas might not be perfectly uncorrelated thereby diminishing the diversity gains that we get from multiple receive antennas.  Yet, the performance gap is sufficiently large to support the conclusion that multiple receive antennas and frequency hopping provide enough diversity for URLLC systems. Based on these results, we have the following observation:

SFBC does not bring any gains when frequency hopping is employed. URLLC systems in NR, where multiple RX antennas and frequency hopping are supported, are not diversity deficient.

Figure 5: Performance evaluation of tx diversity in 2-symbol PUCCH with 2 RX antennas, 4 PRBs and with frequency hopping

Figure 6: Performance evaluation of tx diversity in 2-symbol PUCCH with 4 RX antennas, 4 PRBs and with frequency hopping

# Conclusion

In section 2 we made the following observations:

Observation 1	Transmit diversity such as CDD can provide performance gain at low BLER especially when used for relatively flat channel

Observation 2	SFBC outperforms CDD for medium payload transmission in sPUCCH.

Observation 3	SFBC can outperform no diversity for short PUCCH without frequency hopping, yet the performance is not sufficient for the purposes of URLLC.

Observation 4	SFBC does not bring any gains when frequency hopping is employed. URLLC systems in NR, where multiple RX antennas and frequency hopping are supported, are not diversity deficient.

# References

- RP-171485 – “Revised WID on New Radio Access Technology”, NTT DOCOMO, INC, RAN75, West Palm Beach, USA, June 2017.

- 3GPP TR 38.913 – “Study on Scenarios and Requirements for Next Generation Access Technologies”, v14.2.0

R1-1718640, “On Transmit Diversity Schemes for PUCCH”, Oct 2017, Ericsson