3GPP TSG RAN WG1 Meeting #93	                       	R1-1806131

Busan, Korea, May 21st – 25th, 2018

Source:				ZTE

Title:						Remaining issues on short PUCCH

Agenda item:		7.1.3.2.1

Document for:	Discussion and Decision

# Introduction

In previous meeting, agreements related to simultaneous transmission of NR short PUCCH and other UCI were reached [1][2]. Some remaining points are to be concluded. In this contribution, we present our further views on several remaining issues for short PUCCH.

# SR indication

We had agreements of multiple SRs with PUCCH Format 2/3/4.

Agreements: - If a UCI transmission on a PUCCH form a UE using format 2 or 3 or 4 overlaps in time with K PUCCH resources, each configured for a SR, X bits are used to represent a SR being indicated by the UE and appended to the end of the end of HARQ-ACK followed by CSI. - Note: X is used to indicate both the presence or absence of SR and which of the K configured SR is embedded. - FFS: X=ceil(log2(K+1)) - FFS PUCCH formats 0 & 1

For PUCCH formats 0/1, the payload limit of 2 bits will results some information drops. The currents proposals are try to convey as much information as possible.

## SR multiplexed with A/N on PUCCH format 0

In case a UCI transmission of format 0 (that is a UCI with 1-2 A/N bits), overlaps with multiple configured SR PUCCH resources in time. However, RAN1 92bis did not have progress capturing it. Some proposals were given in RAN1 92 in below.

- Alt 1 (1a/1b/1c):

For 1-2 bits A/N with PUCCH format 0, A/N/SR are transmitted using the PUCCH format 0.Alt 1a need 1 additional CS based on A/N PUCCH resource. Alt 1b/1c needs 2 additional CS

- Alt 2:

Depending on the number of overlapping configured SR, the PUCCH format is changed. For more than one overlapping configured SR, a PUCCH format more than 2 bits is used.

- Alt 3:

The multiplexing is supported only for one overlapping configured SR. For multiple overlapping configured SRs, SRs are dropped and only A/N is transmitted.

Analysis for Alt1b/1c

For Alt 1b/1c, a simple method is applied. For 2 HARQ-ACK bits, 2 configured SR resources can be indicated while in case of 1 HARQ-ACK bit, this number is increased to 5 as described in [4]. However, a UE can be configured by higher layer with maximum 8 scheduling request configurations (maxNrofSR-ConfigPerCellGroup). It means always dropping the other lower prioritized SRs than the first 2 SRs. This alternative does not make much different than the current already supported 1SR plus 1HARQ in the specs. We consider indicating SRs as much as possible in our proposal.

One possible solution is provided as follows.

The HARQ-ACK format 0 and multiple SRs with format 1 could be looked as error case. This is due to format 0 only occupy up to 2 symbols. It is not reasonable to overlap with multiple SRs. It is suggested only to consider The HARQ-ACK format 0 and multiple SRs with format 0.

K (K>1) configured SRs are divided into several numbered groups. Each group contains maximum 5 configured SRs for 1 bit HARQ-ACK and maximum 2 configured SRs for 2 bits HARQ-ACK. One group can be conveyed by a PUCCH resource or an SR resource. In each group, different CSs derived from the initial CS of HARQ-ACK PUCCH are used to distinguish different positive SRs. If all SRs are negative, the HARQ-ACK PUCCH resource and the initial CS are used.

Figure 1. A modified example of Alt1b and Alt1c

The scheme is illustrated by Fig 1 with a 2-bit HARQ-ACK PUCCH and 4 configured SRs. The 4 configured SRs are divided into 2 groups. The first group contains SR ID0 and SR ID1. It uses the time-frequency resources of HARQ-ACK PUCCH and cyclic shift to represent which SR. The second group contains SR ID2 and SR ID3. It uses the time-frequency resources of SR ID0 PUCCH and cyclic shift to represent which SR. By this solution, all the SRs could be possibly indicated based on priority.

Proposal 1: If A/N transmission on a PUCCH for a UE using format 0 fully overlaps in time with K PUCCH resources:

  - K (K>1) configured SRs are divided into several numbered groups.

  - Each group contains up to 5 configured SR for 1 bit HARQ-ACK. If the number of SRs is less than 5, then they are grouped together in one group.

  - Each group contains up to 2 configured SR for 2 bits HARQ-ACK.

  - If all SRs are negative, the initial CS of the HARQ-ACK PUCCH is transmitted in the time-frequency resources of the HARQ-ACK PUCCH.

  - Each group of SRs uses Pre-defined time-frequency resources from the A/N PUCCH or SR resources to transmit HARQ-ACK and positive SR.

  - In each group, different CSs derived from the initial CS of HARQ-ACK PUCCH are used to distinguish different positive SRs.

Alt2 is another choice, but it needs an ARI dynamically indicating the format and resource of PUCCH. In case of semi-persistent scheduling, it doesn’t work because of ARI absence.

## SR multiplexed with A/N on PUCCH format 1

In case a UCI transmission of format 1 (that is a UCI with 1-2 A/N bits), overlaps with multiple configured SR PUCCH resources in time, some methods are also summarized in RAN1 #92bis:

Proposals:

- Select one of the following two alternatives:

  - Alt 1: If a HARQ-ACK transmission from a UE using PUCCH format 1 determined by ARI overlaps with K SR occasions each using PUCCH format 1, in case of negative SR, the HARQ-ACK is transmitted on the HARQ-ACK resource. In case of positive SR, the HARQ-ACK is transmitted on the SR resource corresponding to the positive SR.

  - Alt 2: PUCCH format 2 or 3 or 4 is used to carry L = 1 or 2 bits HARQ-ACK and K > 1 SR.  That implies when a UE has K SRs configured in a same time position,

    - For 1 or 2 bits HARQ-ACK, if K=0 or 1, ARI field in DCI indicates PUCCH format 0 or 1; otherwise, ARI field in DCI indicates PUCCH format 2 or 3 or 4.

It concluded to discuss further based on this.

The alternative 1 needs to detect possible combinations over multiple symbols during the overlapping region. Then for gNB, it may have to detect all the SR resources until the last one to find out which SR is turning on. Last SR may not be the least prioritized one and need to be detected. The problem is more common for Format 1 than Format 0, since Format 1 take much longer duration. Using alternative 2 will indicate resource directly and simplify the processing.

For Alternative 2 we should consider additionally for the case without ARI.

One possible improvement for is provided on top of Alternative 2.

When a UE has K (K>1) SRs configured in a same time position and HARQ-ACK is transmitted through PUCCH format 1, the UE can determine the PUCCH resource using the following rules:

  - If the ARI is absence:

      - The UE can determine the corresponding PUCCH set according to the total number of HARQ-ACK and X bits, wherein, X=ceil(log2(K+1)), K is the number of overlapping SRs.

    - If this PUCCH set is present, then first resource of PUCCH format 3 or 4 that has the same number of symbols as the PUCCH format 0 in this PUCCH set is used.

- If the required PUCCH Format 3 or 4 resources are obtained, HARQ-ACK and X bits are transmitted through PUCCH Format 3 or 4 resources.

    - Else, HARQ-ACK is only transmitted through the resource of PUCCH format 1 and all SRs are dropped.

Proposal 2: If A/N transmission on a PUCCH for a UE using format 1 fully overlaps in time with K(K>1) PUCCH resources, the UE can determine the PUCCH resource using the following rules:

  - If the ARI is available:

- PUCCH format 2 or 3 or 4 is used to carry L = 1 or 2 bits HARQ-ACK and K > 1 SR.  That implies when a UE has K SRs configured in a same time position,

  - For 1 or 2 bits HARQ-ACK, if K=0 or 1, ARI field in DCI indicates PUCCH format 0 or 1; otherwise, ARI field in DCI indicates PUCCH format 2 or 3 or 4.

  - If the ARI is absence:

      - The UE can determine the corresponding PUCCH set according to the total number of HARQ-ACK and X bits, wherein, X=ceil(log2(K+1)), K is the number of overlapping SRs.

    - If this PUCCH set is present, then first resource of PUCCH format 3 or 4 that has the same number of symbols as the PUCCH format 1 in this PUCCH set is used.

- If the required PUCCH Format 3 or 4 resources are obtained, HARQ-ACK and X bits are transmitted through PUCCH Format 3 or 4 resources.

    - Else, HARQ-ACK is only transmitted through the resource of PUCCH format 1 and all SRs are dropped.

# A/N/SR and CSI PUCCH multiplexing

In the RAN1 92bis meetings most of remaining issues for A/N/SR and CSI PUCCH are solved by the 2 agreements.

Agreements: - When AN/SR and CSI PUCCH resources have the same starting symbols in a slot, for a wideband semi-persistent/periodic CSI reporting based on a PUCCH format 2, if the PUCCH resource based on ARI for transmitting HARQ-ACK/SR and CSI is based on format 3: - CSI (only CSI part 1) is jointly encoded with HARQ-ACK/SR, following the existing procedure of encoding and mapping on resources for PUCCH format 3 or 4 to transmit the UCI. Agreements: - When a configured CSI PUCCH resource overlaps with one or more configured SR PUCCH resources in a slot, - The UE multiplexes all UCIs on the CSI PUCCH resource using the corresponding UCI multiplexing rules for the overlapping PUCCH resources with the same starting symbol. - FFS: When one SR PUCCH resource overlaps with two non-overlapping CSI PUCCH resources

One open issue is how to handle the transmission if UE is only configured with one PUCCH resource set. This configuration allows UE to be configured with less PUCCH resource in case no much payload variation of PUCCH feedback is expected. The discussion leads to proposals.

- When HARQ-ACK/SR PUCCH resource determined by ARI and configured CSI PUCCH resources have the same starting symbols in a slot, if the UE is configured with only one PUCCH resource set and simultaneous transmission of HARQ-ACK/SR and CSI is enabled, select from one of the following alternatives:

  - Alt 1: The UE is not expected to transmit the corresponding HARQ-ACK/SR and the CSI report in a PUCCH resource in that slot.

  - Alt 2: HARQ-ACK/SR and CSI are transmitted together by PUCCH resource for CSI

  - Alt 3: HARQ-ACK/SR is transmitted on the HARQ-ACK/SR PUCCH resource and CSI reporting is dropped in that slot.

RAN1 have to defer the discussion further.

As mentioned above, UE may be configured with one set. In the mean time, the CSI PUCCH could be configured periodically. Collision is still possible to happen. Looking this as a miss configuration would results in higher unused configured resources. In addition, if wider range of CSI PUCCH payload is assumed, there would need even large number of PUCCH resource set to be reserved.

Proposal 3:

- When HARQ-ACK/SR PUCCH resource determined by ARI and configured CSI PUCCH resources have the same starting symbols in a slot, if the UE is configured with only one PUCCH resource set and simultaneous transmission of HARQ-ACK/SR and CSI is enabled, select from one of the following alternatives:

  - HARQ-ACK/SR and CSI are transmitted together by PUCCH resource for CSI.

Typically, the semi-static configuration of PUCCH results in case that ARI is not applicable. If it cannot be solved in above proposals, we still need to clarify it in the UCI multiplexing agenda item.

# Conclusion

In this contribution, several remaining issues related to short PUCCH are further analyzed to complete the functionality. In summary, we have the following proposals for capturing.

Proposal 1: If A/N transmission on a PUCCH for a UE using format 0 fully overlaps in time with K PUCCH resources:

  - K (K>1) configured SRs are divided into several numbered groups.

  - Each group contains up to 5 configured SR for 1 bit HARQ-ACK. If the number of SRs is less than 5, then they are grouped together in one group.

  - Each group contains up to 2 configured SR for 2 bits HARQ-ACK.

  - If all SRs are negative, the initial CS of the HARQ-ACK PUCCH is transmitted in the time-frequency resources of the HARQ-ACK PUCCH.

  - Each group of SRs uses Pre-defined time-frequency resources from the A/N PUCCH or SR resources to transmit HARQ-ACK and positive SR.

  - In each group, different CSs derived from the initial CS of HARQ-ACK PUCCH are used to distinguish different positive SRs.

Proposal 2: If A/N transmission on a PUCCH for a UE using format 1 fully overlaps in time with K(K>1) PUCCH resources, the UE can determine the PUCCH resource using the following rules:

  - If the ARI is available:

- PUCCH format 2 or 3 or 4 is used to carry L = 1 or 2 bits HARQ-ACK and K > 1 SR.  That implies when a UE has K SRs configured in a same time position,

  - For 1 or 2 bits HARQ-ACK, if K=0 or 1, ARI field in DCI indicates PUCCH format 0 or 1; otherwise, ARI field in DCI indicates PUCCH format 2 or 3 or 4.

  - If the ARI is absence:

      - The UE can determine the corresponding PUCCH set according to the total number of HARQ-ACK and X bits, wherein, X=ceil(log2(K+1)), K is the number of overlapping SRs.

    - If this PUCCH set is present, then first resource of PUCCH format 3 or 4 that has the same number of symbols as the PUCCH format 1 in this PUCCH set is used.

- If the required PUCCH Format 3 or 4 resources are obtained, HARQ-ACK and X bits are transmitted through PUCCH Format 3 or 4 resources.

    - Else, HARQ-ACK is only transmitted through the resource of PUCCH format 1 and all SRs are dropped.

Proposal 3:

- When HARQ-ACK/SR PUCCH resource determined by ARI and configured CSI PUCCH resources have the same starting symbols in a slot, if the UE is configured with only one PUCCH resource set and simultaneous transmission of HARQ-ACK/SR and CSI is enabled, select from one of the following alternatives:

  - HARQ-ACK/SR and CSI are transmitted together by PUCCH resource for CSI.

# References

- 3GPP TSG RAN1 AH_1801, Chairman’s Note.

- 3GPP TSG RAN1 #92, Chairman’s Note.

- 3GPP TSG RAN Meeting #92,R1-1803258,Summary of Contributions on PUCCH structure in short-duration, Ericsson.

- 3GPP RAN1 AH_1801, R1-1801262 Summary of Discussions on Multiplexing Different UCI types on a PUCCH resource, Ericsson.

- 3GPP TS 38.213 V15.0.1 (2018-03) Physical layer procedures for control (Release 15).