3GPP TSG-RAN WG1 Meeting #93	Tdoc R1-1806245

Busan, South Korea, May 21-25 2018

Agenda Item:	7.4.5

Source:	Ericsson

Title:	Observations on NOMA traffic models

Document for:	Discussion

# 1	Introduction

Traffic models for NOMA traffic have not been studied in much detail in RAN1. We therefore perform some simple analysis of the ITU density requirements for 5G mMTC together with traffic models from earlier cellular IoT evaluations to gain some insight on how mMTC traffic might behave in NOMA applications. In particular, we discuss the loads and levels of multiplexing that can be expected based on the ITU requirements on device density and traffic-models used for cellular IoT.

This paper is a resubmission of R1-1805007.

# 2	Discussion

In the ITU requirements for evaluation of the mMTC scenario a minimum requirement for connection density is 106 devices per km2(section 4.8 of [1]). With a rural Hexagonal grid with ISD = 1732m, thereby an area of 0.866 km2/sector, and independent uniform distribution of devices, the number of devices per sector, N , is ~Poissonλ, where the mean λ=865 975 devices/sector and the standard deviation λ=931 devices/sector. Thus, the most loaded sectors are insignificantly higher loaded than the average sector in terms of devices. The 99% percentile most loaded sector has ~ 868 139 devices, as shown in in the cumulative illustration in Figure 1.

Figure 1	Distribution of number of devices per Hexagonal sector with ISD=1732m and an average user density of 106 devices/km2

Observation: Uniform device density on mMTC ITU requirement level result in practically same number of devices in all sectors of a homogenous deployment scenario.

Following the Mobile Autonomous Reporting (MAR) periodic reports traffic-model for Cellular IoT in Annex E.2.1 of [2], with a uniform distribution of device-reporting timing-offsets, the number of packets arriving from devices with a reporting rate of R reports/day/device in an interval ΔT s (shorter than the reporting periodicity), given a number of NR devices in a sector with this reporting rate, is ~Poisson(λR,ΔT,NR), where

|  | λR,ΔT,NR=NRΔTR/(24⋅60⋅60⋅1000) reporting packets/millisecond. | (1) |
| --- | --- | --- |

With 40%, 40%, 15%, and 5% of devices reporting with rates 1, 12, 24 and 48 times a day, respectively, as in Annex E.2.1 of [2], and the high density of devices as in the ITU requirement and low variability of total number of devices between sectors in comparison to the average N. The approximation NR~ N(NfR, NfR(1-fR) )  is decent. Again the variation around the mean is very small. So, with NR≈fRN, using the mean values of N and fR we in Table 1 list the mean packet arrival numbers for ΔT= 1 ms and 10 ms. With the weighted mean user packet frequency RΣ=r=14RrfRr=11.2 packets/day,  fRΣ=1 , and  NRΣ=N,  we can also approximate the total number of packets on average per sector and time period using Eq. (1) resulting in values in the rightmost column of Table 1.

Table 1	Mean number of report arrivals λR,ΔT,NR, Eq. (1), within time-period ΔT

| Reporting frequency R | 1 report/day | 12 reports/day (every 2 hours) | 24 reports/day (every hour) | 48 reports/day (every 30 min) | RΣ=11.2 reports/day |
| --- | --- | --- | --- | --- | --- |
| Device fraction fR | fR=0.4 | fR=0.4 | fR=0.15 | fR=0.05 | fRΣ=1 |
| ΔT=1ms | 4.01⋅10-3 | 0.0481 | 0.0361 | 0.0241 | 0.112 |
| ΔT=10ms | 0.0401 | 0.481 | 0.361 | 0.241 | 1.12 |

Looking at the cumulative distribution of the number of packet arrivals in Figure 2, we find there are very few multiplexing opportunities for packets arriving during 1ms: only about 0.6% of the time do one or more packets occur in the same subframe. During a 10ms time frame there is a 30% chance of more than 1 packet arrival among the devices in a sector and less than 1% chance of more than 4 packets. If the mMTC opportunities are reduced to every 100ms, more packets need to be multiplexed.

Figure 2	Probabilities that more than x number of packets arrive in a time interval ΔT with Cellular IoT traffic model and 106 Devices/km2 (rightmost column of Table 1). Order magnitude shift of ΔT is equivalent to corresponding shift of device density or reporting frequency.

Since the Poisson parameter scales linearly with the packet frequency and user density we expect that traffic intensity will have to increase by an order of magnitude over the traffic model of 45.820 and ITU requirements to have a significant amount of even 2 user multiplexing opportunities.  If the connection density is increased by 10, then this corresponds to 10 UEs per square meter.  If instead the reporting frequency or packet size is increased, then the assumptions about the underlying smart traffic characteristic and use case may be incorrect.  Greater multiplexing opportunities could also be had if NOMA were scheduled periodically.  However, if only a few PRBs are used for NOMA transmission, then periodic NOMA scheduling would use even less UL resource, and so even substantial NOMA capacity gains may result in only a small net gain.

We therefore make the following observation and proposal

Observations:

  - Current mMTC traffic models and ITU density requirement offer virtually no pairing opportunities (<1% of subframes).

    - Very large connection densities (~6M UEs/km2 or more) are required to have significant opportunities for even 2 user pairing.

    - It is unclear that such a high connection density is realistic

    - Higher reporting frequency or packet sizes may not be relevant to the traffic model or mMTC use case.

Proposal:

  - Further study traffic parameters (such as connection densities, arrival rates, and packet sizes) that are both realistic in mMTC traffic models and use cases and that have suitable pairing probabilities for NOMA study.

# 3	Conclusion

We performed a simple analysis of the ITU density requirements for 5G mMTC together with traffic models used in cellular IoT evaluations, finding that limited system-level NoMA gains can be expected under those models. We made the following observations which led to the proposal below:

Observations:

  - Current mMTC traffic models and ITU density requirement offer virtually no pairing opportunities (<1% of subframes).

    - Very large connection densities (~6M UEs/km2 or more) are required to have significant opportunities for even 2 user pairing.

    - It is unclear that such a high connection density is realistic

    - Higher reporting frequency or packet sizes may not be relevant to the traffic model or mMTC use case.

Proposal:

  - Further study traffic parameters (such as connection densities, arrival rates, and packet sizes) that are both realistic in mMTC traffic models and use cases and that have suitable pairing probabilities for NOMA study.

# 4	References

- ITU-R Report M.2410, “Minimum requirements related to technical performance for IMT-2020 radio interface(s)”, Nov, 2017

- 3GPP TR 45.820: " Cellular system support for ultra-low complexity and low throughput Internet of Things (CIoT) (v13.1.0)".