3GPP TSG RAN WG1 Meeting #93                                          R1-1806409

Busan, Korea, May 21st – 25th, 2018

Agenda Item:	7.2.1

Source:	Spreadtrum Communications

Title:	Consideration on CQI and MCS tables for NR URLLC

Document for:	Discussion and decision

- Introduction

In RAN1 92, the following agreements were made:

Agreements:

The two BLER targets for CQI reporting that are configurable for URLLC are to be down-selected from one of the following options:

- Option A. (10-1, 10-4)

- Option B. (10-1, 10-5)

- Option C. (10-3, 10-5)

- Option D. (10-2, 10-4)

In RAN1 92BIS, the following agreements were made:

Agreements:

- The two BLER targets that are configurable for URLLC for CSI reporting are:

  - Option B. (10-1, 10-5)

  - Note: The definition of the test case for the BLER target of 10-5 should take into account channel and interference variations and estimation errors.

Agreements:

- In total, there are two CQI tables for URLLC CQI reporting

  - The first table for URLLC CQI reporting is the same as the existing 64QAM CQI table without any change, which is for BLER target 10-1 for URLLC

    - Note: this means the agreement on “Highest spectral efficiency for CQI based on 10-1 BLER target for URLLC is no more than 873/1024*6” is overturned

  - The new table will have entries corresponding to BLER target 10-5

- For CSI reporting, the CQI field is 4-bit.

Agreements:

- For BLER 10-5,

- Companies are encouraged to perform simulations for the new CQI table for URLLC, including

  - The lowest SE entry

    - E.g., 30~50/1024*2

  - Note that the highest SE entry of no more than 772/1024*6 is already agreed

  - Consider using approximately equally spaced SNR values

    - Other options are not precluded

    - Whether or not some existing CQI entries for BLER 10-1 can be reused

      - Consider exsiting CQI entires when applicable

  - In total 15 CQI entries (+1 OOR entry)

- In performing the simulations, consider

  - Fading channel (TDL-A, 30ns) & (TDL-C, 300ns)

    - Other options are not precluded

  - Payload of 32 bytes

    - Other payload sizes can also be considered, up to each company

  - SNR at 5% geometry for the lowest SE entry

    - Other options are not precluded

  - For other simulation assumptions, refer to agreements from RAN1#92

- Similar considerations are also applicable to the MCS table evaluations

In this contribution, we discuss CQI table design for URLLC and present an example table following the proposed design goals.

- Discussion

  - Target BLER

URLLC requirements are such that a small packet of size 32 byte can be transmitted within 1ms latency with success probability of 1-10-5. In LTE, CQI report is derived based on 10% BLER. Several retransmissions can be performed to achieve high reliability if required. However, since much stricter requirements on both reliability and latency are set, CQI report for URLLC should be based on lower target BLER.

In last meeting, it was agreed the two BLER targets that are configurable for URLLC for CSI reporting are (10-1, 10-5). The main motivation behind this decision is to support multiple URLLC services which may require different reliability and latency requirements. According to this agreement, there are two CQI tables for URLLC CQI reporting. The first table for URLLC CQI reporting is the same as the existing 64QAM CQI table without any change, which is for BLER target 10-1 for URLLC. The secondary table will have entries corresponding to BLER target 10-5, the values in this table are undetermined.

  - CQI Table Design for URLLC

The target BLER should provide different CQI indices for a given TBS. As we know, it is meaningless to introduce both 10-1 and 10-2 as BLER targets if the SNRs are close to each other. To see that, we investigate the variation of the BLER performance assuming more realistic channel model assumptions TDL.

For URLLC, it is important to have an accurate CQI report from the UE and accuracy can be improved by assuming good granularity of SE and a higher number of entries. Considering also on the feedback overhead, we think that 4-bit CQI feedback is reasonable for URLLC. If CQI entries are also used in the MCS table, the scheduler could use CQI report in the scheduling decisions, and this is already the case in LTE. Considering this aspect, we give an example CQI table for URLLC is shown in Table 1. Compared with the EMBB CQI table, the minimum SE is reduced from 0.1523 to 0.0586.

Table 1 4-bit CQI table for URLLC

| CQI index for BLER 10^-5 | modulation | code rate x 1024 | efficiency |
| --- | --- | --- | --- |
| 0 | out of range |  |  |
| 1 | QPSK | 30 | 0.0586 |
| 2 | QPSK | 50 | 0.0977 |
| 3 | QPSK | 78 | 0.1523 |
| 4 | QPSK | 120 | 0.2344 |
| 5 | QPSK | 193 | 0.3770 |
| 6 | QPSK | 308 | 0.6016 |
| 7 | QPSK | 449 | 0.8770 |
| 8 | QPSK | 602 | 1.1758 |
| 9 | 16QAM | 378 | 1.4766 |
| 10 | 16QAM | 490 | 1.9141 |
| 11 | 16QAM | 616 | 2.4063 |
| 12 | 64QAM | 466 | 2.7305 |
| 13 | 64QAM | 567 | 3.3223 |
| 14 | 64QAM | 666 | 3.9023 |
| 15 | 64QAM | 772 | 4.5234 |

Link level simulation has been performed using the proposed CQI tables. The achievable spectral efficiency (bits/symbol) for different MCSs in the CQI table with 10-5 target BLER are shown as a function of SNR for TDL-A channel. Simulation assumption is summarized in the table2 in appendix. In simulation, four code rates are tested. We hope well-distributed simulation results curve, so code rates entries 30/1024 and 50/1024 are selected for Table1.

Figure 1   Performance of CQI table for TDL-A channel

- Conclusion

In this contribution we discussed the open issues related to the BLER targets for CQI reporting and the design of CQI table for URLLC in NR. Based on the discussion, following proposals are made:

Proposal 1: Adopt Table 1 of current document for CQI table designed for configured BLER 10-5.

- References

- 3GPP TR 38.213 3rd Generation Partnership Project; Technical Specification Group Radio Access Network; NR; Physical layer procedures for control(Release 15)

- Chairman’s note RAN1#92

- Chairman’s note RAN1#92 bis

- R1-1803919 CQI and MCS Tables for URLLC, Ericsson,  3GPP RAN1 Meeting #92b, April 2018.

- Appendix

Table 2: Link level simulation assumption

| Parameters | Value |
| --- | --- |
| System bandwidth | 20 MHz |
| Carrier Frequency | 4 GHz |
| Subcarrier spacing | 30 kHz |
| Modulations | QPSK |
| Channel coding | LDPC: LNMS, SF=0.8, 20 iterations |
| Allocation in time | Slot transmission 14os |
| Allocation in frequency | 4PRB |
| Channel estimation | Ideal |
| Channel model | TDL-A (delay spread: 300ns) |
| UE speed | 3 km/h |
| Number of BS antennas | 1Tx |
| Number of UE antennas | 1Rx |