3GPP TSG RAN WG1 Meeting #93	R1-1806706

# Busan, Korea, May 21st – 25th, 2018

Agenda item:	7.1.1.5.1

Source:	Samsung

Title:	Corrections on Measurements for Mobility Management

Document for:	Discussion and decision

- Introduction

This contribution discusses the remaining issues on the measurements for mobility management. It is updated from R1-1804350.

- C-DRX UE’s assumption on CSI-RS reception

For C-DRX UE’s assumption on CSI-RS reception, RAN1#91 identified two alternatives as following:

- Alt 1: UE assumes that the periodicity of CSI-RS resources before entering C-DRX mode is kept during C-DRX mode, and it is up to UE implementation that UE measures CSI-RS resources of neighbour cell even outside active time.

- Alt 2: Network is not required to transmit the configured CSI-RS resources for L3 mobility that occur outside the active time of UE configured with C-DRX.

Taking into account current DRX operation specified in [1], we would need to understand what the specification impact from above alternatives is.

3GPP TS 38.321 V15.1.0 (2018-3) - 5.7 Discontinuous Reception (DRX) … 1> if the MAC entity is in Active Time: 2> monitor the PDCCH; 2> if the PDCCH indicates a DL transmission or if a DL assignment has been configured: 3> start the drx-HARQ-RTT-TimerDL for the corresponding HARQ process immediately after the corresponding PUCCH transmission; 3> stop the drx-RetransmissionTimerDL for the corresponding HARQ process. 2> if the PDCCH indicates a UL transmission: 3> start the drx-HARQ-RTT-TimerUL for the corresponding HARQ process immediately after the first repetition of the corresponding PUSCH transmission; 3> stop the drx-RetransmissionTimerUL for the corresponding HARQ process. 2> if the PDCCH indicates a new transmission (DL or UL): 3> start or restart drx-InactivityTimer. 1> else (i.e. not part of the Active Time): 2> not transmit type-0-triggered SRS defined in TS 38.214 [7]. 1> if CQI masking (cqi-Mask) is setup by upper layers: 2> if drx-onDurationTimer is not running: 3> not report CSI on PUCCH. 1> else: 2> if the MAC entity is not in Active Time: 3> not report CSI on PUCCH.

From our perspective, the following RAN1#90 agreement would be enough.

RAN1#90 Agreement: - UE is not required to measure CSI-RS configured for L3 mobility outside the active time

Proposal 1: For C-DRX UE’s assumption on CSI-RS reception, no additional clarification of the specification is needed.

- Conclusion

Proposal 1: For C-DRX UE’s assumption on CSI-RS reception, no additional clarification of the specification is needed.

Reference

- 3GPP TS 38.321 v15.1.0 (2018-3)