3GPP TSG RAN WG1 Meeting #93	R1-1806707

# Busan, Korea, May 21st – 25th, 2018

Agenda item:	7.1.1.5.2

Source:	Samsung

Title:	Corrections on Radio Link Monitoring for Mobility Management

Document for:	Discussion and decision

- Introduction

This contribution discusses the remaining issues of RLM including;

- Relationship between RLF and BFD and BFR

- The maximum number of RLM-RS(s) and BFD-RS(s)

- Relationship between RLM-RS and serving PDCCH monitoring

- Relationship between RLF and BFD and BFR

The relationship between RLM and beam management has been discussed in previous meetings. Finally, RAN2#101bis achieved following progress and consequently the pending issue on aperiodic indication based on BFR are addressed:

Agreements:

1:	No aperiodic indication of a successful beam recovery will be reported to RRC.

2:	BFR failure will result in a RACH failure reported to RRC and will trigger RRC to perform either re-establishment or SCG failure. This is already the behaviour according to the current MAC and RRC specs (nothing extra to specify)

Observation: RAN2 already concluded on the aperiodic indication based on BFR.

- The maximum number of RLM-RS(s) and BFD-RS(s)

Among the following agreements on the maximum number of RLM-RS(s) and BFD-RS(s), there remain working assumptions which are band specific:

Agreements in RAN1#92bis: - To reply the LS from RAN2 in R1-1803577 - Answer 1: - The maximum number of BFD-RS(s) is 2 per BWP. - The maximum number of RLM-RS(s) and BFD-RS(s) should depend on whether same RS(s) is shared between RLM and BFD. The maximum number of unique RS(s), each RS using different set of resources, for both RLM-RS(s) and BFD-RS(s) are: - X RS(s) per BWP for below 3 GHz, - X=2 (working assumption) - Y RS(s) per BWP for above 3 GHz and below 6 GHz, - Y=6 (working assumption) - Z RS(s) per BWP for above 6 GHz, - Z=8 (working assumption) - where maximum number of BFD-RS(s) is 2 RSs per BWP and maximum number of RLM-RS(s) is 2 RSs per BWP for below 3 GHz, 4 RSs per BWP for above 3 GHz and below 6 GHz, 8 RSs per BWP for above 6 GHz. - Please note that support of 8 RLM-RSs and 2 BFD-RS for above 6 GHz is feasible if the 2 BFD-RS are a subset of 8 RLM-RSs. - Answer 2: - Yes. They can be completely orthogonal, depending on NW configuration.

It appears there is no compelling reason not to confirm the working assumptions.

Proposal 1: Confirm the working assumptions on the maximum number of unique RS(s), i.e., X=2, Y=6, Z=8.

- Relationship between RLM-RS and serving PDCCH monitoring

Previously, RAN2 agreed following for RLM-RS:

RAN2 Agreements#1 1 Introduce one list of RSs and indicate for each whether it is used for beam- and/or cell-RLM. 1a If no RSs are provided for Beam-Monitoring, the UE performs Beam-Monitoring based on the TCI-State for PDCCH (as agreed by RAN1) 2 If no RSs are provided in this list at all (neither for Cell- nor for Beam-RLM), the UE performs Cell-RLM based on TCI-State of PDCCH

A text proposal was suggested [1] to reflect above RAN2 agreement to be concluded in this RAN1 meeting. Further, RAN1 has clarified above RAN2 agreements as following:

Agreements in RAN1#92bis: - Further clarification of RAN2 agreement (will require additional physical layer text proposals): - Working assumption: If the TCI-states refer to CSI-RS for tracking, it is up to UE to select a NZP-CSI-RS resource from the configured resources for CSI-RS for tracking for RLM - FFS on the UE behavior when TCI-states indicate a combination of SSB, CSI-RS, and CSI-RS for tracking

Regarding the UE behavior when TCI-states indicate a combination of RSs, it is our understanding that current description in TS38.214 is good enough.

In addition, RAN2 LS [2] indicates following additional agreements:

RAN2 agreements#2 => If RS for beam failure detection are explicitly configured then RSs for RLM are always explicitly configured by the network

Combining above RAN2 agreements #1 and #2, the valid RS configurations for beam failure detection and radio link monitoring are cases #1, #2, and #4 as summarized in Table 1.

Table 1: RS configuration for beam failure detection and radio link monitoring

| Case | RS for beam failure detection | RS for radio link monitoring | Remark |
| --- | --- | --- | --- |
| #1 | Configured | Configured |  |
| #2 | Not configured | Configured |  |
| #3 | Configured | Not configured | Not supported based on above RAN2 agreements#2 |
| #4 | Not configured | Not configured |  |

- Conclusions

Observation: RAN2 already concluded on the aperiodic indication based on BFR.

Proposal 1: Confirm the working assumptions on the maximum number of unique RS(s), i.e., X=2, Y=6, Z=8.

References

[1] R1-1805697, Summary of Offline Discussion for NR Radio Link Monitoring, Intel

[2] R1-1805811 (R2-1806504), LS to RAN1 on Removing RLM-RS Restriction in TS 38.331 ASN.1, RAN2

[3] R2-1806391, CR to TS38.331, Ericsson

Annex: TS38.331 (based on the latest endorsed CR [3])

–	RadioLinkMonitoringConfig

The RadioLinkMonitoringConfig IE is used to configure radio link monitoring for detection of beam- and/or cell radio link failure. See also 38.321, section 5.1.1.

RadioLinkMonitoringConfig information element

-- ASN1START

-- TAG-RADIOLINKMONITORINGCONFIG-START

RadioLinkMonitoringConfig ::=		SEQUENCE {

failureDetectionResourcesToAddModList			SEQUENCE (SIZE(1..maxNrofFailureDetectionResources)) OF RadioLinkMonitoringRS	OPTIONAL, 	-- Need N

failureDetectionResourcesToReleaseList	SEQUENCE (SIZE(1..maxNrofFailureDetectionResources)) OF RadioLinkMonitoringRS-Id	OPTIONAL,-- Need N

beamFailureInstanceMaxCount				ENUMERATED {n1, n2, n3, n4, n5, n6, n8, n10}									OPTIONAL,	-- Need S

beamFailureDetectionTimer      			ENUMERATED {pbfd1, pbfd2, pbfd3, pbfd4, pbfd5, pbfd6, pbfd8, pbfd10}			OPTIONAL,	-- Need R

...

}

RadioLinkMonitoringRS ::=			SEQUENCE {

radioLinkMonitoringRS-Id				RadioLinkMonitoringRS-Id,

purpose								ENUMERATED {beamFailure, rlf, both},

detectionResource					CHOICE {

ssb-Index							SSB-Index,

csi-RS-Index						NZP-CSI-RS-ResourceId

},

...

}

-- TAG-RADIOLINKMONITORINGCONFIG-STOP

-- ASN1STOP

–	TCI-State

The TCI-State IE associates one or two DL reference signals with a corresponding quasi-colocation (QCL) type.

TCI-State information element

-- ASN1START

-- TAG-TCI-STATE-START

TCI-State ::= 						SEQUENCE {

tci-StateId							TCI-StateId,

qcl-Type1							QCL-Info,

qcl-Type2							QCL-Info																OPTIONAL,	-- Need R

nrofPTRS-Ports						ENUMERATED {n1, n2}														OPTIONAL,	-- Need R

...

}

QCL-Info ::=						SEQUENCE {

cell								ServCellIndex															OPTIONAL,	-- Need R

bwp-Id								BWP-Id																	OPTIONAL, -- Cond CSI-RS-Indicated

referenceSignal						CHOICE {

csi-rs								NZP-CSI-RS-ResourceId,

ssb									SSB-Index,

csi-RS-for-tracking					NZP-CSI-RS-ResourceSetId

},

qcl-Type							ENUMERATED {typeA, typeB, typeC, typeD},

...

}

-- TAG-TCI-STATE-STOP

-- ASN1STOP

QCL-Info field descriptions

bwp-Id The DL BWP which the RS is located in.

cell The carrier which the RS is located in. If the field is absent, it applies to the serving cell in which the TCI-State is configured. The RS can be located on a serving cell other than the serving cell in which the TCI-State is configured only if the qcl-Type is configured as typeD. See TS 38.214 section 5.1.5.

csi-RS-for-tracking A set of CSI-RS resources for tracking

referenceSignal Reference signal with which quasi-collocation information is provided as specified in TS 38.3214 subclause 5.1.5.

qcl-Type QCL type as specified in TS 38.214 subclause 5.1.5.

| Conditional Presence | Explanation |
| --- | --- |
| CSI-RS-Indicated | This field is mandatory present if csi-rs or csi-RS-for-tracking is included, absent otherwise |