3GPP TSG RAN WG1 Meeting #93	 		R1-1806709

# Busan, Korea, 21st – 25th May 2018

Agenda item:	7.1.2.1.2

Source: 	Samsung

Title: 	Issues on codebook based UL transmission

Document for:	Discussion and Decision

# Introduction

In this contribution, the following two issues about codebook based UL transmission have been discussed: (1) UL power control and (2) SRS time domain behavior.

# UL power control

Table 1: Coherent port groups for 4 antenna ports

Table 2: Total power of precoding matrix for 4 antenna ports

| Rank | NC TPMIs |  | PC TPMIs |  | FC TPMIs |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | TPMI indices | Total power | TPMI indices | Total power | TPMI indices | Total power |
| 1 | 0-3 | ¼ | 4-11 | ½ | 12-27 | 1 |
| 2 | 0-5 | ½ | 6-13 | 1 | 14-21 | 1 |
| 3 | 0 | ¾ | 1-2 | 1 | 3-6 | 1 |
| 4 | 0 | 1 | 1-2 | 1 | 3-4 | 1 |

For codebook based UL transmission, a pre-coding matrix  is indicated to a UE from an UL codebook that comprises pre-coding matrices of one of the three coherent types, nonCoherent (NC), partialAndNonCoherent (PC+NC), fullAndPartialAndNonCoherent (FC+PC+NC), where the UL codebook is configured to the UE depending on the capability reported by the UE. In particular, if the UE reports full-coherence as its capability, then the UE can be configured with any of the three coherence types, i.e., NC, PC+NC, or FC+PC+NC, for TPMI. If the UE reports partial-coherence as its capability, then the UE can be configured with one of the two coherence types, NC or PC+NC, for TPMI. If the UE reports non-coherence as its capability, then the UE can only be configured with NC as its coherence type for TPMI. Based on the UE capability, the antenna ports at the UE are grouped into coherent port groups. As an example, for 4 antenna ports, the coherent port groups are illustrated in Table 1 for the three coherent types. The UL codebook in NR comprises pre-coding matrices of these three coherent types. The total power of the pre-coding matrices for different ranks and coherence types is summarized in Table 2. As discussed in [1], we can observe the following.

- Issue 1: for PC and NC TPMIs, total power increases as rank increases, which implies that the TPMI selection will be biased to higher rank. In particular, even for cell-edge UEs, rank 1 TPMI may not be selected, which can severely affect cell-edge performance.

- Issue 2: for a given rank, total power of NC TPMIs ≤ total power of PC TPMIs ≤ total power of FC TPMIs. The reason for this trend is that the power of non-zero (NZ) antenna ports does not changes across three types of TPMIs.

In our view, Issue 1 will affect UL performance for NC and PC UEs significantly, and hence should be addressed. Issue 2 on the other hand can be argued to be necessary/beneficial for UE power saving by turning off antenna(s), similar to LTE UL codebook.

A solution (cf. 38.213, section 7.1) to address Issue 1 is proposed in [1], which is copied below.

>>>>>>>>>>>> Start text proposal in [1] >>>>>>>>>>>>

For PUSCH, a UE first scales a linear value  of the transmit power  on UL BWP , as described in Subclause 12, of carrier  of serving cell , with parameters as defined in Subclause 7.1.1, by β and the resulting scaled power is then split equally across the antenna ports on which the non-zero PUSCH is transmitted, where β=1 for single antenna port transmission, and for multi-antenna port transmission:

- β=min{1,α} with α=Kρ0ρ where

  - ρ is the number of antenna ports  according to 38.211 6.3.1.5

  - ρ0 be the number of non zero antenna ports in according to 38.211 6.3.1.5.

  - For non-codebook based transmission K=1. For codebook based transmission K is given from the table below, where ULCodebookSubset is a higher layer parameter.

| ULCodebookSubset | Number of UE antenna ports | K |
| --- | --- | --- |
| fullAndPartialAndNonCoherent | 2 | 1 |
| fullAndPartialAndNonCoherent | 4 | 1 |
| partialCoherent | 4 | 2 |
| nonCoherent | 2 | 2 |
| nonCoherent | 4 | 4 |

>>>>>>>>>>>> End text proposal in [1] >>>>>>>>>>>>

However, there is one issue with this solution. For 4 antenna ports and PC+NC coherence type, as highlighted in Table 3, the power per NZ port changes across NC and PC TPMIs (for a fixed rank of 2 or 3). In our view, it is better to keep power per NZ ports fixed for a given rank. This is to ensure that the UE doesn’t need to change its power setting if only TPMI changes (and TRI does not change), and may have to change power only when TRI value changes, and TRI value change is expected to be slower than TPMI change.

Table 3: power per NZ port according to proposal in [1]

| Coherence type | Rank | NC TPMIs |  |  | PC TPMIs |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | TPMI | β | Power/NZ ports | TPMI | β | Power/NZ ports |
| PC+NC | 2 | 0-5 | 1 | 1/2 | 6-13 | 1 | ¼ |
|  | 3 | 0 | 1 | 1/3 | 1-2 | 1 | ¼ |

In particular, the solution to address Issue 1 should meet the following constraints.

- Constraint 1: at least for the “most coherent” TPMIs, total power should not change across rank (to solve Issue 1)

  - for FC+PC+NC, the most coherent TPMIs = FC TPMIs;

  - for PC+NC, the most coherent TPMIs = PC TPMIs; and

  - for NC, the most coherent TPMIs = NC TPMIs

- Constraint 2: For a given rank, power per NZ antenna port should not change (to save UE power) across all TPMIs.

For codebook-based UL transmission, a solution which satisfies the two constraints is the scaling factor β=β1β2, where

- β1 scales the transmit power equally across the coherent port groups (K in proposal [1]) on which a non-zero PUSCH is transmitted, i.e., β1=1K0, and

- for each coherent port group with a non-zero PUSCH, β2 further scales the transmit power equally across the ports within the coherent port group on which a non-zero PUSCH is transmitted, i.e., β2=k=0K-1ρ0,kρk=Kk=0K-1ρ0,kρ=Kρ0ρ=α in proposal [1]

  - K = #coherent port groups (for the “most coherent” TPMIs) as in Table 4

  - K0 = #coherent port groups (for the “most coherent” TPMIs) with a non-zero PUSCH

  - ρk = #configured ports for the transmission in the k-th coherent port group,

  - ρ0,k = #ports with a non-zero PUSCH transmission in the k-th coherent port group,

  - ρ = #configured ports for the transmission, and

  - ρ0 = #ports with a non-zero PUSCH transmission.

Table 4: Number of coherent antenna port groups (for the “most coherent” TPMIs) (K)

| Coherence type | Number of antenna ports = 2 | Number of antenna ports = 4 |
| --- | --- | --- |
| nonCoherent | 2 | 4 |
| partialAndNonCoherent |  | 2 |
| fullAndPartialAndNonCoherent | 1 | 1 |

Effectively, this solution replaces the min operation in proposal [1] with another scaling β1 to distribute power over NZ coherent port groups. We can verify that the power per NZ port is fixed for a given rank. For example, for 4 antenna ports and PC+NC coherence type, as highlighted in Table 5, the power per NZ port is fixed across NC and PC TPMIs (for a fixed rank of 2 or 3).

Table 5: power per NZ port according to proposal in this contribution

| Coherence type | Rank | β1 | NC TPMIs |  |  |  | PC TPMIs |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | TPMI | β2 | β | Power/NZ ports | TPMI | β2 | β | Power/NZ ports |
| PC+NC | 2 | ½ | 0-5 | 1 | ½ | ¼ | 6-13 | 2 | 1 | ¼ |
|  | 3 | ½ | 0 | 3/2 | ¾ | ¼ | 1-2 | 2 | 1 | ¼ |

For non-codebook based UL transmission, β=1.

Proposal: for UL power control, PUSCH is scaled by a factor β and the resulting scaled power is then split equally across the antenna ports on which the non-zero PUSCH is transmitted, where

- for non-codebook based UL transmission, β=1

- for codebook based UL transmission, β=β1β2

  - β1=1K0, and

  - β2=Kρ0ρ, where

    - K = #coherent port groups (for the “most coherent” TPMIs) as in Table 4,

    - K0 = #coherent port groups (for the “most coherent” TPMIs) with a non-zero PUSCH,

    - ρ = #configured ports for the transmission, and

    - ρ0 = #ports with a non-zero PUSCH transmission.

The corresponding text proposal is provided in Section 5.

# SRS time domain behavior

The following agreement was made in RAN1 NR-AH #1801 [2].

Agreement

- For codebook based uplink transmission, SRS resources in the same resource set should have the same time domain behavior on periodic, aperiodic and semi-persistent SRS

- Send the above as part of LS to RAN2

Based on the offline summary [3], the following alternatives about time domain behavior of SRS resources in a SRS resource set were discussed during RAN1#92 and RAN1#92bis.

- Alt 1: UE can be configured with one SRS resource set for codebook based UL transmission with only one type of time-domain behaviour (agreement made in RAN1 NR-AH #1801)

- Alt 2: UE can be configured with multiple SRS resource sets for codebook based UL transmission, each with only one type of time-domain behaviour

- Alt 3: UE can be configured with one SRS resource set for codebook based UL transmission, each SRS resource can have a SRS-SlotConfig parameter for periodic/semi-persistent transmission and can be triggered by SRS request field.

Alt 1 was agreed because it is simple, hence preferred over a more complex solution such as Alt 2 or Alt 3.

Both Alt 2 and Alt 3, though provide more flexibility, are unclear to provide any significant performance gain over Alt 1. Also, the following issues were identified at least for Alt 3 (they may also be applicable to Alt 2):

- If Alt 3 is also used for non-codebook based UL transmission, then it can complicate the association of CSI-RS resource to a SRS resource.

- It is unclear why aperiodic SRS is not included.

Alt 2, on the other hand, can be argued to be simpler than Alt 3 (since similar functionality can be achieved by allowing, e.g., up to two SRS resource sets) with less specification impact on the existing parameters such as SRS-SlotConfig. But again, it is not clear how much benefit it can bring when compared with Alt 1.

In summary, in our view, there is no critical issue with Alt 1, and there is no clear understanding about how much benefit can be achieved with either Alt 2 or Alt 3 over Alt 1. Such improvements require more discussion, hence can be discussed in future release.

Proposal: Further improvements to time domain behaviour of SRS resources for codebook-based UL transmission is not supported in Rel. 15; such improvements can be discussed in future release.

# 4	Conclusion

In this contribution, the following proposals are made regarding codebook-based UL transmission.

- For UL power control, PUSCH is scaled by a factor β and the resulting scaled power is then split equally across the antenna ports on which the non-zero PUSCH is transmitted, where

  - for non-codebook based UL transmission, β=1

  - for codebook based UL transmission, β=β1β2

    - β1=1K0, and

    - β2=Kρ0ρ, where

      - K = #coherent port groups (for the “most coherent” TPMIs) as in Table 4,

      - K0 = #coherent port groups (for the “most coherent” TPMIs) with a non-zero PUSCH,

      - ρ = #configured ports for the transmission, and

      - ρ0 = #ports with a non-zero PUSCH transmission.

- Further improvements to time domain behaviour of SRS resources for codebook-based UL transmission is not supported in Rel. 15; such improvements can be discussed in future release.

# 5	Text Proposal for TS 38.213, Section 7.1

For PUSCH, a UE first scales a linear value  of the transmit power  on UL BWP , as described in Subclause 12, of carrier  of serving cell , with parameters as defined in Subclause 7.1.1, by the ratio of the number of antenna ports with a non-zero PUSCH transmission to the number of configured antenna ports for the transmission scheme β and . T the resulting scaled power is then split equally across the antenna ports on which the non-zero PUSCH is transmitted, where

- β=1 for single antenna port transmission or non-codebook based transmission, and

- β=β1β2 for codebook-based port transmission, where

  - β1=1K0, and

  - β2=Kρ0ρ, where

    - ρ is the number of configured ports for transmission,

    - ρ0 is the number of ports with non-zero PUSCH, and

    - K is the number of coherent port groups (for the “most coherent” TPMIs) given by

| codebookSubset | Number of antenna ports = 2 | Number of antenna ports = 4 |
| --- | --- | --- |
| nonCoherent | 2 | 4 |
| partialAndNonCoherent |  | 2 |
| fullAndPartialAndNonCoherent | 1 | 1 |

    - K0 is number of coherent port groups (for the “most coherent” TPMIs) with a non-zero PUSCH.

# Reference

- R1-1805205, Power control for UL MIMO, Ericsson

- 3GPP, RAN1 NR-AH#1801, Chairman’s Notes

- R1-1803321, Offline Discussion Summary on Codebook Based UL Transmission, Intel