3GPP TSG RAN WG1 Meeting #93			R1-1806777

Busan, Korea, May 21st – May 25th, 2018

Title:	[Draft] Reply LS on subcarrier spacing for BWPs and TDD configurations

Release:	Rel-15

Work Item:	NR_newRAT-Core

Source:	MediaTek, [RAN1]

To:	RAN2

CC:

Contact Person:

#### Name:	Pei-Kai Liao

E-mail Address:	pk.liao@mediatek.com

1. Overall Description:

Regarding to RAN2’s questions, RAN1 provides answers as follows.

Q1: Some of the fields are redundant (i.e. could be removed)?

Answer: None of them can be removed due to the following reasons.

- SCS in a DL BWP configuration is applied to PDCCH, PDSCH and corresponding DMRS when the DL BWP becomes active

- Reference SCS in SIB1 TDD configuration is used as a reference subcarrier spacing to align DL/UL transmission direction in time domain based on the signalled TDD configuration when different subcarrier spacings are applied to actual transmission in a carrier

- Reference SCS in slotFormatCombinationsPerCell is used as a reference subcarrier spacing to align DL/UL transmission in a slot based on the signalled SFI

- SCS list in scs-SpecificCarrierList is used to provide a look-up table of offsetToCarrier, k0 and carrierBandwidth for all applicable subcarrier spacings for a cell and a UE can use this look-up table to locate the configured BWPs in frequency domain based on the SCS in BWP configuration.

Q2: Some of the fields have to be set to the same values (e.g. SIB1 SCS vs. BWP SCS, FrequencyInfoDL SCS list vs BWP SCS)?

Answer: There is no such requirement to set the same values for any pair among SCS in BWP, reference SCS in SIB1 TDD configuration, reference SCS in slotFormatCombinationsPerCell and SCS list in scs-SpecificCarrierList.

Q3: Some of the fields have a more complicated relationship (e.g. use valueX only if valueY is used)?

Answer: While configuring values for SCS in BWP, reference SCS in SIB1 TDD configuration, reference SCS in slotFormatCombinationsPerCell and SCS list in scs-SpecificCarrierList, the following conditions should be met.

- The reference SCS in SIB1 TDD configuration is not larger than any SCS of configured BWPs for Pcell

- The reference SCS in slotFormatCombinationsPerCell is not larger than any SCS of configured BWPs for a cell

- SCS list in scs-SpecificCarrierList should contain all SCSs of configured BWPs for a cell

2. Actions:

To RAN2 group

ACTION:

RAN1 respectfully asks RAN2 to take the above information into account in their future work.

3. Date of Next RAN1 Meetings:

RAN1#94	20th – 24th August 2018	Gothenburg, Sweden

RAN1#94bis	8th – 12th October 2018	Chengdu, China