3GPP TSG RAN WG1 Meeting #93		R1-1807103

Busan, Korea, May 21st – 25th, 2018

Agenda item:	6.2.6.2

Source: 	Qualcomm Incorporated

Title: 	Physical layer aspects of early data transmission

Document for:	Discussion/Decision

# Background

In RAN1#92b, the following agreements were reached regarding early data transmission:

Agreement

For EDT scheduled by RAR grant,

- For CE Mode A, 2–6 PRBs are supported with 4 bits resource allocation field.

  - Whether to support 1 PRB can be reconsidered if there is an available spare bit to expand resource allocation field.

- For CE Mode B, 1-2 PRBs are supported with at least 3 bits resource allocation field.

  - Resource allocation may or may not be modified, picking at least 8 resource allocations.

  - Whether to expand resource allocation can be reconsidered if there is an available spare bit to expand resource allocation field.

Agreement

- The 8 maximum TBS values in CE mode A (i.e. PRACH levels 0 and 1) are {328 408 504 600 712 808 936 1000}.

  - Only values of NPRB and values of TBS in legacy Rel-13 PUSCH table can be used for EDT

- The 8 maximum TBS values in CE mode B (i.e. PRACH levels 2 and 3) are {328 408 456 504 600 712 808 936}.

  - Only values of NPRB and values of TBS in legacy Rel-13 PUSCH table can be used for EDT

- For each of the 8 maximum TBS values

  - The number of repetitions changes depending on the actual TBS, potentially including numbers of repetitions which are not included in legacy PUSCH repetition numbers (FFS how)

Agreement

For EDT Msg. 3 transmission, 16QAM modulation is not supported.

Agreement

- The use of TBS smaller than the maximum configured is configured per CE level in SIB.

- Per cell, in the below, Ti < Ti+1, eNB can configure that the UE chooses from:

  - When there are 4 permitted actual transmitted TBS {T1, T2, T3, T4}

    - T2 or T4

    - T1, or T2, or T3, or T4

  - When there are 3 permitted actual transmitted TBS {T1, T2, T3}

    - T2 or T3

    - T1 or T2 or T3

  - When there are 2 permitted actual transmitted TBS {T1, T2}

    - T1 or T2

Agreement

The possible smaller TBS values are predefined from the configured maximum TBS and the configured maximum number of blind decodes.

Agreement

Use the ‘R’ bit in RAR to differentiate EDT and non-EDT.

Agreement in email discussion [92b-LTE-09] on tables for EDT:

Table 1: Exact TBS table on EDT, CEModeA

|  | 328 | 408 | 504 | 600 | 712 | 808 | 936 | 1000 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| T1 | 328 | 328 | 328 | 328 | 328 | 328 | 328 | 328 |
| T2 |  | 408 | 408 | 408 | 456 | 504 | 504 | 536 |
| T3 |  |  | 456 | 504 | 600 | 712 | 712 | 776 |
| T4 |  |  | 504 | 600 | 712 | 808 | 936 | 1000 |

Table 2: Exact TBS table on EDT, CEModeB

|  | 328 | 408 | 456 | 504 | 600 | 712 | 808 | 936 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| T1 | 328 | 328 | 328 | 328 | 328 | 328 | 328 | 328 |
| T2 |  | 408 | 408 | 408 | 408 | 456 | 504 | 504 |
| T3 |  |  | 456 | 456 | 504 | 600 | 712 | 712 |
| T4 |  |  |  | 504 | 600 | 712 | 808 | 936 |

# Handling of retransmissions

Upon transmitting the initial msg3, the UE may receive a DCI 6-0 triggering a retransmission. The agreement in RAN1#92b allows the UE to pick a TBS and number of repetitions depending on the amount of transmitted data.

If the initial msg3 transmission fails, then the eNB may not be aware of the TBS/number of repetitions used by the UE (especially in very low SNR conditions). Thus, the UE should use the TBS determined in the RAR, and scale the number of retransmissions using the same principle as in RAR.

Proposal 1: For a retransmission of msg3 carrying early data transmission, the UE should use the same TBS as previous transmission (i.e., ignoring the TBS determination in DCI format 6-0), and interpret the repetition number field in the same way as RAR grant.

# Scaling of repetitions

In Sections 5/6 we evaluated the performance of (the majority of) TBS included in the EDT tables. By inspecting the results, we choose the scaling on the number of repetitions that provides a similar BLER as the maximum TBS, for a given SNR value.

Note that, at this early stage of transmission, the eNB may have only a rough estimation of the SNR of the UE, so, in our view, it is unnecessary to include new repetition factors. For the cases of R=1, there is no scaling. We propose the following scaling values for the TBS values, where R is the number of repetitions indicated in DCI or RAR.

Table 1: Exact TBS table on EDT with corresponding repetition level, CEModeA

|  | 328 | 408 | 504 | 600 | 712 | 808 | 936 | 1000 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| T1 | 328 (R) | 328 (R) | 328 (R/2) | 328 (R/2) | 328 (R/2) | 328 (R/2) | 328 (R/2) | 328 (R/2) |
| T2 |  | 408 (R) | 408 (R) | 408 (R) | 456 (R/2) | 504 (R/2) | 504 (R/2) | 536 (R/2) |
| T3 |  |  | 456 (R) | 504 (R) | 600 (R) | 712 (R) | 712 (R) | 776 (R) |
| T4 |  |  | 504 (R) | 600 (R) | 712(R) | 808 (R) | 936 (R) | 1000(R) |

Table 2: Exact TBS table on EDT with corresponding repetition level, CEModeB

|  | 328 | 408 | 456 | 504 | 600 | 712 | 808 | 936 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| T1 | 328 (R) | 328 (R) | 328 (R) | 328 (R/2) | 328 (R/2) | 328 (R/2) | 328 (R/2) | 328 (R/2) |
| T2 |  | 408 (R) | 408 (R) | 408 (R) | 408 (R) | 456 (R/2) | 504 (R/2) | 504 (R/2) |
| T3 |  |  | 456 (R) | 456 (R) | 504 (R) | 600 (R) | 712 (R) | 712 (R) |
| T4 |  |  |  | 504 (R) | 600 (R) | 712 (R) | 808 (R) | 936 (R) |

Proposal 2: Adopt the repetition scaling values in Table 1 and Table 2. For the cases where DCI/RAR indicates R=1, there is no repetition scaling.

# Summary

In this contribution we presented our views on remaining issues for early data transmission in eMTC. We made the following proposals:

Proposal 1: For a retransmission of msg3 carrying early data transmission, the UE should use the same TBS as previous transmission (i.e., ignoring the TBS determination in DCI format 6-0), and interpret the repetition number field in the same way as RAR grant.

Proposal 2: Adopt the repetition scaling values in Table 1 and Table 2. For the cases where DCI/RAR indicates R=1, there is no repetition scaling.

# Simulation results for 1PRB

# Simulation results for 2PRBs