3GPP TSG RAN WG1 Meeting #93	R1-1807138

Busan, Korea, May 21st – 25th, 2018

Agenda Item:	7.1.3.3.7

Source:	Huawei, HiSilicon

Title:	Discussion on semi-static HARQ-ACK codebook when slot aggregation for UCI reliability

Document for:	Discussion and decision

# Introduction

In this contribution, we discuss some remaining issues on semi-static HARQ-ACK codebook determination including fallback operation and codebook determination for UEs configured with slot aggregation.

# Discussion

## Fallback operation for semi-static HARQ-ACK codebook

According to [1], if a UE reports HARQ-ACK information in a PUSCH or a PUCCH only for a PDSCH reception or SPS release scheduled by DCI format 1_0 with a DAI field value of 1 on the PCell, the UE determines a HARQ-ACK codebook only for the PDSCH or the SPS release. This is the fallback operation for semi-static HARQ-ACK codebook.

The current specification is not clear regarding how to determine the HARQ-ACK information reporting by the UE to be only for a SPS PDSCH release or only for a PDSCH reception within the K1 set. Actually, this should be based on the value of the PDSCH-to-HARQ-timing field in the corresponding DCI. Therefore, we have following proposal and the corresponding text proposal in the Appendix.

Proposal 1: The UE reporting HARQ-ACK information should be based on the value of the PDSCH-to-HARQ-timing field in the corresponding DCI.

## Fallback operation for semi-static HARQ-ACK codebook considering slot aggregation

If slot aggregation is configured, e.g. aggregationFactorDL = 4, as shown in Figure 1, there is one PDSCH transmission spanning 4 slots. According to the current specification, the UE should report 8 HARQ-ACK bits, i.e. there are many redundant bits.

A reasonable UE behavior is that the UE determines only one HARQ-ACK bit feedback for the aggregated PDSCH transmission, i.e. fallback operation should be applied.

Figure 1. Fallback operation of semi-static HARQ-ACK codebook for PDSCH with slot aggregation.

According to the above discussion, therefore we have following proposal and the corresponding text proposal in the Appendix.

Proposal 2: If a UE reports HARQ-ACK information in a PUSCH or a PUCCH only for multiple PDSCH receptions scheduled by one DCI format 1_0 with a DAI field value of 1 on PCell, the UE should determine a HARQ-ACK only for the PDSCHs.

## Semi-static HARQ-ACK codebook for PDSCH with slot aggregation

In RAN1#92bis, the semi-static HARQ-ACK codebook for PDSCH with slot aggregation was discussed and the following alternatives were listed in [1].

For PDSCH with slot aggregation

- Alt1: Keep current specifications by mapping HARQ-ACK bit for TB in each corresponding PDSCH occasion (R1-1803758, R1-1804008)

- Alt2: Keep current specifications and include that number of slots can be >1 (R1-1804381)

- Alt3: Update specifications – consider number of non-overlapped PDSCH repetitions and HARQ-ACK timing (R1-1803758, R1-1804431, R1-1804562)

- Revisit at next meeting (not enough time for discussion, first time for the issue to be discussed)

According to section 9.1.2 in [2], the semi-static HARQ-ACK codebook is determined by a set of semi-static configurations by RRC, i.e. the values of K1 set, SLIV and K0 for non-overlapping PDSCH per slot, and UL-DL TDD configuration. Accordingly, the semi-static HARQ-ACK codebook size is dependent on the following parameters: number of values in the K1 set, number of CCs, non-overlapped PDSCH per slot and number of CBGs per CC.

Table 1 Assumption for semi-static HARQ-ACK codebook

| Parameters | Value |
| --- | --- |
| K1 set | {0,1, 2, 3, 4, 8} |
| aggregationFactorDL | 4 |
| DL-UL configuration | FDD system |
| CW number per PDSCH | one |
| CGB feedback | Disable |

Alt.1 and Alt.2 basically propose to reuse the existing HARQ-ACK codebook determination for UEs configured with DL slot aggregation. Alt.3 targets to optimize the codebook size by removing some redundant bits. To ease the analysis, one example is given based on the assumptions provided in Table 1. To simply the case, it is assumed that there is only one non-overlapped PDSCH reception occasion in each slot. Assuming slot aggregation is not configured, the size of semi-static HARQ-ACK codebook is 5 bits. However, when slot aggregation is configured, i.e. aggregationFactorDL = 4, UE can not receive two overlapping PDSCH transmisions within PDSCH 0 to PDSCH 3 simultaneously. Therefore, the semi-static HARQ-ACK codebook size would include redundant bits. As shown in Figure 2, there would have three redundant bits in this case.

It should be noted that even for single CC case without CBG configuration, it is beneficial to compress the HARQ-ACK codebook size whenever possible

- The detection performance can be improved with a reduced number of HARQ-ACK payload at a given SNR. According to Alt.1, there would be 5 bits as shown in Figure 2. As discussed earlier, three redundant bits can be removed in this case, i.e. 60% reduction.

- With a reduced HARQ-ACK payload, a different PUCCH format can be used so that a better A/N detection performance can be achieved. As an example shown in Figure 3, the 2-bit HARQ-ACK can be transmitted on PUCCH format 0/1 while the 5-bit HARQ-ACKs can only be transmitted using PUCCH format 2/3/4. PUCCH format 0/1 would have the better performance than the other PUCCH format, since it uses the sequence to receive PUCCH format 0/1 instead of using modulation or channel coding in the other format.

Furthermore, the HARQ-ACK bits reduction becomes more significant when CBG or carrier aggregation is configured and/or when there is more than one PDSCH reception occasions per slot. The fundamental reason is that the value in K1 set will be not all valid when PDSCH slot aggregation is configured.

Figure 2. Semi-static HARQ-ACK codebook for PDSCH with slot aggregation (Alt.1 and Alt.2)

Based on the above analysis, the size of the semi-static HARQ-ACK codebook should be reduced in case that slot aggregation is configured. To determine semi-static HARQ-ACK codebook for UEs configured with slot aggregation, the above problem can be summarized as determining the non-overlapping PDSCH transmissions. This issue is similar to determining the non-slot transmission within one slot. Therefore, the solution can also be very similar. One possible solution is that the UE can reduce the valid entries in the configured K1 set based on the number of aggregated slots. This procedure is illustrated in the following three steps.

- Step 1: Find the smallest K1 value in the current K1 set;

- Step 2: Remove the K1 values from the K1 set, whose value is smaller than the smallest K1 value plus configured value of aggregationFactorDL;

  - The removed K1 values are associated with the same HARQ-ACK bit(s) and the HARQ-ACK bit(s) is appended to the HARQ-ACK codebook

- Step 3: Iterate Step 1 and Step 2 until the current K1 set is empty.

Figure 3. Semi-static HARQ-ACK codebook size for PDSCH with slot aggregation (Alt.3)

According to above discussion, we have following proposal and the corresponding TP in the Appendix.

Proposal 3: Adopt the following three steps to determine the semi-static HARQ-ACK codebook for UEs configured with DL slot aggregation.

- Step 1: Find the smallest K1 value in the current K1 set;

- Step 2: Remove the K1 values from the K1 set, whose value is smaller than the smallest K1 value plus configured value of aggregationFactorDL;

  - The removed K1 values are associated with the same HARQ-ACK bit(s) and the HARQ-ACK bit(s) is appended to the HARQ-ACK codebook

- Step 3: Iterate Step 1 and Step 2 until the current K1 set is empty.

# Conclusions

In this contribution, the semi-static HARQ-ACK codebook size considering PDSCH slot aggregation is discussed. The following proposals are made and the corresponding text proposals are provided in Appendix:

Proposal 1: The UE reporting HARQ-ACK information should be based on the value of the PDSCH-to-HARQ-timing field in the corresponding DCI.

Proposal 2: If a UE reports HARQ-ACK information in a PUSCH or a PUCCH only for multiple PDSCH receptions scheduled by one DCI format 1_0 with a DAI field value of 1 on PCell, the UE should determine a HARQ-ACK only for the PDSCHs.

Proposal 3: Adopt the following three steps to determine the semi-static HARQ-ACK codebook for UEs configured with DL slot aggregation.

- Step 1: Find the smallest K1 value in the current K1 set;

- Step 2: Remove the K1 values from the K1 set, whose value is smaller than the smallest K1 value plus configured value of aggregationFactorDL;

  - The removed K1 values are associated with the same HARQ-ACK bit(s) and the HARQ-ACK bit(s) is appended to the HARQ-ACK codebook

- Step 3: Iterate Step 1 and Step 2 until the current K1 set is empty.

# References

- R1-1805776, Draft CR to TS 38.213 capturing the RAN1#92bis meeting agreements,

- R1-1805719, Offline summary on CA aspects, Samsung.

# Appendix: Text Proposals

TS 38.213 in [2]

------------------------------------------ Start of Text Proposal ----------------------------------------------

### 9.1.2	Type-1 HARQ-ACK codebook determination

This subclause applies if the UE is configured with pdsch-HARQ-ACK-Codebook = semi-static.

If a UE reports HARQ-ACK information in a PUSCH or a PUCCH only for a SPS PDSCH release or only for a PDSCH reception or only for multiple PDSCH receptions configured with aggregation-Factor-DL > 1, within the occasions for candidate PDSCH receptions based on the value of the PDSCH-to-HARQ-timing-indicator field in DCI format 1_0, as determined in Subclause 9.1.2.1, that is scheduled by DCI format 1_0 with a counter downlink assignment indicator (DAI) field value of 1 on the PCell, the UE determines a HARQ-ACK codebook only for the SPS PDSCH release or only the PDSCH reception; otherwise, the following procedures for a HARQ-ACK codebook determination apply.

#### 9.1.2.1	Type-1 HARQ-ACK codebook in physical uplink control channel

For a serving cell  and the active DL BWP and the active UL BWP, as described in Subclause 12, the UE determines a set of  occasions for candidate PDSCH receptions for which the UE can transmit corresponding HARQ-ACK information in a PUCCH in slot . The determination is based:

a)	on a set of slot timing values  associated with the active UL BWP

a)	If the UE is configured to monitor PDCCH for DCI format 1_0 and is not configured to monitor PDCCH for DCI format 1_1 on serving cell ,  is provided by the slot timing values {1, 2, 3, 4, 5, 6, 7, 8} for DCI format 1_0;

b)	If the UE is configured to monitor PDCCH for DCI format 1_1 on serving cell ,  is provided by higher layer parameter dl-DataToUL-ACK for DCI format 1_1.

b)	when provided, on a set of row indexes of a table provided by higher layer parameter PDSCH-TimeDomainResourceAllocation associated with the active DL BWP and defining respective sets of slot offsets , start and length indicators SLIV, and PDSCH mapping types for PDSCH reception as described in [6, TS 38.214]; and

c)	when provided, on higher layer parameter tdd-UL-DL-ConfigurationCommon, higher layer parameter tdd-UL-DL- ConfigurationCommon2, and higher layer parameter tdd-UL-DL-ConfigDedicated as described in Subclause 11.1.

If the UE is configured with aggregationFactorDL > 1, the UE determines the total number of candidate subset from the set of slot timing values according to the following pseudo-code.

Set  - index of candidate subset

Set

Set  to the cardinality of set

Set k =0 – index of slot timing values in set

While

Set  to the index of the smallest slot timing values, among all slot timing values in set ;

while

if  where  is the k-th slot timing value and  is the m-th slot timing value in set

; - candidate subset index associated with the k-th index slot timing values

;

end if

;

end while

;

;

end while

The UE determines  occasions for candidate PDSCH receptions or SPS PDSCH releases according to the following pseudo-code.

Otherwise, for the set of slot timing values , the UE determines  occasions for candidate PDSCH receptions or SPS PDSCH releases according to the following pseudo-code.

end if

Set  - index of occasion for candidate PDSCH reception or SPS PDSCH release

Set

Set

Set  to the total number of candidate subset  if aggregationFactorDL > 1, otherwise to the cardinality of set

Set k =0 – subset index  for serving cell if aggregationFactorDL > 1, otherwise index of slot timing values  in set  for serving cell

while

Set  to the set of rows provided by PDSCH-TimeDomainResourceAllocation

Set  to the cardinality of ,

Set  – index of row provided by PDSCH-TimeDomainResourceAllocation

if slot  is after a slot for an active DL BWP change on serving cell  or an active UL BWP change on the PCell and slot  is before the slot for the active DL BWP change on serving cell  or the active UL BWP change on the PCell, or slot  is after a slot for an active DL BWP change on serving cell  or an active UL BWP change on the PCell and each slot  is before the slot for the active DL BWP change on serving cell  or the active UL BWP change on the PCell if aggregationFactorDL > 1 where  is the slot timing values in k-th candidate subset

;

else

while

if aggregationFactorDL > 1

if the UE is provided higher layer parameter tdd-UL-DL-ConfigurationCommon, or higher layer parameter tdd-UL-DL- ConfigurationCommon2, or higher layer parameter tdd-UL-DL-ConfigDedicated and at least one OFDM symbol of the PDSCH time resource derived by row  for each slot  is configured as UL where  is the slot timing values in k-th candidate subset

;

otherwise,

if the UE is provided higher layer parameter tdd-UL-DL-ConfigurationCommon, or higher layer parameter tdd-UL-DL- ConfigurationCommon2, or higher layer parameter tdd-UL-DL-ConfigDedicated and at least one OFDM symbol of the PDSCH time resource derived by row  in slot  is configured as UL where  is the k-th slot timing value in set ,

;

end if

end if

;

end while

If the UE does not indicate a capability to receive more than one unicast PDSCH per slot and,

The UE does not expect to receive SPS PDSCH release and unicast PDSCH in a same slot;

else

Set  to the cardinality of

Set  to the smallest last OFDM symbol index, as determined by the SLIV, among all rows of

while

Set

while

if  for start OFDM symbol index  for row

; - index of occasion for candidate PDSCH reception or SPS PDSCH release associated with row

;

end if

;

end while

;

Set  to the smallest last OFDM symbol index among all rows of ;

end while

end if

;

end if

end while

For rows of PDSCH-TimeDomainResourceAllocation associated with a same value of , where , the UE is not expected to receive more than one PDSCH in a same slot.

---------------------------------------- Unchanged parts omitted --------------------------------------------