3GPP TSG-RAN WG1 Meeting #93	R1-1807284

Busan, Korea, 21st - 25th May 2018

Agenda item:		7.8.2

Source:	Nokia, Nokia Shanghai Bell

Title:	Spectral Efficiency Results for the IMT-2020 Self-Evaluation

Document for:		Discussion and Decision

# 1	Introduction

In this contribution, we provide the results of system simulations for the average spectral efficiency and 5th percentile user spectral efficiency for the NR IMT-2020 self-evaluation.  We report results for all three of the eMBB test environments for both downlink and uplink.  This contribution is an updated version of R1-1805145.

# 2	Evaluation Configurations

We evaluated the average cell spectral efficiency and 5th percentile user spectral efficiency for the Indoor Hotspot – eMBB, Dense Urban – eMBB, and Rural – eMBB test environments as described in [1] for both downlink and uplink.  The configurations used for NR generally follow the guidelines given in [2] with details given in Table 4.  For the FDD downlink, the simulations used zero-forcing MU-MIMO transmission with a maximum UE rank of one, pairing up to 12 users.  The transmissions are based on Type II CSI feedback every 5ms with a feedback delay of 5ms.  For TDD downlink, the simulations used zero-forcing MU-MIMO with a maximum UE rank of one, pairing up to 8 users.  CSI is obtained from 4 port SRS channel sounding.  The SRS signals are configured using 4 combs, 4 cyclic shifts, and 4 SRS symbols per frame, yielding 64 orthogonal resources.  Since 10 users are dropped per sector an average of 40 resources are used in each sector.  Only 4 UL antenna ports are sounded in Dense Urban – eMBB with 8 Rx antennas used at the UEs.  The TDD DL/UL split is 50-50 so 4 out of 70 OFDM symbols are used for uplink sounding.  Uplink sounding is non-ideal, where the CSI is obtained using a realistic channel estimator.  For FDD uplink, the simulations used MU-MIMO transmission with a maximum UE rank of two, pairing up to 6 users.  Uplink transmission is codebook-based with CSI feedback every 10ms with a delay of 5ms.  The downlink environments are simulated with both FDD and TDD duplexing schemes; the uplink environments are simulated with FDD.  The Indoor Hotspot - eMBB environment is simulated only with one TRxP per site for a total of 12 TRxP’s.

The antenna configurations used for each environment are also given in Table 4.  A single antenna panel is used is all cases and the antenna configurations are described using (M,N,P), where M is the number of rows (elevation), N is the number of columns (azimuth), and P is the number of polarizations.  These definitions apply to both antenna elements and virtualized antenna ports.  A single antenna port configuration was used for each test environment except for the Dense Urban – eMBB FDD DL simulations, where both 4 and 8 receive antennas were simulated.

# 3	Results

The downlink (DL) FDD spectral efficiency (SE) simulation results are tabulated in Table 1, the DL TDD results are tabulated in Table 2, and the uplink (UL) FDD simulation results are in Table 3.  In all cases, channel estimation error is included, except for the Indoor Hotspot – eMBB results where channel estimation is ideal.  In addition, the UL channel estimation error model for the Rural – eMBB environment is not optimized.  Revised results will be submitted when these models have been updated.  The final indoor results will therefore be lower than reported here, but the rural results may not change significantly.  The results have been calculated assuming DL overhead of 4 OFDM symbols and UL overhead of 2 OFDM symbols, out of 14 symbols in each case.  These overhead assumptions require revision to more accurately account for NR overhead.

The results for all test environments show that NR is capable of satisfying the IMT-2020 requirements for spectral efficiency [3].  The indoor results have a margin which appears sufficient to allow for the incorporation of channel estimation error.  The Dense Urban FDD DL results meet the requirements with 8 receive antennas at the UE, but are marginal with only 4 receive antennas.  Rural results meet the IMT-2020 requirements by a wide margin with the simulated configurations.  Note that the 5th percentile user spectral efficiency does not need to be satisfied in the rural environment with an inter-site distance (ISD) of 6000m.

Observation:  NR satisfies the IMT-2020 cell average and 5th percentile user spectral efficiency requirements in all three of the eMBB test environments for FDD and TDD DL and for FDD UL.

Table 1.  DL FDD spectral efficiency results

| Test Environment | Average Cell SE |  |  | 5th Percentile User SE |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Simulated | Required | Units | Simulated | Required | Units |
| Indoor Hotspot – eMBB, 4GHz | 9.8 | 9 | bits/s/Hz/TRxP | 0.35 | 0.30 | bits/s/Hz |
| Dense Urban – eMBB, 4GHz: |  |  |  |  |  |  |
| (1,4,2) Rx antennas | 9.2 | 7.8 | bits/s/Hz/TRxP | 0.33 | 0.225 | bits/s/Hz |
| (1,2,2) Rx antennas | 7.8 | 7.8 | bits/s/Hz/TRxP | 0.23 | 0.225 | bits/s/Hz |
| Rural – eMBB, 4GHz/1732m | 9.7 | 3.3 | bits/s/Hz/TRxP | 0.269 | 0.12 | bits/s/Hz |
| Rural – eMBB, 700MHz/6000m | 5.4 | 3.3 | bits/s/Hz/TRxP | 0.179 | NA | bits/s/Hz |

Table 2.  DL TDD spectral efficiency results

| Test Environment | Average Cell SE |  |  | 5th Percentile User SE |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Simulated | Required | Units | Simulated | Required | Units |
| Indoor Hotspot – eMBB, 4GHz | 10.9 | 9 | bits/s/Hz/TRxP | 0.491 | 0.30 | bits/s/Hz |
| Dense Urban – eMBB, 4GHz: |  |  |  |  |  |  |
| (1,4,2) Rx antennas | 8.3 | 7.8 | bits/s/Hz/TRxP | 0.250 | 0.225 | bits/s/Hz |
| Rural – eMBB, 4GHz/1732m | 9.4 | 3.3 | bits/s/Hz/TRxP | 0.206 | 0.12 | bits/s/Hz |
| Rural – eMBB, 700MHz/6000m | 5.7 | 3.3 | bits/s/Hz/TRxP | 0.174 | NA | bits/s/Hz |

Table 3.  UL FDD Spectral efficiency results

| Test Environment | Average Cell SE |  |  | 5th Percentile User SE |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Simulated | Required | Units | Simulated | Required | Units |
| Indoor Hotspot – eMBB, 4GHz | 7.4 | 6.75 | bits/s/Hz/TRxP | 0.40 | 0.21 | bits/s/Hz |
| Dense Urban – eMBB, 4GHz | 6.8 | 5.4 | bits/s/Hz/TRxP | 0.28 | 0.15 | bits/s/Hz |
| Rural – eMBB, 4GHz/1732m | 9.0 | 1.6 | bits/s/Hz/TRxP | 0.124 | 0.045 | bits/s/Hz |
| Rural – eMBB, 700MHz/6000m | 4.8 | 1.6 | bits/s/Hz/TRxP | 0.120 | NA | bits/s/Hz |

# 4	Conclusions

In this contribution, we have presented system simulation results for the average spectral efficiency and 5th percentile spectral efficiency performance indicators for IMT-2020.  The observation in this contribution is:

Observation:  NR satisfies the IMT-2020 cell average and 5th percentile user spectral efficiency requirements in all three of the eMBB test environments for FDD and TDD DL and for FDD UL.

# References

- “Guidelines for evaluation of radio interface technologies for IMT-2020,” Report ITU-R M.2412-0, October 2017.

- R1-1803386, “Summary of offline discussion for IMT-2020 self evaluation,” Huawei, RAN1#92, Athens, Greece, 26 February-2 March, 2018.

- “Minimum requirements related to technical performance for IMT-2020 radio interface(s),” Report ITU-R M.2410‑0, November 2017.

# Appendix

Table 4.  Simulation Parameters

| Parameter | Value |
| --- | --- |
| Environments | - Indoor Hotspot – eMBB, Configuration A (4GHz), Channel model A, 12 TRxP’s - Dense Urban – eMBB, Configuration A (4GHz), Channel model A - Rural – eMBB, Configuration B (4GHz/1732m), Channel model A - Rural – eMBB, Configuration C (700MHz/6000m), Channel model A |
| System bandwidth | 10MHz |
| Subcarrier spacing | 15kHz |
| Duplex mode | FDD and TDD (50/50) for DL, FDD for UL |
| Traffic model | Full buffer |
| gNB transmit power | - 21 dBm - 41 dBm - 46 dBm - 46 dBm |
| UE transmit power | 23 dBm for all environments |
| gNB antenna element configuration | - (M,N,P) = (4,4,2). (dV,dH) = ( 0.5, 0.5 ) λ, 180° mechanical tilt (GCS) - (M,N,P) = (16,8,2), (dV,dH) = ( 0.8, 0.5 ) λ, 90° mechanical tilt (GCS) - (M,N,P) = (8,16,2), (dV,dH) = ( 0.8, 0.5 ) λ, 90° mechanical tilt (GCS) - (M,N,P) = (4,8,2), (dV,dH) = ( 0.8, 0.5 ) λ, 90° mechanical tilt (GCS) +45°, -45° polarization in all cases |
| gNB antenna virtualization | - (M,N,P) = (4,4,2), Electrical tilt = 90° (LCS) - (M,N,P) = (2,8,2), Electrical tilt = 100° (LCS) - (M,N,P) = (1,16,2), Electrical tilt = 100° (LCS) - (M,N,P) = (1,8,2), Electrical tilt = 96° (LCS) |
| UE Rx antenna config. | - (M,N,P) = (1,4,2), dH = 0.5 λ - (M,N,P) = (1,2,2) and (1,4,2), dH = 0.5 λ - (M,N,P) = (1,4,2), dH = 0.5 λ - (M,N,P) = (1,2,2), dH = 0.5 λ 0°, 90° polarization in all cases |
| UE Tx antenna config. | (M,N,P) = (1,2,2), dH = 0.5 λ, 0°, 90° polarization for all environments |
| UE antenna pattern | Omni |
| Receiver | MMSE with channel estimation error and interference modeling. Indoor Hotspot channel estimation was ideal. |
| DL FDD CSI feedback | CQI, PMI, RI reporting every 5ms with a delay of 5ms. Type II codebook, 4 beams, 3 phase bits, subband amplitudes |
| DL TDD CSI feedback | 4 port uplink channel sounding (SRS) every frame 4 combs, 4 cyclic shifts, 4 OFDM symbols Non-ideal CSI estimation using a realistic channel estimator |
| DL MIMO transmission scheme | MU-MIMO with zero-forcing transmission, maximum rank of 1, maximum number of paired users is 12 for FDD and 8 for TDD |
| DL scheduler | PF with frequency selective scheduling |
| UL CSI feedback | Codebook-based transmission, CSI feedback period is 10ms, CSI feedback delay is 5 ms |
| UL MIMO transmission scheme | MU-MIMO with maximum UE rank of 2, maximum number of paired users is 6 |
| UL scheduler | PF with wideband scheduling |