3GPP TSG RAN WG1 Meeting #93		                                                                           R1-1807335

Busan, Korean, May 21st – 25th, 2018

Agenda item:	7.1.1.5.2

Source: 	Qualcomm Incorporated

Title: 	Radio link monitoring consideration

Document for:	Discussion/Decision

# Introduction

This contribution discusses the remaining aspects of RLM.

# Clarification on TCI-state for RLM

RAN1 #92b made the following agreements:

Agreements: - Further clarification of RAN2 agreement (will require additional physical layer text proposals): - Working assumption: If the TCI-states refer to CSI-RS for tracking, it is up to UE to select a NZP-CSI-RS resource from the configured resources for CSI-RS for tracking for RLM - FFS on the UE behavior when TCI-states indicate a combination of SSB, CSI-RS, and CSI-RS for tracking

Proposal 1: Confirm the following working assumption

- Working assumption: If the TCI-states refer to CSI-RS for tracking, it is up to UE to select a NZP-CSI-RS resource from the configured resources for CSI-RS for tracking for RLM

Per the existing RRC structure, “referenceSignal” in “QCL-Info” of TCI-state configuration allows only one reference signal to be configured. Hence, there is no need to define UE behavior as indicated in FFS of the agreements.

Observation: Per the existing RRC structure, “referenceSignal” in TCI-state configures one reference signal only.

# C-DRX consideration

RLM in C-DRX is specified in TS 38.213. However, an open issue is on RLM reference signal configuration. It should be more sufficient if the NW transmits the reference signal (CSI-RS or SSB) when the UE is actually assessing the radio link quality i.e., the active time. Hence, we make the following proposal:

Proposal 2: A UE in C-DRX is configured with either CSI-RS or SSB within the active time for performing RLM.

# Conclusions

This contribution has provided our views on remaining details of radio link monitoring. The following observations and proposals have been made:

Observation: Per the existing RRC structure, “referenceSignal” in TCI-state configures one reference signal only.

Proposal 1: Confirm the following working assumption

- Working assumption: If the TCI-states refer to CSI-RS for tracking, it is up to UE to select a NZP-CSI-RS resource from the configured resources for CSI-RS for tracking for RLM

Proposal 2: A UE in C-DRX is configured with either CSI-RS or SSB within the active time for performing RLM.