3GPP TSG-RAN WG1#93	R1-1807608

Busan, Korea, 21st – 25th May 2018

Agenda Item:	7.1.1.2.1

Source:	Ericsson

Title:	Summary of 7.1.1.2.1 NR-PBCH

Document for:	Discussion, Decision

# Introduction

This is the summary for 7.1.1.2.1 on the remaining details on NR-PBCH signals based on the views expressed by companies in the contributions listed in the appendix and providing proposals for agreement.

# Time alignment in TDD

Huawei proposed that to avoid reading of PBCH, UE assumes that the SFN is aligned among cells of the same carrier frequency.

ZTE proposed that the UE can derive the SS block index of neighbor cell according to the serving cell timing as long as the timing offset between the two cells is smaller than 2 OFDM symbols. RAN4 should set requirements when useServingCellTimingForSync is set as true. The setting of useServingCellTimingForSync decide if the UE has to read the PBCH to derive the SS/PBCH index.

LG Electronics proposed that if CSI-RS resource for target cell (or neighbor cell) is configured, UE acquires half frame information by detection of CSI-RS sequence. Otherwise, the UE assumes that gNBs are tightly synchronized.

OPPO proposed that in a TDD band, the UE always assumes SFN alignment among cells on the same carrier frequency. In an FDD band, SFN alignment among cells on the same carrier frequency is not assumed by UE or inform to the UE whether SFN is aligned or not.

InterDigital proposed that the UE always assumes SFN alignment among cells on the same carrier frequency in TDD bands.

NTT DOCOMO proposed that at least one of either indication SFN alignment in the handover command, the UE always assume alignment in TDD, or alignment is assumed UseServingCellTimingForSync is enabled.

Nokia proposed to extend the UE assumption for ‘useServingCellTimingForSync’ (when set TRUE) to cover also SFN alignment between cells.

Feature lead comment: Discuss further during the week.

# Fraudulent use of valid SS/PBCH block indication

To capture some UE behaviour in the specification for this case was supported by ZTE, NTT DOCOMO and Nokia. Qualcomm is fine to include some text if strong desire is there. Ericsson suggested that an informative note might be used to avoid specifying exact UE behaviour. Samsung stated that this can be handled by UE implementation and no text is needed.

Feature lead comment: Discuss further during the week to settle on a text proposal.

# QCL

Lenovo and Motorola Mobility propose to correct the QCL assumptions related to SS/PBCH blocks since spatial Rx parameters are used only for above 6GHz.

Proposal:

- Make the change “…to be quasi co-located with respect to Doppler shift, Doppler spread, average delay, delay spread, and, when applicable, spatial Rx parameters.” to sections 7.4.1.1.2, 7.4.1.3.2 and 74.3.1.

# Other

These are issues raised by single companies that require further discussion.

## MIB extension

Huawei suggested clarifying in 38.212 when BCCH message type is not MIB.

Feature lead comment: Discuss further during the week, checking with RAN2 their intentions and include any backwards compatibility issues.

## RMSI CORESET Configuration with minimum bandwidth 10MHz

LG Electronics suggested including a bit in PBCH to indicate the minimum carrier bandwidth in the band. Alternatively, a 5 bit RMSI CORESET configuration table can be considered.

This topic can be discussed in the general discussion on 10 MHz minimum bandwidth in agenda item 7.1.1.2.2.

## Closest Second SS/PBCH Block

InterDigital proposed that should be possible to indicate any SS/PBCH block not only the closest. Hence, they proposed removing the word “closest” from 38.213.

# Appendix: Contributions used as basis for the summary

Additionally, some companies’ views were taken from contributions in AI 7.1.1.1(R1-1807607).

| R1-1805872 | Remaining details of NR PBCH design | Huawei, HiSilicon |
| --- | --- | --- |
| R1-1805942 | Remaining details of NR-PBCH Design | ZTE |
| R1-1806347 | Correction on QCL of SS blocks | Lenovo, Motorola Mobility |
| R1-1806420 | Remaining details of PBCH | Ericsson |
| R1-1806601 | Remaining Details on PBCH design and contents | LG Electronics |
| R1-1806700 | Corrections on NR-PBCH | Samsung |
| R1-1806873 | Remaining issues on NR-PBCH | OPPO |
| R1-1807009 | On Remaining Details of NR Physical Broadcast Signal and Channel | InterDigital, Inc. |
| R1-1807050 | Remaining issues on NR-PBCH | NTT DOCOMO, INC. |
| R1-1807171 | Remaining details related to SS/PBCH block | Nokia, Nokia Shanghai Bell |
| R1-1807329 | Remaining details on NR-PBCH | Qualcomm Incorporated |