3GPP TSG RAN WG1 Meeting #93	R1-1807748

Busan, Korea, May 21st – 25th, 2018

Source:	Ericsson

Title:	Offline Discussion on Support of Separate CQI and MCS table(s) for URLLC

Agenda Item:	7.2.1

Document for:	Discussion and Decision

# Introduction

In RAN1#92bis, the following agreements were reached.

Agreements:

- The two BLER targets that are configurable for URLLC for CSI reporting are:

  - Option B. (10-1, 10-5)

  - Note: The definition of the test case for the BLER target of 10-5 should take into account channel and interference variations and estimation errors.

Agreements:

- Highest spectral efficiency for CQI based on 10-5 BLER target for URLLC is no more than 772/1024*6

- Highest spectral efficiency for CQI based on 10-1 BLER target for URLLC is no more than 873/1024*6

  - It doesn’t necessarily mean that the CQI table introduced for eMBB can not be directly reused for URLLC – it’s still a separate discussion

- Note that

  - Whether or not to have two tables or a single table covering both BLER targets is a separate issue

Agreements:

- In total, there are two CQI tables for URLLC CQI reporting

  - The first table for URLLC CQI reporting is the same as the existing 64QAM CQI table without any change, which is for BLER target 10-1 for URLLC

    - Note: this means the agreement on “Highest spectral efficiency for CQI based on 10-1 BLER target for URLLC is no more than 873/1024*6” is overturned

  - The new table will have entries corresponding to BLER target 10-5

- For CSI reporting, the CQI field is 4-bit.

Agreements:

- For BLER 10-5,

- Companies are encouraged to perform simulations for the new CQI table for URLLC, including

  - The lowest SE entry

    - E.g., 30~50/1024*2

  - Note that the highest SE entry of no more than 772/1024*6 is already agreed

  - Consider using approximately equally spaced SNR values

    - Other options are not precluded

    - Whether or not some existing CQI entries for BLER 10-1 can be reused

      - Consider exsiting CQI entires when applicable

  - In total 15 CQI entries (+1 OOR entry)

- In performing the simulations, consider

  - Fading channel (TDL-A, 30ns) & (TDL-C, 300ns)

    - Other options are not precluded

  - Payload of 32 bytes

    - Other payload sizes can also be considered, up to each company

  - SNR at 5% geometry for the lowest SE entry

    - Other options are not precluded

  - For other simulation assumptions, refer to agreements from RAN1#92

- Similar considerations are also applicable to the MCS table evaluations

In RAN1#93, the following agreements are reached.

Agreements:

- For CQI table for URLLC with target BLER of 10^-5,

  - At least some entries for CQI table for target BLER of 10^-1 are re-used

  - The highest SE entry is 772/1024*6

  - The lowest SE entry is between (30 ~ 40)/1024*2

Agreements:

- The MCS table for CP-OFDM based PUSCH is separate from the MCS table for DFT-s-OFDM based PUSCH

- The same MCS table is used for PDSCH and CP-OFDM based PUSCH

- At least one new MCS table is introduced for URLLC

Discuss further offline – Yufei (Ericsson)

R1-1807730	Offline Discussion on Support of Separate CQI and MCS table(s) for URLLC	Ericsson

Agreements:

- The CQI table for URLLC with target BLER of 10^-5 is composed of 16 entries, including one entry of “out of range”.

Agreements:

- In CSI report, RRC parameter cqi-Table implicitly provides the BLER target for CQI reporting.

  - New 64QAM table implies BLER target =10^-5 only.

  - Existing 64QAM table and 256QAM table imply BLER target =10^-1 only.

- From RAN1 perspective, RRC parameter bler-Target is not to be used by RAN1 specification in Rel-15. Thus, from RAN1 perspective, this parameter can be removed from 38.331 in Rel-15

  - Spec update is necessary since the parameter already appears in RAN1 specs

- Send an LS to RAN2 to inform the above agreements – R1-1807743

Agreements:

- Lowest SE entry for the new CQI table is 30/1024 * 2

Agreements:

- The CQI table for URLLC with target BLER of 10^-5 is:

| CQI index | modulation | code rate x 1024 | efficiency |
| --- | --- | --- | --- |
| 0 | out of range |  |  |
| 1 | QPSK | 30 | 0.0586 |
| 2 | QPSK | 50 | 0.0977 |
| 3 | QPSK | 78 | 0.1523 |
| 4 | QPSK | 120 | 0.2344 |
| 5 | QPSK | 193 | 0.3770 |
| 6 | QPSK | 308 | 0.6016 |
| 7 | QPSK | 449 | 0.8770 |
| 8 | QPSK | 602 | 1.1758 |
| 9 | 16QAM | 378 | 1.4766 |
| 10 | 16QAM | 490 | 1.9141 |
| 11 | 16QAM | 616 | 2.4063 |
| 12 | 64QAM | 466 | 2.7305 |
| 13 | 64QAM | 567 | 3.3223 |
| 14 | 64QAM | 666 | 3.9023 |
| 15 | 64QAM | 772 | 4.5234 |

Agreements:

- For both CP-OFDM and DFT-s-OFDM, the new MCS table for URLLC has a size of 32 entries, and the MCS information field in DCI is 5-bit.

Agreements:

- For CP-OFDM, the 32-entry MCS table for URLLC includes at least 3 reserved entries for modulation order.

  - FFS if more reserved entries are needed

- For DFT-s-OFDM, the 32-entry MCS table for URLLC includes at least 4 reserved entries for modulation order (pi/2-BPSK, QPSK, 16QAM, 64QAM).

  - FFS if more reserved entries are needed

- Regarding pi/2-BPSK support for PUSCH, the same signalling and procedure are used for URLLC and eMBB.

Agreements:

- The lowest SE entry in the new MCS table is the same as the lowest SE entry of the CQI table for BLER target of 10^-5.

R1-1807742

R1-1807748

Agreements:

- For PDSCH and PUSCH with CP-OFDM, one new MCS table is introduced for URLLC:

| MCS Index | Modulation Order | Code rate R × 1024 | Spectral efficiency |
| --- | --- | --- | --- |
| 0 | 2 | 30 | 0.0586 |
| 1 | 2 | 40 | 0.0781 |
| 2 | 2 | 50 | 0.0977 |
| 3 | 2 | 64 | 0.1250 |
| 4 | 2 | 78 | 0.1523 |
| 5 | 2 | 99 | 0.1934 |
| 6 | 2 | 120 | 0.2344 |
| 7 | 2 | 157 | 0.3066 |
| 8 | 2 | 193 | 0.3770 |
| 9 | 2 | 251 | 0.4902 |
| 10 | 2 | 308 | 0.6016 |
| 11 | 2 | 379 | 0.7402 |
| 12 | 2 | 449 | 0.8770 |
| 13 | 2 | 526 | 1.0273 |
| 14 | 2 | 602 | 1.1758 |
| 15 | 4 | 340 | 1.3281 |
| 16 | 4 | 378 | 1.4766 |
| 17 | 4 | 434 | 1.6953 |
| 18 | 4 | 490 | 1.9141 |
| 19 | 4 | 553 | 2.1602 |
| 20 | 4 | 616 | 2.4063 |
| 21 | 6 | 438 | 2.5664 |
| 22 | 6 | 466 | 2.7305 |
| 23 | 6 | 517 | 3.0293 |
| 24 | 6 | 567 | 3.3223 |
| 25 | 6 | 616 | 3.6094 |
| 26 | 6 | 666 | 3.9023 |
| 27 | 6 | 719 | 4.2129 |
| 28 | 6 | 772 | 4.5234 |
| 29 | 2 | reserved |  |
| 30 | 4 |  |  |
| 31 | 6 |  |  |

Agreements:

- For PUSCH with transform precoding, one new MCS table is introduced for URLLC:

| MCS Index IMCS | Modulation Order Qm | Code rate R × 1024 | Spectral efficiency |
| --- | --- | --- | --- |
| 0 | q | 60/q | 0.0586 |
| 1 | q | 80/q | 0.0781 |
| 2 | q | 100/q | 0.0977 |
| 3 | q | 128/q | 0.1250 |
| 4 | q | 156/q | 0.1523 |
| 5 | q | 198/q | 0.1934 |
| 6 | 2 | 120 | 0.2344 |
| 7 | 2 | 157 | 0.3066 |
| 8 | 2 | 193 | 0.3770 |
| 9 | 2 | 251 | 0.4902 |
| 10 | 2 | 308 | 0.6016 |
| 11 | 2 | 379 | 0.7402 |
| 12 | 2 | 449 | 0.8770 |
| 13 | 2 | 526 | 1.0273 |
| 14 | 2 | 602 | 1.1758 |
| 15 | 2 | 679 | 1.3262 |
| 16 | 4 | 378 | 1.4766 |
| 17 | 4 | 434 | 1.6953 |
| 18 | 4 | 490 | 1.9141 |
| 19 | 4 | 553 | 2.1602 |
| 20 | 4 | 616 | 2.4063 |
| 21 | 4 | 658 | 2.5703 |
| 22 | 4 | 699 | 2.7305 |
| 23 | 4 | 772 | 3.0156 |
| 24 | 6 | 567 | 3.3223 |
| 25 | 6 | 616 | 3.6094 |
| 26 | 6 | 666 | 3.9023 |
| 27 | 6 | 772 | 4.5234 |
| 28 | q | reserved |  |
| 29 | 2 |  |  |
| 30 | 4 |  |  |
| 31 | 6 |  |  |

Agreements:

- Regarding configuration of CQI reporting and MCS table:

  - The new MCS table can be used even when the new CQI table is NOT configured

This contribution provides a summary of offline discussion on the support of separate CQI/MCS tables for URLLC.

# Discussion

## Signalling of the MCS tables

It is noted that without implementation based improvement, FAR = 44 * 2-21 = 2.1e-5.

Potential offline Proposal:

For both initial transmission and re-transmission,

- For UL configured grant, the MCS table is configured by the existing parameter associated with the RRC configured grant configuration, which is extended to include the new 64QAM MCS table.

- For DL SPS, a new RRC parameter mcs-table is added to the SPS-Config in RRC, and the mcs-table include (existing 64QAM table, existing 256QAM table, new 64QAM table).

Offline Proposal:

For grant based transmission, URLLC MCS table signalling method is select from one of the options below.

- Option A-1.

  - When the new 64QAM MCS table is configured:

    - For DCI formats 0_0/1_0 in CSS, existing 64QAM MCS table is used.

    - For DCI formats 0_0/1_0/0_1/1_1 in USS, the new 64QAM MCS table is used.

  - Otherwise, follow existing behaviour.

- Benefits:

  - False alarm rate is not affected;

  - RNTI space is not affected.

- Potential problems:

  - Due to the fact that new MCS table is used for both eMBB and URLLC, 256QAM MCS table, and the two highest SE entries of existing 64QAM table, are not available for USS when the channel condition is very good for eMBB traffic. This degrade eMBB performance when the channel condition is very good.

  - Some companies think this is a drawback: higher number of MIMO layers cannot be used with MCS entries with 256QAM, or the two highest SE entries in the existing 64QAM table.

  - Some companies have concern that this may affect blind decoding budget.

  - The 0_0/1_0 associated with unicast data using existing 64QAM table is limited to CSS.

  - During RRC reconfiguration of the relevant RRC parameter, there is an ambiguity period. The DCI is limited to CSS during the ambiguity period. This also affects MIMO efficiency during reconfiguration.

    - Some companies think this is a concern, some companies think that this is not an important scenario since RRC reconfiguration is rare.

Option A-2. Two RRC parameters are provided, indicator1-MCS-table and indicator2-MCS-table. Both indicators select from 3 MCS tables (existing 64QAM MCS table, existing 256QAM MCS table, new 64QAM MCS table).

- For DCI formats 0_0/1_0 in CSS, existing 64QAM MCS table is used.

- For DCI formats 0_0/1_0 in USS, indicator1-MCS-table is used to select the MCS table.

    - In terms of search space configuration, DCI format 0_0/1_0 may or may not be configured in USS.

- For DCI formats 0_1/1_1 in USS, indicator2-MCS-table is used to select the MCS table.

    - In terms of search space configuration, DCI format 0_1/1_1 may or may not be configured in USS.

    - If indicator2-MCS-table is absent, then for DCI format 0_1/1_1 in USS, the new MCS table is always used.

- Benefits:

  - False alarm rate is not affected;

  - RNTI space is not affected.

  - Dynamic switching of MCS tables is supported when two DCI payload sizes exist in USS;

- Potential problems:

  - During RRC reconfiguration of the two RRC parameters indicator1-MCS-table and indicator2-MCS-table, there is an ambiguity period. The DCI is limited to CSS during the ambiguity period. This also affects MIMO efficiency during reconfiguration, if the 256QAM entries, or the two highest SE entries of existing 64QAM, can be leveraged when the UE experiences very good channel condition.

    - Some companies think this is a concern, some companies think that this is not an important scenario since RRC reconfiguration is rare.

  - Some companies have concern that this may affect blind decoding budget, if two DCI sizes are configured simultaneously for USS.

  - If only one DCI size is used in USS, only one MCS table is available for both eMBB and URLLC data, as scheduled by DCI in USS. Some companies have concern that only MCS entries of a single MCS table is available.

- Option B. Introduce one RRC parameter for configuring a new RNTI.

  - When the new RNTI (via RRC) is configured, RNTI based DCI differentiation is used to choose MCS table:

    - If the DCI CRC is scrambled with the new RNTI, the new MCS table is used; otherwise, follow existing behaviour.

  - When the new RNTI is not configured, follow existing behaviour.

  - UE capability:

    - Support of the new RNTI is a bundled UE feature together with the new MCS table.

- Benefits:

  - RNTI-based switching between two MCS tables incurs very low latency in switching.

- Potential problems:

  - False alarm rate (FAR) is likely degraded. In one example (worst case), the FAR is doubled: FAR = 44*2^(-21) * 2 = 4.2 e-5. The actual FAR degradation depends on the search space configuration.

    - For all candidates that C-RNTI is monitored, new-RNTI is also monitored. For these candidates, the number of monitoring is doubled, which causes FAR degradation.

    - One solution to make the FAR problem is to reduce the number of blind decoding candidates. This applies additional constraints of search space configuration, compared to solutions when new RNTI is not monitored.

  - RNTI space is reduced.

A compromise is captured as Option C.

- Option C. Introduce one RRC parameter for configuring a new RNTI.

  - When the new RNTI is not configured, existing RRC parameter mcs-table is extended to select from 3 MCS tables (existing 64QAM MCS table, existing 256QAM MCS table, new 64QAM MCS table).

    - When mcs-table indicates the new 64QAM MCS table:

      - For DCI format 0_0/1_0 in CSS, existing 64QAM MCS table is used.

      - For DCI formats 0_0/1_0/0_1/1_1 in USS, new 64QAM MCS table is used.

    - Otherwise, follow existing behaviour.

  - When the new RNTI (via RRC) is configured, RNTI scrambling of DCI CRC is used to choose MCS table:

    - If the DCI CRC is scrambled with the new RNTI, the new 64QAM MCS table is used; otherwise, follow existing behaviour.

  - UE capability:

    - Support of the new RNTI is a stand-alone UE feature, independent of other UE features.

## Connection between the new CQI table vs. the new MCS table

Offline Recommendation:

- Support of new CQI table is: optional with UE capability signalling

- Support of new MCS table is: optional with UE capability signalling

Discussion notes:

- Regarding UE feature for supporting MCS table and CQI reporting:

  - Alternative 1: Support of new CQI table and support of new MCS table are two independent UE features.

  - Alternative 2: Support of new CQI table and support of new MCS table are bundled in a single UE feature.

## Other

From Chairman notes:

Discuss further offline regarding whether or not to address the discrepancy in the higest SE(s) in designing MCS table vs. the target packet BLER (e.g., 10^-5)

Offline discussion status:

There is no consensus that the discrepancy needs to be addressed.

# References

- R1-1805898	MCS and CQI design for URLLC	Huawei, HiSilicon

- R1-1806004	CQI and MCS Tables for URLLC	Ericsson

- R1-1806137	Considerations on CQI /MCS table(s) for URLLC	ZTE

- R1-1806529	CQI/MCS enhancements for URLLC	Intel Corporation

- R1-1806747	CQI and MCS Tables for URLLC	Samsung

- R1-1807376	Considerations for CQI and MCS for URLLC	Qualcomm Incorporated

- R1-1806077	Discussion on CSI report for URLLC	vivo

- R1-1806140	On details of CQI and MCS tables for NR URLLC	Panasonic

- R1-1806304	CQI and MCS design for NR URLLC	CATT

- R1-1806371	Discussion on CQI and MCS design for URLLC	CMCC

- R1-1806409	Consideration on CQI and MCS tables for NR URLLC	Spreadtrum Communications

- R1-1806634	Discussion on CQI and MCS table design for URLLC	LG Electronics

- R1-1806658	Remaining details of CQI and MCS for URLLC	Nokia, Nokia Shanghai Bell

- R1-1806804	On CQI and MCS design for URLLC	MediaTek Inc.

- R1-1806977	CQI/MCS Reporting for NR URLLC	AT&T

- R1-1807018	CQI and MCS Tables for NR URLLC 	InterDigital, Inc.

# Appendix. Existing CQI and MCS Tables

## Appendix A. Existing 64-QAM CQI table for CP-OFDM

Table 5.2.2.1-2: 4-bit CQI Table

| CQI index | modulation | code rate x 1024 | efficiency |
| --- | --- | --- | --- |
| 0 | out of range |  |  |
| 1 | QPSK | 78 | 0.1523 |
| 2 | QPSK | 120 | 0.2344 |
| 3 | QPSK | 193 | 0.3770 |
| 4 | QPSK | 308 | 0.6016 |
| 5 | QPSK | 449 | 0.8770 |
| 6 | QPSK | 602 | 1.1758 |
| 7 | 16QAM | 378 | 1.4766 |
| 8 | 16QAM | 490 | 1.9141 |
| 9 | 16QAM | 616 | 2.4063 |
| 10 | 64QAM | 466 | 2.7305 |
| 11 | 64QAM | 567 | 3.3223 |
| 12 | 64QAM | 666 | 3.9023 |
| 13 | 64QAM | 772 | 4.5234 |
| 14 | 64QAM | 873 | 5.1152 |
| 15 | 64QAM | 948 | 5.5547 |

## Appendix B. Existing 64-QAM MCS table for CP-OFDM

Table 5.1.3.1-1: MCS index table 1 for PDSCH

| MCS Index IMCS | Modulation Order Qm | Target code Rate R x [1024] | Spectral efficiency |
| --- | --- | --- | --- |
| 0 | 2 | 120 | 0.2344 |
| 1 | 2 | 157 | 0.3066 |
| 2 | 2 | 193 | 0.3770 |
| 3 | 2 | 251 | 0.4902 |
| 4 | 2 | 308 | 0.6016 |
| 5 | 2 | 379 | 0.7402 |
| 6 | 2 | 449 | 0.8770 |
| 7 | 2 | 526 | 1.0273 |
| 8 | 2 | 602 | 1.1758 |
| 9 | 2 | 679 | 1.3262 |
| 10 | 4 | 340 | 1.3281 |
| 11 | 4 | 378 | 1.4766 |
| 12 | 4 | 434 | 1.6953 |
| 13 | 4 | 490 | 1.9141 |
| 14 | 4 | 553 | 2.1602 |
| 15 | 4 | 616 | 2.4063 |
| 16 | 4 | 658 | 2.5703 |
| 17 | 6 | 438 | 2.5664 |
| 18 | 6 | 466 | 2.7305 |
| 19 | 6 | 517 | 3.0293 |
| 20 | 6 | 567 | 3.3223 |
| 21 | 6 | 616 | 3.6094 |
| 22 | 6 | 666 | 3.9023 |
| 23 | 6 | 719 | 4.2129 |
| 24 | 6 | 772 | 4.5234 |
| 25 | 6 | 822 | 4.8164 |
| 26 | 6 | 873 | 5.1152 |
| 27 | 6 | 910 | 5.3320 |
| 28 | 6 | 948 | 5.5547 |
| 29 | 2 | reserved |  |
| 30 | 4 | reserved |  |
| 31 | 6 | reserved |  |

## Appendix C. Existing 64-QAM MCS table for PUSCH with Transform Precoding

Table 6.1.4.1-1: MCS index table for PUSCH with transform precoding and 64QAM

| MCS Index IMCS | Modulation Order Qm | Target code Rate R x 1024 | Spectral efficiency |
| --- | --- | --- | --- |
| 0 | q | 240/ q | 0.2344 |
| 1 | q | 314/ q | 0.3066 |
| 2 | 2 | 193 | 0.3770 |
| 3 | 2 | 251 | 0.4902 |
| 4 | 2 | 308 | 0.6016 |
| 5 | 2 | 379 | 0.7402 |
| 6 | 2 | 449 | 0.8770 |
| 7 | 2 | 526 | 1.0273 |
| 8 | 2 | 602 | 1.1758 |
| 9 | 2 | 679 | 1.3262 |
| 10 | 4 | 340 | 1.3281 |
| 11 | 4 | 378 | 1.4766 |
| 12 | 4 | 434 | 1.6953 |
| 13 | 4 | 490 | 1.9141 |
| 14 | 4 | 553 | 2.1602 |
| 15 | 4 | 616 | 2.4063 |
| 16 | 4 | 658 | 2.5703 |
| 17 | 6 | 466 | 2.7305 |
| 18 | 6 | 517 | 3.0293 |
| 19 | 6 | 567 | 3.3223 |
| 20 | 6 | 616 | 3.6094 |
| 21 | 6 | 666 | 3.9023 |
| 22 | 6 | 719 | 4.2129 |
| 23 | 6 | 772 | 4.5234 |
| 24 | 6 | 822 | 4.8164 |
| 25 | 6 | 873 | 5.1152 |
| 26 | 6 | 910 | 5.3320 |
| 27 | 6 | 948 | 5.5547 |
| 28 | q | reserved |  |
| 29 | 2 | reserved |  |
| 30 | 4 | reserved |  |
| 31 | 6 | reserved |  |