3GPP TSG-RAN WG1 93	R1-1807897

Busan, Korea, 21st May – 25th May, 2018

Agenda item:		7.1.1.4.2

Source:		Qualcomm Incorporated

Title:		Summary of Remaining Details on RACH Procedure

Document for:		Discussion and Decision

# Agreements on May 21 evening

Offline agreement:

Confirm the following working assumption with updates in red:

For a TA command in MAC-CE received on slot n, the corresponding adjustment of the uplink transmission timing shall apply from the beginning of slot n+k where k-1 is equal to ceil((duration of N1 + L2 + duration of N2 + TA_max)/slot_duration) and where

- N1 refers to the value determined in control session with front loaded plus additional DMRS and UE capability #1 and is determined by the reference SCS

- N2 also refers to the value determined in control session with UE capability #1 and is determined by the reference SCS

- L2 refers to the MAC processing latency and is equal to 0.5 msec.

- TA_max is equal to the maximum timing advance value that the 12 bit TA command can provide with respect to the reference SCS. UL SCS  minimum SCS among the subcarrier spacings of all the configured BWPs in all the configured uplink carriers in a TAG and initial active UL BWP

- Note: Spec captures the value k for different reference SCS

- UE is not expected that UL transmissions in the same slot shall apply different TA values

  - If this happens, it is up to UE’s implementation.

- The reference SCS is the minimum SCS among the subcarrier spacings of all the configured BWPs in all the configured uplink carriers in a TAG, and their corresponding configured downlink carriers/BWPs.

  - Slot n, slot n+k, the time duration of (k-1) slots, and slot_duration are defined based on the reference SCS.

--------------------------------------------------------------------------------------------------

Offline agreement:

Adopt the following TP in TS 38.213.

## 7.4 Physical random access channel (Unchanged parts omitted) If the PRACH transmission from the UE is not in response to a detection of a PDCCH order by the UE which triggers non-contention based random access procedure, referenceSignalPower is provided by ss-PBCH-BlockPower or, when the UE is configured resources for a periodic CSI-RS reception, is obtained by higher layer parameter ss-PBCH-BlockPower and powerControlOffsetSS; where powerControlOffsetSS provides an offset of CSI-RS transmission power relative to SS/PBCH block transmission power [6, TS 38.214], depending on the used DL RS for RACH association and transmission. If the PRACH transmission from the UE is in response to a detection of a PDCCH order by the UE which triggers non-contention based random access procedure, referenceSignalPower is ss-PBCH-BlockPower or, when the UE is configured resources for a periodic CSI-RS reception, is obtained by higher layer parameter ss-PBCH-BlockPower and powerControlOffsetSS; where powerControlOffsetSS that provides an offset of CSI-RS transmission power relative to SS/PBCH block transmission power [6, TS 38.214], depending on the DL RS that the DMRS of the PDCCH order is quasi-collocated with as described in Subclause 10.1. The UE is provided by higher layer parameters CFRA-SSB-Resource and CFRA-CSIRS-Resource an association between PRACH preamble indexes and corresponding indexes for SS/PBCH blocks and periodic CSI-RS resource configurations. (Unchanged parts omitted)

Offline agreement:

Adopt the following TP in TS 38.213.

## 8.1 Random access preamble Physical random access procedure is triggered upon request of a PRACH transmission by higher layers or by a PDCCH order. A configuration by higher layers for a PRACH transmission includes the following: - A configuration for PRACH transmission [4, TS 38.211]. - A preamble index, a preamble subcarrier spacing, , a corresponding RA-RNTI, and a PRACH resource. A preamble is transmitted using the selected PRACH format with transmission power , as described in Subclause 7.4, on the indicated PRACH resource. UE is not expected to be configured with more than one preamble format and more than one subcarrier spacing in a cell.

Offline agreement:

Adopt the following TP in TS 38.213.

## 8.2 Random access response (Unchanged parts omitted) A UE receives the PDCCH with DCI format 1_0 with the CRC scrambled by the corresponding RA-RNTI and the corresponding PDSCH that includes the DL-SCH transport block with the same DM-RS antenna port quasi co-location properties, as described in [6, 38.214], as for a detected SS/PBCH block or a received CSI-RS the SS/PBCH block or the CSI-RS resource the UE used for RACH association and transmission. If the UE attempts to detect the PDCCH with the corresponding RA-RNTI in response to a PRACH transmission initiated by a PDCCH order which triggers non-contention based random access procedure, the UE assumes that the PDCCH and the PDCCH order have same DM-RS antenna port quasi co-location properties. (Unchanged parts omitted)

Offline agreement:

Adopt the following TP in TS 38.213.

## 5 Physical downlink shared channel related procedures ## 5.1 UE procedure for receiving the physical downlink shared channel (Unchanged parts omitted) When receiving PDSCH for Random Access Response (RAR) the UE may assume that the DM-RS port of PDSCH is quasi co-located with the SS/PBCH block or the CSI-RS resource the UE selected use for RACH association and transmission with respect to Doppler shift, Doppler spread, average delay, delay spread, spatial RX parameters when applicable. When receiving a RAR triggered by a PDCCH order which triggers non-contention based random access procedure, the UE may assume that the DM-RS port of the received PDCCH order and the PDSCH of the corresponding RAR are quasi co-located with the same SS/PBCH block or CSI-RS with respect to Doppler shift, Doppler spread, average delay, delay spread, spatial RX parameters when applicable. (Unchanged parts omitted)

Offline agreement:

Proposal 6: Adopt the following CR for 38.331 and send an LS to RAN2.

CHANGE START

-- ASN1START

-- TAG-RACH-CONFIG-COMMON-START

RACH-ConfigCommon ::= 				SEQUENCE {

rach-ConfigGeneric			RACH-ConfigGeneric,

totalNumberOfRA-Preambles			INTEGER (1..63)																		OPTIONAL,	-- Need S

ssb-perRACH-OccasionAndCB-PreamblesPerSSB	CHOICE {

oneEighth								ENUMERATED {n4,n8,n12,n16,n20,n24,n28,n32,n36,n40,n44,n48,n52,n56,n60,n64},

oneFourth								ENUMERATED {n4,n8,n12,n16,n20,n24,n28,n32,n36,n40,n44,n48,n52,n56,n60,n64},

oneHalf									ENUMERATED {n4,n8,n12,n16,n20,n24,n28,n32,n36,n40,n44,n48,n52,n56,n60,n64},

one										ENUMERATED {n4,n8,n12,n16,n20,n24,n28,n32,n36,n40,n44,n48,n52,n56,n60,n64},

two										ENUMERATED {n4,n8,n12,n16,n20,n24,n28,n32},

four									INTEGER (1..16),

eight									INTEGER (1..8),

sixteen									INTEGER (1..4)

}																														OPTIONAL,	-- Need M

groupBconfigured 					SEQUENCE {

ra-Msg3SizeGroupA					ENUMERATED { b56, b144, b208, b256, b282, b480, b640,

b800, b1000, spare7, spare6, spare5, spare4, spare3, spare2, spare1},

messagePowerOffsetGroupB			ENUMERATED { minusinfinity, dB0, dB5, dB8, dB10, dB12, dB15, dB18},

numberOfRA-PreamblesGroupA			INTEGER (1..64)

ssb-perRACH-OccasionAndnumberOfRA-PreamblesGroupAPerSSBPerRACH-Occasion	CHOICE {

oneEighth								ENUMERATED {n4,n8,n12,n16,n20,n24,n28,n32,n36,n40,n44,n48,n52,n56,n60,n64},

oneFourth								ENUMERATED {n4,n8,n12,n16,n20,n24,n28,n32,n36,n40,n44,n48,n52,n56,n60,n64},

oneHalf									ENUMERATED {n4,n8,n12,n16,n20,n24,n28,n32,n36,n40,n44,n48,n52,n56,n60,n64},

one										ENUMERATED {n4,n8,n12,n16,n20,n24,n28,n32,n36,n40,n44,n48,n52,n56,n60,n64},

two										ENUMERATED {n4,n8,n12,n16,n20,n24,n28,n32},

four									INTEGER (1..16),

eight									INTEGER (1..8),

sixteen									INTEGER (1..4)

}

}																														OPTIONAL,	-- Need R

ra-ContentionResolutionTimer			ENUMERATED { sf8, sf16, sf24, sf32, sf40, sf48, sf56, sf64},

rsrp-ThresholdSSB						RSRP-Range																		OPTIONAL,	-- Need R

rsrp-ThresholdSSB-SUL					RSRP-Range																		OPTIONAL,	-- Cond SUL

prach-RootSequenceIndex					CHOICE {

l839									INTEGER (0..837),

l139									INTEGER (0..137)

},

msg1-SubcarrierSpacing					SubcarrierSpacing,

restrictedSetConfig						ENUMERATED {unrestrictedSet, restrictedSetTypeA, restrictedSetTypeB},

msg3-transformPrecoding					ENUMERATED {enabled}															OPTIONAL,	-- Need R

...

}

CHANGE END

# Agreements on May 23

## LS on NumberOfRA-PreamblesGroupA

Reply LS uploaded in inbox with Tdoc R1-1807707.

## 1.2. LS on RACH mask index

DOCOMO has proposed to update the agreement in the following way (with change shown in red):

- PDCCH order for RACH procedure includes the following fields:

  - Random Access Preamble index – 6 bits. Indicating which Random access preamble to use in case of contention-free random access procedure, or the value 000000 in case of contention-based random access procedure

  - For CFRA only:

    - FFS BWP index. Indicating which BWP to transmit the Random access preamble on

    - SUL indicator – 1 bit. Indicating whether to transmit the Random access preamble on SUL or normal uplink carrier

    - SSB index - 6 bits are used to indicate an SSB index, which, in turn, identifies a group of RACH occasions

    - PRACH Mask index – 4 bits are used to indicate PRACH Mask index that corresponds to the RO(s) in the set of RACH occasions identified by the indicated SSB index

      - RAN1 supports RAN2’s agreements regarding the PRACH table as in R1-1805803.

      - The set of RACH occasions identified by the indicated SSB index within a cycle of SSB to RACH mapping of one association period are ordered in the following way:

        - In the order of increasing frequency multiplexed RACH occasions and then

        - In the order of increasing time multiplexed RACH occasions within a RACH slot and then

        - In the order of increasing RACH slot indices

Decision to be made:

- Should we update the agreement based on DOCOMO’s suggestion?

- Should send a reply LS back to RAN2 with the agreement?

  - Note: Agreement shows inclusion of BWP index as FFS.

  - Reply LS with the agreement (with DOCOMO’s suggestion) uploaded in inbox with Tdoc R1-1807746.

## CSI-RS based RACH

- To adopt the following TP for section 7.3 of 38.213 (with changes in red).

If the PRACH transmission from the UE is not in response to a detection of a PDCCH order by the UE, referenceSignalPower is provided by ss-PBCH-BlockPower if SS block is used for RACH association and transmission.

If the PRACH transmission from the UE is in response to a detection of a PDCCH order by the UE which triggers non-contention based random access procedure, referenceSignalPower is ss-PBCH-BlockPower or, when the UE is configured resources for a periodic CSI-RS reception, is obtained by higher layer parameter  ss-PBCH-BlockPower and powerControlOffsetSS; where powerControlOffsetSS  that provides an offset of CSI-RS transmission power relative to SS/PBCH block transmission power [6, TS 38.214], depending on the DL RS that the DMRS of the PDCCH order is quasi-collocated with as described in Subclause 10.1. The UE is provided by higher layer parameters CFRA-SSB-Resource and CFRA-CSIRS-Resource an association between PRACH preamble indexes and corresponding indexes for SS/PBCH blocks and periodic CSI-RS resource configurations.

## Number of preamble formats in a cell

- To adopt the following TP to Section 8.1 of 38.213:

A preamble is transmitted using the selected PRACH format with transmission power , as described in Subclause 7.4, on the indicated PRACH resource. UE is not expected to be configured with more than one preamble format for PRACH in a cell.

## Update typo in previous agreements:

Update the previous agreement while correcting the typo (change shown in yellow mark):

Agreements:

- To adopt the following to section 5.1 of 38.213 38.214:

When receiving PDSCH for Random Access Response (RAR) the UE may assume that the DM-RS port of PDSCH is quasi co-located with the SS/PBCH block or the CSI-RS resource the UE selected used for RACH association and transmission with respect to Doppler shift, Doppler spread, average delay, delay spread, spatial RX parameters when applicable. When receiving a RAR triggered by a PDCCH order which triggers non-contention based random access procedure, the UE may assume that the DM-RS port of the received PDCCH order and the PDSCH of the corresponding RAR are quasi co-located with the same SS/PBCH block or CSI-RS with respect to Doppler shift, Doppler spread, average delay, delay spread, spatial RX parameters when applicable.

## Msg3 size

Question:

- Should NR support both 56 bits and 72 bits MSG3 sizes?

  - Yes

- Should RAN1 consider also supporting of 72 bits MSG3 size with same coverage as for 56 bits?

  - Yes: Huawei, HiSIlicon, MediaTek, Ericsson, Lenovo, Motorola Mobility

  - No: LG Electronics

## Default search space during RACH procedure

Offline agreement:

- From a UE’s perspective, if a Msg2/3/4 search space overlaps in time with UL part and valid RACH occasions, the search space in that particular slot is invalid

Offline conclusion:

RAN1 selects a new default search space during RACH procedure.

- Supported by: Huawei, Hisilicon, ZTE, Sanechips, Ericsson, Qualcomm, LGE, Nokia, CATT

- Objected by: Samsung

Offline conclusion:

If a new default search space for RACH procedure is defined, support following properties.

- Monitoring periodicity of default search space for RACH procedure is every slot.

- If the total number of monitoring occasions for the reception of RMSI PDCCH is indicated as one per slot in PBCH, UE monitors the same set of symbols as RMSI PDCCH.

  - FFS: The set of symbols that UE monitors if the number of monitoring occasions for the reception of RMSI PDCCH is indicated as more than one per slot in PBCH

      - Supported by: Ericsson, Huawei, HiSilicon, Qualcomm, ZTE, Sanechips, LGE, Nokia, CATT

- If a search space overlaps in time with an actually transmitted SSB, the search space in that particular slot is invalid

    - Supported by: Ericsson, Qualcomm

### Different variation of search space invalidation due to SSBs.

  - If a search space overlaps in time with an actually transmitted SSB other than the SSB that got used for corresponding Msg1 transmission, the search space in that particular slot is invalid

    - Supported by: Qualcomm, Ericsson

### 1.7.2. Further discussion on default search space for multiple RMSI search locations

- If the total number of monitoring occasions for the reception of RMSI PDCCH is indicated as more than one per slot in PBCH, Msg 2/3/4 search space is down-selected from the following alternatives:

      - Alt 1: The search space set associated with SSB used for corresponding Msg1 transmission

        - Supported by: ZTE, Sanechips, Qualcomm, Ericsson

      - Alt 2: The search space with the smallest first symbol index within a slot

        - Suppored by: Huawei, HiSilicon

      - Alt 3: search space depends on RA-RNTI

        - Supported by: LGE

      - Alt 4:

        - For RMSI multiplexing pattern 1, search space is the first one if Msg1 is associated to even SSBs. Search space is the 2nd one if Msg1 is the 2nd one if Msg1 is associated to odd SSB.

        - For RMSI multiplexing pattern 2 and 3, starting symbol of the search space is 0 if Msg1 is associated to even SSBs. Starting symbol of the search space is when Msg1 is associated to odd SSBs.

          - Supported by: Nokia

## CSI-RS based handover

Offline agreement:

For CSI-RS based Msg1 transmission, network configures UE with power offset of CSI-RS resource RE, configured by CSI-RS-Resource-Mobility IE, to SS RE in handover command.

- Note: Transmit power of SSB is provided by ss-PBCH-BlockPower.

- RAN1 sends an LS (R1-1807833) to RAN2 informing this agreement.

## Text proposal related to CSI-RS based RACH

Update previous agreement with the following (change shown in yellow marks):

“

If the PRACH transmission from the UE is not in response to a detection of a PDCCH order by the UE, referenceSignalPower is provided by ss-PBCH-BlockPower if SS block is used for RACH association and transmission.

If the PRACH transmission from the UE is in response to a detection of a PDCCH order by the UE which triggers non-contention based random access procedure, referenceSignalPower is provided by ss-PBCH-BlockPower or, when the UE is configured resources for a periodic CSI-RS reception, is obtained by higher layer parameter  ss-PBCH-BlockPower and powerControlOffsetSS; where powerControlOffsetSS  that provides an offset of CSI-RS transmission power relative to SS/PBCH block transmission power [6, TS 38.214], depending on the DL RS that the DMRS of the PDCCH order is quasi-collocated with as described in Subclause 10.1. The UE is provided by higher layer parameters CFRA-SSB-Resource and CFRA-CSIRS-Resource an association between PRACH preamble indexes and corresponding indexes for SS/PBCH blocks and periodic CSI-RS resource configurations.

If the PRACH transmission from the UE is in response to a detection of a PDCCH order by the UE which triggers contention based random access procedure, referenceSignalPower is provided by ss-PBCH-BlockPower.

“

## Number of preamble formats in a cell

No need to have a CR on number of preamble formats in a cell.

## Reply LS to RAN2’s LS on ‘PRACH design table’

Reply LS is uploaded in R1-1807892.

## Search space during RACH procedure

Proposed agreement #1:

If RAN1 cannot converge to design a default search space for RACH procedure in RAN 93, support the following TP:
---------------------------------------------------------------------------

If a UE is not provided higher layer parameter pagingSearchSpace for Type2-PDCCH common search space set, the association between PDCCH monitoring occasions for Type2-PDCCH common search space set and the SS/PBCH block index are same as the association of PDCCH monitoring occasions for Type0-PDCCH common search space set as described in Subclause 13. The CCE aggregation levels and the number of PDCCH candidates per CCE aggregation level are given in Table 10.1-1.

If a UE is not provided higher layer parameter ra-SearchSpace for Type1-PDCCH common search space set, the control resource set and the association between PDCCH monitoring occasions for Type1-PDCCH common search space set and the SS/PBCH block index are same as the control resource set and the association of PDCCH monitoring occasions for Type0-PDCCH common search space set as described in Subclause 13. The CCE aggregation levels and the number of PDCCH candidates per CCE aggregation level are given in Table 10.1-1.

The UE may assume that the DM-RS antenna port associated with PDCCH receptions in the control resource set configured by pdcch-ConfigSIB1 in MasterInformationBlock and for corresponding PDSCH receptions, and the DM-RS antenna port associated with a corresponding SS/PBCH reception are quasi co-located with respect to delay spread, Doppler spread, Doppler shift, average delay, and spatial Rx parameters. The value for the DM-RS scrambling sequence initialization is the cell ID. A subcarrier spacing is provided by higher layer parameter subCarrierSpacingCommon in MasterInformationBlock.

--------------------------------------------------------------------------------------------------------------

Proposed agreement #2:

From a UE’s perspective, if a Msg 2/3/4 search space overlaps in time with an actually transmitted SSB, the search space in that particular slot is invalid.

    - Supported by: Ericsson, Qualcomm

## Reply LS to RAN2’s questions

---------------------------------------------------------

Offline working assumption

  - In single CC and intra-band CA, if the SSB/CSI-RS that UE selects for RACH association and Msg1 (re)transmission is not quasi-co-located (QCLed) - with respect to type D - with the DM-RS antenna port associated with PDCCH reception in reference search spaces, UE is not expected to decode the PDCCH and or PDSCH scrambled by an RNTI other than those associated with Type1-PDCCH common search space, only if they are it is overlapped with PDCCH or PDSCH scrambled by the RNTIs associated with Type1-PDCCH common search space in at least one symbol.

  - Reference search spaces denote the Type0-PDCCH common search space, the Type0A-PDCCH common search space, the Type2-PDCCH common search space, or Type3-PDCCH common search space.

--------------------------------------------------------------------------------------------

Offline agreement:

- Whether tci-StatesPDCCH is ignored during the RA procedure (for BFR it seems already clear) or the tci-StatePDCCH should have been in the SearchSpace (e.g. if it is meant to be used for USS only and not for CSS)?

  - While monitoring Type 1 PDCCH common search space during the RA procedure, UE can ignore the tci-StatesPDCCH of the associated CORESET, if any tci-StatesPDCCH is configured with the associated CORESET.

  - Note: In case of PDCCH ordered CFRA, UE follows the same procedure that got agreed in RAN1 92 RACH procedure session.

- Whether it is allowed to configure UEs with CSI-RS based RA and send the RAR on CORESET #0 (associated with SSBs)?

  - RAN1 is still discussing this.

--------------------------------------------------------------------------------------------------

# 2. TA command timeline

## 2.1 Company positions

Nokia:

Proposal 2: The reference SCS of slot n+k and the time duration of (k-1) slots when UL and DL have different SCSs is minimum of DL SCS and UL SCS.

Proposal 3: For a TA command in MAC-CE received on slot n, the corresponding adjustment of the uplink transmission timing shall apply from the beginning of slot

- n+6 for min(UL SCS, DL SCS)=15 kHz and min(UL SCS, DL SCS)=30 kHz

- n+9 for min(UL SCS, DL SCS)=60 kHz

- n+11 for min(UL SCS, DL SCS)=120 kHz

Mediatek:

Proposal 2: For a TA command in MAC-CE received on slot n, the corresponding adjustment of the uplink transmission timing shall apply from the beginning of slot n+k where k-1 is equal to ceil((duration of N1 + L2 + duration of N2 + TA_max)/slot_duration) and where

- N1 refers to the value determined in control session with front loaded plus additional DMRS and UE capability #1 and is determined by the reference SCS

- N2 also refers to the value determined in control session with UE capability #1 and is determined by the reference SCS

- L2 refers to the MAC processing latency and is equal to 0.5 msec.

- TA_max is equal to the maximum timing advance value that the 12 bit TA command can provide with respect to the UL SCS  reference SCS

- Note: Spec captures the value k for different reference SCS

- UE is not expected that UL transmissions in the same slot shall apply different TA values

  - If this happens, it is up to UE’s implementation.

- The reference SCS is the minimum SCS among the subcarrier spacings of all uplink carriers in a TAG and their corresponding downlink carriers.

  - Slot n, slot n+k, the time duration of (k-1) slots, and slot_duration are based on the reference SCS.

Huawei:

Confirm the working assumption with the following updates:

If UE is configured with multiple active UL BWPs within one TAG:

- The smallest UL SCS within the TAG is the reference UL SCS to determine N1 and N2

- TA_max is equal to the maximum timing advance value that the 12 bit TA command can provide with respect to the smallest SCS of Msg3 SCSs configured in RMSI and UL SCSs within the TAG

- The reference SCS of the time duration of (k-1) slots is the smallest UL SCS

CATT:

Proposal 1: When a TA command is received, if UL and DL have different SCSs, the reference SCS for the corresponding adjustment of the uplink transmission timing after receiving the TA command should be µ= min(µDL, µUL), where µDL corresponds to the subcarrier spacing of the downlink transmission and µUL the subcarrier spacing of the uplink ttransmission.

Qualcomm:

Confirm previous working assumption.

## 2.2 Suggested agreement

Offline agreement:

Confirm the following working assumption with updates in red:

For a TA command in MAC-CE received on slot n, the corresponding adjustment of the uplink transmission timing shall apply from the beginning of slot n+k where k-1 is equal to ceil((duration of N1 + L2 + duration of N2 + TA_max)/slot_duration) and where

- N1 refers to the value determined in control session with front loaded plus additional DMRS and UE capability #1 and is determined by the reference SCS

- N2 also refers to the value determined in control session with UE capability #1 and is determined by the reference SCS

- L2 refers to the MAC processing latency and is equal to 0.5 msec.

- TA_max is equal to the maximum timing advance value that the 12 bit TA command can provide with respect to the reference SCS. UL SCS  minimum SCS among the subcarrier spacings of all the configured BWPs in all the configured uplink carriers in a TAG and initial active UL BWP

- Note: Spec captures the value k for different reference SCS

- UE is not expected that UL transmissions in the same slot shall apply different TA values

  - If this happens, it is up to UE’s implementation.

- The reference SCS is the minimum SCS among the subcarrier spacings of all the configured BWPs in all the configured uplink carriers in a TAG, their corresponding configured downlink carriers/BWPs.

  - Slot n, slot n+k, the time duration of (k-1) slots, and slot_duration are defined based on the reference SCS.

# 3. RACH procedure timeline

Qualcomm:

Confirm the working assumption of the following agreement.

- For PDCCH ordered CFRA, the minimum timing gap between PDCCH order reception and Msg1 transmission is

  - N2+BWPSwitching+ Delay,

    - If BWP switching is not required, BWPSwitching=0; otherwise, BWPSwitching is up to RAN4

    - N2 refers to the UE processing time value determined in the HARQ/scheduling session with front loaded plus additional DMRS and UE capability #1

    - N2 is dependent on the SCS of Msg1

      - Working assumption: For Msg1 with 1.25 kHz or 5 kHz SCS, calculation of N2 uses 15 kHz SCS.

    - Delay includes at least MAC layer delay in initializing PRACH

      - Delay = 250 us for FR2, Delay = 500 us for FR1

## 3.1 Suggested agreement

Confirm above working assumption.

# 4. Occasions/mask index in PDCCH ordered RACH

## 4.1. Company positions

Huawei, HiSilicon

Proposal 3: The following contents should be included in the PDCCH order and inform RAN2.

- Random Access Preamble index – 6 bits. Indicating which Random access preamble to use in case of contention-free random access procedure, or the value 000000 in case of contention-based random access procedure

- BWP index. Indicating which BWP to transmit the Random access preamble on

- SUL indicator – 1 bit. Indicating whether to transmit the Random access preamble on SUL or normal uplink carrier

- For CFRA only:

  - SSB index - 6 bits are used to indicate an SSB index, which, in turn, identifies a group of RACH occasions

  - PRACH mask index - 4 bits are used indicates 16 indices in the PRACH table. Within the table: (a) one index for all the RACH-Occasions; (b) 8 indices for 8 RACH-Occasions; (c) 1 index for the even number of RACH-Occasions and 1 index for the odd number of RACH-Occasions

Ericsson

Proposal 8	RAN1 supports RAN2’s agreements on the design of PRACH table and on increasing the number of bits from 3 to 4 to indicate the RACH occasions for PRACH transmission.

Intel

Proposal 3:

- For PDCCH-triggered CFRA procedure

  - Change the name “relative PRACH occasion index” to “PRACH mask index”

  - Increase the number of bits to indicate the “PRACH mask index” from 3 bits to 4

LGE

Proposal 1:

- Define a RO indexing rule for PRACH mask.

  - Within the SSB to RO association pattern period (e.g. maximum 160ms), the number of available ROs for the indicated SSB index is counted.

  - The index (i.e. 0 to 7) is cyclically mapped from the first RO to the final RO.

  - One RO group is composed by logically consecutive eight ROs.

  - The indicated RO index is applied for all RO groups.

Proposal 4:  update the agreements

    - RACH occasion index - 3 bits are used to indicate the relative RACH occasion index that corresponds to the indicated SSB index identified group of RACH occasions

to be

    - PRACH Mask index - 4 bits are used to indicate the PRACH mask index that corresponds to the RO(s) in the set of RACH occasions identified by the indicated SSB index.

Sharp

- RAN1 should clarify the definition of RACH occasion index in PDCCH order

  - RACH occasion indices should be mapped with enough timing flexibility

DOCOMO

Proposal 1: For PDCCH order RACH, indexing order for 3 bits indication of relative time/frequency location of ROs for each SSB is

-		first, in increasing order of frequency resource indexes for frequency multiplexed PRACH occasions,

-		second, in increasing order of time resource indexes for time multiplexed PRACH occasions within a PRACH slot,

-		third, in increasing order of indexes for PRACH slots

## 4.2 Suggested agreement

Update the previous RAN1 agreement in the following way:

Agreements:

- PDCCH order for RACH procedure includes the following fields:

  - Random Access Preamble index – 6 bits. Indicating which Random access preamble to use in case of contention-free random access procedure, or the value 000000 in case of contention-based random access procedure

  - For CFRA only:

    - FFS BWP index. Indicating which BWP to transmit the Random access preamble on

    - SUL indicator – 1 bit. Indicating whether to transmit the Random access preamble on SUL or normal uplink carrier

    - SSB index - 6 bits are used to indicate an SSB index, which, in turn, identifies a group of RACH occasions

    - PRACH occasion Mask index – 3 4 bits are used to indicate the relative RACH occasion PRACH mask index that corresponds to the RO(s) in the set of RACH occasions identified by the indicated SSB index the indicated SSB index identified group of RACH occasions

      - RAN1 supports RAN2’s agreements on the design of PRACH mask index to indicate the RO(s) for PRACH transmission.

      - RACH occasions identified by the indicated SSB index are ordered in the following way:

        - In the order of increasing the number of frequency multiplexed RACH occasions and then

        - In the order of increasing the number of time multiplexed RACH occasions within a RACH slot

        - In the order of increasing the number of RACH slots

# 5. Default Search Space during RACH Procedure

## 5.1 Company positions

Huawei, Hisilicon

Proposal 1: Default search space for RACH procedures is the search space for RMSI plus reusing the search space with first symbol index within a slot from RMSI search space to other slots without RMSI search space.

ZTE, Sanechips

Proposal 2: The UE monitors Msg2 PDCCH in every slot in the RAR window based on the RMSI-PDCCH-Config in MIB. If there are multiple search spaces sets in a slot (associated with different SSBs for RMSI reception), then the UE only needs to monitor one of them (based on the search space set associated with SSB used for corresponding Msg1 transmission).

Ericsson

Proposal 1	A new default search space for RACH procedure should be designed.

Proposal 2	The default search space for the RACH procedure and type0-PDCCH CSS share the same configuration except having different periodicities. The monitoring periodicity for the new default search space for the RACH procedure is one slot.

Proposal 3	UE should assume that the SSB overlapped with default search space is not transmitted when monitoring the new default search space for RACH procedure if the actually transmitted SSB is not known. If SIB1 has already been decoded and the UE knows that the SSB is transmitted based on the SSB transmission bit map, then, the UE shall not monitor the default search space overlapped with this SSB in that slot.

Vivo

Proposal 2: It is preferred to associate RACH search space with ROs instead of SSB candidates.

Proposal 3: Considering the limited time budget of Rel-15 NR work item, no new default RACH search space set should be introduced at current stage. The NW should guarantee proper configurations of search space and PRACH resource.

Proposal 4: UE assumes that only the RACH PDCCH monitoring occasions within the corresponding monitoring window are valid and is not expected to receive RACH PDCCH outside the monitoring window:

- For Msg2, the monitoring window is the corresponding RAR window.

- For Msg3 retransmission and Msg4 transmission, it depends on the Timer of contention resolution, e.g., ra-ContentionResolutionTimer.

Proposal 5: For RACH search space configured by high layer, the periodicity should be no longer than the configured PRACH configuration period, otherwise it should be considered as an error case.

LGE

- UE monitors every slot within the monitoring window for the reception of RACH MSG2/MSG3 retransmission/ MSG4 scheduling DCI.

- If the number of monitoring occasions for the reception of system information(e.g., RMSI) is indicated as one in a slot in PBCH, all UEs in the system monitor PDCCH monitoring occasion starting from the first symbol in each slot during the monitoring window.

- If the number of monitoring occasions for the reception of system information (e.g., RMSI) is indicated as two in a slot in PBCH.

  - Part of UEs monitor the PDCCH in the monitoring occasion starting from the first symbol in each slot within the monitoring window

  - Rest of UEs monitor the PDCCH in the monitoring occasion in the middle of each slot within the monitoring window

  - The value of RA-RNTI (e.g. even or odd) can be the reference of determining the position of monitoring occasion for each UE.

Samsung

Proposal 8: no need for new default search space for RACH procedure.

Proposal 9: it’s up to gNB implementation whether separately configure the RACH search space in RMSI.

Nokia

Observation 4: The network should be able to transmit RAR in any of slots within the RAR window, i.e. Alt B – Alt 1 is not feasible as such.

Proposal 4: For RMSI and SSB multiplexing pattern 1, the UE monitors PDCCH for RAR in all downlink slots within a RAR window in the first CORESET in case of Msg1 is associated to even SS/PBCH block index or even CSI-RS index (when PRACH preamble is associated to CSI-RS like in handover), and in the second CORESET in case of Msg1 is associated to odd SS/PBCH block index or odd CSI-RS index.

Proposal 5: For RMSI and SSB multiplexing patterns 2 and 3, the UE monitors PDCCH for RAR in all downlink slots within a RAR window and the starting symbol of the search space is given by

- 0, if SSB index or CSI-RS index is even to which PRACH preamble is associated

- , if SSB index or CSI-RS index is odd to which PRACH preamble is associated

where  is defined by pdcch-ConfigSIB1.

DOCOMO

UE monitors Type-1 PDCCH common search space only on duration between start of RAR window and end of random access procedure, i.e., duration between Msg2 and Msg4.

Lenovo

Proposal 1: UE monitors a search space configured via the RRC parameter ‘ra-SearchSpace’ within a RAR window, only if the RAR window does not include any PDCCH monitoring occasion for the selected SS/PBCH block i based on the Type0-PDCCH common search space.

Proposal 2: gNB can indicate in a RAR message which search space (Type0-PDCCH common search space vs. the search space configured by the RRC parameter ‘ra-SearchSpace’) UE has to monitor for Msg3 re-transmission and Msg4 reception.

Qualcomm

Default search space of Msg 2/3/4 uses the first symbol index within a slot from RMSI search space, configured in MIB, but applies it in every slot

- In case there are multiple search spaces sets in a slot, (corresponding to different SSBs), a UE only monitors the search space set associated with SSB used for corresponding Msg1 transmission

- If a search space overlaps in time with an actually transmitted SSB other than the SSB used for corresponding Msg1 transmission or with UL part, the search space in that particular slot becomes invalid.

## 5.2 Suggested agreement

RAN1 selects a new default search space during RACH procedure.

- Supported by: Huawei, Hisilicon, ZTE, Sanechips, Ericsson, Qualcomm, LGE, Nokia

- Not supported by: Samsung, Vivo, Docomo

Default search space for RACH procedure:

## 5.3 Discussion on May 23 night

Offline agreement:

- From a UE’s perspective, if a Msg2/3/4 search space overlaps in time with UL part and valid RACH occasions, the search space in that particular slot is invalid

Offline proposal:

RAN1 selects a new default search space during RACH procedure.

- Supported by: Huawei, Hisilicon, ZTE, Sanechips, Ericsson, Qualcomm, LGE, Nokia, CATT

- Objected by: Samsung, Docomo

If a Default search space for RACH procedure is defined, support following properties.

- Monitoring periodicity of default search space for RACH procedure is every slot.

  - Supported by all companies that support redesigning default search space for RACH procedure.

  - A) If a search space overlaps in time with an actually transmitted SSB, the search space in that particular slot is invalid

    - Supported by: Ericsson, Qualcomm

    - Not supported by:

    - Objected by:

  - B) If a search space overlaps in time with an actually transmitted SSB other than the SSB that got used for corresponding Msg1 transmission, the search space in that particular slot is invalid

    - Supported by: Qualcomm, [Ericsson],

    - Not supported by:

    - Objected by:

- Following alternatives are considered while re-designing default search space of Msg 2/3/4:

  - If the total number of monitoring occasions for the reception of system information (e.g., RMSI) is indicated as one in a slot in PBCH,

    - UE monitors the same search space symbols as RMSI.

      - Supported by: Ericsson, Huawei, HiSilicon, Qualcomm, ZTE, Sanechips, LGE, [Nokia]

  - If the number of monitoring occasions for the reception of system information(e.g., RMSI) is indicated as more than one in a slot in PBCH, UE monitors,

      - Alt 1: The search space set associated with SSB used for corresponding Msg1 transmission

        - Supported by: ZTE, Sanechips, Qualcomm, Ericsson

      - Alt 2: The search space with the smallest first symbol index within a slot

        - Suppored by: Huawei, HiSilicon

      - Alt 3: search space depends on RA-RNTI

        - Supported by: LGE

      - Alt 4:

        - For pattern 1, search space is the first one if Msg1 is associated to even SSBs. Search space is the 2nd one if Msg1 is the 2nd one if Msg1 is associated to odd SSB.

        - For pattern 2 and 3, starting symbol of the search space is 0 if Msg1 is associated to even SSBs. Starting symbol of the search space is when Msg1 is associated to odd SSBs.

          - Supported by: Nokia

# 6. Simultaneous transmission of uplink signals

## 6.1 Company positions

Intel

Proposal 1:

- UE does not simultaneously transmit PRACH and PUSCH/PUCCH/SRS in inter-band CA, except

  - PRACH in FR1 and PUSCH/PUCCH/SRS in FR2, or vice versa

Samsung:

Proposal 5: Transmission of PRACH or PUSCH, if any, is up to UE implementation.

Proposal 6: Transmission of PRACH is prioritized over transmission of SRS.

Nokia:

Observation 1: UE capability to transmit simultaneously may be per band combination.

Proposal 1: Define dropping and/or power scaling rules for the case the UE (capable of transmitting simultaneously on different carriers in inter-band CA case) is configured to transmit simultaneously PRACH and PUSCH/PUCCH/SRS:

1)	When PRACH and SRS are configured to be transmitted simultaneously drop SRS if the total transmit power would exceed the max allowed transmit power value

2)	When PRACH and PUCCH/PUSCH are configured to be transmitted simultaneously scale transmission power of PUCCH/PUSCH so that the total transmit power does not exceed the max allowed transmit power value

## 6.2 Suggested action

Discuss further, possibly jointly with other uplink sessions.

# 7. Msg3 TBS and Slot Aggregation

## 7.1 Company positions

Huawei

Observation 1: For MSG3 enhancement, gNB can indicate repetition in MSG2 RAR or MSG3 retransmission DCI. In case the MSG3 OFDM symbols collided with DL/X part or RMSI CORESET or SSB, UE can skip the slot and continue the repetition at the next available UL slot.

Ericsson

Proposal 5	To support 9 bytes Msg3 payload with same coverage as 7 bytes Msg3 payload while avoiding the latency suffered due to HARQ retransmissions, NR should support repetition of Msg3.

Proposal 6	Due to time constraints to finalize Rel-15, we propose that similar structure as in normal PUSCH be used for Msg3 repetitions, wherever possible.

Proposal 7	2 of the 3 reserved bits of RAR can indicate the aggregationFactor of {2,4,8} to indicate to the UE how many repetitions UE should use.

Mediatek

Proposal 5: One or two bits of the reserved bits in RAR can be used to indicate the slot aggregation level for Msg3 transmission.

Lenovo

Proposal 4: NR supports Msg3 PUSCH aggregation based on the TBS of Msg3.

Proposal 5: A UE in RRC CONNECTED state performing a contention-based random access procedure should ignore UE-specifically and BWP specifically configured PUSCH aggregation factor and transmit Msg3 PUSCH based on the pre-defined or cell-specifically configured aggregation factor.

Qualcomm:

NR supports network configuring Msg3 PUSCH aggregation via Msg2 RAR or Msg3 retransmission DCI based on TBS of Msg3.

- When Msg3 PUSCH aggregation is configured, UE repeats Msg3 in consecutive slots.

- UE assumes that the configured consecutive slots for Msg3 PUSCH aggregation do not conflict with any DL transmission.

- Note: Msg3 PUSCH aggregation can only be configured for Msg3 size over 56 bits.

## 7.2 Suggested action

“RAN2 therefore kindly ask whether RAN1 can consider support of MSG3 size as follow:

RRC Connection Request: 56 bits

RRC Connection Resume Request: 72 bits. “

Issue 1:

Does NR support Msg3 size over 56 bits?

Issue 2:

Does NR support Msg3 slot aggregation?

Issue 3:

Whether/how to handle when Msg3 collides with DL transmission?

# 8. Additional details of CORESET, Search Space and TCI states during connected mode RACH

ZTE

Proposal 5: As in LTE, CBRA following PDCCH order with preamble index=0 follows the normal 4-step CBRA procedure, as configured in SIB1.

- The same CORESET and Type1-PDCCH C-SS as for initial access is used.

Mediatek

Proposal 7: When UE conducts CBRA in RRC connected mode, UE selects RACH preamble indices/resources that are associated with the downlink reference signal that QCL’ed with the DM-RS antenna port(s) associated with PDCCH reception in common search space(s) where UE is expected to monitor the random access response.

Proposal 8: If the SSB that UE selects for RACH association and Msg1 (re)transmission is not quasi-co-located (QCLed) with the DM-RS antenna port associated with PDCCH reception in the Type0-PDCCH common search space, the Type0A-PDCCH common search space, the Type2-PDCCH common search space, or Type3-PDCCH common search space, UE is not expected to decode PDSCH indicated by a PDCCH scrambled by an RNTI other than RA-RNTI if they are assigned in the same time instance.

Suggested action: Discuss above proposals.

# 9. Additional details of SSB to RO mapping

## 9.1 Misalignment of SSB to RO mapping between connected mode and handover scenario

### 9.1.1. Company positions

Vivo:

Observation 1: Different SSB-RO associations will be assumed between UEs in different states if connected UE uses the SSB indicated in RRC to establish SSB-RO association.

Observation 2: One simple way to avoid such misaligned assumptions is that all UEs assume SSB-RO association based on the actually transmitted SSB indicated by RMSI.

Observation 3: Based on the current 38.331, UE cannot obtain the compressed SSB position indication of the target cell/Scell/Pscell through HO/Scell/PScell configuration; therefore, it may still have a different understanding of SSB-RO association with other UEs.

Proposal 1: To address the misaligned SSB-RO association issue during HO/Scell/PScell configuration, RAN1 should consider introducing inOneGroup and groupPresence into ssb-PositionsInBurst IE in RRC.

Qualcomm:

Proposal 4: For CFRA during handover scenarios, NR considers allowing network to configure a different set of actually transmitted SSBs to UEs.

- This can be achieved by including ssb-PositionsInBurst IE within RACH-configGeneric IE.

### 9.1.2. Suggestion action

Discuss above proposals

## 9.2 Updating the placement of CBRA preambles in RACH occasion

ZTE:

Proposal 7: Preambles for on-demand OSI request can be allocated in some PRACH occasions used for CBRA (for initial access). A PRACH occasion/preamble per SIB(s) is allocated for each actually transmitted SSB using one unique preamble index (similar to ra-PreambleIndex). The time instance in which the SIB(s) can be requested is up to RAN2.

Samsung:

Proposal 1:　Update the agreement to be:

- For the case that N SSBs are associated with one RACH occasion, where N >= 1, the subset of cb-preamblePerSSB consecutive CBRA preambles associated with SSB i (i=0, …, N-1) starts from preamble index i* floor(totalNumberOfRA-Preambles /N).

- totalNumberOfRA-Preambles = 40

- 4 SSBs per RO. 8 CBRA preambles per SSB

- Previous assumption: 1:8, 17:24, 33:40, 49:56

- New proposal: 1:8, 11:18, 21:28, 31:38

Discuss above proposals.

# 10. Msg1 power control during CSI-RS based PDCCH order and handover

Offline agreement:

Adopt the following TP in TS 38.213.

## 7.4 Physical random access channel (Unchanged parts omitted) If the PRACH transmission from the UE is not in response to a detection of a PDCCH order by the UE which triggers non-contention based random access procedure, referenceSignalPower is provided by ss-PBCH-BlockPower or, when the UE is configured resources for a periodic CSI-RS reception, is obtained by higher layer parameter ss-PBCH-BlockPower and powerControlOffsetSS; where powerControlOffsetSS provides an offset of CSI-RS transmission power relative to SS/PBCH block transmission power [6, TS 38.214], depending on the used DL RS for RACH association and transmission. If the PRACH transmission from the UE is in response to a detection of a PDCCH order by the UE which triggers non-contention based random access procedure, referenceSignalPower is ss-PBCH-BlockPower or, when the UE is configured resources for a periodic CSI-RS reception, is obtained by higher layer parameter ss-PBCH-BlockPower and powerControlOffsetSS; where powerControlOffsetSS that provides an offset of CSI-RS transmission power relative to SS/PBCH block transmission power [6, TS 38.214], depending on the DL RS that the DMRS of the PDCCH order is quasi-collocated with as described in Subclause 10.1. The UE is provided by higher layer parameters CFRA-SSB-Resource and CFRA-CSIRS-Resource an association between PRACH preamble indexes and corresponding indexes for SS/PBCH blocks and periodic CSI-RS resource configurations. (Unchanged parts omitted)

Intel:

Proposal 5:

- Reference transmit power information for CSI-RS has to be given to UE for enabling open loop power control of PRACH when the PRACH is associated with CSI-RS during handover

Discuss above proposals.

# 11. Other Proposals

Motorola:

Proposal 3: UE shall not perform soft combining of PDSCHs scrambled by the same RA-RNTI but associated with different SSBs or CSI-RS resources.

ZTE

Proposal 1: The number of SSBs per RACH occasion = min(SSB-per-rach-occasion, the number of actually transmitted\ SSBs).

Proposal 3: It may be necessary to explicitly configure the C-SS for Msg2 PDCCH reception in SIB1 besides the default configuration for UE monitors Msg2 PDCCH in every slot .

Proposal 4: UE may assume that the DMRS of PDCCH and the DMRS of PDSCH conveying Msg2 are QCL'ed with the SS block that the UE selected for RACH association and transmission, regardless of the TCI state configuration in ra-ControlResourceSet, if any.

CATT:

Proposal 2: If there is a change of the subcarrier spacing in either UL or DL or both, the slot for uplink transmission adjustment after change of the subcarrier spacing can be determined with one of the following alternatives:

- the TA command still takes effective at the slot n+k, where the k is determined when the TA command is received even though the duration of the slot is changed due to the change of thesubcarrier spacing;

- the TA command takes effective at the slot n+k’, where the k’ is the k scaled with the radio of  the subcarrier spacing changes;

LGE:

Proposal 2:

- All of the messages involved in the PDCCH transmission such as MSG2/MSG3 retransmission/MSG4 scheduling share the same CORESET during the RACH procedure.

Proposal 3:

- Monitoring window(s) for the reception of DCIs scheduling MSG2/ MSG3 retransmission/MSG4 is configured. The length of monitoring window for each message reception can be the same.

Mediatek

Proposal 3: UE is not expected to receive a DCI carrying a PDCCH order in OFDM symbols other than the first 3 OFDM symbols of a slot.

Proposal 4: NR selects one of the following proposals:

- Alt. 1: PDCCH order does not include a BWP index.

- Alt. 2: PDCCH order includes a BWP index for CFRA.

  - The number of bits for the BWP index is 2. Proposal 6: NR should discuss how UE selects RACH preamble indices/resources for contention-based random access in RRC_CONNECTED mode.

Samsung:

Observation 2: Msg3 PUSCH time domain resource assignment in RAR as provided in 6.1.2.1.1 of R1-1805796 [4] captures the intended behaviour

Proposal 2: update the Table 8.2-1 as following:

Table 8.2-1: Random Access Response Grant Content field size

| RAR grant field | Number of bits |
| --- | --- |
| Frequency hopping flag | 1 |
| Msg3 PUSCH frequency resource allocation | 12 |
| Msg3 PUSCH time resource allocation | 4 |
| MCS | 4 |
| TPC command for Msg3 PUSCH | 3 |
| CSI request | 1 |

Proposal 3: the complete of CFRA procedure could be based on detecting PDCCH addressed to C-RNTI.

Vivo

Proposal 5: For RACH search space configured by high layer, the periodicity should be no longer than the configured PRACH configuration period, otherwise it should be considered as an error case.

Proposal 6: It should be clarified that whether the agreement “UE can assume that DMRS of both the received PDCCH order and the PDCCH of the corresponding Msg2 are QCLed with the same SSB/CSI-RS” can be applied in the following two cases:

- Where the CORESETs of PDCCH order and RAR receptions are different.

- Where the DL BWPs of PDCCH order and RAR receptions are different.

Ericsson

Proposal 4	This new default search space is proposed to be used for the RACH procedure, Paging, OSI.

Intel

Proposal 2:

- If a single SSB is configured in a cell,

  - UE chooses the earlist RO for the transmission of PRACH.

- Otherwise,

  - UE chooses the earliest set of ROs associated with the chosen SSB

    - The set includes ROs of the number of 1 over ssb-perRACH-OccasionAndCB-PreamblesPerSSB

    - Inside the set, ROs are consecutive in either time domain or frequency domain or both time and frequency domains

  - UE randomly chooses one RO amongst the chosen set of ROs with the equal probability

- Send an LS to RAN2 about RAN1 decision

Proposal 3:

- For PDCCH-triggered CFRA procedure

  - Change the name “relative PRACH occasion index” to “PRACH mask index”

  - Increase the number of bits to indicate the “PRACH mask index” from 3 bits to 4

Proposal 4:

- For PDCCH-triggered CFRA procedure

  - CSI-RS resource index can be indicated for CSI-RS associated PRACH

    - 7 bits are used to indicate the CSI-RS resource index to support maxRA-CSIRS-Resources which is 96 as defined in 38.331 [3]

    - 1 additional bit may be needed to indicate whether the PDCCH order is for CSI-RS associated PRACH or SSB associated PRACH

QUALCOMM:

Proposal 5: In Rel-15, UE is not required to decode paging PDSCH during RACH procedure.

Proposal 7: For a PDSCH scheduled by a DCI whose CRC is scrambled by RA-RNTI, a scaling factor can be applied to N_info in TBS calculation (Subclause 5.1.3.2 in 38.214). The scaling factor can be signalled via a subset of the reserved fields. Recommended scaling factors to be signalled are {1, 1/2, 1/4, 1/8}.

Proposal 8: For Msg3 Transmission scheduled by the UL grant carried by RAR, a scaling factor can be applied to N_info in TBS calculation (Subclause 5.1.3.2 in 38.214). The scaling factor can be signalled via a subset of the reserved fields in the UL grant. Recommended scaling factors to be signalled are {1, 1/2,1/4, 1/8}.

# 12. The power ramping suspension notification for power prioritization

The power ramping suspension notification for UL beam switch is captured in Section 7.4 in TS 38.213.  RAN2 has agreed that the power ramping suspension notification can be reused for the lower prioritized PRACH transmission in dual-connectivity case as in LTE.

RAN2 #101: - The power ramping suspension notification defined in NR for UL beam switch case can be reused for the lower prioritized PRACH transmission in dual-connectivity case

However, RAN1 specification does not capture above case, and the condition on indication of the power ramping suspension should be captured in RAN1 specification as in LTE. In addition, in NR, power prioritization including CA and SUL, and power sharing for dual connectivity in TS38.213 7.5 and 7.6. Then the power ramping suspension notification can be reused for these cases.

For information, in LTE, the related sentence in TS36.213 5.1.4 is the following.

“If the UE drops the PRACH transmission, it sends power ramping suspension indicator to the higher layers. If the UE adjusts the power of PRACH transmission, it may send power ramping suspension indicator to the higher layers.”

Proposal from DOCOMO:

For Prioritizations for transmission power reductions and power sharing for dual connectivity,

- if the UE drops the PRACH transmission, it sends power ramping suspension indicator to the higher layer, and. If the UE adjusts the power of PRACH transmission, it may send power ramping suspension indicator to the higher layers.

# 13. CRs from Different Companies

## 12.1 Vivo

Observation 10: According to 38.331, the high layer signaling is able to configure different PRACH formats and PRACH resources for different UL BWPs.

Proposal 7. As preamble format is a cell-specific parameter, following solutions can be taken into consideration:

- Opt1.Send an LS to RAN2 to ask for the possibility of revising the structure of RACH configuration to be cell-specific.

- Opt2.Adopt the following TP in 38.213.

If Opt.2 is agreed, the following TP is suggested for clarification in 38.213.

Offline agreement:

## 8.1 Random access preamble Physical random access procedure is triggered upon request of a PRACH transmission by higher layers or by a PDCCH order. A configuration by higher layers for a PRACH transmission includes the following: - A configuration for PRACH transmission [4, TS 38.211]. - A preamble index, a preamble subcarrier spacing, , a corresponding RA-RNTI, and a PRACH resource. A preamble is transmitted using the selected PRACH format with transmission power , as described in Subclause 7.4, on the indicated PRACH resource. UE is not expected to be configured with more than one preamble format and more than one subcarrier spacing in a cell.

## 12.2 DOCOMO

Proposal 3: Adopt the following TP in TS 38.213.

Offline agreement:

## 8.2 Random access response (Unchanged parts omitted) A UE receives the PDCCH with DCI format 1_0 with the CRC scrambled by the corresponding RA-RNTI and the corresponding PDSCH that includes the DL-SCH transport block with the same DM-RS antenna port quasi co-location properties, as described in [6, 38.214], as for a detected SS/PBCH block or a received CSI-RS the SS/PBCH block or the CSI-RS resource the UE used for RACH association and transmission. If the UE attempts to detect the PDCCH with the corresponding RA-RNTI in response to a PRACH transmission initiated by a PDCCH order which triggers non-contention based random access procedure, the UE assumes that the PDCCH and the PDCCH order have same DM-RS antenna port quasi co-location properties. (Unchanged parts omitted)

Offline agreement:

## 5 Physical downlink shared channel related procedures ## 5.1 UE procedure for receiving the physical downlink shared channel (Unchanged parts omitted) When receiving PDSCH for Random Access Response (RAR) the UE may assume that the DM-RS port of PDSCH is quasi co-located with the SS/PBCH block or the CSI-RS resource the UE selected use for RACH association and transmission with respect to Doppler shift, Doppler spread, average delay, delay spread, spatial RX parameters when applicable. When receiving a RAR triggered by a PDCCH order which triggers non-contention based random access procedure, the UE may assume that the DM-RS port of the received PDCCH order and the PDSCH of the corresponding RAR are quasi co-located with the same SS/PBCH block or CSI-RS with respect to Doppler shift, Doppler spread, average delay, delay spread, spatial RX parameters when applicable. (Unchanged parts omitted)

Proposal 6: Adopt the following TP in TS 38.213.

## 8.3 Msg3 PUSCH (Unchanged parts omitted) If in slot a UE receives a PDSCH with a RAR message for a corresponding preamble transmission from the UE, the UE transmits a Msg3 PUSCH in slot , where is provided in [6, TS 38.214]. The UE may assume a minimum time between the last symbol of a PDSCH reception conveying a RAR and the first symbol of a corresponding Msg3 PUSCH transmission scheduled by the RAR in the PDSCH for a UE is equal to msec. is a time duration of symbols corresponding to a PDSCH reception time for PDSCH processing capability 1 when additional PDSCH DM-RS is configured and is a time duration of symbols corresponding to a PUSCH preparation time for PUSCH processing capability 1 [6, TS 38.214]. A UE receives the PDCCH with DCI format 0_0 with CRC scrambled by the corresponding TC-RNTI conveying Msg3 retransmission grant with the same DM-RS antenna port quasi co-location properties as for the SS/PBCH block or the CSI-RS resource the UE selected for RACH association and transmission with respect to Doppler shift, Doppler spread, average delay, delay spread, spatial RX parameters when applicable.

Proposal 7: Adopt the following TP in TS 38.213.

## 8.4 PDSCH with UE contention resolution identity In response to an Msg3 PUSCH transmission when a UE has not been provided with a C-RNTI, the UE attempts to detect a DCI format 1_0 with CRC scrambled by a corresponding TC-RNTI scheduling a PDSCH that includes a UE contention resolution identity [11, TS 38.321]. A UE receives the PDCCH with DCI format 1_0 with CRC scrambled by the corresponding TC-RNTI and the corresponding PDSCH that includes a UE contention resolution identity with the same DM-RS antenna port quasi co-location properties, as described in [6, 38.214], as for the SS/PBCH block or the CSI-RS resource the UE selected for RACH association and transmission. In response to the PDSCH reception with the UE contention resolution identity, the UE transmits HARQ-ACK information in a PUCCH. The PUCCH transmission is within a same initial active UL BWP as the Msg3 PUSCH transmission. A minimum time between the last symbol of the PDSCH reception and the first symbol of the corresponding HARQ-ACK transmission is equal to msec. is a time duration of symbols corresponding to a PDSCH reception time for PDSCH processing capability 1 when additional PDSCH DM-RS is configured.

## 12.3 Qualcomm

Offline agreement:

Proposal 6: Adopt the following CR for 38.331 and send an LS to RAN2.

CHANGE START

-- ASN1START

-- TAG-RACH-CONFIG-COMMON-START

RACH-ConfigCommon ::= 				SEQUENCE {

rach-ConfigGeneric			RACH-ConfigGeneric,

totalNumberOfRA-Preambles			INTEGER (1..63)																		OPTIONAL,	-- Need S

ssb-perRACH-OccasionAndCB-PreamblesPerSSB	CHOICE {

oneEighth								ENUMERATED {n4,n8,n12,n16,n20,n24,n28,n32,n36,n40,n44,n48,n52,n56,n60,n64},

oneFourth								ENUMERATED {n4,n8,n12,n16,n20,n24,n28,n32,n36,n40,n44,n48,n52,n56,n60,n64},

oneHalf									ENUMERATED {n4,n8,n12,n16,n20,n24,n28,n32,n36,n40,n44,n48,n52,n56,n60,n64},

one										ENUMERATED {n4,n8,n12,n16,n20,n24,n28,n32,n36,n40,n44,n48,n52,n56,n60,n64},

two										ENUMERATED {n4,n8,n12,n16,n20,n24,n28,n32},

four									INTEGER (1..16),

eight									INTEGER (1..8),

sixteen									INTEGER (1..4)

}																														OPTIONAL,	-- Need M

groupBconfigured 					SEQUENCE {

ra-Msg3SizeGroupA					ENUMERATED { b56, b144, b208, b256, b282, b480, b640,

b800, b1000, spare7, spare6, spare5, spare4, spare3, spare2, spare1},

messagePowerOffsetGroupB			ENUMERATED { minusinfinity, dB0, dB5, dB8, dB10, dB12, dB15, dB18},

numberOfRA-PreamblesGroupA			INTEGER (1..64)

ssb-perRACH-OccasionAndnumberOfRA-PreamblesGroupAPerSSBPerRACH-Occasion	CHOICE {

oneEighth								ENUMERATED {n4,n8,n12,n16,n20,n24,n28,n32,n36,n40,n44,n48,n52,n56,n60,n64},

oneFourth								ENUMERATED {n4,n8,n12,n16,n20,n24,n28,n32,n36,n40,n44,n48,n52,n56,n60,n64},

oneHalf									ENUMERATED {n4,n8,n12,n16,n20,n24,n28,n32,n36,n40,n44,n48,n52,n56,n60,n64},

one										ENUMERATED {n4,n8,n12,n16,n20,n24,n28,n32,n36,n40,n44,n48,n52,n56,n60,n64},

two										ENUMERATED {n4,n8,n12,n16,n20,n24,n28,n32},

four									INTEGER (1..16),

eight									INTEGER (1..8),

sixteen									INTEGER (1..4)

}

}																														OPTIONAL,	-- Need R

ra-ContentionResolutionTimer			ENUMERATED { sf8, sf16, sf24, sf32, sf40, sf48, sf56, sf64},

rsrp-ThresholdSSB						RSRP-Range																		OPTIONAL,	-- Need R

rsrp-ThresholdSSB-SUL					RSRP-Range																		OPTIONAL,	-- Cond SUL

prach-RootSequenceIndex					CHOICE {

l839									INTEGER (0..837),

l139									INTEGER (0..137)

},

msg1-SubcarrierSpacing					SubcarrierSpacing,

restrictedSetConfig						ENUMERATED {unrestrictedSet, restrictedSetTypeA, restrictedSetTypeB},

msg3-transformPrecoding					ENUMERATED {enabled}															OPTIONAL,	-- Need R

...

}

CHANGE END

## 12.4 CATT

Proposal 3:

========== Text Proposal (In Section 8.1, TS 38.213) ======

An association period, starting from frame 0, for mapping SS/PBCH blocks to PRACH occasions is the smallest value in the set determined by the PRACH configuration period according Table 8.1-1 such that  SS/PBCH blocks are mapped at least once to the PRACH occasions within the association period, where a UE obtains  from the value of higher layer parameter ssb-PositionsInBurst . If after an maximum allowed  integer number of SS/PBCH blocks to PRACH occasions mapping cycles within the association period there is a set of PRACH occasions that are not mapped to SS/PBCH blocks, no SS/PBCH blocks are mapped to the set of PRACH occasions. An association pattern period consists of one or more association periods and is determined so that a pattern between PRACH occasions and SS/PBCH blocks repeats at most every 160 msec. PRACH occasions not associated with SS/PBCH blocks after an maximum allowed integer number of association periods within the association pattern period, if any, are not used for PRACH transmissions.

Proposal 4:

========== Text Proposal (In Section 8.1, TS 38.213) ======

For single cell operation or for operation with carrier aggregation in a same frequency band, a UE does not transmit PRACH and PUSCH/PUCCH/SRS in a same slot, and UE does not transmit both PRACH and PUSCH/PUCCH/SRS or when a gap between the first/last symbol of a PRACH (or a PUSCH/PUCCH/SRS) transmission in a first slot is separated by less than  symbols from the last/first symbol of a PUSCH/PUCCH/SRS (or a PRACH) transmission in a second slot, where  for  or ,  for  or , and  is the subcarrier spacing configuration for the active UL BWP. It will be up to UE to transmit the PRACH or the PUSCH/PUCCH/SRS.

Proposal 5:

========== Text Proposal (In Section 8.2, TS 38.213) ======

If the UE does not detect the DCI format 1_0 with the CRC scrambled by the corresponding RA-RNTI within the window, or if the UE does not correctly receive the DL-SCH transport block in the PDSCH within the window, the UE shall be ready to transmit a PRACH no later than  msec after the last symbol of the window, if requested by higher layers. or ifIf the higher layers do not identify the RAPID associated with the PRACH transmission, the higher layers can indicate to the physical layer to transmit a PRACH. If requested by higher layers, the UE shall be ready to transmit a PRACH no later than  msec after the last symbol of the window, or the last symbol of the PDSCH reception,   where  is a time duration of  symbols corresponding to a PDSCH reception time for PDSCH processing capability 1 when additional PDSCH DM-RS is configured.

……….

If a UE does not detect the DCI format 1_0 with CRC scrambled by a corresponding RA-RNTI or does not correctly receive a corresponding DL-SCH transport block within the window, the UE procedure is as described in [11, TS 38.321].

# 14. Duration of RACH Search Space

DOCOMO:

UE monitors Type-1 PDCCH common search space only on duration between start of RAR window and end of random access procedure, i.e., duration between Msg2 and Msg4.

# 15.References

| Agenda item | Type | TDoc | Title | Source | Is revision of | TDoc Status |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 7.1.1.4.2; other; R1-1805876; Remaining details of RACH Procedures; Huawei, HiSilicon; available. 7.1.1.4.2; discussion; R1-1805945; Remaining details of RACH procedure; ZTE; available. 7.1.1.4.2; discussion; R1-1806035; Remaining issues on RACH procedure; vivo; available. 7.1.1.4.2; discussion; R1-1806272; Further details on NR 4-step RA Procedure; CATT; available. 7.1.1.4.2; discussion; R1-1806425; Remaining details of RACH procedure; Ericsson; available. 7.1.1.4.2; discussion; R1-1806501; Remaining issues in RACH procedure; Intel Corporation; available. 7.1.1.4.2; discussion; R1-1806606; RACH Procedure; LG Electronics; available. 7.1.1.4.2; other; R1-1806705; Corrections on RACH Procedure; Samsung; available. 7.1.1.4.2; discussion; R1-1806771; On remaining issues of RACH procedure; MediaTek Inc.; available. 7.1.1.4.2; discussion; R1-1806923; Remaining details on RACH procedure; Nokia, Nokia Shanghai Bell; available. 7.1.1.4.2; other; R1-1806941; Remaining issues on RACH procedure; Sharp; available. 7.1.1.4.2; discussion; R1-1807603; Remaining issues on RACH procedure; NTT DOCOMO, INC.; available. 7.1.1.4.2; discussion; R1-1807275; Remaining details for Msg/2/3/4 transmission and reception; Motorola Mobility, Lenovo; available. 7.1.1.4.2; discussion; R1-1807333; Remaining details on RACH procedure; Qualcomm Incorporated; available. |  |  |  |  |  | available |  |