3GPP TSG RAN WG1 Meeting #94				                          R1-1808157

Gothenburg, Sweden, August 20th –24th, 2018

Source:           ZTE

Title:             Link-level simulation results for eMBB scenario

Agenda item:     7.2.1.5

Document for:    Discussion and Decision

# Background

The link-level performance metrics and simulation assumptions for NoMA study were agreed in RAN1#92, RAN1#92bis and RAN1#93 meetings [1][2][3].

In this contribution, we present some preliminary link-level evaluation results for the QAM-sequence based symbol-level spreading scheme (MUSA) based on the agreed LLS assumptions. MMSE hard-SIC receiver is assumed.

# Preliminary link-level evaluation results for MUSA

## Simulation parameters

The spreading sequences used for the simulation are listed in the Appendix 1. For each set of TB size and number of UEs, the spreading factor, and modulation order are adjusted in order to achieve better performance in some of the simulation cases. In addition, as analyzed in [4], multi-layer transmission per UE could be considered to further enhance the performance for very high per UE spectral efficiency.

Linear MMSE with hard-SIC receiver is assumed where the details can be found in [5]. For realistic channel estimation, NR DMRS design with 1/7 overhead is adopted as starting point. For mMTC and eMBB with 14 OFDM symbols, NR DMRS can support up to 12 ports in the case of DMRS type 2. For the case of more than 12 UEs, we simply double the number of FDM combs to increase the orthogonal ports to e.g. 24. Consequently, the accuracy of channel estimation would suffer due to the less number of estimation samples in frequency domain.

## Preliminary simulation results

- eMBB, fixed MA signature allocation

Due to more number of Rx antennas assumed for eMBB use cases, the overloading capability of symbol-level linear spreading can be further enhanced. Significant performance degradation can be observed for realistic channel estimation, which means the current NR design of DMRS may need enhancement for the NOMA transmission under mini-slot based assumptions.

The results for eMBB scenario with 40-Byte TBS under different number of UEs are shown in Figure 1. Compared with OMA single user which is the upper bound of per UE performance, 20 UEs can be multiplexed with less than 1dB SNR loss for both ideal and realistic channel estimation. The achievable sum throughput corresponding to 20 UEs with 0.1 BLER is 2.67 bps/Hz where the per UE SE = (40*8bits)/(12*12*15kHz)/1ms.

Figure 1 shows the results with the same assumptions at ideal and realistic channel estimation. It can be found that for TDL-A model, there will be about 0.5 dB loss for realistic channel estimation (RCE) compared with ideal channel estimation (ICE).

| (a). Ideal channel estimation | (b). Realistic channel estimation |
| --- | --- |

Figure 1 Performance evaluation for eMBB with 40 bytes with equal avg. SNR .

- eMBB, Random MA signature allocation

| (a). TBS=40, TDLA-30ns, 1T4R | (b). TBS=80, TDLA-30ns, 1T4R |
| --- | --- |

Figure 2 Performance evaluation of eMBB with Enhanced DMRS in random scenario, unequal avg. SNR, Realistic CE, TO = (1/2)*CP, FO = 140Hz, Realistic TO/FO estimation

Figure 2 shows some preliminary simulation results on BLER vs SNR for with Enhanced DMRS NOMA scheme with symbol-level linear spreading. In this scheme Enhanced DMRS with overhead 1/7 is used for UE detection, TOFO estimation and channel estimation.

Observation 1: With MMSE hard-SIC receiver, NOMA with QAM-sequence based symbol-level linear spreading can achieve excellent overloading capability with very limited per UE performance loss in all the interested use scenarios.

Proposal 1: Support QAM-sequence based symbol-level linear spreading for NOMA.

# Conclusion

In this contribution, we provided some link level simulations and preliminary performance comparisons among different NOMA schemes from various aspects. Some proposals and observations were made as follows.

Observation 1: With MMSE hard-SIC receiver, NOMA with QAM-sequence based symbol-level linear spreading can achieve excellent overloading capability with very limited per UE performance loss in all the interested use scenarios.

Proposal 1: Support QAM-sequence based symbol-level linear spreading for NOMA.

# Reference

- 3GPP Chairman’s Notes RAN1#92, Feb. 2018.

- 3GPP Chairman’s Notes RAN1#92bis, April. 2018.

- 3GPP Chairman’s Notes RAN1#93, May. 2018.

- 3GPP R1-1805840, Key processing modules at transmitter side for NOMA, ZTE

- 3GPP R1-1805841, Multi-user receivers for NOMA, ZTE

# Appendix 1

Table A1 Simulation assumptions used for preliminary evaluations

| Parameters | mMTC | URLLC | eMBB | Further specified values |
| --- | --- | --- | --- | --- |
| Carrier Frequency | 700 MHz | 700 MHz or 4 GHz | 4 GHz, 700 MHz as optional |  |
| Waveform (data part) | CP-OFDM and DFT-s-OFDM | CP-OFDM as starting point | CP-OFDM as starting point |  |
| Channel coding | URLLC: NR LDPC eMBB: NR LDPC mMTC: NR LDPC |  |  | The choice of channel coding here is only for the performance evaluation purpose for NOMA study |
| Numerology (data part) | SCS = 15 kHz, #OS = 14 | Case 1: SCS = 60 kHz, #OS = 7 (normal CP), optionally 6 (ECP) Case 2: SCS = 30 kHz, #OS = 4 | SCS = 15 kHz #OS = 14 |  |
| Allocated bandwidth | 6 as the starting point | 12 for 60 kHz SCS and 24 for 30kHz SCS as the starting point | 12 as the starting point | For high payload such as 75 bytes, larger number of RBs can be considered. |
| TBS per UE | At least five TBS that are [10, 20, 40, 60, 75] bytes. Other values higher than 10 bytes are not precluded. Lower than 0.1 bits/RE is optional | At least five TBS that are [10, 20, 40, 60, 75] bytes. Other values higher than 10 bytes are not precluded. | At least five TBS that are [20, 40, 80, 120, 150] bytes. Other values higher than 20 bytes are not precluded. | For ideal channel estimation, DMRS overhead is 1/7 for #OS 7 and 14, and 1/4 for #OS 4 |
| Target BLER for one transmission | 10% | 0.10% | 10% |  |
| Number of UEs multiplexed in the same allocated bandwidth | Indicated in the figures |  |  | Companies are encouraged to perform evaluations with various number of UEs Note: refined set of numbers of UEs should be further discussed in the next meeting. |
| BS antenna configuration | 2 Rx or 4 Rx for 700MHz, 4 Rx or 8 Rx for 4 GHz 8 Rx as optional |  |  | CDL model in 38.901 should be considered for 8Rx |
| UE antenna configuration | 1 Tx |  |  |  |
| Propagation channel & UE velocity | TDL-A 30ns and TDL-C 300ns in TR38.901, 3km/h, CDL optional |  |  |  |
| Max number of HARQ transmission | 1 as starting point. | 1 as starting point. More values, 2 for URLLC can be used. | 1 as starting point. |  |
| Channel estimation | Ideal channel estimation results should be reported for calibration Realistic channel estimation |  |  |  |
| MA signature allocation (for data and DMRS) | Fixed/Random |  |  | Proponents report the details of random MA signature allocation (whether without or with collision) |
| Distribution of avg. SNR | Both equal and unequal | Equal | Both equal and unequal | Uniform discrete values for unequal case,, range [x - a, x + a] (dB) with 1 dB step, where x is the average SNR among UEs, and the deviation [a=3] |
| Timing offset | 0 as starting point. For grant-free without perfect TA (asynchronous), timing offset is within [0, y] as starting point, where y has two values at least for the purpose of evaluation: • Case 1: CP/[2] < y <= CP+rms_DS, with detailed value FFS • Case 2: 2*CP>=y > CP, with detailed value FFS |  |  | Additional value(s) for y are not precluded Possible down-selection can still be discussed |
| Frequency error | 0 as starting point. Also evaluate uniform distribution between -70 and 70 Hz for 700MHz carrier frequency, and uniform distribution between [-140] and [140] Hz for 4GHz carrier frequency. |  |  |  |
| Traffic model for link level | Full buffer as starting point. Non-full-buffer model (like Poisson arrival of fixed packet size) is optional. |  |  |  |
| For link level calibration purpose only | OMA single user whose spectral efficiency is the same as per UE SE in NOMA. AWGN curves can be provided also. |  |  |  |

Table A2 Sequence pool (before normalization) used for the symbol-level linear spreading scheme.

| Linear spreading codebook for SF=2 (the table is repeated for the simulation with more than 6 UEs) |  |
| --- | --- |
| Linear spreading codebook for SF=3 (the table is repeated for the simulation with more than 9 UEs) |  |
| Linear spreading codebook for SF=4 | See Table A3 |
| Linear spreading codebook for SF=6 | See Table A4 |

Table A3 Example of QPSK sequence with SF = 4, pool size = 64 (before normalization).

| No. | c1 | c2 | c3 | c4 | No. | c1 | c2 | c3 | c4 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 | 1 | 1 | 1 | 33 | 1 | 1 | 1 | -j |
| 2 | 1 | 1 | -1 | -1 | 34 | 1 | 1 | -1 | j |
| 3 | 1 | -1 | 1 | -1 | 35 | 1 | -1 | 1 | j |
| 4 | 1 | -1 | -1 | 1 | 36 | 1 | -1 | -1 | -j |
| 5 | 1 | 1 | -j | j | 37 | 1 | 1 | -j | 1 |
| 6 | 1 | 1 | j | -j | 38 | 1 | 1 | j | -1 |
| 7 | 1 | -1 | -j | -j | 39 | 1 | -1 | -j | -1 |
| 8 | 1 | -1 | j | j | 40 | 1 | -1 | j | 1 |
| 9 | 1 | -j | 1 | j | 41 | 1 | -j | 1 | 1 |
| 10 | 1 | -j | -1 | -j | 42 | 1 | -j | -1 | -1 |
| 11 | 1 | j | 1 | -j | 43 | 1 | j | 1 | -1 |
| 12 | 1 | j | -1 | j | 44 | 1 | j | -1 | 1 |
| 13 | 1 | -j | -j | -1 | 45 | 1 | -j | -j | j |
| 14 | 1 | -j | j | 1 | 46 | 1 | -j | j | -j |
| 15 | 1 | j | -j | 1 | 47 | 1 | j | -j | -j |
| 16 | 1 | j | j | -1 | 48 | 1 | j | j | j |
| 17 | 1 | 1 | 1 | -1 | 49 | 1 | 1 | 1 | j |
| 18 | 1 | 1 | -1 | 1 | 50 | 1 | 1 | -1 | -j |
| 19 | 1 | -1 | 1 | 1 | 51 | 1 | -1 | 1 | -j |
| 20 | 1 | -1 | -1 | -1 | 52 | 1 | -1 | -1 | j |
| 21 | 1 | 1 | -j | -j | 53 | 1 | 1 | -j | -1 |
| 22 | 1 | 1 | j | j | 54 | 1 | 1 | j | 1 |
| 23 | 1 | -1 | -j | j | 55 | 1 | -1 | -j | 1 |
| 24 | 1 | -1 | j | -j | 56 | 1 | -1 | j | -1 |
| 25 | 1 | -j | 1 | -j | 57 | 1 | -j | 1 | -1 |
| 26 | 1 | -j | -1 | j | 58 | 1 | -j | -1 | 1 |
| 27 | 1 | j | 1 | j | 59 | 1 | j | 1 | 1 |
| 28 | 1 | j | -1 | -j | 60 | 1 | j | -1 | -1 |
| 29 | 1 | -j | -j | 1 | 61 | 1 | -j | -j | -j |
| 30 | 1 | -j | j | -1 | 62 | 1 | -j | j | j |
| 31 | 1 | j | -j | -1 | 63 | 1 | j | -j | j |
| 32 | 1 | j | j | 1 | 64 | 1 | j | j | -j |

Table A4 Example of BPSK or {+1/-1} sequence with SF = 6, pool size = 16 (before normalization).

| No. | c1 | c2 | c3 | c4 | c5 | c6 |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 | 1 | 1 | 1 | 1 | 1 |
| 2 | 1 | 1 | 1 | 1 | -1 | -1 |
| 3 | 1 | 1 | 1 | -1 | 1 | -1 |
| 4 | 1 | 1 | 1 | -1 | -1 | 1 |
| 5 | 1 | 1 | -1 | 1 | 1 | -1 |
| 6 | 1 | 1 | -1 | 1 | -1 | 1 |
| 7 | 1 | 1 | -1 | -1 | 1 | 1 |
| 8 | 1 | 1 | -1 | -1 | -1 | -1 |
| 9 | 1 | -1 | 1 | 1 | 1 | -1 |
| 10 | 1 | -1 | 1 | 1 | -1 | 1 |
| 11 | 1 | -1 | 1 | -1 | 1 | 1 |
| 12 | 1 | -1 | 1 | -1 | -1 | -1 |
| 13 | 1 | -1 | -1 | 1 | 1 | 1 |
| 14 | 1 | -1 | -1 | 1 | -1 | -1 |
| 15 | 1 | -1 | -1 | -1 | 1 | -1 |
| 16 | 1 | -1 | -1 | -1 | -1 | 1 |

# Appendix 2 Evaluation results

- eMBB scenario, fixed MA signature

| (a). OMA reference | (b). TBS=20 | (c). TBS=40 |
| --- | --- | --- |
| (d). TBS=80 | (e). TBS=120 | (f). TBS=150 |

Figure A2.a-1. BLER Performance for eMBB (Equal SNR, TDL-A, CP-OFDM, Ideal CE)

| (a). OMA reference | (b). TBS=20 | (c). TBS=40 |
| --- | --- | --- |
| (d). TBS=80 | (e). TBS=120 | (f). TBS=150 |

Figure A2.a-2. BLER Performance for eMBB (Equal SNR, TDL-C, CP-OFDM, Ideal CE)

| (a). OMA reference | (b). TBS=20 | (c). TBS=40 |
| --- | --- | --- |
| (d). TBS=80 | (e). TBS=120 | (f). TBS=150 |

Figure A2.a-3. BLER Performance for eMBB (Equal SNR, TDL-A, CP-OFDM, Realistic CE)

| (a). OMA reference | (b). TBS=20 | (c). TBS=40 |
| --- | --- | --- |
| (d). TBS=80 | (e). TBS=120 | (f). TBS=150 |

Figure A2.a-4. BLER Performance for eMBB (Equal SNR, TDL-C, CP-OFDM, Realistic CE)

| (a). OMA reference | (b). TBS=20 | (c). TBS=40 |
| --- | --- | --- |
| (d). TBS=80 | (e). TBS=120 | (f). TBS=150 |

Figure A2.a-5. BLER Performance for eMBB (Unequal SNR, TDL-A, CP-OFDM, Ideal CE)

| (a). OMA reference | (b). TBS=20 | (c). TBS=40 |
| --- | --- | --- |
| (d). TBS=80 | (e). TBS=120 | (f). TBS=150 |

Figure A2.a-6. BLER Performance for eMBB (Unequal SNR, TDL-C, CP-OFDM, Ideal CE)

| (a). OMA reference | (b). TBS=20 | (c). TBS=40 |
| --- | --- | --- |
| (d). TBS=80 | (e). TBS=120 | (f). TBS=150 |

Figure A2.a-7. BLER Performance for eMBB (Unequal SNR, TDL-A, CP-OFDM, Realistic CE)

| (a). OMA reference | (b). TBS=20 | (c). TBS=40 |
| --- | --- | --- |
| (d). TBS=80 | (e). TBS=120 | (f). TBS=150 |

Figure A2.a-8. BLER Performance for eMBB (Unequal SNR, TDL-C, CP-OFDM, Realistic CE)

- eMBB scenario, Random MA signature

| (a). TBS=40, TDLA-30ns, 1T4R | (b). TBS=80, TDLA-30ns, 1T4R |
| --- | --- |

Figure A2.b-1. BLER Performance for eMBB(Enhanced DMRS, Unequal SNR, CP-OFDM, Realistic CE, TO = (1/2)*CP, FO = 140Hz, Realistic TO/FO estimation)

- eMBB scenario, Sum-throughput vs total SNR

| (a). TDLA-30ns | (b). TDLC-300ns |
| --- | --- |

Figure A2.c-1. Sum-throughput vs total SNR for eMBB (Equal SNR, CP-OFDM, Ideal CE)

| (a). TDLA-30ns | (b). TDLC-300ns |
| --- | --- |

Figure A2.c-2. Sum-throughput vs total SNR for eMBB (Equal SNR, CP-OFDM, Realistic CE)

| (a). TDLA-30ns | (b). TDLC-300ns |
| --- | --- |

Figure A2.c-3. Sum-throughput vs total SNR for eMBB (Unequal SNR, CP-OFDM, Ideal CE)

| (a). TDLA-30ns | (b). TDLC-300ns |
| --- | --- |

Figure A2.c-4. Sum-throughput vs total SNR for eMBB (Unequal SNR, CP-OFDM, Realistic CE)