3GPP TSG RAN WG1 Meeting #94			R1-1808396

Gothenburg, Sweden, August 20th – 24th, 2018

Source:	CATT

Title:	Initial Access and Mobility for NR Unlicensed Operations

Agenda Item:	7.2.2.4.2

Document for:	Discussion and Decision

# Introduction

The following agreements were made on initial access and mobility for NR unlicensed operations in RAN1#93 [1]:

Agreement:

The following modifications to initial access procedures are beneficial

- Modifications to initial access procedures considering limitations on access to the channel based on LBT

  - Develop techniques to handle reduced SS/PBCH block and RMSI transmission opportunities due to LBT failure

- Enhancement to 4-step RACH

  - Mechanisms to handle reduced msg 1/2/3/4 transmission opportunities due to LBT failure

- 2-step RACH potentially has benefit for channel access

Agreement:

Potential modifications to RLM/RRM procedures due to reduced transmission opportunities for DL signals and channels due to LBT failure should be identified and studied.

Agreement:

Modifications to paging procedures due to reduced transmission opportunities for paging due to LBT failure are beneficial and should be identified and studied.

In this contribution, we further discuss the issues associated with initial access and mobility for NR unlicensed operations.

# Discussions

## NR-U Broadcasting Signals

In RAN1#93, the following agreement was made:

Agreement: The following modifications to initial access procedures are beneficial - Modifications to initial access procedures considering limitations on access to the channel based on LBT - Develop techniques to handle reduced SS/PBCH block and RMSI transmission opportunities due to LBT failure

In NR, a burst of SSBs, called SSB burst set, is transmitted within a half frame. The transmission patterns of the candidate SSBs within a burst of SSBs within a half frame are predefined in the specs according to the SSB subcarrier spacings, which are predefined per band. A UE knows the SSB transmission patterns for a band in which the UE searches for the SSBs.

For NR, the transmission periodicity of the half frame with SSB burst set can be configured with the periodicity of {5, 10, 20, 40, 80, 160} ms, as shown in Figure 1. The PBCH within the SSB is the first channel for the UE to decode after the detection of PSS/SSS of the SSB. NR PBCH carries a part of the minimum system information required for the UE to access the system, including the MIB. MIB is transmitted with the TTI of 80 ms and with the repetitions within TTI. MIB contains the parameters that are needed to acquire RMSI. RMSI, which has the TTI of 160ms, contains the parameters that are needed to acquire OSI.

Figure 1. Illustration of NR SSB Transmission Patterns

In NR-U operation, the transmission opportunities of the SSB and RMSI transmission may be smaller than the configured transmission periodicity due to LBT failure. Thus, there is a need to compensate for the loss of the transmission opportunities of the SSB and RMSI.

One approach is to modify the SSB transmission pattern as shown in Figure 2, where before the transmission of each SSB in the SSB burst set in the half frame for an SSB transmission period, the NR-U gNB first perform the LBT for that SSB to see if the gNB is allowed to transmit that SSB in its beam direction. If the LBT fails, the NR-U gNB will have another LBT check in next half frame. This procedure continues until the LBT succeeds or until it reaches the end of the SSB transmission period. If LBT succeeds, the NR-U gNB will complete the transmission of the SSB within the half frame, and will not perform LBT and SSB transmission for that SSB within the SSB transmission period. When an SSB is transmitted, the associated PDCCH/PDSCH for RMSI are also transmitted without performing another LBT to avoid the delay for a UE to receive the RMSI.

Figure 2. Illustration of NR-U SSB Transmission Patterns

Proposal 1: For each SSB transmission period, before the transmission of any SSB within the SSB burst set, the LBT is carried out for that SSB. If the LBT fails, the next LBT is carried out at the following half subframe for that SSB. The procedure continues until either when the LBT succeeds or when it reaches the end of the SSB transmission period.

Proposal 2: When the LBT succeeds for an SSB in an SSB burst set, the SSB is transmitted. The PDCCH/PDSCH for the RMSI associated with the SSB should also be transmitted without LBT.

## LBT during NR-U PRACH Procedure

In RAN1#93, the following agreement was made:

Agreement: The following modifications to initial access procedures are beneficial - … - Enhancement to 4-step RACH - Mechanisms to handle reduced msg 1/2/3/4 transmission opportunities due to LBT failure - 2-step RACH potentially has benefit for channel access

The 4-step random access procedure is currently supported for NR licensed operation, which includes:

- Msg1: UE transmits a random access preamble sequence on PRACH (Msg1)

- Msg2: BS transmits a random access response (RAR) on the PDCCH/PDSCH back to UE (Msg2)

- Msg3: UE transmits a message on the PUSCH channel in response to Msg2

- Msg4: BS transmits a message on PDSCH with the contention resolution

For Msg1 transmission, NR Rel-15 has adopted very flexible design for PRACH resource configurations. In general, the network should be able to find a suitable PRACH configuration that provides adequate PRACH opportunities with a targeting LBT failure rate within the PRACH configuration period for supporting the NR-U PRACH operation. There seems no compelling reason to modify the PRACH resource configurations for NR-U operation. In addition, NR Rel-15 PRACH resource configurations are currently already quite complicated, and we should avoid changing the current configurations unless absolutely necessary.

Observation 1: NR Rel-15 PRACH resource configurations for Msg1 transmission is very flexible. Modification of the NR Rel-15 PRACH resource configurations for NR-U operation should be avoided unless absolutely necessary.

From gNB side, when it detects a PRACH preamble, it transmits RAR with the DL beam based on the association between the SSB and the RO that the UE selected for Msg1 transmission. In UE side, after sending Msg1, the UE attempts to detect RAR from gNB in respond to the transmitted Msg1 within a window provided by higher layer parameter ra-ResponseWindow. If for any reason, the UE does not successfully detect the Msg2 corresponding to the transmitted Msg1, e.g., the UE does not detect the DCI format 1_0 with the CRC scrambled by the corresponding RA-RNTI from the CORESET#1 for Type1-PDCCH CSS, or do not identify the RAPID associated with the transmitted Msg1, the UE may re-transmit the Msg1. For NR-U operation, gNB needs to perform the LBT first before it can send the RAR in respond to Msg1. If LBT fails, the transmission of the RAR may be delayed. It is highly desirable in this situation to prevent the UE from retransmitting the Msg1 since gNB already received the Msg1.

Observation 2: LBT failure may cause the delay in responding to Msg1. It is highly desirable to prevent the UE from retransmitting the Msg1 under this situation.

Configuring larger ra-ResponseWindow to account for the LBT failure may help in reducing the probability that the UE retransmits Msg1 due to the delay caused LBT failure. On the other hand, the  ra-ResponseWindow cannot be configured too large. Otherwise, this approach may potentially cause larger delays for the case that gNB actually does not receive the Msg1.

Observation 3: The configuration of ra-ResponseWindow needs to consider both the LBT failure and the gNB failed in receiving the Msg1.

In NR operation in licensed spectrum, after receiving RAR, the UE transmits an UL-SCH in a Msg3 PUSCH scheduled by a RAR grant.  Retransmissions, if any, of the UL-SCH in a Msg3 PUSCH are scheduled by a DCI format 0_0 with CRC scrambled by a TC-RNTI provided in the corresponding RAR message [11, TS 38.321]. If in the slot  a UE receives a PDSCH with a RAR message for a corresponding preamble transmission from the UE, the UE transmits a Msg3 PUSCH in the slot , where  is provided in TS 38.214.

The issue with NR-U is that the UE may not be able to transmit an UL-SCH in a Msg3 PUSCH scheduled by a RAR grant if LBT fails. How to handle the case needs further careful investigation.

In response to a Msg3 PUSCH transmission when a UE has not been provided with a C-RNTI, the UE attempts to detect a DCI format 1_0  with CRC scrambled by a corresponding TC-RNTI scheduling a PDSCH that includes a UE contention resolution identity. In response to the PDSCH reception with the UE contention resolution identity, the UE transmits HARQ-ACK information in a PUCCH. The PUCCH transmission is within a same active UL BWP as the Msg3 PUSCH transmission. The minimum time between the last symbol of the PDSCH reception and the first symbol of the corresponding HARQ-ACK information transmission is specified in TS 38.213. For NR-U operation, the response time needs again take into the consideration of the LBT.

Observation 4: The response time between messages during PRACH procedure in NR-U operation are all impacted by the LBT process.

Proposal 3: The response time between PRACH messages during the PRACH procedure in NR-U operation should be defined based on the response times between PRACH messages during the PRACH procedure defined in NR operation with additional consideration of the failure of LBT.

# Conclusion

In this contribution, we further discussed the issues associated with initial access and mobility for NR unlicensed operations. Based on the discussion, the following proposals were made:

Proposal 1: For each SSB transmission period, before the transmission of any SSB within the SSB burst set, the LBT is carried out for that SSB. If the LBT fails, the next LBT is carried out at the following half subframe for that SSB. The procedure continues until either when the LBT succeeds or when it reaches the end of the SSB transmission period.

Proposal 2: When the LBT succeeds for an SSB in an SSB burst set, the SSB is transmitted. The PDCCH/PDSCH for the RMSI associated with the SSB should also be transmitted without LBT.

Proposal 3: The response time between PRACH messages during the PRACH procedure in NR-U operation should be defined based on the response times between PRACH messages during the PRACH procedure defined in NR operation with additional consideration of the failure of LBT.

# References

- Chairman’s Note 3GPP TSG RAN WG1 #93

- R1-1805920	Initial access in NR unlicensed	Huawei, HiSilicon

- R1-1806087	Discussion on RACH procedure for NR-U	vivo

- R1-1806109	On Initial Access and Mobility for NR-U	Nokia, Nokia Shanghai Bell

- R1-1806254	On initial access, RRM, mobility and RLM	Ericsson

- R1-1806463	Discussion on initial access and mobility for NR-U	ZTE Corporation

- R1-1806545	Potential enhancements to NR initial access and mobility to support unlicensed operation	Intel Corporation

- R1-1806570	Considerations on initial access procedures for NR unlicensed operations	Sony

- R1-1806646	Initial access and mobility for NR unlicensed operation	LG Electronics

- R1-1806762	Initial access and mobility procedure for NR-U	Samsung

- R1-1806798	Considerations on initial access and mobility for NR-U	MediaTek Inc.

- R1-1806853	Initial access procedure on NR-U	OPPO

- R1-1806990	Beam measurement for NR-U	AT&T

- R1-1807037	On Initial Access in NR-Unlicensed Spectrum 	InterDigital, Inc.

- R1-1807142	Initial access and mobility for NR-U	ITL

- R1-1807390	Initial access and mobility for NR-U	Qualcomm Incorporated