3GPP TSG RAN WG1 Meeting #94	R1-1808752

Gothenburg, Sweden, August 20th – 24th, 2018

Agenda item:	7.1.2.5

Source: 	Samsung

Title: 	Simultaneous reception of physical channels and reference signals

Document for:	Discussion and Decision

Introduction

In RAN1#92bis and RAN1#93, following agreement and further studies are captured:

For further discussion in RAN1#93 - RAN4 agreement exists for SSB-RRM + PDSCH multiplexing for non-CA and CA cases - Consider at least the following alternatives. At least for cases outside SMTC window (SSB-RLM+PDSCH, SSB-BM+PDSCH) including CA: - Alt 1: (multiplexing restriction) Extend the RAN4 agreement to the case of SSB+PDSCH multiplexing for both CA and non-CA without PDSCH associated with SI-RNTI, P-RNTI. This means that the UE is not expected to receive PDSCH on SSB symbols (for same numerology in FR2) - Alt 2: gNB will ensure at least spatial QCL - Alt 3: The UE is not expected to perform Rx beam refinement on SSB locations (Rx beam refinement may be associated with BM, RLM, BFD) Note: The above applies to the case with same numerology for SSB and PDSCH. The case with different numerologies for SSB and PDSCH is to be addressed. Agreement For the case of CSIRS+PDCCH on the same OFDM symbols where spatial QCL is configured for CSI-RS - NW configuration should ensure spatial QCL - UE is not expected to be configured with CSI-RS with repetition=ON in the symbols UE is configured to monitor the CORESET Above agreement applies to both same BWP as well as intra-band CA if spatial QCL is applicable. For further study until the next meeting Alt 1: For the case of CSIRS+PDCCH on the same OFDM symbols, if CSI-RS is not configured with spatial QCL, the UE follows the spatial QCL associated with the PDCCH Alt 2: No need for further definition of UE behavior for the case of CSIRS not configured with spatial QCL overlapping with PDCCH on an OFDM symbol For further study until the next meeting For the case of CSIRS+PDSCH on the same OFDM symbols where spatial QCL is configured for CSI-RS - NW configuration should ensure spatial QCL - UE is required to decode PDSCH that is overlapping with one or more symbols configured with CSI-RS with repetition=ON - Note: The UE is not expected to perform Rx beam refinement on CSIRS symbols configured with CSI-RS with repetition=ON - NW configuration should ensure spatial QCL - UE is expected to symbol level rate-match PDSCH that is overlapping with one or more symbols on the symbols configured with CSI-RS with repetition=ON - DMRS should not be affected by rate matching - NW configuration should ensure spatial QCL - UE is not expected to receive PDSCH that is overlapping with one or more symbols configured with CSI-RS with repetition=ON - DMRS should not be affected by rate matching For further study until the next meeting For the case of PDSCH+PDCCH simultaneous RX in case spatial RX is configured and is different between the two channels - Alt1: CORESET is prioritized - spatial QCL associated with CORESET is applied to the PDSCH - FFS: PDSCH slot aggregation case - Alt2: PDSCH is prioritized depending on scheduling offset - Before threshold, CORESET is prioritized - After threshold, PDSCH is prioritized

In this contribution, we provide our view for each multiplexing case for DL and UL, respectively.

Discussion

In this section, our views on the cases are provided for DL and UL, respectively.

- Downlink

| Channels/RS in same OFDM symbol | Same CC / BWP | Different CCs |
| --- | --- | --- |
| SSB + PDCCH | Inside SMTC window - No PDCCH can be FDMed* Outside SMTC window - The UE is not expected to perform RX beam refinement on SSB locations | No need for further definition |
| SSB + PDSCH | Inside SMTC window - No PDSCH can be FDMed* Outside SMTC window - PDSCH with SI-RNTI, RA-RNTI, P-RNTI can be FDMed* for 120 KHz SCS* - For PDSCH with other than SI-RNTI, RA-RNTI, P-RNTI, the UE is not expected to perform RX beam refinement on SSB locations | No need for further definition |
| CSI-RS + PDSCH (including DM-RS/PT-RS) - A-CSI-RS (below and above threshold), P CSI-RS, SP CSI-RS - BM CSI-RS, CSI-RS for CSI, CSI-RS for tracking - CSI-RS Repetition ‘ON’, ‘OFF’ - CSI-IM | CSI-RS configured with spatial QCL - NW should ensure spatial QCL (no repetition) CSI-RS configured without spatial QCL - No need for further definition of UE behavior for the case of CSI-RS not configured with spatial QCL overlapping with PDSCH on an OFDM symbol | CSI-RS configured with spatial QCL - Prioritize beam indication of pCell or cells which has lower ID CSI-RS configured without spatial QCL - No need for further definition of UE behavior |
| PDCCH / CORESET+ PDSCH - Default CORESET (CORESET with lowest ID which is used for obtaining default QCL assumption) - Non default CORESET | CORESET is prioritized - Spatial QCL associated with CORESET is applied to the PDSCH | Prioritize beam indication of pCell or cells which has lower ID |
| CSI-RS + PDCCH / CORESET | CSI-RS configured with spatial QCL - NW should ensure spatial QCL (no repetition) * CSI-RS configured without spatial QCL - No need for further definition of UE behavior for the case of CSI-RS not configured with spatial QCL overlapping with PDCCH on an OFDM symbol | CSI-RS configured with spatial QCL - Prioritize beam indication of pCell or cells which has lower ID CSI-RS configured without spatial QCL - No need for further definition of UE behavior |
| PDCCH / CORESET + PDCCH / CORESET | Rx spatial QCL should be ensured by the NW configuration | Prioritize beam indication of pCell or cells which has lower ID |
| CSI-RS + SSB | Rx spatial QCL should be ensured by the NW configuration* | No need for further definition |
| CSI-RS + CSI-RS | Rx spatial QCL should be ensured by the NW configuration | No need for further definition |
| PDSCH + PDSCH with different RNTIs | Prioritize PDSCH with C-RNTI | No need for further definition |
| SSB + SSB | N/A | No need for further definition |
| PDSCH + PDSCH (C-RNTI + C-RNTI) | N/A | Prioritize beam indication of pCell or cells which has lower ID |

*Note: Already agreed in the previous meetings

- Uplink

| Channels/RS in same OFDM symbol | Same CC | Different CCs |
| --- | --- | --- |
| PUCCH + PUCCH | N/A | Prioritize beam indication of pCell or cells which has lower ID |
| PUSCH + PUSCH | N/A | Prioritize beam indication of pCell or cells which has lower ID |
| SRS + SRS | Rx spatial QCL should be ensured by the NW configuration |  |

- References

- 3GPP TSG RAN1#92bis, Chairman’s Note

- 3GPP TSG RAN1#93, Chairman’s Note.