3GPP TSG-RAN1 Meeting #94			R1-1808963

Gothenburg, Sweden, August 20th – 24th, 2018

Title:	Draft Reply LS on RA preambles mapping to SSBs

Response to:	R1-1808175 (R2-1810969)

Work Item:	NR_newRAT-Core

Source:	Nokia, NSB

To:	RAN WG2

Cc:

Contact Person:

#### Name: Emad Farag

E-mail Address:	Emad (dot) Farag (at) Nokia (dot) com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:

1. Overall Description:

RAN1 would like to thank RAN2 for their LS in R1-1808175(R2-1810969).

There are 64 physical preamble indices in a PRACH occasion. N SS/PBCH blocks can be associated with one PRACH occasion. The higher layer parameter “totalNumberOfRA-Preambles” indicates the total number of preamble available in a PRACH occasion for contention-based and contention-free random access. As clarified in RAN2 NR AH-1807, “totalNumberOfRA-Preambles” is a multiple of N, i.e. each SSB is associated with totalNumberOfRA-Preambles /N preambles for contention-free and contention-base random access.

Based on prior RAN1 agreements and as described in TS38.213, the contention-based and contention-free preambles of SS/PBCH block n start at physical preamble index n.64/N, using totalNumberOfRA-Preambles /N preamble indices for contention-based and contention-free random access for this SS/PBCH block. The following preamble indices until the start of the next SS/PBCH Block with index (n+1) are available for other purposes, e.g. SI requests.

RAN1 doesn’t see the need to change the formulas included in the RAN1 specifications.

2. Actions:

To RAN WG2

ACTION: 	RAN1 respectfully asks RAN2 to take the above information into consideration.

3. Date of Next TSG-RAN1 Meetings:

TSG-RAN1 Meeting #94bis		8th – 12th October 2018	Chengdu, China

TGS-RAN1 Meeting #95		12th – 16th November 2018	Spokane, USA