3GPP TSG-RAN WG1 Meeting #94	R1-1809040

Gothenburg, Sweden, August 20th – 24th 2018

Agenda Item:	7.2.6.4

Source:	Ericsson

Title:	Latency Evaluation of Rel-15 URLLC

Document for:	Discussion, Decision

# Introduction

the last meeting [2], details for UE processing capability#2 were discussed and following agreements were made. In this contribution we evaluate the baseline URLLC latency assuming NR Rel. 15 design, and then extend the evaluation to potential enhancements of Rel-16.

Agreements: The Capability #2 for (Aggressive) UE processing time in Rel-15 is supported under the following conditions - Non-CA - Note: this does not preclude EN-DC - FFS CA case with Capability #2 supported on only one or more of the carriers, and potential handling of some special cases - Single numerology for PDCCH, PDSCH, and PUSCH for the serving cell - PDSCH/PUSCH allocation with mapping Type A and Type B - For PDSCH mapping type A with last PDSCH symbol ending in symbol ‘i' of a slot, where i < 7 - N1 processing time is increased by (7-i) relative to the case where i=7. - (Working assumption) For PDSCH mapping type B with 4 or 2 symbols - N1 processing time is increased by ‘d’ symbols relative to the case of PDSCH with 7 symbols, where ‘d’ is the amount of time-domain overlap in symbols between the scheduling PDCCH and the scheduled PDSCH - FFS: handling of 3-symbol CORESET where first 2 symbols of CORESET are overlapped with a 2-symbol PDSCH - No UCI multiplexing - FFS: whether similar multiplexing rule as with Capability #1 may be included - For C-RNTI - FFS: simultaneous reception with broadcast PDSCH - Note: The UE signals whether Capability #2 is supported for each SCS, and separately for uplink and downlink

Agreements: The Capability #2 UE processing times are given below, where the PDSCH allocation length is assumed to be at least 7 symbols. - N1 - 15kHz: N1 = 3 - 30kHz: N1 = 4.5 - Note: as part of UE capability, - UE may report support with maximum scheduled RB allocation of 136 RBs - If RB allocation by scheduling exceeds the maximum signaled, UE defaults to Capability #1 processing time - OR UE may report support with no restriction on maximum scheduled RB allocation - 60kHz for FR1: 9 - N2 - 15kHz: N2 = 5 for CP-OFDM & For DFT-S-OFDM - 30kHz: N2 = 5.5 - 60kHz for FR1: 11 - If 1st symbol of PUSCH is data-only or FDM data with DMRS, then add 1 symbol to N2.

# Discussion

The evaluation considers different transmission scenarios such as DL, SR-based UL, and configured grant (SPS) UL in FDD. We perform latency analysis of different transmission duration options for 15 and 30 kHZ SCS, with the goal of meeting the ITU requirement of 1 ms latency.

The evaluation is based on assumptions of

- PDCCH periodicity

- PUCCH periodicity

- SR periodicity

- UE processing capability and

- gNB processing time

For Rel. 15, we use UE processing capability #2. To further improve latency in NR Rel. 16, we consider potential improved UE processing time (e.g. UE capability #3) targeting eURLLC use cases [1]. This includes e.g. more aggressive values for N1 and N2 at 15kHz and 30 kHz SCS.

We note that assumption on PDCCH periodicity is related to limits on number of blind decodes and CCEs. For Rel. 16 potential we also consider improved number of blind decodes and CCEs and thus possible improved PDCCH monitoring periodicity.

In the following we will analyze the UP latency in FDD after a first transmission and up to 3 HARQ retransmissions. We will follow the ITU definition of UP latency as being defined from L2/L3 ingress to L2/L3 egress.

## Assumptions

Processing time

The UE processing times between receiving PDSCH and providing HARQ feedback, and between receiving a dynamic grant and transmitting PUSCH are according to capability #2 (Tables 1 and 2). The gNB processing times are assumed to be the same as UE processing time. For DL the gNB processing is similar to data preparation time so we assume it is equal to N2. For UL the gNB processing is similar to data decoding time plus preparing for feedback so we assume it is equal to N1.

We assume that the processing time is dominated by demodulation and decoding of the physical channel and assume that the earliest time that data can be delivered to L2/L3 egress is the same time as the earliest time that HARQ feedback could be transmitted.

Table 1. PDSCH processing time in OFDM symbols for the UE capabilities with only front-loaded DMRS.

| #Symbols | N1 |  |
| --- | --- | --- |
|  | 15kHz SCS | 30kHz SCS |
| Capability 1 | 8 | 10 |
| Capability 2 | 3 | 4.5 |

Table 2: PUSCH preparation procedure time.

| #Symbols | N2 |  |
| --- | --- | --- |
|  | 15kHz SCS | 30kHz SCS |
| Capability 1 | 10 | 12 |
| Capability 2 | 5 | 5.5 |

Alignment delay

The alignment delay is the time required after being ready to transmit until transmission can start. We assume the worst-case latency meaning the alignment delay is assumed to the longest possible, i.e., equal to one TTI.

PUCCH duration is assumed to be equal to one TTI.

PDCCH duration is assumed to be 1 symbol. PDCCH opportunities depends on PDCCH monitoring periodicity. We consider different PDCCH periodicity values. For example, with PDCCH periodicity of 5 symbols we mean that PDCCH can be transmitted starting at symbols 0, 5, and 10 within a slot.

PDSCH immediately follows the PDCCH that schedules it. PDSCH duration of 2/4/7/14 OFDM symbols are considered, as shown in Table 3.

For HARQ-ACK on PUCCH, the evaluation assumes that at most one HARQ-ACK is allowed per slot for Capability #2. For Capability #3 evaluation, multiple HARQ-ACK per slot is allowed for achieving lowest latency.

For UL data, the scheduling can either be based on SR or SPS UL. PUSCH periodicity corresponding to opportunity for PUSCH transmission is assumed to be every TTI, where the TTI is 2/4/7/14 OFDM symbol as shown in Table 3. We assume that SR periodicity is 2 symbols corresponding to the shortest periodicity allowed in the Rel. 15 specification [3].

Figure 1: Illustration of latency components for DL and UL data.

## Latency analysis

### Rel. 15 Capability 2

Evaluation results are shown below for Rel. 15 capability #2 with a PDCCH periodicity of 5 symbols. Note that with the CCE limit per slot of 56 [4], it is allowed up to 3 PDCCH monitoring occasions per slot where each occasion contains at least one AL16 candidate.

Table 3. Capability 2 with PDCCH periodicity = 5 os

| Latency (ms) | HARQ | 15kHz SCS |  |  |  | 30kHz SCS |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 14-os TTI | 7-os TTI | 4-os TTI | 2-os TTI | 14-os TTI | 7-os TTI | 4-os TTI | 2-os TTI |
| DL data | 1st transmission | 2.50 | 1.64 | 1.21 | 1.07 | 1.34 | 0.91 | 0.70 | 0.62 |
|  | 1 retx | 6.50 | 3.64 | 2.57 | 2.07 | 3.34 | 1.91 | 1.52 | 1.27 |
|  | 2 retx | 10.50 | 5.64 | 3.86 | 3.07 | 5.34 | 2.91 | 2.34 | 1.95 |
|  | 3 retx | 14.50 | 7.64 | 5.21 | 4.07 | 7.34 | 3.91 | 3.02 | 2.45 |
| UL data (SR) | 1st transmission | 4.50 | 3.07 | 2.36 | 1.79 | 2.34 | 1.63 | 1.27 | 1.05 |
|  | 1 retx | 6.50 | 4.57 | 3.64 | 2.79 | 3.34 | 2.38 | 1.91 | 1.55 |
|  | 2 retx | 8.50 | 6.07 | 5.07 | 3.79 | 4.34 | 3.13 | 2.63 | 2.05 |
|  | 3 retx | 10.50 | 7.57 | 6.36 | 4.79 | 5.34 | 3.88 | 3.27 | 2.55 |
| UL data (Configured grant) | 1st transmission | 2.50 | 1.50 | 1.21 | 0.79 | 1.34 | 0.84 | 0.70 | 0.48 |
|  | 1 retx | 4.50 | 3.00 | 2.50 | 1.79 | 2.34 | 1.59 | 1.34 | 1.05 |
|  | 2 retx | 6.50 | 4.50 | 3.79 | 2.79 | 3.34 | 2.34 | 1.98 | 1.55 |
|  | 3 retx | 8.50 | 6.00 | 5.21 | 3.79 | 4.34 | 3.09 | 2.70 | 2.05 |

It is observed that with Rel. 15 capability #2 and PDCCH periodicity of 5 symbols, SR-based UL transmission with 15 kHz or 30 kHz SCS is not possible within 1 ms latency. Retransmission is also not possible within 1 ms for all transmission scenarios.

With Rel. 15 capability #2 with PDCCH periodicity of 5 symbols, retransmission is not possible within the 1ms target for 15 and 30 kHZ SCS.

With Rel. 15 capability #2 with PDCCH periodicity of 5 symbols, SR-based UL transmission is not possible within the 1ms target for 15 kHz and 30 kHz SCS.

With Rel. 15 capability #2 with PDCCH periodicity of 5 symbols, it is not possible to achieve DL or UL data latency as low as 0.5 ms for 15 and 30 kHz SCS.

### Rel. 16 potential

We now study the potential for latency improvement from a new capability (e.g. capability #3) for Rel. 16 with improved values of N1 and N2. Also, we consider PDCCH periodicity of 2 symbols as a consequence of potential improvement on the limits of number of blind decodes and CCEs.

Table 4. PDSCH processing time in OFDM symbols for the UE capabilities with only front-loaded DMRS.

| #Symbols | N1 |  |
| --- | --- | --- |
|  | 15kHz SCS | 30kHz SCS |
| Capability 2 | 3 | 4.5 |
| Capability 3 | 2.5 | 2.5 |

Table 5: PUSCH preparation procedure time.

| #Symbols | N2 |  |
| --- | --- | --- |
|  | 15kHz SCS | 30kHz SCS |
| Capability 2 | 5 | 5.5 |
| Capability 3 | 2.5 | 2.5 |

Table 4. Capability 3 with PDCCH periodicity = 2 os

| Latency (ms) | HARQ | 15kHz SCS |  |  |  | 30kHz SCS |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 14-os TTI | 7-os TTI | 4-os TTI | 2-os TTI | 14-os TTI | 7-os TTI | 4-os TTI | 2-os TTI |
| DL data | 1st transmission | 2.32 | 1.39 | 0.96 | 0.68 | 1.16 | 0.70 | 0.48 | 0.34 |
|  | 1 retx | 6.32 | 3.39 | 2.11 | 1.39 | 3.16 | 1.70 | 1.05 | 0.70 |
|  | 2 retx | 10.32 | 5.39 | 3.11 | 2.11 | 5.16 | 2.70 | 1.55 | 1.05 |
|  | 3 retx | 14.32 | 7.39 | 4.11 | 2.82 | 7.16 | 3.70 | 2.05 | 1.41 |
| UL data (SR) | 1st transmission | 4.32 | 2.39 | 1.89 | 1.18 | 2.16 | 1.20 | 0.95 | 0.59 |
|  | 1 retx | 6.32 | 3.39 | 2.89 | 1.75 | 3.16 | 1.70 | 1.45 | 0.88 |
|  | 2 retx | 8.32 | 4.39 | 3.89 | 2.32 | 4.16 | 2.20 | 1.95 | 1.16 |
|  | 3 retx | 10.32 | 5.39 | 4.89 | 2.89 | 5.16 | 2.70 | 2.45 | 1.45 |
| UL data (Configured grant) | 1st transmission | 2.32 | 1.32 | 1.04 | 0.61 | 1.16 | 0.66 | 0.52 | 0.30 |
|  | 1 retx | 4.32 | 2.32 | 2.04 | 1.18 | 2.16 | 1.16 | 1.02 | 0.59 |
|  | 2 retx | 6.32 | 3.32 | 3.04 | 1.75 | 3.16 | 1.66 | 1.52 | 0.88 |
|  | 3 retx | 8.32 | 4.32 | 4.04 | 2.32 | 4.16 | 2.16 | 2.02 | 1.16 |

With proposed values for potential Rel. 16 capability #3 UE processing time and PDCCH periodicity of 2 symbols, SR-based UL transmission with 30 kHz SCS is now possible within 1 ms latency. Moreover, one to two retransmissions become possible within 1 ms for DL, UL, and configured grant UL transmissions.

With potential Rel. 16 UE capability #3 and improved PDCCH periodicity of 2 symbols, one retransmission is possible within the 1ms target for DL, UL, and UL SPS transmissions with 30 kHz SCS.

With potential Rel. 16 UE capability #3 and improved PDCCH periodicity of 2 symbols, SR-based UL transmission is possible within the 1ms target for 30 kHz SCS.

With potential Rel. 16 UE capability #3 and improved PDCCH periodicity of 2 symbols, DL or UL data latency as low as 0.5 ms can be achieved for 30 kHz SCS.

While only SCS 15 and 30 kHz are evaluated here, 60 and 120 kHz are even better for achieving low latency. Hence Rel-16 UE Capability #3 should consider 60 kHz and 120 kHz also [5].

Introduce a new UE capability #3 with improved N1 and N2 values for 15/30/60/120 kHz SCS in NR Rel. 16 targeting URLLC.

Increase limits of blind decodes and CCEs for channel estimations to support more frequent PDCCH monitoring periodicity with reasonable PDCCH scheduling flexibility in NR Rel. 16 targeting URLLC.

# Conclusion

In section 2 we made the following observations:

Observation 1	With Rel. 15 capability #2 with PDCCH periodicity of 5 symbols, retransmission is not possible within the 1ms target for 15 and 30 kHZ SCS.

Observation 2	With Rel. 15 capability #2 with PDCCH periodicity of 5 symbols, SR-based UL transmission is not possible within the 1ms target for 15 kHz and 30 kHz SCS.

Observation 3	With Rel. 15 capability #2 with PDCCH periodicity of 5 symbols, it is not possible to achieve DL or UL data latency as low as 0.5 ms for 15 and 30 kHz SCS.

Observation 4	With potential Rel. 16 UE capability #3 and improved PDCCH periodicity of 2 symbols, one retransmission is possible within the 1ms target for DL, UL, and UL SPS transmissions with 30 kHz SCS.

Observation 5	With potential Rel. 16 UE capability #3 and improved PDCCH periodicity of 2 symbols, SR-based UL transmission is possible within the 1ms target for 30 kHz SCS.

Observation 6	With potential Rel. 16 UE capability #3 and improved PDCCH periodicity of 2 symbols, DL or UL data latency as low as 0.5 ms can be achieved for 30 kHz SCS.

Based on the discussion in section 2 we have the following proposals:

Proposal 1	Introduce a new UE capability #3 with improved N1 and N2 values for 15/30/60/120 kHz SCS in NR Rel. 16 targeting URLLC.

Proposal 2	Increase limits of blind decodes and CCEs for channel estimations to support more frequent PDCCH monitoring periodicity with reasonable PDCCH scheduling flexibility in NR Rel. 16 targeting URLLC.

# References

RP-181477 New SID on Physical Layer Enhancements for NR URLLC, Huawei, HiSilicon, Nokia, Nokia Shanghai Bell

Chairman’s note RAN1 #93

3GPP TS 38.331, Radio Resource Control (RRC) protocol specification, V15.2.0

3GPP TS 38.213, Physical layer procedures for control, V15.2.0

R1-1808147	Enhancements to Uplink and Downlink Physical Channels for NR URLLC	Ericsson