3GPP TSG RAN WG1 Meeting #94			R1-1809166

Gothenburg, Sweden, August 20th – 24th, 2018

Source:	NTT DOCOMO, INC.

Title:	Updated evaluation results of Indoor Hotspot-eMBB for IMT-2020 self-evaluation

Agenda Item:	7.2.7.2

Document for: 	Discussion

- Introduction

At the RAN1#92bis meeting, the eMBB evaluation method and parameters for FR2 have been agreed [1]. Based on the agreed evaluation assumptions on FR2, we present the updated evaluation results of Indoor Hotspot-eMBB Config A, Config B and Config C for both downlink and uplink.

- ITU Requirements

The minimum requirement for the 5th percentile user spectral efficiency and average spectral efficiency for Indoor Hotspot-eMBB sceanrio is provided as follow [2].

TABLE 1

5th percentile user spectral efficiency

| Test environment | Downlink (bit/s/Hz) | Uplink (bit/s/Hz) |
| --- | --- | --- |
| Indoor Hotspot – eMBB | 0.3 | 0.21 |

TABLE 2

Average spectral efficiency

| Test environment | Downlink (bit/s/Hz/TRxP) | Uplink (bit/s/Hz/TRxP) |
| --- | --- | --- |
| Indoor Hotspot – eMBB | 9 | 6.75 |

The defination of area traffic capacity is also defined in [2]. Area traffic capacity is the total traffic throughput served per geographic area (in Mbit/s/m2). The throughput is the number of correctly received bits, i.e. the number of bits contained in the SDUs delivered to Layer 3, over a certain period of time.

This can be derived for a particular use case (or deployment scenario) of one frequency band and one TRxP layer, based on the achievable average spectral efficiency, network deployment (e.g., TRxP (site) density) and bandwidth. Let W denote the channel bandwidth and ρ the TRxP density (TRxP/m2). The area traffic capacity Carea is given by:

Carea = ρ × W × SEavg                                                               (1)

The minimum requirement of area traffic capacity is listed as follow:

TABLE 3

Area traffic capacity

| Test environment | Downlink (Mbit/s) |
| --- | --- |
| Indoor Hotspot – eMBB | 10 |

- Evaluation Results of Spectral Efficiency

3.1 Downlink Spectral Efficiency

In this section, we update the downlink results of Indoor Hotspot–eMBB Config B/C, which are enhancend by multi-pannel combination at UE side. The detailed evaluation assumptions can be found in Annex.

The evaluation results of DL average spectral efficiency and DL 5th percentile spectral efficiency are summarized in Table 4, Table 5 and Table 6. The area traffic capacity results are provided based on results of the DL average spectral efficiency.

TABLE 4

Average spectral efficiency and 5th percentile user spectral efficiency for Indoor Hotspot–eMBB Config A

| Test environment | Antenna configuration | SEavg (bit/s/Hz/TRxP) | ITU Requirement of SEavg (bit/s/Hz/TRxP) | SEuser (bit/s/Hz) | ITU Requirement of SEuser (bit/s/Hz) |
| --- | --- | --- | --- | --- | --- |
| 12 TRxPs | 32/4 AE for TRxP/UE 32/4 TXRU for TRxP/UE | 12.230 | 9 | 0.4880 | 0.3 |
| 36 TRxPs | 128/4 AE for TRxP/UE 32/4 TXRU for TRxP/UE | 14.08 | 9 | 0.4548 | 0.3 |

TABLE 5

Average spectral efficiency and 5th percentile user spectral efficiency for Indoor Hotspot–eMBB Config B

| Test environment | Antenna configuration | SEavg (bit/s/Hz/TRxP) | ITU Requirement of SEavg (bit/s/Hz/TRxP) | SEuser (bit/s/Hz) | ITU Requirement of SEuser (bit/s/Hz) |
| --- | --- | --- | --- | --- | --- |
| 12 TRxPs | 64/32 AE for TRxP/UE 8/16 TXRU for TRxP/UE | 11.3537 | 9 | 0.5707 | 0.3 |
| 12 TRxPs | 64/32 AE for TRxP/UE 32/32 TXRU for TRxP/UE | 19.7189 | 9 | 0.7489 | 0.3 |

TABLE 6

Average spectral efficiency and 5th percentile user spectral efficiency for Indoor Hotspot–eMBB Config C

| Test environment | Antenna configuration | SEavg (bit/s/Hz/TRxP) | ITU Requirement of SEavg (bit/s/Hz/TRxP) | SEuser (bit/s/Hz) | ITU Requirement of SEuser (bit/s/Hz) |
| --- | --- | --- | --- | --- | --- |
| 12 TRxPs | 256/32 AE for TRxP/UE 8/16 TXRU for TRxP/UE | 10.0323 | 9 | 0.5305 | 0.3 |
| 12 TRxPs | 256/32 AE for TRxP/UE 32/32 TXRU for TRxP/UE | 13.7262 | 9 | 0.6318 | 0.3 |

Based on above evaluation results, we have following observation.

Observation 1: The minimum requirement for DL 5th percentile user spectral efficiency and DL average spectral efficiency can be met for Indoor Hotspot-eMBB configuration A, B and C.

3.2 Uplink Spectral Efficiency

In this section, we provide the uplink results of Indoor Hotspot–eMBB Config A/B/C. The detailed evaluation assumptions can be found in Annex.

The evaluation results of UL average spectral efficiency and UL 5th percentile spectral efficiency are summarized in Table 7, Table 8 and Table 9. It is noted that, in some antenna configurations, there are 8 TxRU for UE in two panels. UE only utilizes 4 TxRU in one panel for data transmission after panel selection.

TABLE 7

Average spectral efficiency and 5th percentile user spectral efficiency for Indoor Hotspot–eMBB Config A

| Test environment | Antenna configuration | SEavg (bit/s/Hz/TRxP) | ITU Requirement (bit/s/Hz/TRxP) | SEuser (bit/s/Hz) | ITU Requirement (bit/s/Hz) |
| --- | --- | --- | --- | --- | --- |
| 12 TRxPs | 32/4 AE for TRxP/UE 32/4 TXRU for TRxP/UE | 7.0695 | 6.75 | 0.3810 | 0.21 |

TABLE 8

Average spectral efficiency and 5th percentile user spectral efficiency for Indoor Hotspot–eMBB Config B

| Test environment | Antenna configuration | SEavg (bit/s/Hz/TRxP) | ITU Requirement (bit/s/Hz/TRxP) | SEuser (bit/s/Hz) | ITU Requirement (bit/s/Hz) |
| --- | --- | --- | --- | --- | --- |
| 12 TRxPs | 64/32 AE for TRxP/UE 8/8 TXRU for TRxP/UE | 7.1343 | 6.75 | 0.4382 | 0.21 |

TABLE 9

Average spectral efficiency and 5th percentile user spectral efficiency for Indoor Hotspot–eMBB Config C

| Test environment | Antenna configuration | SEavg (bit/s/Hz/TRxP) | ITU Requirement (bit/s/Hz/TRxP) | SEuser (bit/s/Hz) | ITU Requirement (bit/s/Hz) |
| --- | --- | --- | --- | --- | --- |
| 12 TRxPs | 256/32 AE for TRxP/UE 8/8 TXRU for TRxP/UE | 7.4218 | 6.75 | 0.4417 | 0.21 |

Based on above evaluation results, we have following observation.

Observation 2: The minimum requirement for UL 5th percentile user spectral efficiency and UL average spectral efficiency can be met for Indoor Hotspot-eMBB configuration A, B and C.

- Evaluation Results of Area Traffic Capacity

In this section, we will analyze the minimum bandwidth to meet the ITU requirement on area traffic capacity based on initial results of the DL average spectral efficiency in Table 10, Table 11 and Table 12.

TABLE 10

Area traffic capacity for Config A

| Test environment | Antenna configuration | Bandwidth, W (MHz) | Area traffic capacity (Mbit/s) | ITU requirement (Mbit/s) |
| --- | --- | --- | --- | --- |
| 12 TRxPs | 32/4 AE for TRxP/UE 32/4 TXRU for TRxP/UE | 400 | 9.78 | 10 |
|  |  | 450 | 11.0 | 10 |
| 36 TRxPs | 128/4 AE for TRxP/UE 32/4 TXRU for TRxP/UE | 100 | 8.45 | 10 |
|  |  | 150 | 12.67 | 10 |

TABLE 11

Area traffic capacity for Config B

| Test environment | Antenna configuration | Bandwidth, W (MHz) | Area traffic capacity (Mbit/s) | ITU requirement (Mbit/s) |
| --- | --- | --- | --- | --- |
| 12 TRxPs | 64/32 AE for TRxP/UE 32/32 TXRU for TRxP/UE | 250 | 9.86 | 10 |
|  |  | 300 | 11.83 | 10 |

TABLE 12

Area traffic capacity for Config C

| Test environment | Antenna configuration | Bandwidth, W (MHz) | Area traffic capacity (Mbit/s) | ITU requirement (Mbit/s) |
| --- | --- | --- | --- | --- |
| 12 TRxPs | 256/32 AE for TRxP/UE 32/32 TXRU for TRxP/UE | 350 | 9.61 | 10 |
|  |  | 400 | 10.98 | 10 |
|  |  | 450 | 12.35 | 10 |

Based on above results, we have following observations.

Observation 3: The requirement on area traffic capacity can be met for Indoor Hotspot-eMBB configuration A, when the bandwidth is larger than 450MHz for 12 TRxPs and 150MHz for 36 TRxPs.

Observation 4: The requirement on area traffic capacity can be met for Indoor Hotspot-eMBB configuration B, when the bandwidth is larger than 300MHz for 12 TRxPs.

Observation 5: The requirement on area traffic capacity can be met for Indoor Hotspot-eMBB configuration C, when the bandwidth is larger than 400MHz for 12 TRxPs.

- Summary

In this contribution, we provide both the downlink and uplink results of Indoor Hotspot-eMBB Config B and C, including the DL/UL average spectral efficiency, the DL/UL 5th percentile user spectral efficiency and the area traffic capacity. The observations are summarized as follow:

Observation 1: The minimum requirement for DL 5th percentile user spectral efficiency and DL average spectral efficiency can be met for Indoor Hotspot-eMBB configuration A, B and C.

Observation 2: The minimum requirement for UL 5th percentile user spectral efficiency and UL average spectral efficiency can be met for Indoor Hotspot-eMBB configuration A, B and C.

Observation 3: The requirement on area traffic capacity can be met for Indoor Hotspot-eMBB configuration A, when the bandwidth is larger than 450MHz for 12 TRxPs and 150MHz for 36 TRxPs.

Observation 4: The requirement on area traffic capacity can be met for Indoor Hotspot-eMBB configuration B, when the bandwidth is larger than 300MHz for 12 TRxPs.

Observation 5: The requirement on area traffic capacity can be met for Indoor Hotspot-eMBB configuration C, when the bandwidth is larger than 400MHz for 12 TRxPs.

References

- R1-1805644, “Summary of discussion on eMBB evaluation method and parameters for FR2”, Huawei, April 2018.

- Report ITU-R M.2410, “Minimum requirements related to technical performance for IMT-2020 radio interface(s)”, Nov. 2017

Annex: Evaluation assumptions

| Indoor Hotspot – eMBB |  | Config A | Config B | Config C |
| --- | --- | --- | --- | --- |
| Carrier frequency for evaluation |  | 4 GHz | 30GHz | 70GHz |
| Total transmit power per TRxP |  | 21 dBm for 10MHz bandwdith | 20 dBm for 40 MHz bandwidth | 18 dBm for 40 MHz bandwidth |
| TRxP number per site |  | 1 | 1 | 1 |
| Number of antenna elements per TRxP |  | 32Tx/Rx, (M,N,P,Mg,Ng) = (4,4,2,1,1), (dH,dV) = (0.5, 0.5)λ +45°, -45° polarization | 64Tx/Rx= (M,N,P,Mg,Ng) = (4,8,2,1,1), (dH,dV) = (0.5, 0.5)λ +45°, -45° polarization | 256Tx/Rx = (M,N,P,Mg,Ng) = (8,16,2,1,1), (dH,dV) = (0.5, 0.5)λ +45°, -45° polarization |
| Number of TXRU per TRxP |  | 32TXRU, (Mp,Np,P,Mg,Ng) = (4,4,2,1,1) (1-to-1 mapping) | 8TXRU, (Mp,Np,P,Mg,Ng) = (2,2,2,1,1) 32TXRU,(Mp,Np,P,Mg,Ng) = (4,4,2,1,1) | 8TXRU, (Mp,Np,P,Mg,Ng) = (2,2,2,1,1) 32TXRU,(Mp,Np,P,Mg,Ng) = (4,4,2,1,1) |
| Number of UE antenna elements |  | 4Tx/Rx, (M,N,P,Mg,Ng) = (1,2,2,1,1), (dH,dV) = (0.5, N/A)λ 0°,90° polarization | 32Tx/Rx = (M,N,P,Mg,Ng) = (2,4 ,2,1,2), (dH,dV) = (0.5, 0.5)λ (dg,V,dg,H) = (0, 0)λ. Θmg,ng=90; Ω0,1=Ω0,0+180; 0°,90° polarization | 32Tx/Rx = (M,N,P,Mg,Ng) = (2,4 ,2,1,2), (dH,dV) = (0.5, 0.5)λ (dg,V,dg,H) = (0, 0)λ. Θmg,ng=90; Ω0,1=Ω0,0+180; 0°,90° polarization |
| Number of TXRU per UE |  | 4TXRU, (Mp,Np,P,Mg,Ng) = (1,2,2,1,1) (1-to-1 mapping) | 4TXRU,(Mp,Np,P,Mg,Ng) = (1,1,2,1,2) 8TXRU,(Mp,Np,P,Mg,Ng) = (1,2,2,1,2) 16TXRU,(Mp,Np,P,Mg,Ng) = (1,4,2,1,2) 32TXRU,(Mp,Np,P,Mg,Ng) = (2,4,2,1,2) | 4TXRU,(Mp,Np,P,Mg,Ng) = (1,1,2,1,2) 8TXRU,(Mp,Np,P,Mg,Ng) = (1,2,2,1,2) 16TXRU,(Mp,Np,P,Mg,Ng) = (1,4,2,1,2) 32TXRU,(Mp,Np,P,Mg,Ng) = (2,4,2,1,2) |
| Traffic model |  | Full buffer | Full buffer | Full buffer |
| UE density |  | 10 UEs per TRxP |  |  |
| UT attachment |  | Based on RSRP (formula (8.1-1) in TR36.873) from port 0 | Based on RSRP (formula) from port 0 The UE panel with the best receive SNR is chosen. i.e. no combining is done between panels. | Based on RSRP (formula) from port 0 The UE panel with the best receive SNR is chosen. i.e. no combining is done between panels. |
| Wrapping around method |  | Geographical distance based wrapping |  |  |
| Polarized antenna model |  | Model-2 in TR36.873 |  |  |
| Scheduler |  | UL: SU PF (Max 12 layer) | UL: SU PF (Max 4 layer) DL: SU PF (Max 8 layer) | UL:SU PF (Max 4 layer) DL:SU PF (Max 8 layer) |
| Subband number |  | 10 | 5 | 5 |
| Overhead uplink | PDCCH | 14 OS, 4PRB for FDD | 14 OS, 4PRB for FDD | 14 OS, 4PRB for FDD |
|  | DMRS (Type II) | 6, 12 ports |  |  |
|  | SRS(expect 4RBs for PUCCH) | 2 OFDM symbols per 5 slots |  |  |
|  | PT-RS | N.A | time density is 4 OS, frequency density is 4 PRB | time density is 4 OS, frequency density is 4 PRB |
| Overhead downlink | SS/PBCH block | 1 SS/PBCH block per 20ms | 24 SS/PBCH block per 20ms | 24 SS/PBCH block per 20ms |
|  | PDCCH | 2 OFDM symbols per slot | 0.5 OFDM symbols per slot | 0.5 OFDM symbols per slot |
|  | DMRS (Type II) | 4, 8, 12 ports |  |  |
|  | CSI-RS | 4, 8, 16, 24, 32 ports with periodicity of 5ms |  |  |
|  | TRS | 4 CSI-RS resource in 2 consecutive slots per 20ms, 50PRB |  |  |
|  | PT-RS | N.A. | time desity is 4OS, frequency density is 4PRB | time desity is 4OS, frequency density is 4PRB |
| Receiver |  | Ideal-IRC |  |  |
| CSI feedback |  | Ideal |  |  |
| Channel estimation |  | Ideal |  |  |