3GPP TSG-RAN WG1 Meeting #94	R1-1809277

Gothenburg, Sweden, August 20th  – 24th , 2018				         (Resubmission of R1-1806442)

Agenda Item:	7.2.7.2

Source:	Ericsson

Title:	IMT-2020 self-evaluation: UP latency in NR

Document for:	Information

# Introduction

The ITU target for user plane (UP) latency in IMT 2020 has been set to 1ms [1] for URLLC, and 4ms for eMBB. In this paper, we make an evaluation of the UP latency in NR with different configurations, and show that the targets can be reached in both FDD and TDD. In the ITU target for reliability there is also a requirement on latency (1ms) during which the packet should be delivered with a certain probability. For this case, it is useful to evaluate how many retransmissions can be done within the latency limit.

# Discussion

In the following we will analyze the worst-case UP latency after a first transmission and up to 3 retransmissions. We will follow the ITU definition [1] of UP latency as being defined from L2/L3 ingress to L2/L3 egress.

## Assumptions

Processing delay

Processing delay is delay caused at TX by preparation of transmission and at RX by reception procedures and decoding. For downlink the processing delay in the UE includes the reception and decoding procedure. For uplink there is also processing delay in UE due to reception and decoding of the uplink grant. In gNB there is also processing delay as in the UE, with the addition that processing delay in gNB also comprise delay caused by scheduling.

Alignment delay

The alignment delay is the time required after being ready to transmit until transmission can start. We assume the worst-case latency meaning the alignment delay is assumed to the longest possible. PDCCH opportunities and PUCCH opportunities are assumed to be every scheduled TTI.

gNB timing

The minimum response timing in the gNB between SR and UL grant, and between DL HARQ and retransmission, is assumed to be 1 TTI. For higher SCS and fewer symbols in the mini-slot, the TTI is shorter, and more TTIs should be used for processing. The processing in gNB consists of three main components:

- Reception processing (PUSCH processing, SR/HARQ-ACK processing)

- Scheduling processing (including SDU/PDU processing for DL)

- L1 preparation processing for PDSCH and PDCCH

For simplicity we refer to gNB processing time as the total processing time and further that the processing time is equal for the cases that can occur. For example, the same processing time is assumed for scheduling first transmission and re-transmission. Same processing time is also assumed for DL and UL. The processing time is a lower limit for gNB response time, where the assumptions on gNB processing time are given in Table 1.

UE timing

The minimum response timing in the UE between DL data and DL HARQ, and between UL grant and UL data. In DL the UE processing time is according to N1 value (Table 2) while in UL the UE processing time is according to N2 value (Table 3) for UE capability #2.

UL scheduling

For UL data, the scheduling can either be based on SR or SPS UL. We assume that SR periodicity is 2os corresponding to the shortest periodicity allowed.

Table 1. Processing time (in # of OFDM symbols) assumptions for gNB.

| Timing | 15/30kHz SCS |  |  | 120kHz SCS |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| #Symbols | 7os TTI | 4os TTI | 2os TTI | 7os TTI | 4os TTI | 2os TTI |
| gNB processing | 7 | 4 | 4 | 14 | 12 | 10 |

Table 2. PDSCH processing time in OFDM symbols for the UE capabilities with front-loaded DMRS.

| #Symbols | N1 |  |  |
| --- | --- | --- | --- |
|  | 15kHz SCS | 30kHz SCS | 120kHz SCS |
| Capability 2 | 3 | 4.5 | 20 |

Table 3: PUSCH preparation procedure time.

| #Symbols | N2 |  |  |
| --- | --- | --- | --- |
|  | 15kHz SCS | 30kHz SCS | 120kHz SCS |
| Capability 2 | 5 | 5.5 | 36 |

TTI length and pattern

In this evaluation, we study slot lengths of 14 symbols as well as mini-slots of 7, 4, and 2 symbols. For TDD an alternating DL-UL pattern has been assumed, to represent the most latency-optimized setup in a carrier. With TDD slot/mini-slots of 14, 7, and 4 symbols are studied.

Figure 1: illustration of latency components for DL and UL data.

## FDD

For the case of FDD the HARQ RTT is n+k TTI according to Table 1. The resulting UP latency for SCS of 15, 30 and 120 kHz is shown in Table 4. As can be seen, the 1ms requirement can be reached for SCS 15kHz and up depending on mini-slot configuration. In UL configured grants reduces the latency considerably compared to SR-based scheduling.

Table 4: FDD UP one-way latency for data transmission with HARQ-based retransmission, compared to the 1ms (green) and 4ms (pink) requirements.

| Latency (ms) | HARQ | 15kHz SCS |  |  |  | 30kHz SCS |  |  |  | 120kHz SCS |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 14-os TTI | 7-os TTI | 4-os TTI | 2-os TTI | 14-os TTI | 7-os TTI | 4-os TTI | 2-os TTI | 14-os TTI | 7-os TTI | 4-os TTI | 2-os TTI |
| DL data | 1st transmission | 3.2 | 1.7 | 1.3 | 0.86 | 1.7 | 0.91 | 0.7 | 0.48 | 0.55 | 0.43 | 0.38 | 0.31 |
|  | 1 retx | 6.2 | 3.2 | 2.6 | 1.7 | 3.1 | 1.6 | 1.3 | 0.96 | 1.1 | 0.87 | 0.76 | 0.63 |
|  | 2 retx | 9.2 | 4.7 | 3.6 | 2.6 | 4.7 | 2.4 | 2 | 1.5 | 1.6 | 1.3 | 1.1 | 0.96 |
|  | 3 retx | 12 | 6.2 | 4.6 | 3.4 | 6.1 | 3.1 | 2.7 | 2 | 2.1 | 1.7 | 1.5 | 1.3 |
| UL data (SR) | 1st transmission | 5.5 | 3 | 2.5 | 1.8 | 2.8 | 1.5 | 1.3 | 0.93 | 1.2 | 1.1 | 1 | 0.89 |
|  | 1 retx | 9.4 | 4.9 | 3.9 | 2.6 | 4.7 | 2.4 | 2 | 1.4 | 1.9 | 1.7 | 1.6 | 1.3 |
|  | 2 retx | 12 | 6.4 | 4.9 | 3.5 | 6.2 | 3.2 | 2.6 | 1.9 | 2.6 | 2.3 | 2.1 | 1.8 |
|  | 3 retx | 15 | 7.9 | 5.9 | 4.4 | 7.7 | 3.9 | 3.3 | 2.3 | 3.2 | 2.8 | 2.6 | 2.2 |
| UL data (CG) | 1st transmission | 3.4 | 1.9 | 1.4 | 0.93 | 1.7 | 0.95 | 0.7 | 0.48 | 0.7 | 0.57 | 0.52 | 0.45 |
|  | 1 retx | 6.4 | 3.4 | 2.6 | 1.8 | 3.2 | 1.7 | 1.4 | 0.93 | 1.3 | 1.1 | 1.1 | 0.89 |
|  | 2 retx | 9.4 | 4.9 | 3.9 | 2.6 | 4.7 | 2.4 | 2 | 1.4 | 1.9 | 1.7 | 1.6 | 1.3 |
|  | 3 retx | 12 | 6.4 | 4.9 | 3.5 | 6.2 | 3.2 | 2.6 | 1.9 | 2.6 | 2.3 | 2.1 | 1.8 |

NR FDD can fulfill the 4ms UP latency target with 15kHz SCS.

NR FDD can fulfill the 1ms UP latency target with 15kHz SCS, mini-slots, and UL configured grants.

## TDD

With TDD there are additional alignment delays caused by the sequence of DL and UL slots. Depending on when the data arrives in the transmit buffer the latency may be same or longer than the FDD latency. For a DL-UL pattern with HARQ RTT of n+4 TTI and higher (again following Table 1), the resulting latency is as indicated in Table 5. As can be seen in the table, the 4ms target can be reached with a SCS of 15kHz for 7-symbol mini slot, while 30 kHz SCS is possible also with slot length transmission. The 1ms target can be reached with 120kHz SCS and mini-slots for DL and UL configured grant transmissions.

Table 5. TDD UP one-way latency for data transmission with alternating DL-UL slot pattern, compared to the 1ms (green) and 4ms (pink) requirements.

| Latency (ms) | HARQ | 15kHz SCS |  |  | 30kHz SCS |  |  | 120kHz SCS |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 14-os TTI | 7-os TTI | 4-os TTI | 14-os TTI | 7-os TTI | 4-os TTI | 14-os TTI | 7-os TTI | 4-os TTI |
| DL data | 1st transmission | 4.2 | 2.7 | 2.3 | 2.2 | 1.4 | 1.2 | 0.68 | 0.55 | 0.51 |
|  | 1 retx | 8.2 | 4.7 | 4.3 | 4.1 | 2.4 | 2.2 | 1.4 | 1.1 | 1 |
|  | 2 retx | 12 | 6.7 | 6.3 | 6.2 | 3.4 | 3.2 | 2.2 | 1.6 | 1.5 |
|  | 3 retx | 16 | 8.7 | 8.3 | 8.1 | 4.4 | 4.2 | 2.9 | 2.1 | 2 |
| UL data (SR) | 1st transmission | 7.5 | 4.5 | 4.1 | 3.8 | 2.3 | 2.1 | 1.5 | 1.2 | 1.2 |
|  | 1 retx | 12 | 6.9 | 6.4 | 6.2 | 3.4 | 3.2 | 2.3 | 1.9 | 1.7 |
|  | 2 retx | 16 | 8.9 | 8.4 | 8.2 | 4.5 | 4.2 | 3.1 | 2.5 | 2.2 |
|  | 3 retx | 20 | 11 | 10 | 10 | 5.4 | 5.2 | 3.8 | 3.2 | 2.7 |
| UL data (CG) | 1st transmission | 4.4 | 2.9 | 2.4 | 2.2 | 1.4 | 1.2 | 0.82 | 0.7 | 0.64 |
|  | 1 retx | 8.4 | 4.9 | 4.4 | 4.2 | 2.5 | 2.2 | 1.6 | 1.3 | 1.2 |
|  | 2 retx | 12 | 6.9 | 6.4 | 6.2 | 3.4 | 3.2 | 2.3 | 1.9 | 1.7 |
|  | 3 retx | 16 | 8.9 | 8.4 | 8.2 | 4.5 | 4.2 | 3.1 | 2.5 | 2.2 |

NR TDD can fulfil the 4ms UP latency target with 15 kHz SCS, mini-slot and configured UL grants.

NR TDD can fulfil the 1ms UP latency target with 120 kHz SCS, mini-slots and configured UL grants.

# Conclusion

In section 2 we made the following observations:

- NR FDD can fulfill the 4ms UP latency target with 15kHz SCS.

NR FDD can fulfill the 1ms UP latency target with 15kHz SCS, mini-slots, and UL configured grants.

NR TDD can fulfil the 4ms UP latency target with 15 kHz SCS, mini-slot and configured UL grants.

NR TDD can fulfil the 1ms UP latency target with 120 kHz SCS, mini-slots and configured UL grants.

From the above we conclude that:

NR fulfills the IMT-2020 requirements on latency.

# References

[1]	5D/TEMP/216-E, Preliminary draft new report ITU-R M., [IMT-2020.TECH PERF REQ], ITU-R Working Party 5D, (WP5D-TD-0300)

In NR Rel. 15 no value (lower than for Capability 1) for 120 kHz SCS was agreed.

In NR Rel. 15 no value (lower than for Capability 1) for 120 kHz SCS was agreed.