3GPP TSG RAN WG1 Meeting #94	R1-1809565

Gothenburg, Sweden, August 20 - 24, 2018

Agenda Item:	6.1.4.2

Source:	Huawei, HiSilicon

Title:	Feature lead summary of Maintenance for Early data transmission in eMTC

Document for:	Discussion and decision

WID objective

Improved latency: - Support early data transmission [RAN2 lead, RAN1, RAN3] - Evaluate power consumption/latency gain and specify necessary support for DL/UL data transmission on a dedicated resource during the Random Access procedure (after PRACH transmission and before the RRC connection setup is completed) at least in the RRC Suspend/Resume case.

Background

Early data transmission during RACH for Rel-15 eMTC has been discussed in Rel-15 and was concluded completed in RAN1#93. The related agreements are attached in the Appendix for reference. In RAN1#94, three contributions are proposed in the maintenance session of Release 15 EDT for eMTC.

Summary of proposals on maintenance of EDT for eMTC

Missing specification of number of PUSCH repetitions for EDT Msg3 retransmissions

In RAN1#93, it was agreed as following that the repetition number indicated in the DCI scheduling retransmission of EDT Msg3is based on the configured maximum TBS. However, there is no explicit agreement on how to scaling the repetition number of retransmission of EDT Msg3.

Agreement Msg3 retransmission with EDT uses the same TBS as the initial transmission assuming repetition number indicated in the DCI for configured maximum TBS;

Therefore, 36.213 V15.2.0 section 6.2 specifies the number of PUSCH repetitions for the initial EDT Msg3 transmission, but there is no corresponding specification text for the number of PUSCH repetitions for the EDT Msg3 retransmissions.

It is proposed in R1-1808985 to specify the number of PUSCH repetition for EDT Msg3 retransmissions.

In 36.213, specify the number of PUSCH repetition for EDT Msg3 retransmissions.

- It is suggested to agree the following potential proposal:

Potential proposal#1: In 36.213, specify the number of PUSCH repetition for EDT Msg3 retransmissions, similarly as that in 36.213 for NB-IoT.

New PUSCH repetition numbers for EDT Msg3

In both R1-1808985 and R1-1808624, it is proposed to introduce new repetition number for the scaled repetition for EDT, rather than using legacy repetition in Rel-14 only.

In R1-1808985, the proposal is as following in the table:

In 36.213, specify a finer granularity for the down-scaling of the number of PUSCH repetitions for EDT Msg3, for example in steps of Nacc, i.e. 1 subframe in CE mode A and 4 or 5 subframes in CE mode B (for FDD and TDD, respectively).

In R1-1808624, the following proposal 1 is proposed and a corresponding text proposal is also proposed.

Proposal 1: If the scaled down repetition number is larger than 4, the actual repetition number can be scaled down proportionally to the nearest equal or higher repetition value which meets a multiple of 4. The example of TP in 36.213 for the proposal above is provided as below: -------------------------------------------Start of Text proposal---------------------- ## 6.2 Random Access Response Grant <unchanged part> The repetition number field in the random access response grant configured by higher layers indicates the repetition level () for the initial transmission of Msg3 PUSCH as given in Table 6.2-C for CEmodeA and Table 6.2-D for CEmodeB, where - is determined by higher layer parameter pusch-maxNumRepetitionCEmodeA-r13 if it is signaled, otherwise = 8, - is determined by higher layer parameter pusch-maxNumRepetitionCEmodeB-r13 if it is signaled, otherwise = 512. - If the higher layers indicate EDT to the physical layer as defined in [8] and if the UE is configured with higher layer parameter edt-smallTBS-Enabled-r15, the repetition number for the initial transmission of Msg3 PUSCH is the smallest integer number of M that is equal to or larger than where is the TBS of Msg3 PUSCH as determined in subclause 8.6.2, and is the value of the higher layer parameter edt-TBS-r15. M = 4 if > 4, M = 1 otherwise. <unchanged part> ----------------------------------- end of Text proposal-----------------------

- Considering it is beneficial from UE power consumption perspective, it is suggested to add new repetition numbers for EDT Msg3 transmission. Based on the input from the two contributions, it is suggested to consider the following potential proposals:

Potential proposal#2:

- For CE mode B, specify a finer granularity for the down-scaling of the number of PUSCH repetitions for EDT Msg3 by adopting the following TP.

- For CE mode A, further discuss whether finer granularity is introduced.

-------------------------------------------Start of Text proposal----------------------

6.2   Random Access Response Grant

<unchanged part>

The repetition number field in the random access response grant configured by higher layers indicates the repetition level () for the initial transmission of Msg3 PUSCH as given in Table 6.2-C for CEmodeA and Table 6.2-D for CEmodeB, where

-	 is determined by higher layer parameter pusch-maxNumRepetitionCEmodeA-r13 if it is signaled, otherwise  = 8,

-	 is determined by higher layer parameter pusch-maxNumRepetitionCEmodeB-r13 if it is signaled, otherwise  = 512.

- If the higher layers indicate EDT to the physical layer as defined in [8] and if the UE is configured with higher layer parameter edt-smallTBS-Enabled-r15, the repetition number for the initial transmission of Msg3 PUSCH is the smallest integer number of M  that is equal to or larger than where  is the TBS of Msg3 PUSCH as determined in subclause 8.6.2, and  is the value of the higher layer parameter edt-TBS-r15. M = 4 if > 4, M = 1 otherwise.

<unchanged part>

----------------------------------- end of Text proposal-----------------------

Discussion on fallback to legacy Msg3

In RAN1#93, it was agreed that the DCI format that schedules the retransmission of EDT Msg3 can indicate the UE to transmit legacy Msg3 as fallback.

Agreement

DCI for Msg3 re-transmission can instruct physical layer in UE by using one unused state of MCS to indicate EDT Msg. 3 retransmission and the rest to indicate to fallback legacy Msg. 3 transmission.

Note: Spec. editor to choose one unused state of MCS

Note: Whether to introduce higher layer support for this mechanism is up to RAN2.

It should be noticed that whether to adopt this feature in higher layers depends on RAN2’s decision. In R1-1808125, it refers to RAN2 discussion that it may impact the UE to introduce the fallback to legacy Msg3 transmission unless the timing relationships are defined sufficiently conservatively. To resolve RAN2’s concern, it is proposed in R1-1808125 as following that for EDT fallback to legacy Msg3, the minimum scheduling delay is relaxed to 12 ms.

Proposal 1: For EDT fallback to legacy Msg3, relax the minimum scheduling delay to be 12ms. Proposal 2: Adopt the TP in Annex A for TS 36.213. - Annex A. TP on EDT fallback -------------------------------------------- Unchanged parts omitted -------------------------------------- Table 8-2f: k for TDD configurations 0-6 TDD UL/DL Configuration; subframe number n. 0; 1; 2; 3; 4; 5; 6; 7; 8; 9. 0; 12; 12; 12; 12. 1; 12; 12; 12; 12. 2; 12; 12. 3; 12; 12. 4; 12; 12. 5; 12. 6; 12; 12; 12; 12. For BL/CE UEs, the higher layers indicate the set of BL/CE UL subframes according to fdd-DownlinkOrTddSubframeBitmapBR and fdd-UplinkSubframeBitmapBR [11]. A BL/CE UE shall upon detection on a given serving cell of an MPDCCH with DCI format 6-0A/6-0B intended for the UE, perform a corresponding PUSCH transmission in subframe(s) n+ki if a transport block corresponding to the HARQ process of the PUSCH transmission is generated as described in [8] with i = 0, 1, …, N-1 according to the MPDCCH, where - subframe n is the last subframe in which the MPDCCH is transmitted; and - x≤k0<k1<…,kN-1 and the value of is determined by the repetition number field in the corresponding DCI, where - if the UE is configured with higher layer parameter ce-pdsch-puschEnhancement-config with value ‘On’ are given by {1,2,4,8,12,16,24,32} - otherwise, are given in Table 8-2b and Table 8-2c; and - in case N>1, subframe(s) n+ki with i=0,1,…,N-1 are N consecutive BL/CE UL subframe(s) starting with subframe n+x, and in case N=1, k0=x; - for FDD, x=12 when the UE is configured with higher layer parameter edt-parameters-r15, and the uplink scheduling grant corresponding to the PUSCH transmission indicates a retransmission as part of the contention based random access procedure with , otherwise x = 4; - for TDD UL/DL configurations 1-6, or for TDD UL/DL configuration 0 and a BL/CE UE in CEModeB, when the UE is configured with higher layer parameter edt-parameters-r15, and the uplink scheduling grant corresponding to the PUSCH transmission indicates a retransmission as part of the contention based random access procedure with , the value of x is given as the value of k in Table 8-2f, otherwise the value of x is given as the value of k in Table 8-2 for the corresponding TDD UL/DL configuration; If the value x is not given in Table 8-2 for subframe n, denote subframe n’ as the first downlink/special subframe which has a value in Table 8-2 after subframe n, and substitute n with n’ in the above procedure for performing the PUSCH transmission. - for TDD UL/DL configuration 0 and a BL/CE UE in CEModeA and N=1, if the MSB of the UL index in the MPDCCH with DCI format 6-0A is set to 1, the value of x is given as the value of k in Table 8-2 for the corresponding TDD UL/DL configuration; if the LSB of the UL index in the MPDCCH with DCI format 6-0A is set to 1, x = 7. The UE is not expected to receive DCI format 6-0A with both the MSB and LSB of the UL index set to 1 when N>1. In case both the MSB and LSB of the UL index are set to 1, the HARQ process number of the PUSCH corresponding the MSB of the UL index is and the HARQ process number of the PUSCH corresponding the LSB of the UL index is , where is determined according to the HARQ process number field in DCI format 6-0A -------------------------------------------- Unchanged parts omitted -------------------------------------- ----------------------------------------------- End of Text Proposal ---------------------------------------

- It is suggested to further discuss the issue.

Discussion on collision issue due to EDT retransmission

In R1-1808125, another issue is raised. As shown in Figure 1, for EDT Msg3 retransmission eNB can assign resource based on the detected TBS rather than the broadcasted TBS. This enables eNB to reduce the resource overhead of EDT Msg3 retransmission, rather than always allocating the large maximum resource that can convey the maximum TBS.

However, as shown in the figure, the actual used TBS (TBSMsg3) may be different from the detected TBS (TBSMsg3,detected). When TBSMsg3,detected < TBSMsg3, the duration of PUSCH transmission by UE may be larger than the duration allocated by eNB. In this case, there is collision between EDT Msg3 retransmission and the transmission from other UEs. Similar issue is also discussed in NB-IoT and the details can be found in R1-1808129.

Figure 1 illustration of collision issue

To resolve the issue, R1-1808129 proposed to indicate the detected TBS in the DCI format N0 that schedules the EDT Msg3 retransmission. The corresponding proposals are attached as following.

Proposal 2: Indicate one of the up to 4 actual TBS candidates in DCI for scheduling EDT Msg3 retransmission. Proposal 3: In EDT Msg3 retransmission, if the indicated TBS in DCI( TBSindicated) is smaller than the TBS the UE selected (TBSMsg3), the UE uses TBSMsg3 to retransmit the Msg3 and scales repetition number by TBSMsg3, indicated/TBSMsg3, max instead of TBSMsg3,detected/TBSMsg3.

- It is suggested to discuss the issue.

Other corrections

The following corrections are also proposed in R1-1808985.

In 36.213 section 8.6.2, in Tables 8.6.2-1 and 8.6.2-2, clarify in the table headings that the tables concern the case when edt-smallTBS-Enabled-r15 is configured. In 36.213 section 8.6.2, in Tables 8.6.2-1 and 8.6.2-2, insert the two missing commas and the missing space character as shown below. 36.213 Table 8.6.2-1: EDT TBS for CEModeA with edt-smallTBS-Enabled-r15 configured. edt-TBS-r15; edt-SmallTBS-Subset-r15; Allowable TBS values. 408; not configured; 328, 408. 504; not configured; 328, 408, 456, 504. 504; enabled; 408, 504. 600; not configured; 328, 408, 504, 600. 600; enabled; 408, 600. 712; not configured; 328, 456, 600, 712. 712; enabled; 456, 712. 808; not configured; 328, 504, 712, 808. 808; enabled; 504, 808. 936; not configured; 328, 504, 712, 936. 936; enabled; 504, 936. 1000; not configured; 328, 536, 776, 1000. 1000; enabled; 536, 1000. 36.213 Table 8.6.2-2: EDT TBS for CEModeB with edt-smallTBS-Enabled-r15 configured. edt-TBS-r15; edt-SmallTBS-Subset-r15; Allowable TBS values. 408; not configured; 328, 408. 456; not configured; 328, 408, 456. 456; enabled; 408, 456. 504; not configured; 328, 408, 456, 504. 504; enabled; 408, 504. 600; not configured; 328, 408, 504, 600. 600; enabled; 408, 600. 712; not configured; 328, 456, 600, 712. 712; enabled; 456, 712. 808; not configured; 328, 504, 712, 808. 808; enabled; 504, 808. 936; not configured; 328, 504, 712, 936. 936; enabled; 504, 936. 36.213 section 6.2 refers to various tables related to RAR UL grant definitions. In the following paragraph, the text could be clarified by removing the colon sign after the highlighted word “otherwise”. - The content of these Nr bits starting with the MSB and ending with the LSB are given in Table 6-2 for CEmodeA and CEmodeB if the higher layers does not indicate EDT to the physical layer as defined in [8], Table 6.2-F otherwise: - where = and. Also, in both the above paragraph and the below paragraph, “higher layers does not indicate” should be corrected to “higher layers do not indicate”. - For CEmodeB, the Msg3 PUSCH narrowband index indicates the narrowband to be used for first subframe of Msg3 PUSCH transmission as given in Table 6.2-A if the higher layers does not indicate EDT to the physical layer as defined in [8], Table 6.2-G otherwise.. In 36.213 section 6.2, remove the colon sign after the text “Table 6.2-F otherwise”. In 36.213 section 6.2, in two places, change “higher layers does not indicate” to “higher layers do not indicate”.

- It seems not essential to change the title of Table 8.6.2-1 and Table 8.6.2-1;

- Other corrections on typos are suggested to be accepted.

- References

| No. | Tdoc | Type | Source | Title |
| --- | --- | --- | --- | --- |
| 1 | R1-1808125 | discussion | Huawei, HiSilicon | Corrections on EDT for eMTC |
| 2 | R1-1808624 | discussion | ZTE | Remaining issue for MTC EDT |
| 3 | R1-1808985 | discussion | Ericsson | Maintenance issues for Early data transmission for Rel-15 LTE-MTC |

- Appendix: Agreements regarding EDT in Rel-15

Agreements in RAN1#90bis - From RAN1 point of view, it is feasible to support early UL data transmission in Msg3 from a BL/CE UE using some TBS value(s) from the TBS range specified for BL/CE UEs in Rel-13 with a maximum total TBS of 1000 bits. - Note: For Msg3 for Rel-13 BL/CE UEs, the maximum total TBS is 712 bits in CE mode A and 328 bits in CE mode B. - FFS if and how there will also be a larger supported maximum total TBS (than 1000 bits) - The detailed value(s) should consider the payload size of early data packets from RAN2. - From RAN1 perspective, the physical layer design will assume eNB is not required to always provide a grant of a larger TBS for Msg3 and can decide to just provide a grant corresponding to Rel-13 Msg3 TBS instead. Agreements in RAN1#91 - Maximum TBS for early data transmission in Msg3 is 1000 bits for PRACH CE levels 0 and 1 and 936 bits for PRACH CE levels 2 and 3 - Ask RAN2 whether one reserved bit in MAC RAR can be used for EDT feature Agreement in RAN1#92 - The maximum TBS broadcasted in system information are selected from 8 values which are taken from the Rel-13 PUSCH tables. - The up to 4 possible TBS which is smaller than or equal to the maximum broadcast TBS values for the UE to choose among are FFS. FFS: How the UE obtains the up to 4 possible values. - Support NW enabling the use of TBS smaller than the maximum configured. FFS details. Agreement in RAN1#92Bis - For EDT scheduled by RAR grant, - For CE Mode A, 2–6 PRBs are supported with 4 bits resource allocation field. - Whether to support 1 PRB can be reconsidered if there is an available spare bit to expand resource allocation field. - For CE Mode B, 1-2 PRBs are supported with at least 3 bits resource allocation field. - Resource allocation may or may not be modified, picking at least 8 resource allocations. - Whether to expand resource allocation can be reconsidered if there is an available spare bit to expand resource allocation field. - The 8 maximum TBS values in CE mode A (i.e. PRACH levels 0 and 1) are {328 408 504 600 712 808 936 1000}. - Only values of NPRB and values of TBS in legacy Rel-13 PUSCH table can be used for EDT - The 8 maximum TBS values in CE mode B (i.e. PRACH levels 2 and 3) are {328 408 456 504 600 712 808 936}. - Only values of NPRB and values of TBS in legacy Rel-13 PUSCH table can be used for EDT - For each of the 8 maximum TBS values - The number of repetitions changes depending on the actual TBS, potentially including numbers of repetitions which are not included in legacy PUSCH repetition numbers (FFS how) - For EDT Msg. 3 transmission, 16QAM modulation is not supported. - The use of TBS smaller than the maximum configured is configured per CE level in SIB. - Per cell, in the below, Ti < Ti+1, eNB can configure that the UE chooses from: - When there are 4 permitted actual transmitted TBS {T1, T2, T3, T4} - T2 or T4 - T1, or T2, or T3, or T4 - When there are 3 permitted actual transmitted TBS {T1, T2, T3} - T2 or T3 - T1 or T2 or T3 - When there are 2 permitted actual transmitted TBS {T1, T2} - T1 or T2 - The possible smaller TBS values are predefined from the configured maximum TBS and the configured maximum number of blind decodes. Email agreement on exact table Table 1: Exact TBS table on EDT, CEModeA 328; 408; 504; 600; 712; 808; 936; 1000. T1; 328; 328; 328; 328; 328; 328; 328; 328. T2; 408; 408; 408; 456; 504; 504; 536. T3; 456; 504; 600; 712; 712; 776. T4; 504; 600; 712; 808; 936; 1000. Table 2: Exact TBS table on EDT, CEModeB 328; 408; 456; 504; 600; 712; 808; 936. T1; 328; 328; 328; 328; 328; 328; 328; 328. T2; 408; 408; 408; 408; 456; 504; 504. T3; 456; 456; 504; 600; 712; 712. T4; 504; 600; 712; 808; 936. - Use the ‘R’ bit in RAR to differentiate EDT and non-EDT. Agreement in RAN1#93 Agreement For CEModeA, 5 bits resource allocation to support 1~6PRBs in RAR UL grant for EDT. For CEModeB, reuse Rel-13 legacy resource allocation in RAR UL grant for EDT. Agreement Scaled down proportionally to the nearest equal or higher legacy repetition number. - New number can be considered during CR phase. Agreement For UL grant in MAC RAR scheduling EDT: - In CEModeA, bits for any Msg3/4 MPDCCH narrowband index in CEModeA. - In CEModeB, 3 bits for Msg3/4 MPDCCH narrowband index, and 3 bits for Msg3 PUSCH narrowband index. Agreement DCI for Msg3 re-transmission can instruct physical layer in UE by using one unused state of MCS to indicate EDT Msg. 3 retransmission and the rest to indicate to fallback legacy Msg. 3 transmission. Note: Spec. editor to choose one unused state of MCS Note: Whether to introduce higher layer support for this mechanism is up to RAN2. Agreement Msg3 retransmission with EDT uses the same TBS as the initial transmission assuming repetition number indicated in the DCI for configured maximum TBS; Agreement The timing for monitoring DCI is based on the number of Msg3 PUSCH repetitions indicated in RAR/DCI