3GPP TSG RAN WG1 Meeting #94 	R1-1809576

# Gothenburg, Sweden, 20 – 24 August 2018

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.212 | CR | 0300 | rev | - | Current version: | 15.2.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on the partial PUSCH mode field for FeLAA in 36.212 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_unlic-Core |  |  |  |  |  |  | Date: |  | 2018-08-22 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The current version of 36.212 specifies that the 2 bits field for indicating partial PUSCH modes in DCI format 0A/4A is present only when a RRC parameter is configured, but RRC configuration for Mode 1, Mode 2, and Mode 3 are separately defined in the running CR of 36.331. So the condition for the presence of the partial PUSCH mode field in format 0A/4A needs to be redefined. In addition, the names of the RRC parameters for Mode 1, Mode 2, and Mode 3 in 36.212 are not aligned with 36.331. Details can refer to R1-1808587. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Correct that the ‘partial PUSCH mode’ field in DCI format 0A/4A is only present when at least one RRC parameter among laa-PUSCH-Mode1, laa-PUSCH-Mode2, and laa-PUSCH-Mode3 is configured. In addition, correct the names of RRC parameters for configuring partial PUSCH mode 1/2/3 in DCI format 0B/4B. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The condition on the presence of the partial PUSCH mode field for DCI format 0A/4A is not clear. RRC parameter names between 36.212 and 36.331 are not aligned. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.3.3.1.1A, 5.3.3.1.1B, 5.3.3.1.8A, 5.3.3.1.8B |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

< Unchanged parts are omitted >

## 5.3.3.1.1A	Format 0A

DCI format 0A is used for the scheduling of PUSCH in a LAA SCell, or activating/releasing AUL transmission as defined in [3], or indicating AUL downlink feedback information (AUL-DFI) to a UE that is activated with AUL transmission.

The following information is transmitted by means of the DCI format 0A:

< Unchanged parts are omitted >

-	Partial PUSCH Mode – 2 bits as specified in Table 5.3.3.1.1A-3. This field is only present when at least one of laa-PUSCH-Mode1, laa-PUSCH-Mode2, and laa-PUSCH-Mode3 is configured. The UE is not expected to be indicated with a partial PUSCH mode that is not configured by RRC.

< Unchanged parts are omitted >

## 5.3.3.1.1B	Format 0B

DCI format 0B is used for the scheduling of PUSCH in each of multiple subframes in a LAA SCell.

The following information is transmitted by means of the DCI format 0B:

< Unchanged parts are omitted >

-	Partial PUSCH Mode 1 – 1 bit only present if laa-PUSCH-Mode1 is configured by higher layer, and applicable to each of the scheduled subframe(s), except for the first scheduled subframe in case the 'Partial PUSCH Mode 2' field is value 1 and the last scheduled subframe in case the 'Partial PUSCH Mode 3' field is value 1, where

-	value 0 indicates the starting position of the PUSCH is determined following Partial PUSCH Mode 0 as defined in Table 5.3.3.1.1A-3;

-	value 1 indicates the starting position of the PUSCH of the first transmitted subframe is determined following  Partial PUSCH Mode 1 as defined in Table 5.3.3.1.1A-3.

-	Partial PUSCH Mode 2 – 1 bit. This field is only present if laa-PUSCH-Mode2 is configured by higher layer, and applicable to only the first scheduled subframe.

-	Partial PUSCH Mode 3 – 1 bit. This field is only present if laa-PUSCH-Mode3 is configured by higher layer, and applicable to only the last scheduled subframe.

< Unchanged parts are omitted >

## 5.3.3.1.8A	Format 4A

DCI format 4 is used for the scheduling of PUSCH in a LAA SCell with multi-antenna port transmission mode, or activating/releasing AUL transmission for a UE with multi-antenna port transmission mode, or indicating AUL-DFI to a UE that is activated with AUL transmission with multi-antenna port transmission mode.

The following information is transmitted by means of the DCI format 4A:

< Unchanged parts are omitted >

-	Partial PUSCH Mode – 2 bits as specified in Table 5.3.3.1.1A-3. This field is only present when at least one of laa-PUSCH-Mode1, laa-PUSCH-Mode2, and laa-PUSCH-Mode3 is configured. The UE is not expected to be indicated with a partial PUSCH mode that is not configured by RRC.

< Unchanged parts are omitted >

## 5.3.3.1.8B	Format 4B

DCI format 4B is used for the scheduling of PUSCH with multi-antenna port transmission mode in each of multiple subframes in a LAA SCell.

The following information is transmitted by means of the DCI format 4B:

< Unchanged parts are omitted >

-	Partial PUSCH Mode 1 – 1 bit only present if laa-PUSCH-Mode1 is configured by higher layer, and applicable to each of the scheduled subframe(s), except for the first scheduled subframe in case the 'Partial PUSCH Mode 2' field is value 1 and the last scheduled subframe in case the 'Partial PUSCH Mode 3' field is value 1, where

-	value 0 indicates the starting position of the PUSCH is determined following Partial PUSCH Mode 0 as defined in Table 5.3.3.1.1A-3;

-	value 1 indicates the starting position of the PUSCH of the first transmitted subframe is determined following Partial PUSCH Mode 1 as defined in Table 5.3.3.1.1A-3.

-	Partial PUSCH Mode 2 – 1 bit. This field is only present if laa-PUSCH-Mode2 is configured by higher layer, and applicable to only the first scheduled subframe.

-	Partial PUSCH Mode 3 – 1 bit. This field is only present if laa-PUSCH-Mode3 is configured by higher layer, and applicable to only the last scheduled subframe.

< Unchanged parts are omitted >