3GPP TSG RAN WG1 Meeting #94                                                                                    R1-1809716

Gothenburg, Sweden, August 20th – 24th, 2018

Source:	CATT

Title:	Self evaluation on Reliability

Agenda Item:	7.2.7.2

Document for:	Discussion and Decision

- Introduction

At RAN#75 meeting, new Study Item on Self Evaluation towards IMT-2020 submission was approved [1]. Self-Evaluation will provide the performance towards all the ITU-R IMT-2020 requirements as defined in Report ITU-R M.2410 [2]. High-level assessment methods for these evaluation characteristics are given in § 6 of Report ITU-R M.2412 [3]. Reliability for URLLC needs to be evaluated with both system-level simulation (SLS) and link-level simulation (LLS). A 5th-percentile SINR provided by SLS will be used in link level simulation to achieve final evaluation results [3].

In this contribution, we provide initial reliability evaluation results, including UL and DL pre-processing SINR CDF, and preliminary evaluation results for reliability.

- Pre-processing SINR

At Ran1#93 meeting, it was agreed that for SU-MIMO pre-processing SINR as defined in [4] is calculated in SLS. In this section, we provide the preliminary evaluation results of UL and DL preprocessing SINR for URLLC scenario. The 5%-tile point pre-processing SINR values are provided in Table 1. Pre-processing SINR CDFs are shown in Figure 1 and Figure 2. The general evaluation SLS assumptions are listed in Appendix I.

Table 1: 5%-tile point pre-processing SINR

| Scenario | Configuration | Channel Model | 5%-tile pre-processing SINR (dB) |
| --- | --- | --- | --- |
| URLLC UL | Config A <α = 0.8, P0 = -90 dBm> | Model A | -9.13 |
|  | Config A <α = 1, P0 = -106 dBm> | Model B | -2.5 |
|  | Config B <α = 0.8, P0 = -80 dBm> | Model A | -4.5 |
|  | Config B <α = 0.8, P0 = -80 dBm> | Model B | -0.34 |
| URLLC DL | Config A | Model A | -3.47 |
|  |  | Model B | -3.46 |
|  | Config B | Model A | -2.55 |
|  |  | Model B | -2.46 |

| Figure 1: UL pre-processing SINR CDF | Figure 2: DL pre-processing SINR CDF |
| --- | --- |

- Link level simulation

At RAN1#93 meeting, analytical calculation methods for total reliability were agreed in [5]. In order to obtain final reliability, probabilities (denoted as p0~p4) as listed in the following table need to be evaluated. p1 and p2 are obtained by running PDCCH and PDSCH/PUSCH simulations respectively.  p0, p3 and p4 are acquired by PUCCH simulations. For a better understanding of these probabilities, notations are also provided in Table 2.

Table 2: Successful probability notations for various events

| Successful Probability | Description | Notations |
| --- | --- | --- |
| p0 | Successful probability of SR detection | P(SR detected ¦ UE sent SR) |
| p1 | Successful probability of PDCCH transmission | P(PDCCH correctly received) |
| p2 | Successful probability of PDSCH/PUSCH transmission | P(PDSCH/PUSCH correctly received) |
| p3 | Successful probability of PUCCH NACK detection | P(NACK or DTX detected ¦ UE sent NACK) = 1 - P(ACK detected ¦ UE sent NACK) |
| p4 | Successful probability of PUCCH DTX detection | P(NACK or DTX detected ¦ UE sent DTX) = 1 - P(ACK detected ¦ UE sent DTX) |

According to [5], Alternative 1 without soft combination and Alternative 2 with soft combination gain can be applied to LLS. In the following section, LLS assumptions and results based on Alternative 1 (without soft combination gain) on PDCCH, PDSCH and PUSCH are provided and then total reliability results are obtained.

LLS assumptions for PDCCH, PDCCH and PUSCH are listed in Table 3, 4, 5. For PDSCH and PUSCH, since TBS is set to be 256 bits, the allocation bandwidth and MCS for PDSCH/PUSCH are derived based on the TBS determination procedure as defined in Section 5.1.3 of TS38.214 [6].  1 symbol DMRS overhead is accounted for TBS calculation. The MCS table for URLLC agreed at RAN1 #93 meeting is applied to the evaluations.

Table 3: PDCCH simulation assumptions

| Numerology | 30kHz SCS |
| --- | --- |
| Waveform | CP-OFDM |
| Aggregation level | 16 |
| PDCCH | 2 symbols |
| RB number | 48 RB |
| Payload size | 34 bits |
| Carrier frequency | Config B: 700MHz |
| Antenna config | Config B: 2T2R |
| Antenna pattern | 0dB omni-directional gain |
| channel model | TDL-C, 363ns |
| Speed | 3km/h |
| Channel estimation | Non-ideal |

Table 4: PDSCH simulation assumptions

| Numerology | 30kHz SCS |
| --- | --- |
| Waveform | CP-OFDM |
| Carrier frequency | 700MHz |
| Antenna config | 2T2R |
| Antenna pattern | 0dB omni-directional gain |
| Transmission scheme | Spatial multiplexing |
| channel model | TDL-C, 363ns |
| Speed | 3km/h |
| Channel estimation | Non-ideal |
| Data and RB allocation | TBS = 256 bits : 7-os, MCS1, 46RB 4-os, MCS5, 37RB |
| Transmission scheme | No HARQ |
| Overhead | 1 symbol DMRS accounted |

Table5 PUSCH simulation assumptions

| Numerology | 30kHz SCS |
| --- | --- |
| Waveform | CP-OFDM |
| Carrier frequency | Config A: 4GHz Config B: 700MHz |
| Antenna config | 1T8R |
| Antenna pattern | 0dB omni-directional gain |
| channel model | TDL-C, 363ns |
| Speed | 3km/h |
| Channel estimation | Non-ideal |
| Data and RB allocation | TBS = 256 bits : 12-os, MCS4,12RB |
| Transmission scheme | No HARQ |
| Overhead | 1 symbol DMRS accounted |

LLS simulation results at preprocessing SINR point are given in Table 6.

Table 6:  LLS simulation results for PDCCH, PDSCH and PUSCH

| Carrier frequency | SINR（5%） | Channel | BLER |
| --- | --- | --- | --- |
| 700MHz | DL: -2.5dB | PDCCH (AL=16, 34bits, 48RB) | 7.1E-06 |
|  |  | PDSCH (7-os, MCS1, 46RB) | 2.86E-06 |
|  |  | PDSCH (4-os, MCS5, 37RB) | <1.67 E-05 |
| 700MHz Molde A | UL: -4.5dB | PUSCH (12-os, MCS4,12RB) | <7.69E-6 |
| 700MHz Molde B | UL: -0.34dB | PUSCH (12-os, MCS4,12RB) | <7.69E-6 |
| 4GHz Molde B | UL: -2.5dB | PUSCH (12-os, MCS4,12RB) | <7.69E-6 |

- Total reliability analysis

- DL reliability

For the DL transmission scheme as shown in figure 3, DL total reliability (without soft combination) can be expressed as：

(1)

Figure 3: DL transmission scheme

There are two cases evaluated:

Case 1. One shot, non-slot transmission (M=1)

For one shot transmission with 7-os PDSCH (MCS1, 46RB), total reliability at 5%-tile SINR point is given in Table 7.

Table 7: One shot non-slot transmission for Config. B

| Carrier frequency | SINR | PDSCH | p1 | p2 | One shot total reliability |
| --- | --- | --- | --- | --- | --- |
| 700MHz | -2.5 | 7-os, MCS1, 46RB | 0.9999929 | 0.9999971 | 0.99999004288 |

Observation 1:  DL one shot non-slot transmission has less than 10-5 error and successful probability can reach the requirement of 1-10-5.

Case 2. One transmission tempt with M>1

For one transmission tempt, i.e. K=1, if (non-)slot aggregation scheme is conducted with M = 2, non-slot length = 4 symbols (with MCS5, 37 RB) total reliability result at 5%-tile SINR point is given in Table 8.

Table 8: One transmission tempt reliability with M = 2 for Config B

| Carrier frequency | SINR | PDSCH | p1 | p2 | K=1， M = 2 total reliability |
| --- | --- | --- | --- | --- | --- |
| 700MHz | -2.5 | 4-os, MCS5, 37RB | 0.9999929 | 0.9999923esultsen in Table 7.t on nfor PDCCH, PDSCH and PUSCH listed in Table 6. | 0.9999929 |

Observation 2:  DL One transmission tempt with M = 2 has less than 10-5 error and successful probability can reach the requirement of 1-10-5.

It is noted that with higher M, e.g. M = 3 would further improve the reliability.

- UL reliability

For the UL grant free transmission scheme as shown in figure 4, UL total reliability (without soft combination) can be expressed as

(2)

Figure 4: UL data transmission without grant

For the following case:

Case 1. One shot, slot-based transmission (M=1)

For one shot transmission with 12-os PDSCH (MCS4,12RB), total reliability at  5%-tile SINR point is given in Table 9.

Table 9: One shot non-slot transmission for Config B

| Carrier frequency | SINR | PUSCH | p2 | One shot total reliability |
| --- | --- | --- | --- | --- |
| 700MHz Model A | -4.5dB | 12-os, MCS4,12RB | 0.9999971 | 0.9999923 |
| 700MHz Model B | -0.34dB | 12-os, MCS4,12RB | 0.9999971 | 0.9999923 |
| 4GHz Model B | -2.5dB | 12-os, MCS4,12RB | 0.9999971 | 0.9999923 |

Observation 3:  UL one shot slot transmission has less than 10-5 error and successful probability can reach the requirement of 1-10-5.

- UP latency analysis

It is found from above results of reliability that when M = 1 with 7OS and 12OS as non-slot based for DL and slot-based forUL respectively and M = 2 with 4OS as non-slot for DL can reach the requirement of 1-10-5. But it is noted that only slot-based slot aggregation is supported in the spec. Even when M=2, the latency following slot-based cases as analyzed in [7] is hard to fulfill 1ms requirements. Furthermore, RB number allocation should not exceed the bandwidth configuration for DL, i.e. 51 RB for 20MHz bandwidth and that for UL should be aligned with RB applied for SLS SINR achievement.

There are two evaluation results:

A.M = 1 with 7OS and 12OS as non-slot for DL and UL respectively. For this single shot non-slot-based evaluation, UP latency is as follows: (this scheme is supported by spec and required RB number is also legal)

Table 11: One shot scheme latency for UE capability #2, 30kHz SCS

| Scheme | UP latency calculation method | FDD | TDD (DSUUD) | TDD (DU) |
| --- | --- | --- | --- | --- |
| 7 OS PDSCH | Average method | 0.5 | 0.77 | 0.45 |
|  | Best case method | 0.43 | 0.43 | 0.43 |
| 12 OS PUSCH | Average method | 0.80 | 1.33 | 0.99 |
|  | Best case method | 0.68 | 0.68 | 0.68 |

B. M = 2 with 4OS as non-slot for DL. This is one transmission tempt with two non-slot aggregation by non-slot based latency (this scheme is not yet supported by spec)

Table 12: M=2 scheme latency for UE capability #2, 30kHz SCS

| Scheme | UP latency calculation method | FDD | TDD (DSUUD) | TDD (DU) |
| --- | --- | --- | --- | --- |
| 4 OS PDSCH with M=2 | Average method | 0.74 | 1.24 | 1.11 |
|  | Best case method | 0.68 | 0.68 | 0.68 |
| 4 OS PUSCH with M=2 | Average method | 0.92 | 1.98 | 1.3 |
|  | Best case method | 0.68 | 0.68 | 0.68 |

Observation 5: Both A and B can fulfill 1 ms latency under conditions of 30kHz SCS for UE capability #2.

- Conclusion

In this contribution, reliability evaluation is discussed and the following observations are provided:

Observation 1:  DL one shot non-slot transmission has less than 10-5 error and successful probability can reach the requirement of 1-10-5.

Observation 2:  DL One transmission tempt with M = 2 has less than 10-5 error and successful probability can reach the requirement of 1-10-5.

Observation 3:  UL one shot slot transmission has less than 10-5 error and successful probability can reach the requirement of 1-10-5.

Observation 4: UL One transmission tempt with M = 2 has less than 10-5 error and successful probability can reach the requirement of 1-10-5.

Observation 5: Both A and B can fulfill 1 ms latency under conditions of 30kHz SCS for UE capability #2.

More evaluation results would be provided in near future.

- References

- RP-170832, “New Study Item on Self Evaluation towards IMT-2020 submission”, Huawei, Ericsson.

- Report ITU-R M.2410, “Minimum requirements related to technical performance for IMT-2020 radio interface(s)”, Nov. 2017.

- Report ITU-R M.2412, “Guidelines for evaluation of radio interface technologies for IMT-2020”, Nov. 2017.

- R1-1805643, “Summary of discussion on mobility evaluation method and parameters”, Huawei

- R1-1807760, “Summary on offline discussion on URLLC evaluation method and parameters for IMT-2020 self-evaluation”, Huawei

- R1-1805796, “draft CR to 38.214 capturing the RAN1#92bis meeting agreements”, April. 2018.

- R1-1808425, “Self evaluation on User Plane latency for NR”, CATT

Appendix I

Simulation assumption of SLS part for reliability

|  | Urban Macro - URLLC |
| --- | --- |
| Evaluation configuration | Configuration A, Configuration B |
| Carrier frequency for evaluation | Configuration A : 4 GHz Configuration B :700 MHz |
| Multiple access (DL/UL) | OFDMA |
| Duplexing | FDD |
| Transmission scheme | DL: SU-MIMO with rank 1 UL: SIMO |
| BS antenna height | 25 m |
| Total transmit power per TRxP | 46 dBm for 10 MHz bandwidth |
| UE power class | 23 dBm |
| Percentage of high loss and low loss building type | 100% low loss (applies to Channel model B) |
| Inter-site distance | 500m |
| Number of antenna elements per TRxP | Config A: 64 Tx/Rx, (M,N,P,Mg,Ng) = (8,4,2,1,1) Config B: 16 Tx/Rx, (M,N,P,Mg,Ng) = (8,1,2,1,1) (dH,dV) = (0.5, 0.8)λ, +45°, -45° polarization |
| Number of TXRU per TRxP | DL Config A: 8TXRU, (Mp,Np,P,Mg,Ng) = (1,4,2,1,1) DL Config B: 2TXRU, (Mp,Np,P,Mg,Ng) = (1,1,2,1,1) UL Config A: 8TXRU, (Mp,Np,P,Mg,Ng) = (1,4,2,1,1) UL Config B: 8TXRU, (Mp,Np,P,Mg,Ng) = (4,1,2,1,1) |
| Number of UE antenna elements | Config A: 4Tx/Rx, (M,N,P,Mg,Ng) = (1,2,2,1,1) Config B: 2Tx/Rx, (M,N,P,Mg,Ng) = (1,1,2,1,1) UL: 1Tx/Rx, (M,N,P,Mg,Ng) = (1,1,1,1,1) (dH,dV) = (0.5, N/A)λ0°,90° polarization |
| Number of TXRU per UE | DL Config A: 4TXRU (1-to-1 mapping) DL Config B: 2TXRU (1-to-1 mapping) UL: 1 TXRU (1-to-1 mapping) |
| Device deployment | 80% outdoor ,20% indoor Randomly and uniformly distributed over the area |
| UE speeds of interest | 3 km/h for indoor and 30 km/h for outdoor |
| Traffic model | Full buffer |
| Simulation bandwidth | 10 MHz |
| UE density | 10 UEs per TRxP |
| UE antenna height | 1.5 m |
| Channel model variant | Alt. 1: Channel model A Alt. 2: Channel model B |
| TRxP number per site | 3 |
| Mechanic tilt | 90° in GCS (pointing to horizontal direction) |
| Electronic tilt | 99° in LCS |
| Handover margin (dB) | 0 |
| TRxP boresight | 30 / 150 / 270 degrees |
| UT attachment | Based on RSRP (formula (8.1-1) in TR36.873) from port 0 |
| Wrapping around method | Geographical distance based wrapping |
| Minimum distance of TRxP and UE | d2D_min=10m |
| Polarized antenna model | Model-2 in TR36.873 |
| Numerology | one slot with 30 kHz SCS |
| Scheduling | PF |
| Receiver | MMSE-IRC |
| Pre-processing SINR calculation | Aligned with Section 2.1.1 in R1-1805643 |