3GPP TSG-RAN WG1 Meeting #94bis	Tdoc R1-1811297

Chengdu, China, 8th – 12th October, 2018

Agenda Item:	7.2.2.1, Evaluation Results for NR-U operations

Source:	Ericsson

Title:	Coexistence evaluation results for NR-U and Wi-Fi

Document for:	Discussion, Decision

# 1	Introduction

In RAN1#92 [1], the following agreements regarding to NR-U coexistence with other networks have been reached

Agreement:

- Coexistence with other networks (e.g. WiFi, LAA LTE, NR-U)

  - When coexistence with WiFi is evaluated, only consider deployed WiFi systems (e.g. 11ac for 5 GHz)

    - Fairness criterion for coexistence with 11ax can be further discussed at plenary level

    - The coexistence evaluation applies to 5GHz band (11ac) and 60GHz (11ad)

    - From SID: NR-based operation in unlicensed spectrum should not impact deployed Wi-Fi services (data, video and voice services) more than an additional Wi-Fi network on the same carrier

  - For sub-7 GHz bands, coexistence simulations will be performed using technology neutral assumptions (eg. channel access mechanism) at an arbitrary carrier frequency in 5GHz band for application to bands other than 5GHz which may become available subject to regulations

    - Note: The study assumes regulation will provide the framework concerning the protection for the technologies not using unlicensed access in those bands

In this contribution, we provide the coexistence evaluation results for NR-U and Wi-Fi in all the simulation scenarios agreed in RAN1 for NR-U operations, including indoor scenario, outdoor scenario 1 and outdoor scenario 2 [2] .

# 2	Coexistence Evaluations

## 2.1	Coexistence evaluation assumptions

Similar to LTE-based LAA, coexistence evaluation methodology is as follow:

- Wi-Fi and NR-U coexistence

  - For each UE and eNB/AP drop

    - Step 1: Performance metrics for two Wi-Fi networks coexisting in a given evaluation scenario are evaluated and recorded.

    - Step 2: Wi-Fi is replaced with NR-U for the group of gNBs and UEs served by one of the Wi-Fi operators. Performance metrics of the Wi-Fi network coexisting with the NR-U network are evaluated and recorded.

  - Performance metrics for the Wi-Fi operator common to the two steps are compared.

- NR-U and NR-U coexistence

  - Performance metrics for two NR-U operators coexisting in a given evaluation scenario are evaluated and recorded.

  - Performance metrics for the two NR-U operators are compared.

In our evaluation, we consider two operators named Operator A and Operator B. In the Wi-Fi and NR-U coexistence scenario, in the first step Operator A and B both use Wi-Fi. In the second step, operator A and its corresponding UEs are replaced by an NR-U operator and NR-U UEs while operator B and its UEs remain unchanged. In the NR-U and NR-U coexistence scenario both operators that coexist with each other are NR-U networks.

The simulation assumptions are based on the agreed coexistence assumptions in RAN1, in which some main parameters are defined in Table 4 and Table 5 in the appendix.

In all the coexistence evaluations, the transmit power of the base station is assumed to be 20 dBm. The average received power CDFs for indoor and outdoor scenarios are given in the appendix. We also assume the mixed traffic scenarios in which the traffic ratio between DL and UL is 50:50. Channel access mechanisms follow the same principles defined for LAA for both DL and UL. The used LBT parameters are listed in the appendix.

NR-U uses self-scheduling for UL transmissions, i.e. the grant is sent on the same unlicensed carrier as the data transmission. Additionally, no licensed carrier is being considered and the Processing delays are modelled according to UE Capability #1.

For Wi-Fi networks, we evaluate with 802.11ac system assumptions. Finally, Table 1 and Table 2 capture our assumptions for Wi-Fi and NR-U systems, respectively.

Table 1 - Additional Wi-Fi system evaluation assumptions

| Parameter |  | Value |
| --- | --- | --- |
| MCS |  | 802.11ac MCS table with 256 QAM |
| Antenna configuration |  | 4Tx2Rx in DL, Cross-polarized |
| Channel coding |  | LDPC |
| Frame aggregation |  | A-MPDU |
| MPDU size |  | 1500B MSDU + 14 B header |
| MCOT |  | 6 ms |
| MAC | Coordination | DCF |
|  | SIFS, DIFS | SIFS, DIFS |
|  | Detection | Energy detection & preamble detection |
|  | RTS/CTS | No |
|  | Contention window | Per DCF |
| CCA-PD |  | -82dBm and preamble decoding |
| CCA-ED |  | -62dBm |
| ACK Modeled (successful reception, resources utilized) |  | Yes |
| Rate control |  | Minstrel algorithm |
| OFDM symbol length |  | 4 micro second |
| Contention Window {min, max} |  | DL: {15,63} UL: {15,1023} |

| Table 2 - Additional NR-U system evaluations assumptions |  |
| --- | --- |
| Parameters | Value |
| PCI planning for each NW | Planned |
| Antenna configuration | 4Tx2Rx in DL, Cross-polarized. |
| Transmission schemes | QPSK/16QAM/64QAM/256QAM |
| Scheduling | Proportional fair |
| Link adaptation | Realistic |
| CCA-ED | -72 dBm |
| Cyclic Prefix | Normal |
| Subcarrier spacing | 30KHz |
| MCOT | 6 ms |
| UE processing capability | Capability #1 |
| Contention Window {min, max} | DL: {15,63} UL: {15,1023} |

## 2.2	Coexistence evaluation results

Based on the simulation assumption that specified in Table 1 and Table2 we set up three evaluation scenarios to study coexistence of Wi-Fi network with Wi-Fi, Wi-Fi with NR-U and finally NR-U with NR-U to see coexistence of two NR-U operators.

The arrival intensities are selected such that the mean buffer occupancy (BO) of the reference WiFi-WiFi scenario are 10%, 35%, and 55%, corresponding to the cases of low, medium, and high load. Accordingly, the arrival intensities are [0.19  0.29  0.35] file/s, [0.14 0.17 0.19] file/s, and [0.14 0.16 0.18] file/s for indoor scenario, outdoor scenario 1, and outdoor scenario 2, respectively.

In the following, Figures 1-3 illustrate the evaluation results for indoor scenario. In which, Figures 1-2 show DL-UL mean object bit rate per UE, and Figure 3 shows the ratio of served traffic to offered traffic for low, medium and high loads.

- (b)

Figure 1: DL mean object bit rate in indoor scenario of (a) Wi-Fi coexisting with Wi-Fi/NR-U, (b) NR-U coexisting with Wi-Fi/NR-U.

- (b)

Figure 2: UL mean object bit rate in indoor scenario of (a) Wi-Fi coexisting with Wi-Fi/NR-U, (b) NR-U coexisting with Wi-Fi/NR-U.

- (b)

Figure 3: DL (a) and UL (b) ratios of served traffic over offered traffic for Wi-Fi in indoor scenario.

In the following, Figures 4-6 illustrate the evaluation results for outdoor scenario 1. In which, Figures 4-5 show DL-UL mean object bit rate per UE, and Figure 6 shows the ratio of served traffic to offered traffic for low, medium and high loads.

- (b)

Figure 4: DL mean object bit rate in outdoor scenario 1 of (a) Wi-Fi coexisting with Wi-Fi/NR-U, (b) NR-U coexisting with Wi-Fi/NR-U.

- (b)

Figure 5: UL mean object bit rate in outdoor scenario 1 of (a) Wi-Fi coexisting with Wi-Fi/NR-U, (b) NR-U coexisting with Wi-Fi/NR-U.

- (b)

Figure 6: DL (a) and UL (b) ratio of served traffic over offered traffic for Wi-Fi in outdoor scenario 1.

In the following, Figures 7-9 illustrate the evaluation results for outdoor scenario 2. In which, Figures 7-8 show DL-UL mean object bit rate per UE, and Figure 9 shows the ratio of served traffic to offered traffic for low, medium and high loads.

- (b)

Figure 7: DL mean object bit rate in outdoor scenario 2 of (a) Wi-Fi coexisting with Wi-Fi/NR-U, (b) NR-U coexisting with Wi-Fi/NR-U.

- (b)

Figure 8: UL mean object bit rate in outdoor scenario 2 of (a) Wi-Fi coexisting with Wi-Fi/NR-U, (b) NR-U coexisting with Wi-Fi/NR-U.

- (b)

Figure 9: DL (a) and UL (b) ratios of served traffic over offered traffic of Wi-Fi in outdoor scenario 2.

The system performance results in Figures 1(a)-9(a) show that not only does NR-U with DL and UL traffic coexists in a friendly manner with Wi-Fi but also boosts Wi-Fi performance as compared to the case where two Wi-Fi networks coexist with each other. Some of the major elements which make NR-U a good neighbor to Wi-Fi in the unlicensed band where both technologies have to share the medium are listed below:

- The NR-U uplink traffic is scheduled by the NR-U gNBs, which reduces the number of contending nodes at any given time. In the Wi-Fi network, all UEs with non-empty UL buffer contend to access the medium and by that introduce intra-cell collisions and higher inter-cell collisions.

- NR-U, as compared to the evaluated 802.11ac Wi-Fi, has more robust interference mitigation, error correction and retransmission schemes which result in greater efficiency in serving the traffic and reducing resource utilization thus providing more opportunities for other systems to access the medium.

- NR-U uses the same sensing threshold irrespective of transmissions from NR-U or Wi-Fi nodes as opposed to Wi-Fi which uses a higher sensing threshold for NR-U as compared to Wi-Fi.

NR-U networks on the unlicensed band can coexist well with Wi-Fi networks in both indoor and outdoor scenarios.

Additionally, the results in Figures 1(b)-9(b) demonstrate that when NR-U coexists with another NR-U network, it can achieve better performance than coexisting with Wi-Fi.

NR-U networks on the unlicensed band can coexist well with each other in both indoor and outdoor scenarios.

## 2.3	Table format for NR-U statistics in Technical Reports

In order to record evaluation data from different companies for the NR-U Technical Report, we propose to use similar table template as was used for LTE-based LAA study. In the following, we include an example table, recording the coexistence results between Wi-Fi and NRU in indoor scenario, corresponding to the results in Figures 1-3.

Table 3 – DL evaluation results for Wi-Fi and NRU coexistence in indoor scenario

| Tdoc / Source | Reported parameters |  | Low load BO range for Wi-Fi in Step 1: 10%~25% |  |  | Medium load BO range for Wi-Fi in Step 1: 35%~50% |  |  | High load BO range for Wi-Fi in Step 1: above 55% |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Wi-Fi in step 1 | Wi-Fi in step 2 | NRU in step 2 | Wi-Fi in step 1 | Wi-Fi in step 2 | NRU in step 2 | Wi-Fi in step 1 | Wi-Fi in step 2 | NRU in step 2 |
| R1-1811297 | DL UPT CDF [Mbps] | 5% | 37.52 | 41.35 | 43.93 | 20.80 | 29.94 | 32.64 | 12.15 | 21.84 | 22.88 |
|  |  | 50% | 82.85 | 88.08 | 103.86 | 53.21 | 69.68 | 82.22 | 31.23 | 52.12 | 59.19 |
|  |  | 95% | 101.82 | 107.58 | 128.25 | 75.56 | 95.57 | 110.13 | 55.78 | 81.17 | 87.66 |
|  |  | Mean | 80.76 | 85.83 | 101.59 | 53.90 | 70.64 | 82.59 | 34.61 | 55.47 | 61.64 |
|  | DL Delay CDF [s] | 5% | 0.036 | 0.033 | 0.027 | 0.046 | 0.047 | 0.063 | 0.077 | 0.074 | 0.117 |
|  |  | 50% | 0.147 | 0.048 | 0.047 | 0.428 | 0.208 | 0.388 | 0.721 | 0.635 | 1.061 |
|  |  | 95% | 1.279 | 0.139 | 0.166 | 4.814 | 1.355 | 1.493 | 7.704 | 3.107 | 4.064 |
|  |  | Mean | 0.431 | 0.075 | 0.076 | 1.557 | 0.487 | 0.667 | 2.573 | 1.266 | 1.785 |
|  | 𝜌 |  | 97.1% | 99.9% | 99.9% | 81.1% | 96.8% | 96% | 68.0% | 93.1% | 88.5% |
|  | Mean BO |  | 10% | 5.5% | 5.4% | 35% | 17.5% | 17.6% | 55% | 33.1% | 34.1% |
|  | 𝜆 |  | 0.19 files/s |  |  | 0.29 files/s |  |  | 0.35 files/s |  |  |
|  |  | Additional comments: - Simulation setup: NR-U indoor scenario, 50/50 DL/UL traffics. - In the table: 𝜌 is the ratio between served traffic and offered traffic for UL+DL. - Simulation assumptions are given in R1-1811297. |  |  |  |  |  |  |  |  |  |

# 3	Conclusion

In the previous sections we made the following observation and proposals for indoor coexistence evaluation for NR-U and Wi-Fi:

Observation 1	NR-U networks on the unlicensed band can coexist well with Wi-Fi networks in both indoor and outdoor scenarios.

Observation 2	NR-U networks on the unlicensed band can coexist well with each other in both indoor and outdoor scenarios.

We also proposed the table format for recording the evaluation results in the NR-U Technical Report.

# References

Chairman’s notes, RAN1#92, Athens, Greece.

Chairman’s notes, RAN1#94, Gothenburg, Sweden.

# Appendix

Table 4 - Simulation parameters for indoor sub-7GHz scenario

| Layout for nodes | Layout dimensions: 120mx80m a=20 meters, b=40 meters, c=20 meters, and d=40 meters |
| --- | --- |
| Carrier Frequency | 5GHz |
| Carrier Channel Bandwidth | 20MHz |
| Number of carriers | 1 |
| Number of users per operator | 5 per gNB |
| SCS | 30kHz |
| Channel Model | NR InH Mixed Office model |
| BS/AP Tx Power | 20dBm (subtract 3dBm from 23dBm to meet EIRP limit with BF) |
| UE/STA Tx Power | 18dBm |
| BS/AP Antenna gain | 0dBi |
| UE/STA Antenna gain | 0dBi |
| BS/AP Noise Figure | 5dB |
| UE/STA Receiver Noise Figure | 9dB |
| Minimum received power from serving cell for UE dropping | -82dBm |
| UE receiver | MMSE-IRC as the baseline receiver |
| BS/AP antenna Array configuration | (M, N, P, Mg, Ng) = (1, 2, 2, 1, 1), dH = dV = 0.5 λ |
| UE/STA antenna Array configuration | (M, N, P, Mg, Ng) = (1, 1, 2, 1, 1), dH = dV = 0.5 λ |
| Traffic model | Use 36.889 Table A.1.1. |
| UE/STA to UE/STA link pathloss model | Directly use InH office pathloss model with proper d_3D with indoor mixed office LOS probability |
| gNB to gNB link pathloss model | Directly use InH office pathloss model with proper d_3D with indoor mixed office LOS probability |
| UE/STA dropping per network | All UEs should be randomly dropped and be within coverage of the gNB/AP in the unlicensed band. Example of a dropping method to achieve this with N=5 UEs/STAs per gNB /AP: - Randomly drop a large enough number of UEs over the whole 120mx80m building. - Each UE/STA select the best gNB /AP (according to RSSI) as its serving gNB /AP. The number of dropping UEs in the first step should be large enough such that at least 5 UEs/STAs are associated to each BS/AP. - Each gNB AP randomly selects 5 UEs/STAs from its associated UEs/STAs. |

Figure 10: CDFs of  Rx power for DL(left) and UL(right) transmissions for indoor scenario.

Table 5 - Simulation parameters for outdoor sub-7GHz scenario

| Parameters | Outdoor Sub-7GHz |
| --- | --- |
| Carrier Frequency | 5GHz |
| Carrier Channel Bandwidth | 20MHz |
| Number of carriers | 1 |
| Number of users per operator | 5 per gNB |
| SCS | 30kHz |
| Channel Model | NR UMi street canyon |
| BS/AP Tx Power | 20dBm (subtract 3dBm from 23dBm to meet EIRP limit with BF) |
| UE/STA Tx Power | 18dBm |
| BS/AP Antenna gain | 0 dBi |
| UE/STA Antenna gain | 0 dBi |
| BS/AP Noise Figure | 5dB |
| UE/STA Receiver Noise Figure | 9dB |
| Minimum received power from serving cell for UE dropping | -82dBm |
| UE receiver | MMSE-IRC as the baseline receiver |
| BS/AP antenna Array configuration | (M, N, P, Mg, Ng) = (1, 2, 2, 1, 1), dH = dV = 0.5 λ |
| UE/STA antenna Array configuration | Tx/Rx: (M, N, P, Mg, Ng) = (1, 1, 2, 1, 1), dH = dV = 0.5 λ |
| Traffic model | Use 36.889 Table A.1.1. |
| UE/STA to UE/STA link pathloss model | Directly use UMi street canyon pathloss model with proper d_3D with UMi street canyon LOS probability |
| gNB to gNB link pathloss model | Directly use UMi street canyon pathloss model with proper d_3D with UMi street canyon LOS probability |

Figure 11: CDFs of  Rx power for DL(left) and UL(right) transmissions for outdoor scenario 1.

Figure 12: CDFs of  Rx power for DL(left) and UL(right) transmissions for outdoor scenario 2.