3GPP TSG-RAN WG1 Meeting #94bis	R1- 1810172

Chengdu, China, October 8 – 12, 2018

Agenda Item:	7.1.6

Source:	Ericsson

Title:	Maintenance of Rel-15 URLLC

Document for:	Discussion, Decision

# 1	Introduction

In RAN1#94, text proposals have been adopted on MCS determination of PDSCH (including DL SPS). In RAN1#94bis, text proposals have been adopted on MCS determination of PUSCH (including UL configured grant).

In this contribution, we identify a few editorial changes to TS38.214.

# 2	Discussion

After checking the specifications, a few minor editorial corrections are found. The text proposal can be found in section 3.

Adopt text proposal from section 3.1 of this contribution to TS 38.214 section 5.1.2

Adopt text proposal from section 3.2 of the contribution to TS 38.214 section 6.2.2

Another issue in current version of specifications is related to correct reference on “PDSCH-Config”. It was found that in other sections of TS38.214 it is captured as “pdsch-Config” (in lower case). It’s noted that in TS38.214 Section 6.1.4.1, ‘pusch-Config’ is used. To make the specification consistent, we propose to adopt text proposals in Sections 3.3. Similar changes should be applied throughput all specifications.

Use consistent references to RRC IEs in RAN1 specifications, for example, use “pdsch-Config” (in lower case) always in stead of mixing in “PDSCH-Config”.

# 3	Text Proposals

## 3.1	TS38.214 for Section 5.1.2

====================== Start of Text Proposal to TS 38.214 ======================

5.1.3.1 Table 5.1.2.1.1-1: Applicable PDSCH time domain resource allocation [Unaffected table lines are excluded]. C-RNTI, MCS-C-RNTI, CS-RNTI; Any common search space associated with CORESET#0; 1, 2, 3; No; -; Default A. 1, 2, 3; Yes; -; pdsch-TimeDomainAllocationList provided in pdsch-ConfigCommon. C-RNTI, MCS-C-RNTI,CS-RNTI; Any common search space not associated with CORESET#0 UE specific search space; 1,2,3; No; No; Default A. 1,2,3; Yes; No; pdsch-TimeDomainAllocationList provided in pdsch-ConfigCommon. 1,2,3; No/Yes; Yes; pdsch-TimeDomainAllocationList provided in pdsch-Config.

## ====================== End of Text Proposal to TS 38.214 ======================

## 3.2	TS38.214 for Section 6.2.2

====================== Start of Text Proposal to TS 38.214 ======================

### 6.2.2 UE DM-RS transmission procedure When transmitted PUSCH is not scheduled by PDCCH format 0_1 with CRC scrambled by C-RNTI, CS-RNTI or MCS-C-RNTI, the UE shall use single symbol front-loaded DM-RS of configuration type 1 on DM-RS port 0 and the remaining REs not used for DM-RS in the symbols are not used for any PUSCH transmission except for PUSCH with allocation duration of 2 or less OFDM symbols with transform precoding disabled, additional DM-RS can be transmitted according to the scheduling type and the PUSCH duration as specified in Table 6.4.1.1.3-3 of [4, TS38.211] for frequency hopping disabled and as specified in Table 6.4.1.1.3-6 of [4, TS38.211] for frequency hopping enabled, and

## ====================== End of Text Proposal to TS 38.214 ======================

## 3.3	TS38.214 for Section 5.1.3.1

====================== Start of Text Proposal to TS 38.214 ======================

#### 5.1.3.1 Modulation order and target code rate determination For the PDSCH scheduled by a PDCCH with DCI format 1_0 or format 1_1 with CRC scrambled by C-RNTI, MCS-C-RNTI, TC-RNTI, CS-RNTI, SI-RNTI, RA-RNTI, or P-RNTI, or for the PDSCH scheduled without corresponding PDCCH transmissions using the higher-layer-provided PDSCH configuration SPS-config, if the higher layer parameter mcs-Table given by PDSCHpdsch-Config is set to 'qam256', and the PDSCH is scheduled by a PDCCH with DCI format 1_1 with CRC scrambled by C-RNTI - the UE shall use IMCS and Table 5.1.3.1-2 to determine the modulation order (Qm) and Target code rate (R) used in the physical downlink shared channel. elseif the UE is not configured with MCS-C-RNTI, the higher layer parameter mcs-Table given by PDSCHpdsch-Config is set to 'qam64LowSE', and the PDSCH is scheduled by a PDCCH in a UE-specific search space with CRC scrambled by C-RNTI - the UE shall use IMCS and Table 5.1.3.1-3 to determine the modulation order (Qm) and Target code rate (R) used in the physical downlink shared channel. elseif the UE is configured with MCS-C-RNTI, and the PDSCH is scheduled by a PDCCH with CRC scrambled by MCS-C-RNTI - the UE shall use IMCS and Table 5.1.3.1-3 to determine the modulation order (Qm) and Target code rate (R) used in the physical downlink shared channel. elseif the UE is not configured with the higher layer parameter mcs-Table given by SPS-config, the higher layer parameter mcs-Table given by PDSCHpdsch-Config is set to 'qam256', - if the PDSCH is scheduled by a PDCCH with DCI format 1_1 with CRC scrambled by CS-RNTI or - if the PDSCH is scheduled without corresponding PDCCH transmission using SPS-config, - the UE shall use IMCS and Table 5.1.3.1-2 to determine the modulation order (Qm) and Target code rate (R) used in the physical downlink shared channel. elseif the UE is configured with the higher layer parameter mcs-Table given by SPS-config set to 'qam64LowSE' - if the PDSCH is scheduled by a PDCCH with CRC scrambled by CS-RNTI or - if the PDSCH is scheduled without corresponding PDCCH transmission using SPS-config, - the UE shall use IMCS and Table 5.1.3.1-3 to determine the modulation order (Qm) and Target code rate (R) used in the physical downlink shared channel. else - the UE shall use IMCS and Table 5.1.3.1-1 to determine the modulation order (Qm) and Target code rate (R) used in the physical downlink shared channel. end

====================== End of Text Proposal to TS 38.214 ======================

# 4	Conclusion

Based on the discussion in the previous sections we propose the following:

Proposal 1	Adopt text proposal from section 3.1 of this contribution to TS 38.214 section 5.1.2

Proposal 2	Adopt text proposal from section 3.2 of the contribution to TS 38.214 section 6.2.2

Proposal 3	Use consistent references to RRC IEs in RAN1 specifications, for example, use “pdsch-Config” (in lower case) always in stead of mixing in “PDSCH-Config”.

# References

Chairman’s notes RAN1#93

3GPP TS 38.214, V15.3.0 (2018-09)