3GPP TSG-RAN WG1 Meeting #95	R1-1813784

# Spokane, USA, November 12-16, 2018

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.213 | CR | 1202 | rev | - | Current version: | 15.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on CQI definition for feCoMP in 36.213 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon, Ericsson, Intel Corporation |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R1 |  |  |  |  |  |  |  |  |  |
| Work item code: | feCOMP_LTE-Core |  |  |  |  |  | Date: |  |  | 2018-11-15 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | In RAN1 #90 meeting, the follow agreement was achieved: - CRI = 2 CSI reporting is performed for both NZP CSI-RS resources …… - The PDSCH layers (υ1 layers for CW1 and υ2 layers for CW2) to antenna port mapping (15,…,14+P1 and 15,…,14+P2 ) for corresponding CW is given by: y1(15)(i)⋮y1(14+P1)(i)=W1(i)x1(0)(i)⋮x1(υ1-1)(i) for CW1 y2(15)(i)⋮y2(14+P2)(i)=W2(i)x2(0)(i)⋮x2(υ2-1)(i) for CW2 - where P1 and P2 are the number of NZP CSI-RS ports for the two configured CSI-RS resources. W1 and W2 are precoding matrices from legacy codebooks for CW1 and CW2. - CSI of one CW is calculated based on interference assumption that inter-CW interference is derived from the CSI-RS resource corresponding to other CW. Inter-CW interference is derived from channel measurement obtained from other NZP CSI-RS resource and precoding matrix according to equation above. However, the equation in above agreement was incorrectly captured and the inter-CW interference was missed in the current specification. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Correct the equation in section 7.2.3 of 36.213 to follow the above agreement. - Capture the inter-CW interference agreement into the specification to support feCoMP. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incorrect specification and the UE may not report the correct CSI for feCoMP. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.2.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

< Unchanged parts are omitted >

7.2.3	Channel Quality Indicator (CQI) definition

The CQI indices and their interpretations are given in Table 7.2.3-1, Table 7.2.3-5, Table 7.2.3-6 for reporting CQI based on QPSK, 16QAM and 64QAM. The CQI indices and their interpretations are given in Table 7.2.3-2 for reporting CQI based on QPSK, 16QAM, 64QAM and 256QAM. The CQI indices and their interpretations are given in Table 7.2.3-3 for reporting CQI based on QPSK and 16QAM. The CQI indices and their interpretations are given in Table 7.2.3-4 for reporting CQI based on QPSK, 16QAM, 64QAM, 256QAM, and 1024QAM.

< Unchanged parts are omitted >

-	The UE-specific reference signal overhead is consistent with the most recent reported rank for the CSI process if more than one CSI-RS port is configured, and is consistent with rank 1 transmission if only one CSI-RS port is configured; and PDSCH signals on antenna ports for layers would result in signals equivalent to corresponding symbols transmitted on antenna ports , as given by,

if the UE is configured with higher layer parameter semiOpenLoop and not configured with higher layer parameter FeCoMPCSIEnabled,

for

for

where  is a vector of symbols from the layer mapping in subclause 6.3.3.2 of [3], is the number of CSI-RS ports configured, and if UE reports a PMI,  where  associated with PMI codebook Table 7.2.4-0A and Table 7.2.4-0B for 4 antenna ports when alternativeCodeBookEnabledFor4TX-r12=TRUE is configured,  associated with PMI codebook Table 7.2.4-1 and Table 7.2.4-2 for 8 antenna ports when higher layer parameter eMIMO-Type is not configured,  associated with PMI codebook Table 7.2.4-10 and Table 7.2.4-11 for 8/12/16/20/24/28/32 antenna ports when higher layer parameter eMIMO-Type is configured, and where  is the column vector associated with the reported first PMI i1 and the second PMI i2 configured according to codebook subset restriction, and otherwise  is the selected precoding matrix corresponding to the reported CQI applicable to . The corresponding PDSCH signals transmitted on antenna ports  would have a ratio of EPRE to CSI-RS EPRE equal to the ratio given in subclause 7.2.5,

otherwise if the UE is not configured with higher layer parameter FeCoMPCSIEnabled or the UE is configured with higher layer parameter FeCoMPCSIEnabled and UE reports CRI with value of 0 or 1,

,

where  is a vector of symbols from the layer mapping in Subclause 6.3.3.2 of [3],  is the number of antenna ports of the associated CSI-RS resource, and if P=1,is 1, otherwise for UE configured for PMI/RI reporting is the precoding matrix corresponding to the reported PMI applicable to  and for UE configured without PMI reporting is the selected precoding matrix corresponding to the reported CQI applicable to . The corresponding PDSCH signals transmitted on antenna ports  would have a ratio of EPRE to CSI-RS EPRE equal to the ratio given in Subclause 7.2.5,

otherwise if the UE is configured with higher layer parameter FeCoMPCSIEnabled and UE reports CRI=2 then the PDSCH signals on antenna ports corresponding to layers of codeword k would result in signals equivalent to corresponding symbols transmitted on antenna ports corresponding to the (k+1)th CSI-RS resource, where are the number of antenna ports for the (k+1)th CSI-RS resource, as given by

yk(15)(i)⋮yk(14+Pk)(i)=Wkixk0i⋮xkυk-1i,k=0,1

where  is a vector of symbols from the layer mapping in Subclause 6.3.3.2 of [3] for codeword k=0, 1 and where the CSI corresponding to a codeword is calculated based on the assumption that inter-codeword interference is derived from channel measurement obtained from the NZP CSI-RS resource and the precoding matrix corresponding to the other codeword. The corresponding PDSCH signals transmitted on antenna ports  would have a ratio of EPRE to CSI-RS EPRE equal to the ratio given in Subclause 7.2.5 for the (k+1)th CSI-RS resource.  If =1, then  is 1 otherwise for UE configured for PMI/RI reporting  is the precoding matrix corresponding to the reported PMI applicable to  and for UE configured without PMI reporting is the selected precoding matrix corresponding to the reported CQI applicable to .

-	Assume no REs allocated for CSI-RS and zero-power CSI-RS

< Unchanged parts are omitted >