3GPP TSG RAN WG1 Meeting #95		R1-1813862

Spokane, US, November 12th – 16th, 2018

Source:	vivo

Title:	UE power Consumption Reduction in RRM Measurements

Agenda Item:	7.2.9.3

Document for:	Discussion and Decision

- Introduction

In RAN1 #94bis, the enhancements to reduce UE power consumption in RRM measurement are discussed, and following agreements are made [1]:

Agreements:

- RRM power saving study should consider RRC IDLE, INACTIVE and CONNECTED states.

Agreements:

- Further study on relaxing/adapting RRM measurement for UE power saving,

  - Relaxing/adapting the number of RRM measurements in time domain for serving cell and neighbor cells based on certain conditions, e.g., number of SSB/CSI-RS/measurement occasions.

  - Relaxing/adapting the number of measured cells/frequencies for neighbor cells based on different conditions

  - Schemes considered should disclose the mechanism used to determine the mobility/speed assumed

  - FFS conditions, e.g., channel condition, UE location, UE speed, measurement configuration, measurement results, cell type, etc.

  - FFS impact on measurement/mobility performance

  - Relaxed/adapting measurement report (if any)

  - Other solutions are not precluded.

Agreements:

- Further study on the benefit/impact for additional resource for RRM measurements for power saving for mobility performance not inferior to that of Rel-15

  - The schemes proposed should at least include the following aspects:

    - The signals/channels used as additional resource

    - Procedure on how to use additional resources.

    - Overhead from network perspective should be considered.

Agreements:

- Focus on stationary (e.g., 0km/h), pedestrian (e.g., 3km/h) and vehicular (e.g., 30km/h) scenarios for further studies when considering UE power saving techniques for RRM measurements,

- Further studies UE power saving for RRM measurement by considering the following issues

  - Multi-beam/multi-cell related RRM measurement issues.

  - Possible misalignment between SSB(s)/SMTC and DRX ON duration

In this contribution, we will discuss the enhancements which may reduce the power consumption in RRM, and simulation results and numerical analysis are provided. This contribution is revised from R1-1812332.

- Power consumption in RRM

In NR, RRM is performed by UE in RRC idle, RRC inactive and RRC connected states.

For both idle state and connected state, RRM measurement includes serving cell and intra/inter frequency measurement. Intra and inter frequency measurement are only performed when serving cell quality does not fulfill certain conditions. Thus, UE performs serving cell measurement more frequently compared with intra/inter frequency measurement.

In RRC connected state, more behaviors should be performed by UE, e.g. more frequent PDCCH and PDSCH reception, therefore RRM may not take great proportion in the overall power consumption. However, in RRC idle state, UE mainly performs two behaviors, which are RRM measurement and paging monitoring. For serving cell measurement in idle state, it is specified in TS38.133 [2] that:

- The UE shall filter the SS-RSRP and SS-RSRQ measurements of the serving cell using at least 2 measurements. Within the set of measurements used for the filtering, at least two measurements shall be spaced by, at least DRX cycle/2.

Accordingly, UE should perform serving cell measurement twice per paging DRX cycle. Based on the power model in [3], assuming 10% group paging rate, the UE energy consumption in IDLE mode for RRM measurement and paging monitoring is shown in Table 1. It can be seen that most of the UE energy (62%) is consumed by RRM measurement. In the analysis only serving cell RRM measurement is considered, thus it is expected that the energy consumption by RRM measurement will be even higher when neighbor cell measurement is also included.

Table 1: UE energy consumption in IDLE mode

| UE behavior | Energy consumption (relative power units) |
| --- | --- |
| RRM (serving cell measurement) | 6400 |
| Paging monitoring (90% PDCCH only and 10% PDCCH+PDSCH) | 4000 |
| Additional assumptions: - The duration of SSB measurement occasion per paging cycle is 1 slot. - The duration of paging monitoring per paging cycle is 1 slot. - The power consumption is counted in 10 seconds duration. - The slot duration is based on 15 kHz SCS. - Pre-sync per paging cycle is not modeled in this simulation. |  |

Observation 1: In idle state, the power consumption caused by RRM measurements takes a great proportion of the overall UE power consumption.

- Enhancements to reduce power consumption in serving cell measurement

As shown in Figure 1, since UE is mandated to measure on two occasions which are equally spaced within each I-DRX cycle, UE have to wake up twice per I-DRX cycle, one is for both paging and RRM measurement, and the other is dedicated for RRM measurement. Based on the power model in [3], except for the PDCCH/PDSCH monitoring and RRM measurement, the power consumption caused by the transition from the deep/light sleep is also non-negligible. Frequent wake-up and active/sleep transition will cause high UE energy consumption.

Figure 1: UE behaviors in idle state

Besides, the SMTC is configured cell-specifically, while the occasions for paging monitoring are UE-group specific. In most cases, the paging and the SMTC measurement occasions are not aligned in time, especially considering SMTC is sparse in time in practical deployment. As shown in Figure 2, to perform both RRM and paging monitoring, UE may have to keep awake between paging occasion and the nearest SSB occasion beforehand, and this will increase the overall active time and UE power consumption correspondingly.

Observation 2: Reducing measurement activities should be targeted for UE power saving in RRM.

To achieve the target, two approaches can be considered, i.e. to reduce the number of required measurement samples in time domain, and to align the measurement occasion with the paging monitoring occasion.

- Reduce the number of requirement measurement samples in time

Reducing number of measurement samples may lead to performance loss in RRM thus the measurement accuracy shall be carefully studied. From the link level simulation results shown in Figure 2 and 3, non-negligible loss on RSRP accuracy is observed when the L1 RRM sample for each paging DRX cycle is reduced from 2 to 1.

In order to maintain the same measurement accuracy with less measurement samples, additional measurement RS, e.g. CSI-RS, can be introduced for UE in idle state. To evaluate the accuracy of measurement based on SSB and CSI-RS, simulation has been taken. Both layer 1 RSRP and layer 3 filtered RSRP are evaluated under 3km/h and 30km/h mobility speed. Other simulation assumptions are given in Table A-1 in Annex.

| (1) L1 RSRP accuracy | (2) L3 filtered RSRP accuracy |
| --- | --- |

Figure 2: RSRP accuracy per evaluation period-TDLC 3km/h

| (1) L1 RSRP accuracy | (2) L3 filtered RSRP accuracy |
| --- | --- |

Figure 3: RSRP accuracy per evaluation period-TDLC 30km/h

* Note: Measurement period (MP) is equal to the DRX cycle.

According to the simulation results in Figure 2 and Figure 3, the measurement accuracy of CSI-RS-RSRP with 1 sample outperforms SSB-RSRP with 2 samples per measurement period, which means UE power consumption can be reduced by perform CSI-RS based measurement with only one measurement sample per DRX cycle without sacrificing measurement accuracy, compared with 2 SSB based measurement samples per DRX cycle. In this simulation, 0dB SNR is considered. In scenarios with high SNR, it can be expected that measurement accuracy can be easily satisfied based on one measurement sample per measurement period, regardless SSB based or CSI-RS based. Which means UE can select and use either SSB or CSI-RS that is closer to paging occasion as the measurement RS to reduce the wake-up duration. As illustrated in Figure 4, in this particular case, UE can use the CSI-RS resource before paging occasion for RRM.

Figure 4: SSB or CSI-RS closer to paging occasion is used for RRM

To obtain the quantitative power saving gain of the enhanced measurement method compared with SSB based measurement, we evaluate the power consumption for these methods based on the power model provided in [1], and power consumption for each power state are given in Table A-2. In this evaluation, the power saving gain takes an average of different paging monitoring occasions (different duration between PO and measurement occasion) into account.

Accordingly, the total power consumption in idle state using CSI-RS and SSB based serving cell measurement are given in the following Table 2.

Table 2: Idle UE power consumption analysis

|  | Average power consumption with different paging offset (relative power units) |
| --- | --- |
| Only SSB is used for RRM (2 samples per DRX cycle) | 52983.5 (baseline) |
| Only CSI-RS is used for RRM (1 sample per DRX cycle) | 35964.5 (-32.1%) |
| SSB or CSI-RS closer to paging occasion is used for RRM (1 sample per DRX cycle) | 34123.5 (-35.6%) |
| Additional assumption: - The duration of SSB/CSI-RS measurement occasion per paging cycle is 1 slot. - Periodicity of SSB is 20ms and SSB is transmitted in the 1st slot of each 20ms period. - Periodicity of CSI-RS is 10ms and CSI-RS is transmitted in the 6th slot of each radio frame. - Paging cycle (DRX cycle) is 320ms, and the paging is monitored in one slot in first 2 radio frames of each paging cycle by different UEs with equal probability. - The power consumption is counted in 10 seconds duration. - The slot duration is based on 15 kHz SCS. |  |

Based on the results above, we have the following observation.

Observation 3: For UE in idle state, about 35% power saving gain can be achieved if CSI-RS can be introduced as an additional measurement RS, based on which the measurement samples can be reduced and the measurement accuracy can still be guaranteed.

Note that, for UE in RRC connected state, CSI-RS is used for different purposes, e.g. CSI acquisition, radio link monitoring, time and frequency tracking, beam failure detection, etc. The CSI-RS resources are usually shared with multiple UEs for same or different purposes. According to the assumptions in IMT-2020 Self-Evaluation [5], the CSI-RS are configured with 5 slots periodicity for CSI measurement. Therefore, it will not cause additional network overhead if the CSI-RS configured for RRC connected state UEs is reused as measurement RS for UEs in idle state.

Furthermore, the CSI-RS based RRM measurement is already supported in Rel-15 as a mandatory UE feature, therefore use of CSI-RS for idle mode RRM measurement does not cause any problem from UE complexity perspective.

Observation 4: Use of CSI-RS for idle state RRM measurement does not cause issues from network overhead or UE complexity perspective.

Proposal 1: CSI-RS is introduced as measurement RS for UE in idle state.

Proposal 2: UE is allowed to take one sample for RRM measurement per paging DRX cycle.

Furthermore, the measurement period can even be extended to two DRX cycles if measurement accuracy is not degraded in the prioritized scenarios for RRM enhancement. Then UE energy consumption for RRM can be further reduced. The accuracy of RSRP after layer 3 filter with different L1 sample rates and L3 measurement periodicities is illustrated in Figure 5, where both SSB and CSI-RS based measurement are considered.

| (1) TDL-C 3km/h | (2) TDL-C 30km/h |
| --- | --- |

Figure 5: Layer 3 RSRP accuracy per evaluation period

As illustrated in Figure 5, the layer 3 RSRP accuracy will not degrade if the layer 1 RSRP sample is reported to layer 3 every two DRX cycles. Therefore, it is feasible for UE to measure once in every two paging DRX cycles as the RSRP accuracy is not degraded. Furthermore, the accuracy of L3 RSRP based on CSI-RS with only 1 L1 sample every two DRX cycles still outperforms that based on SSB with 2 L1 samples every DRX cycle. With less measurement samples provided to layer 3, UE only need to perform RRM measurement in half/quarter of the DRX cycles, which lead to half/quarter power consumption in RRM measurement.

Proposal 3: The L3 RRM measurement periodicity can be relaxed to multiple paging DRX cycles.

For UE in connected state, reducing the number of measurements within a certain time can reduce the UE power consumption. We evaluate the overall power consumption with different measurement sample rates and C-DRX configurations under FTP model 3.

Table 3: Relaxed RRM measurement in connected state (FTP Model 3)

| DRX InactivityTimer [ms] | Measurement reporting period [ms] | Power consumption per ms | Average Packet Delay [ms] | UPT [kbps] |
| --- | --- | --- | --- | --- |
| 40 | 200 | 40.45 (+29%) | 18.35 | 861 |
|  | 400 | 35.80 (+14%) |  |  |
|  | 800 | 31.38 |  |  |
| 100 | 200 | 73.65 (+6.5%) | 11.14 | 1277 |
|  | 400 | 69.98 (+1.2%) |  |  |
|  | 800 | 69.15 |  |  |
| Additional assumptions: - Long DRXCycle = 160ms, onDurationTimer = 8ms, data_rate=300Mbps - FTP Model 3, packet size =0.5MByte |  |  |  |  |

Based on the above results, if the measurement period increase 4 times (i.e., 800ms), the power consumption can be reduced by 6.5 ~ 29%.

Proposal 4: Extending RRM measurement periodicity is beneficial for power saving when C-DRX is configured.

Typically, the network can configured the RRM measurement based on the cell type. For example, an indoor cell can always configure the UEs belongs to its own with a relaxed RRM measurement. UE with assistant information for adapting RRM measurement is also of interests. For example, ue speed, RSRP/RSRQ fluctuation can provide useful information to gNB to decide the measurement related parameters.

Proposal 5: The network can adapt the RRM measurement either with or without UE assisted information.

- Power consumption in intra-frequency measurement

For both RRC idle and connected state UE, when the quality of serving cell is not higher than a certain threshold, intra-frequency measurement should be performed. In [4], power model of intra frequency measurement under synchronous and asynchronous network is provided. According to the power model, it is natural that less number of measured cells leads to lower power consumption.

Similar to the criteria to trigger intra-frequency measurement, one method to reduce the required number of intra-frequency is to allow UE keep tracking and measuring a subset of identified neighbor cells when the quality of these cells are higher than certain threshold. With this method, UE does not have to measure all the slots within SMTC but only measure the slot containing SSBs from “N strong neighbor cells” (e.g. N=2, 4), which can reduce UE energy consumption.  Figure 6 shows an example where the SMTC is 4 slots long but the SSBs from strong neighbor cells are located in only 2 slots within the SMTC. Less number of measured SSB may also means shorter time for RRM measurement, especially in asynchronous scenarios and in cases that SSBs from different cells are located in different slots.

Figure 6: Illustration of less measurements in SMTC window

We evaluated and compared the power consumption for intra frequency measurement with different number of measured cells, according to the power model provided in [4]. Different measurement duration for different number of intra-frequency cells in asynchronous case is assumed. Detailed evaluation results and assumptions are given in Table 3.

Table 3: Comparison of power consumption of intra frequency measurement on different number of cells

| Number of required intra-frequency measurement cells | Synchronous | Asynchronous |
| --- | --- | --- |
| Energy for full measurement (8 cells) | 58034 | 81656 |
| Energy for reduced measurement (only 4 strongest cells) | 54542 | 56870 |
| Power saving gain | -6.02% | -30.4% |
| Additional assumptions - SMTC duration=4 slots - In the synchronous case, SSBs from all the neighbor cells are located in 2 slots within SMTC. UE measures 2 slots for each SMTC - In the asynchronous case, SSB from 8 cells are located in 4 slots, within which the subset of 4 strong cells are located in 2 slots within the SMTC. UE measures 4 slots for full measurement but measures only 2 slots for reduced measurement. - Paging cycle (DRX cycle) is 320ms, 2 measurement samples per paging cycle. - The power consumption is counted in 10 seconds duration. |  |  |

Based on the power evaluation above, we have the following observation and proposal,

Observation 5:  Reducing number of measured intra-frequency cells provides power saving benefit, especially when SSBs from neighbor cells are located at different slots within the SMTC.

Proposal 6: Reducing number of measured intra-frequency cells should be supported for NR UE power saving.

- Conclusion

In this contribution, we focus on the optimizations on power consumption reduction in RRM, and have the following proposals and observations:

Observation 1: In idle state, the power consumption caused by RRM measurements takes a great proportion of the overall UE power consumption.

Observation 2: Reducing measurement activities should be targeted for UE power saving in RRM.

Observation 3: For UE in idle state, about 35% power saving gain can be achieved if CSI-RS can be introduced as an additional measurement RS, based on which the measurement samples can be reduced and the measurement accuracy can still be guaranteed.

Observation 4: Use of CSI-RS for idle state RRM measurement does not cause issues from network overhead or UE complexity perspective.

Observation 5:  Reducing number of measured intra-frequency cells provide power saving benefit, especially when SSBs from neighbor cells are located at different slots within the SMTC.

Proposal 1: CSI-RS is introduced as measurement RS for UE in idle state.

Proposal 2: UE is allowed to take one sample for RRM measurement per paging DRX cycle.

Proposal 3: The L3 RRM measurement periodicity can be relaxed to multiple paging DRX cycles.

Proposal 4: Extending RRM measurement periodicity is beneficial for power saving when C-DRX is configured.

Proposal 5: The network can adapt the RRM measurement either with or without UE assisted information.

Proposal 6: Reducing number of measured intra-frequency cells should be supported for NR UE power saving.

- References

- Chairman’s notes, RAN1#94bis, Chengdu, China, October, 2018.

- 3GPP TS 38.133, “Requirements for support of radio resource management”.

- R1-1811897, “Summary Power model for NR power saving”, MediaTek, Qualcomm, Apple, Intel, CATT, InterDigital, OPPO, vivo, Samsung, Panasonic, RAN1#94bis, Chengdu, China, October, 2018.

- R1-1812079, “[94b-NR-10] Power modelling for RRM measurement /search”, Qualcomm, RAN1#94bis, Chengdu, China, October, 2018.

- RP-182097, “Draft LTI on 3GPP 5G Preliminary Description Template and Self-Evaluation”, Gold Coast, Australia, September 10 - 13, 2018.

- Annex-1 : Simulation assumption for RSRP accuracy

Table A-1: Simulation assumptions for RSRP accuracy

| Simulation parameters | values |
| --- | --- |
| bandwidth | 20 MHz |
| Es/N0 | 0dB |
| Channel model | TDL-C Low 100ns 10Hz 1X2 |
| Subcarrier spacing | The same as SS block subcarrier spacing |
| Measurement period(paging cycle) | 320ms |
| Number of measurement samples per measurement period (MP) | 1,2 |
| Subcarrier spacing | 15 kHz |
| Number of SS blocks per SS burst set/ SS burst periodicity | 1/20ms |
| New measurement RS | CSI-RS: - Density: 3RE/RB - 1 port - 2 symbols per slot - whole bandwidth |
| L3 filter | Defined in section 5.5.3.2 in 38.331 Fn=1-a∙Fn-1+a∙Mn Where a = 1/2(k/4) and k=1 |
| Evaluation metric | Delta RSRP(dB) = measured RSRP – ideal RSRP |

- Annex-2: Power model for RRM

Table A-2: power assumptions for RSRP accuracy

| Power State | Characteristics | Relative Power |
| --- | --- | --- |
| Deep Sleep | Time interval (X>20ms) for the sleep should be larger than the total transition time entering and leaving this state. Accurate timing may not be maintained. | 1 |
| Light Sleep | Time interval (6ms<=X<=20ms)for the sleep should be larger than the total transition time entering and leaving this state. | 20 |
| Micro sleep | Immediate transition is assumed for power saving study purpose from or to a non-sleep state; (X<6ms) | 45 |
| PDCCH-only | No PDSCH and same-slot scheduling; this includes time for PDCCH decoding and any micro-sleep within the slot. | 100 |
| SSB or CSI-RS proc. | SSB can be used for fine time-frequency sync. And RSRP measurement of the serving/camping cell. FFS the power scaling for RRM of neighbor cells . TRS is the considered CSI-RS for sync. FFS the power scaling for processing other configurations of CSI-RS. | 100 |
| PDCCH + PDSCH | PDCCH + PDSCH. ACK/NACK in long PUCCH is modeled by UL power state. FFS the power scaling for PDSCH-only slot. | 300 |