3GPP TSG RAN WG1 Meeting #95 R1-1813915 Spokane, USA, November 12th – 16th, 2018 CR-Form-v11.2. CHANGE REQUEST. 38.214; CR; DRAFT; rev; -; Current version:; 15.3.0. For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests.. Proposed change affects:; UICC apps; ME; X; Radio Access Network; X; Core Network. Title:; Draft CR to TS38.214 on SRS. Source to WG:; ZTE. Source to TSG:. Work item code:; NR_newRAT-Core; Date:; 2018-11-2. Category:; F; Release:; Rel-15. Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900.; Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16). Reason for change:; In current TS 38.214 for the UL beam management, the SRS resources in different SRS resource sets can be transmitted simultaneously. This rule was discussed assuming the same BWP of those SRS resource sets for BM. Or else, this rule is not correct if two SRS resource sets are in different BWPs or CCs.. Summary of change:; The current description for BM should be restricted in the same BWP or CCs.. Consequences if not approved:; The description for UL beam management is not correct if two SRS resource sets are in different BWPs or CCs.. Clauses affected:; 6.2.1. Y; N. Other specs; X; Other core specifications; TS/TR ... CR .... affected:; X; Test specifications; TS/TR ... CR .... (show related CRs); X; O&M Specifications; TS/TR ... CR .... Other comments:.

6.2.1	UE sounding procedure

The UE can be configured with one or more Sounding Reference Signal (SRS) resource sets as configured by the higher layer parameter SRS-ResourceSet. For each SRS resource set, a UE may be configured with SRS resources (higher layer parameter SRS-Resource), where the maximum value of K is indicated by [SRS_capability [13, 38.306]]. The SRS resource set applicability is configured by the higher layer parameter usage in SRS-ResourceSet. When the higher layer parameter usage is set to 'BeamManagement', only one SRS resource in each of multiple SRS sets can be transmitted at a given time instant, andthe SRS resources in different SRS resource sets in the same BWP can be transmitted simultaneously.

<Unchanged part omitted>