3GPP TSG RAN WG1 Meeting #95								    R1-1814062

Spokane, USA, November 12th – 16th, 2018

Agenda Item:	7.2.2.1

Source:	InterDigital Inc.

Title:	NR-U coexistence evaluation results

Document for:	Discussion and Decision

# Introduction

In this contribution, we present evaluation results for coexistence between WiFi and NR-U. The deployment scenarios used for this evaluation are the ones approved and later the layout dimensions were agreed in RAN1#92, 92b and 93 meetings [1,2,3].

This contribution is an update of contribution R1-1813216.

# Coexistence evaluation

To evaluate coexistence between WiFi and NRU, the following scenarios are simulated:

- Wi-Fi + Wi-Fi

- Wi-Fi + NR-U

- NR-U + NR-U

The goal for simulation of the top two scenarios is to evaluate the performance of WiFi with and without presence of NR-U. This is to verify whether the objective of the NR-U SI [4] is met. Particularly, to evaluate the impact of an NR-U network on an existing WiFi network as described in the SID [4]:

“Coexistence methods within NR-based and between NR-based operation in unlicensed and LTE-based LAA and with other incumbent RATs in accordance with regulatory requirements in e.g., 5GHz, 6GHz bands

  - Coexistence methods already defined for 5GHz band in LTE-based LAA context should be assumed as the baseline for 5GHz operation. Enhancements in 5GHz over these methods should not be precluded. NR-based operation in unlicensed spectrum should not impact deployed Wi-Fi services (data, video and voice services) more than an additional Wi-Fi network on the same carrier; ”

## Evalution assumptions

The general assumptions regarding deployment scenarios are taken from the agreements in [2,3], and for convenience reported in Appendix in Table 1 and 2.

Impementation-related assumptions for WiFi is reported in Appendix in Table 3 and the assumptions for NR-U is reported in Appendix in Table 4.

## Evaluation results – Indoor deployment

The table below shows coexistence evaluation results for the three scenarios (Wi-Fi+Wi-Fi, Wi-Fi+NR-U and NR-U+NR-U) under low, medium and high traffic load.

Indoor Scenario

| Tdoc / Source | Reported parameters |  | Low load BO range for Wi-Fi in Wi-Fi+Wi-Fi: 10%~25% |  |  |  | Medium load BO range for Wi-Fi in Wi-Fi+Wi-Fi: 35%~50% |  |  |  | High load BO range for Wi-Fi in Wi-Fi+Wi-Fi: above 55% |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Wi-Fi in WiFi+ WiFi | Wi-Fi in WiFi+ NR-U | NR-U in WiFi+ NR-U | NR-U in NR-U+ NR-U | Wi-Fi in WiFi+ WiFi | Wi-Fi in WiFi+ NR-U | NR-U in WiFi+ NR-U | NR-U in NR-U+ NR-U | Wi-Fi in WiFi+ WiFi | Wi-Fi in WiFi+ NR-U | NR-U in WiFi+ NR-U | NR-U in NR-U+ NR-U |
| R1- 1814062 /InterDigital | DL UPT CDF [Mbps] | 5% | 17.154 | 17.711 | 41.850 | 63.416 | 3.943 | 4.645 | 9.842 | 22.380 | 1.500 | 1.402 | 2.691 | 23.185 |
|  |  | 50% | 77.534 | 86.665 | 148.551 | 171.191 | 32.930 | 36.844 | 82.630 | 146.557 | 13.770 | 19.858 | 53.544 | 122.645 |
|  |  | 95% | 128.170 | 133.654 | 198.427 | 198.431 | 110.541 | 113.586 | 197.805 | 198.370 | 73.161 | 97.293 | 194.578 | 197.587 |
|  |  | Mean | 76.170 | 80.257 | 140.165 | 155.393 | 42.386 | 47.433 | 94.770 | 131.612 | 21.862 | 31.864 | 75.520 | 120.925 |
|  | DL Delay CDF [s] | 5% | 0.032 | 0.031 | 0.021 | 0.021 | 0.036 | 0.036 | 0.021 | 0.021 | 0.063 | 0.044 | 0.021 | 0.021 |
|  |  | 50% | 0.056 | 0.050 | 0.027 | 0.025 | 0.133 | 0.129 | 0.055 | 0.30 | 0.385 | 0.280 | 0.094 | 0.034 |
|  |  | 95% | 0.215 | 0.232 | 0.094 | 0.064 | 1.063 | 1.391 | 1.116 | 0.187 | 2.866 | 2.908 | 2.647 | 0.191 |
|  |  | Mean | 0.085 | 0.081 | 0.043 | 0.037 | 0.278 | 0.325 | 0.225 | 0.055 | 0.782 | 0.658 | 0.505 | 0.062 |
|  | UL UPT CDF [Mbps] | 5% | 28.220 | 31.549 | 30.852 | 26.097 | 13.497 | 10.113 | 7.552 | 18.442 | 7.330 | 3.494 | 4.978 | 16.160 |
|  |  | 50% | 81.430 | 95.070 | 147.554 | 157.696 | 44.765 | 52.213 | 66.107 | 67.717 | 28.108 | 31.130 | 40.481 | 65.771 |
|  |  | 95% | 125.537 | 129.270 | 198.879 | 198.460 | 113.807 | 116.741 | 167.788 | 171.151 | 77.057 | 109.554 | 162.036 | 170.362 |
|  |  | Mean | 80.987 | 89.301 | 133.846 | 141.838 | 53.162 | 56.365 | 75.220 | 79.673 | 33.345 | 41.111 | 59.763 | 78.704 |
|  | UL Delay CDF [s] | 5% | 0.033 | 0.031 | 0.021 | 0.032 | 0.035 | 0.036 | 0.026 | 0.024 | 0.060 | 0.038 | 0.030 | 0.024 |
|  |  | 50% | 0.050 | 0.044 | 0.027 | 0.024 | 0.095 | 0.085 | 0.072 | 0.063 | 0.150 | 0.160 | 0.160 | 0.063 |
|  |  | 95% | 0.128 | 0.127 | 0.134 | 0.122 | 0.306 | 0.913 | 0.726 | 0.224 | 0.659 | 1.241 | 1.241 | 0.226 |
|  |  | Mean | 0.064 | 0.058 | 0.044 | 0.044 | 0.125 | 0.223 | 0.203 | 0.086 | 0.227 | 0.338 | 0.338 | 0.084 |
|  | 𝜌DL |  | 0.9990 | 0.9994 | 0.9994 | 1.00 | 0.9916 | 0.9960 | 0.9960 | 1.00 | 0.9786 | 0.967 | 0.9760 | 1.00 |
|  | 𝜌UL |  | 0.9984 | 0.9995 | 1.00 | 1.00 | 0.9980 | 0.9974 | 1.00 | 1.00 | 0.9957 | 0.976 | 0.9673 | 0.9870 |
|  | BO |  | 11.00 | 10.4 | 5.83 | 4.68 | 39.50 | 42.00 | 28.60 | 16.6 | 67.88 | 62.00 | 40.60 | 18.00 |
|  | 𝜆 |  | 0.167/0.167 |  |  |  | 0.3/0.3 |  |  |  | 0.4/0.4 |  |  |  |
|  |  |  | Additional comments: Laypout and parameters for Indoor Sub-7 deployment Scenario are from R1-1807384. WiFi settings: 802.11ac MCS table including 256 QAM, 2Tx2Rx in DL (cross-polarized, open loop), 2Tx2Rx in UL, 2 streams in both DL and UL. GI:0.8 µs, TXOP=4 ms, LDPC, A-MPDU enabled, RTS/CTS enabled, link adaptation: Minstrel algorithm, CWS: DL{15,63} and UL{15,1023}, CCA: CS=-82dBm, ED=-62dBm. NR-U settings: 4Tx2Rx in DL, Cross-polarized. MCS=4/16/64/256QAM, scheduling: proportional fair, link adaptation realistic, ED=-72 dBm, CP=Normal, SCS=30KHz, TXOP=4 ms, UE Capability #1, MCS: DL{15,63} and UL{15,1023}, COT sharing enabled, COT details: flexible DL/UL only and mixed DL/UL based on traffic needs, 3/11 DL control/data symbols, 3/11 symbols UL control/data. |  |  |  |  |  |  |  |  |  |  |  |

The results above show that in all three various level of traffic loads, Wi-Fi performance in Wi-Fi+NR-U is better than Wi-Fi performance in Wi-Fi+Wi-Fi.

Observation 1: Our results show that in all the indoor simulated scenarios, Wi-Fi performance in Wi-Fi+NR-U is better than Wi-Fi performance in Wi-Fi+Wi-Fi.

## Evaluation results – Outdoor deployment

The tables below shows coexistence evaluation results for both Outdoor scenario 1 and scenario 2, under all the three coexistence scenarios while operating under low, medium and high load.

Outdoor Scenario 1

| Tdoc / Source | Reported parameters |  | Low load BO range for Wi-Fi in Wi-Fi+Wi-Fi: 10%~25% |  |  |  | Medium load BO range for Wi-Fi in Wi-Fi+Wi-Fi: 35%~50% |  |  |  | High load BO range for Wi-Fi in Wi-Fi+Wi-Fi: above 55% |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Wi-Fi in Wi-Fi+ Wi-Fi | Wi-Fi in Wi-Fi+ NR-U | NR-U in Wi-Fi+ NR-U | NR-U in NR-U+ NR-U | Wi-Fi in Wi-Fi+ Wi-Fi | Wi-Fi in Wi-Fi+ NR-U | NR-U in Wi-Fi+ NR-U | NR-U in NR-U+ NR-U | Wi-Fi in Wi-Fi+ Wi-Fi | Wi-Fi in Wi-Fi+ NR-U | NR-U in Wi-Fi+ NR-U | NR-U in NR-U+ NR-U |
| R1- 1814062 /InterDigital | DL UPT CDF [Mbps] | 5% | 11.891 | 12.482 | 27.214 | 40.856 | 1.566 | 4.815 | 7.408 | 13.925 | 0.961 | 2.345 | 5.858 | 8.077 |
|  |  | 50% | 63.790 | 80.543 | 134.350 | 142.631 | 20.597 | 41.713 | 69.150 | 89.247 | 12.230 | 18.196 | 34.205 | 57.872 |
|  |  | 95% | 116.687 | 127.743 | 201.932 | 199.782 | 98.768 | 106.480 | 181.136 | 198.498 | 71.163 | 78.572 | 117.063 | 179.804 |
|  |  | Mean | 64.637 | 73.768 | 131.201 | 148.222 | 31.725 | 47.200 | 79.495 | 104.393 | 20.757 | 26.668 | 46.786 | 80.682 |
|  | DL Delay CDF [s] | 5% | 0.033 | 0.030 | 0.021 | 0.021 | 0.042 | 0.039 | 0.024 | 0.021 | 0.054 | 0.052 | 0.038 | 0.030 |
|  |  | 50% | 0.06 | 0.051 | 0.031 | 0.023 | 0.205 | 0.145 | 0.080 | 0.038 | 0.325 | 0.304 | 0.196 | 0.116 |
|  |  | 95% | 0.355 | 0.359 | 0.165 | 0.162 | 1.654 | 1.319 | 0.612 | 0.301 | 2.264 | 2.700 | 1.162 | 0.645 |
|  |  | Mean | 0.116 | 0.110 | 0.056 | 0.050 | 0.439 | 0.338 | 0.163 | 0.092 | 0.634 | 0.612 | 0.344 | 0.240 |
|  | UL UPT CDF [Mbps] | 5% | 10.305 | 10.514 | 26.193 | 31.033 | 1.327 | 5.022 | 6.596 | 12.632 | 0.737 | 2.713 | 3.013 | 8.462 |
|  |  | 50% | 63.356 | 77.355 | 119.294 | 128.471 | 18.200 | 35.384 | 58.210 | 66.122 | 11.274 | 20.049 | 23.930 | 32.478 |
|  |  | 95% | 110.755 | 117.296 | 191.984 | 194.520 | 93.601 | 94.371 | 160.740 | 177.824 | 78.363 | 81.837 | 117.751 | 164.475 |
|  |  | Mean | 64.278 | 71.860 | 110.596 | 118.240 | 31.913 | 43.966 | 69.836 | 77.030 | 22.160 | 28.613 | 38.368 | 57.172 |
|  | UL Delay CDF [s] | 5% | 0.032 | 0.032 | 0.021 | 0.020 | 0.042 | 0.041 | 0.027 | 0.024 | 0.050 | 0.050 | 0.043 | 0.033 |
|  |  | 50% | 0.063 | 0.052 | 0.033 | 0.319 | 0.222 | 0.168 | 0.113 | 0.110 | 0.371 | 0.315 | 0.291 | 0.287 |
|  |  | 95% | 0.308 | 0.309 | 0.144 | 0.144 | 2.195 | 1.283 | 1.101 | 0.060 | 3.004 | 2.988 | 2.029 | 1.629 |
|  |  | Mean | 0.114 | 0.099 | 0.059 | 0.057 | 0.514 | 0.362 | 0.256 | 0.246 | 0.752 | 0.741 | 0.551 | 0.367 |
|  | 𝜌DL |  | 0.9813 | 0.9800 | 0.9800 | 1.00 | 0.9280 | 0.9560 | 0.9560 | 1.00 | 0.8882 | 0.8880 | 0.8880 | 0.9530 |
|  | 𝜌UL |  | 0.9740 | 0.9700 | 1.00 | 1.00 | 0.9285 | 0.9530 | 0.9846 | 1.00 | 0.9026 | 0.8950 | 0.9325 | 0.9244 |
|  | BO |  | 10.0 | 8.7 | 4.05 | 3.90 | 40.0 | 30.70 | 17.80 | 13.65 | 58.0 | 54.90 | 34.80 | 21.10 |
|  | 𝜆 |  | 0.086/0.086 |  |  |  | 0.114/0.114 |  |  |  | 0.143/0.143 |  |  |  |
|  |  |  | Additional comments: Outdoor Sub-7 deployment Scenario 1 WiFi settings: 802.11ac MCS table including 256 QAM, 2Tx2Rx in DL (cross-polarized, open loop), 2Tx2Rx in UL, 2 streams in both DL and UL. GI:0.8 µs, TXOP=4 ms, LDPC, A-MPDU enabled, RTS/CTS disabled, link adaptation: Minstrel algorithm, CWS: DL{15,63} and UL{15,1023}, CCA: CS=-82dBm, ED=-62dBm. NR-U settings: 4Tx2Rx in DL, Cross-polarized. MCS=4/16/64/256QAM, scheduling: proportional fair, link adaptation realistic, ED=-72 dBm, CP=Normal, SCS=30KHz, TXOP=4 ms, UE Capability #1, MCS: DL{15,63} and UL{15,1023}, COT sharing enabled, COT details: flexible DL/UL only and mixed DL/UL based on traffic needs, 3/11 DL control/data symbols, 3/11 symbols UL control/data. |  |  |  |  |  |  |  |  |  |  |  |

Outdoor Scenario 2

| Tdoc / Source | Reported parameters |  | Low load BO range for Wi-Fi in Wi-Fi+Wi-Fi: 10%~25% |  |  |  | Medium load BO range for Wi-Fi in Wi-Fi+Wi-Fi: 35%~50% |  |  |  | High load BO range for Wi-Fi in Wi-Fi+Wi-Fi: above 55% |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Wi-Fi in Wi-Fi+ Wi-Fi | Wi-Fi in Wi-Fi+ NR-U | NR-U in Wi-Fi+ NR-U | NR-U in NR-U+ NR-U | Wi-Fi in Wi-Fi+ Wi-Fi | Wi-Fi in Wi-Fi+ NR-U | NR-U in Wi-Fi+ NR-U | NR-U in NR-U+ NR-U | Wi-Fi in Wi-Fi+ Wi-Fi | Wi-Fi in Wi-Fi+ NR-U | NR-U in Wi-Fi+ NR-U | NR-U in NR-U+ NR-U |
| R1- 1814062 /InterDigital | DL UPT CDF [Mbps] | 5% | 3.714 | 4.364 | 30.880 | 37.545 | 1.541 | 3.672 | 16.983 | 18.597 | 0.707 | 1.552 | 6.490 | 12.252 |
|  |  | 50% | 43.175 | 62.566 | 123.093 | 147.561 | 23.461 | 37.345 | 100.256 | 109.054 | 15.120 | 22.544 | 46.490 | 52.998 |
|  |  | 95% | 105.741 | 117.282 | 201.034 | 191.196 | 102.653 | 112.321 | 181.664 | 182.641 | 96.035 | 101.527 | 180.281 | 188.214 |
|  |  | Mean | 53.272 | 61.646 | 126.294 | 135.740 | 37.662 | 52.038 | 107.190 | 124.808 | 27.231 | 35.182 | 69.630 | 89.052 |
|  | DL Delay CDF [s] | 5% | 0.039 | 0.033 | 0.021 | 0.022 | 0.035 | 0.034 | 0.021 | 0.020 | 0.042 | 0.041 | 0.023 | 0.021 |
|  |  | 50% | 0.091 | 0.060 | 0.035 | 0.031 | 0.164 | 0.088 | 0.046 | 0.040 | 0.277 | 0.161 | 0.106 | 0.097 |
|  |  | 95% | 1.032 | 0.581 | 0.126 | 0.120 | 1.840 | 0.990 | 0.260 | 0.240 | 3.313 | 3.001 | 1.054 | 0.797 |
|  |  | Mean | 0.276 | 0.198 | 0.054 | 0.053 | 0.458 | 0.262 | 0.086 | 0.068 | 0.690 | 0.594 | 0.253 | 0.171 |
|  | UL UPT CDF [Mbps] | 5% | 4.938 | 11.565 | 14.560 | 32.995 | 1.879 | 5.905 | 7.896 | 26.318 | 0.310 | 1.703 | 3.515 | 15.488 |
|  |  | 50% | 57.731 | 84.192 | 110.640 | 121.620 | 24.350 | 62.856 | 75.543 | 80.462 | 13.294 | 22.741 | 35.754 | 49.990 |
|  |  | 95% | 106.614 | 114.090 | 190.409 | 190.086 | 91.471 | 111.718 | 168.420 | 170.992 | 73.424 | 92.983 | 133.636 | 137.900 |
|  |  | Mean | 57.767 | 70.590 | 103.105 | 106.134 | 34.531 | 59.456 | 84.700 | 87.968 | 20.858 | 33.784 | 49.572 | 65.205 |
|  | UL Delay CDF [s] | 5% | 0.036 | 0.032 | 0.021 | 0.021 | 0.040 | 0.034 | 0.022 | 0.021 | 0.047 | 0.040 | 0.031 | 0.300 |
|  |  | 50% | 0.074 | 0.049 | 0.041 | 0.037 | 0.176 | 0.073 | 0.062 | 0.060 | 0.363 | 0.200 | 0.167 | 0.127 |
|  |  | 95% | 0.737 | 0.369 | 0.360 | 0.184 | 1.809 | 0.604 | 0.528 | 0.163 | 2.641 | 2.254 | 1.533 | 0.377 |
|  |  | Mean | 0.198 | 0.108 | 0.082 | 0.080 | 0.443 | 0.178 | 0.144 | 0.142 | 0.786 | 0.544 | 0.402 | 0.273 |
|  | 𝜌DL |  | 0.9716 | 0.9716 | 0.9714 | 1.00 | 0.9130 | 0.9371 | 0.9371 | 1.00 | 0.9107 | 0.9102 | 0.9102 | 1.00 |
|  | 𝜌UL |  | 0.9662 | 0.9736 | 1.00 | 1.00 | 0.9467 | 0.9670 | 1.00 | 1.00 | 0.8828 | 0.9373 | 0.9735 | 1.00 |
|  | BO |  | 13.0 | 8.44 | 3.02 | 3.00 | 35.0 | 16.48 | 7.90 | 5.60 | 59.0 | 40.40 | 22.40 | 15.60 |
|  | 𝜆 |  | 0.06/0.06 |  |  |  | 0.086/0.086 |  |  |  | 0.114/0.114 |  |  |  |
|  |  |  | Additional comments: Outdoor Sub-7 deployment Scenario 2 WiFi settings: 802.11ac MCS table including 256 QAM, 2Tx2Rx in DL (cross-polarized, open loop), 2Tx2Rx in UL, 2 streams in both DL and UL. GI:0.8 µs, TXOP=4 ms, LDPC, A-MPDU enabled, RTS/CTS disabled, link adaptation: Minstrel algorithm, CWS: DL{15,63} and UL{15,1023}, CCA: CS=-82dBm, ED=-62dBm. NR-U settings: 4Tx2Rx in DL, Cross-polarized. MCS=4/16/64/256QAM, scheduling: proportional fair, link adaptation realistic, ED=-72 dBm, CP=Normal, SCS=30KHz, TXOP=4 ms, UE Capability #1, MCS: DL{15,63} and UL{15,1023}, COT sharing enabled, COT details: flexible DL/UL only and mixed DL/UL based on traffic needs, 3/11 DL control/data symbols, 3/11 symbols UL control/data. |  |  |  |  |  |  |  |  |  |  |  |

The results above show that in all three various level of loads, Wi-Fi performance in Wi-Fi+NR-U is better than Wi-Fi performance in Wi-Fi+Wi-Fi for both Scenario 1 and 2.

Observation 2: Our results show that in all the outdoor simulated scenarios, Wi-Fi performance in Wi-Fi+NR-U is better than Wi-Fi performance in Wi-Fi+Wi-Fi.

# Summary

In this contribution we submitted our simulation results for coexistence between WiFi and NR-U networks.

Observation 1: Our results show that in all the indoor simulated scenarios, Wi-Fi performance in Wi-Fi+NR-U is better than Wi-Fi performance in Wi-Fi+Wi-Fi.

Observation 2: Our results show that in all the outdoor simulated scenarios, Wi-Fi performance in Wi-Fi+NR-U is better than Wi-Fi performance in Wi-Fi+Wi-Fi.

# Appendix

Table 1: Indoor Sub-7 deployment Scenario

|  | Indoor Sub-7GHz |
| --- | --- |
| Layout | From R1-1807384, with (a,b,c,d)=(20,40,20,40) |
| Carrier Frequency | 5GHz |
| Carrier Channel Bandwidth | 20MHz baseline |
| Number of carriers | 1 |
| Number of users per operator | 5 per gNB per 20MHz |
| SCS | 30 KHz |
| Channel Model | NR InH Mixed Office model |
| BS/AP Tx Power | 23dBm (total across all TX antennas) |
| UE/STA Tx Power | 18dBm (total across all TX antennas) |
| BS/AP Antenna gain | 0dBi |
| UE/STA Antenna gain | 0 dBi |
| BS/AP Noise Figure | 5dB |
| UE/STA Receiver Noise Figure | 9dB |
| Minimum received power from serving cell for UE dropping | -82dBm |
| UE receiver | MMSE-IRC |
| BS/AP antenna Array configuration | (M, N, P, Mg, Ng) = (1, 2, 2, 1, 1), dH = dV = 0.5 λ |
| UE/STA antenna Array configuration | Baseline Tx/Rx: (M, N, P, Mg, Ng) = (1, 1, 2, 1, 1), dH = dV = 0.5 λ |
| Traffic model | According to 36.889 Table A.1.1. DL/UL FTP traffic – DL/UL traffic ratio: 50% DL and 50% UL Low load: Buffer occupancy 10% to 25% Medium load: Buffer occupancy 35% to 50% High load: Buffer occupancy above 55% |
| UE/STA to UE/STA link pathloss model | Directly use InH office pathloss model with proper d_3D with indoor mixed office LOS probability |
| gNB to gNB link pathloss model | Directly use InH office pathloss model with proper d_3D with indoor mixed office LOS probability |

Table 2: Outdoor Sub-7 deployment Scenario

| Parameters | Outdoor Sub-7GHz |
| --- | --- |
| Layout for nodes | Micro layer: Random drop(all micro BSs are outdoor) 1 micro BSs per operator per macro BS |
| Carrier Channel Bandwidth | 20MHz |
| Number of carriers | 1 |
| Number of users per operator | 5 UEs associated with each gNB per 20MHz |
| SCS | 30 KHz |
| Macro ISD | Scenario 1: 200 m Scenario 2: 300 m |
| Maximum distance inter-operator | Scenario 1: 30 m Scenario 2: NA |
| Minimum distance inter-operator | Scenario 1: 10 m Scenario 2: 10 m |
| Channel Model | NR UMi street canyon |
| BS/AP Tx Power | 23dBm (total across all TX antennas) |
| UE/STA Tx Power | 18dBm (total across all TX antennas) |
| BS/AP Antenna gain | 0 dBi |
| UE/STA Antenna gain | 0 dBi |
| BS/AP Noise Figure | 5dB |
| UE/STA Receiver Noise Figure | 9dB |
| Minimum received power from serving cell for UE dropping | -82dBm |
| UE receiver | MMSE-IRC as the baseline receiver |
| BS/AP antenna Array configuration | (M, N, P, Mg, Ng) = (1, 2, 2, 1, 1), dH = dV = 0.5 λ |
| UE/STA antenna Array configuration | (M, N, P, Mg, Ng) = (1, 1, 2, 1, 1), dH = dV = 0.5 λ |
| Traffic model | Use 36.889 Table A.1.1. Note: Results based on the mixed traffic models can be used to determine the design. |
| UE/STA to UE/STA link pathloss model | Directly use UMi street canyon pathloss model with proper d_3D with UMi street canyon LOS probability |
| gNB to gNB link pathloss model | Directly use UMi street canyon pathloss model with proper d_3D with UMi street canyon LOS probability |

Table 3: Additional WiFi Evaluation assumptions

| Parameter |  | Wi-Fi system evaluation assumptions |
| --- | --- | --- |
| MCS |  | 802.11ac MCS table with 256 QAM |
| Antenna Configuration |  | 2Tx2Rx in DL, cross-polarized, open loop, 2Tx2Rx in UL |
| MIMO layers |  | 2 streams in both DL and UL |
| Guard interval |  | 0.8 µs |
| TXOP duartion |  | 4 ms |
| Channel coding |  | LDPC |
| Frame aggregation |  | A-MPDU |
| Max PPDU duration |  | 4 ms |
| MAC | Coordination | EDCA AC4 |
|  | SIFS, DIFS | 16 µs, 25µs |
|  | Detection | CCA-CS |
|  | RTS/CTS | Indoor:On, Outdoor: Off |
|  | CWS {min, max} | DL: {15,63}, UL: {15,1023} |
| CCA-CS |  | -82dBm |
| CCA-ED |  | -62dBm |
| Rate control |  | Minstrel algorithm with 100 ms reconfiguration period |
| Channel selection |  | Minimum neighbour |
| OFDM symbol length |  | 4µs |

Table 4: Additional NR-U Evaluation assumptions

| Parameters | NR-U System evaluation assumption |
| --- | --- |
| Antenna configuration | 4Tx2Rx in DL, Cross-polarized. |
| Transmission schemes | QPSK/16QAM/64QAM/256QAM |
| Scheduling | Proportional fair |
| Link adaptation | Realistic |
| CCA-ED | -72 dBm |
| Cyclic Prefix | Normal |
| Subcarrier spacing | 30KHz |
| MCOT | 4 ms |
| UE processing capability | Capability #1 |
| CWS {min, max} | DL: {15,63}, UL: {15,1023} |
| COT details | gNB assisted dynamic COT sharing. Flexible COT reconfiguration - DL only, UL only, Mixed DL/UL (with single switching point). COT reconfiguration based on traffic needs. |
| DL slot symbols | 3 DL control, 11 DL data |
| UL slot symbols | 3 UL control, 11 UL data |

# References

- Chairman’s notes, 3GPP TSG RAN WG1#92, Athens, Greece, Feburary 26 – March 2, 2018

- Chairman’s notes, 3GPP TSG RAN WG1#92b, Sanya, China, April 16 – April 20, 2018

- Chairman’s notes, 3GPP TSG RAN WG1#93, Busan, Korea, May 21 – May 25, 2018

- RP-181339, “Revised SID on NR-based Access to Unlicensed Spectrum”, Qualcomm Incorporated, RAN#80, La Jolla, USA, June 11-14, 2018.