3GPP TSG RAN WG1 #96 meeting                                        	R1-1901709

Athens, Greece, 25th February – 1st March, 2019

Source:	vivo

Title:	Detailed Evaluation Results for UE power saving

Agenda Item:	7.2.9.1

Document for:	Discussion and Decision

- Introductions

In RAN1 #94bis and #95 meeting, the power models for UE adaptation to power and traffic, and for RRM enhancement were agreed [1] [2]. It was agreed that applications including FTP, web-browsing, video streaming, instant messaging, VoIP, gaming, background app sync can be considered for traffic modeling for power saving proposal evaluation. Simulation methodology and scenarios were also agreed. Calibrations for FTP and VoIP were completed in RAN1 #95.

In RAN1 #95 meeting, there are lots of proposed power saving schemes discussed and captured in UE power saving TR. A template table was agreed to capture the simulation results in TR.

In this contribution, the UE power saving gain for different power saving schemes are simulated and compared with Rel-15 existing schemes. The schemes for UE adaptation for traffic are discussed in companion paper [3].

- General simulation consumptions

To evaluate the power saving gain of different schemes, three traffic models including FTP model 3, gaming and VoIP are used. For gaming, the traffic model is based on R1-070674 [4], shown in Annex 2. The mean inter-packet interval for gaming is about 55ms. Each downlink gaming packet can be transmitted in one slot, since the packet size (hundreds of bits) is relatively small. For FTP model 3, the packet size is 0.5 Mbytes and the mean inter-arrival time is 200 ms. VoIP model is also according to R1-070674, as shown in Annex 3.

For FTP packet transmission, each packet is transmitted by 5 slots, 7 slots and 26 slots, for UEs with high SINR (32.4dB), UEs with medium SINR (15.2dB) and UEs with low SINR (2.4dB), respectively. The calculation for the number of slots needs to transmit a packet is based on Table A2 in Annex 1 assuming 100MHz bandwidth. These SINR values are based on 5%, 50% and 95% of downlink geometry of dense urban scenario with single layer, as shown in the Table and Figure 1 and Table 1 below. For VoIP and gaming traffic, since the packet size is small, it is assumed that the number of slots needs to transmit a VoIP or gaming packet is 1 slot.

Figure 1: Geometry CDF for dense urban scenario (single layer, 4GHz frequency)

Table 1: X percentile SINR mapping to the number of slots needs to transmit one packet (dense urban, single layer, 4GHz frequency)

| X percentile SINR | SINR value | the number of slots needs to transmit a 0.5MB ftp model 3 packet | the number of slots needs to transmit a VoIP or gaming packet |
| --- | --- | --- | --- |
| UE of 5 percentile SINR | 2.4dB | 26 slots | 1 slots |
| UE of 50 percentile SINR | 15.2dB | 7 slots | 1 slots |
| UE of 95 percentile SINR | 32.4dB | 5 slots | 1 slots |

FDD is assumed in the simulation with only DL packet modeled.  FR 1 (30KHz SCS with 0.5ms slot) is used. HARQ retransmission is not modeled. Same-slot scheduling is assumed. The power level (slot-averaged) for power saving signal reception is the same as PDCCH only, i.e., the power saving channel is modeled as normal PDCCH, that is, 100 unit for PDCCH only of same slot scheduling. The state transition energy from deep/light sleep is the same as normal PDCCH case.

A periodicity of max (DRX cycle, 320 msec) is modeled for periodic activities, e.g. time/frequency, channel or beam tracking. It is assumed CSI-RS resources with 320ms periodicity is configured for the periodic activities. Every 320ms, there is CSI-RS of 1 ms (2 slots), and the CSI-RS is in the 2 slots just before onduration of CDRX, if CDRX is configured. UL is not modeled. Numerical simulation is used in this paper.

Other assumptions are clarified in the following sections.

- Simulation results for UE adaptation in time domain

  - Simulation for adaptive PDCCH monitoring

In RAN1 #95 meeting, it is agreed that:

The power saving schemes to reduce PDCCH monitoring and blind decoding for further studies are as follows, - Triggering of PDCCH monitoring – dynamic trigger through L1 signal/signaling - Power saving signal triggering PDCCH monitoring - Go-to-sleep signaling to skip PDCCH monitoring - PDCCH skipping - DCI based indication for PDCCH skipping (e.g., indication in DCI content, new SFI state). - L1 signal/signaling (other than DCI) based triggering - Multiple CORESET/search space configurations - Configuration of different PDCCH periodicities with dynamic signaling - Adaptation of CORSET/search space configuration – DCI/timer/HARQ-ACK based indication - Dynamic/semi-persistent CORSET/search space ON/OFF - Adaptation between DRX ONduration timer and inactivitytimer - Separated PDCCH monitoring of DL and UL - L1 signaling triggering to assist UE in reducing the number of PDCCH blind decoding - Reduced PDCCH monitoring on SCell (including cross carrier scheduling) - Network assistance – RS is dynamically transmitted based on the need to assist UE performing synchronization, channel tracking, measurements and channel estimations before PDCCH decoding

In total four schemes are modeled in the evaluation, where scheme 1 and 2 are Rel-15 enabled schemes, and scheme 3 and 4 are for power saving enhancements.

- Scheme 1: Rel-15 one slot PDCCH monitoring periodicity with C-DRX

In this scheme, the UE is configured with one-slot PDCCH monitoring periodicity and C-DRX is enabled. The C-DRX parameters are selected according to the traffic modes, as shown in the following table, in which 30 KHz SCS is assumed.  Other C-DRX parameters, including short DRX cycle timer, drx-RetransmissionTimerDL, drx-RetransmissionTimerUL, drx-HARQ-RTT-TimerDL, or drx-HARQ-RTT-TimerUL are not modelled in the simulation.

Table 2: C-DRX parameter settings for scheme 1 (slot length is 0.5ms)

| C-DRX parameter setting | Gaming | FTP 3 |
| --- | --- | --- |
| DRX cycle | 80 slots | 320 slots |
| Ondurationtimer | 8 slots | 16 slots |
| Inactivitytimer | 20 slots | 200 slots |

- Scheme 2: Rel-15 PDCCH monitoring with different periodicities, no C-DRX

In this scheme, UE monitors PDCCH with a RRC configured PDCCH monitoring periodicity. No CDRX is assumed. The PDCCH monitoring periodicity is selected according to the traffic characteristics, e.g. mean interval of packet arrival. In each monitoring periodicity, a monitoring duration is selected. For gaming traffic, since the packet is relatively small, fixed 1 slot duration is used regardless of UE SINR. For FTP 3, different monitoring durations are selected according to the UE SINR conditions.

Table 3: PDCCH monitoring parameter settings for scheme 2

| PDCCH monitoring parameter setting | Gaming | FTP 3 |
| --- | --- | --- |
| Mean interval of packet arrival | ~55ms | ~200ms |
| PDCCH monitoring periodicities | 80slots | 100, 320, 400 slots |
| PDCCH monitoring durations | Fixed 1 slot | For 100 slots monitoring periodicities - High UE SINR (95%): 3 slots - Medium UE SINR (50%): 4 slots - Low UE SINR (5%): 14 slots For 320 or 400 slots monitoring periodicities - High UE SINR: 5 slots - Medium UE SINR: 7 slots - Low UE SINR: 26 slots |

- Scheme 3: Dynamic switching of PDCCH monitoring periodicities by DCI indication

In this scheme, multiple potential PDCCH monitoring periodicities are configured by RRC signalling, and DCI indicates one of them. No CDRX is assumed. The potential PDCCH monitoring periodicities for dynamic switching are N1 and N2 slots, where N2 is selected according to the traffic characteristics. Since dynamic switching between 1-slot periodicity and N2 slots periodicity is supported, the PDCCH monitoring duration is fixed as 1 slots, regardless of UE SINR conditions.

Table 4: PDCCH monitoring parameter settings for scheme 3

| PDCCH monitoring parameter setting | Gaming | FTP 3 |
| --- | --- | --- |
| Mean interval of packet arrival | ~55ms | ~200ms |
| PDCCH monitoring periodicity N1 | 1 slot | 1 slot |
| PDCCH monitoring periodicity N2 | 80, 120 or 160 slots | 100, 320 or 400 slots |
| PDCCH monitoring durations | Fixed 1 slot | Fixed 1 slot |

- Scheme 4: DCI explicit indication of PDCCH monitoring occasions within a periodicity

In this scheme, UE is explicitly indicated by DCI the exact PDCCH monitoring occasions for the upcoming monitoring periodicity, where the monitoring periodicity is RRC configured. For example, for a 10 slots monitoring periodicity, there can be a 10bits-bitmap to indicate whether the UE is required to monitor PDCCH in each slot. No CDRX is assumed.

Table 5: PDCCH monitoring parameter settings for scheme 4

| PDCCH monitoring parameter setting | Gaming | FTP 3 |
| --- | --- | --- |
| Mean interval of packet arrival | ~55ms | ~200ms |
| PDCCH monitoring periodicities N3 | 80 slots | 100, 320 or 400 slots |

Simulation results for gaming

The power and delay simulation results for gaming are shown in Table 6. The latency of each packet can be obtained by:

(1)

The simulation results for gaming traffic are shown in Table 6. The scheme 1 and scheme 2 (with 80 slots PDCCH monitoring periodicities) are taken as the baseline for comparison. Since the delay requirement for gaming is 60ms [4] (i.e., 120 slots), all the four schemes can meet this requirement. It can be seen that scheme 3 can provide large power saving benefit with similar or longer packet delay. It can also be seen that scheme 4 provides large power saving benefit without sacrificing packet latency.

For fixed PDCCH monitoring periodicity (Scheme 2), it is difficult to determine the accurate PDCCH monitoring periodicity (N) to balance the tradeoff between power consumption and packet delay. However, for dynamic PDCCH monitoring periodicities switching (Scheme 3), power consumption and packet delay are not too sensitive to the periodicity settings. This is also applicable for explicit PDCCH monitoring occasion indication (Scheme 4).

Table 6: Simulation results for gaming traffic

| Power saving schemes | Parameter values (slot) | Average Relative Power [unit] | average packet delay (ms) | power saving gain over CDRX | Power gain over fixed PDCCH monitoring periodicity (N=80) |
| --- | --- | --- | --- | --- | --- |
| Scheme 1: CDRX | {DRX cycle, ondurationtimer, inactivitytimer} = {80,8,20} | 35.18 | 16.83 | 0% (baseline#1) |  |
| Scheme 2: Fixed PDCCH monitoring periodicity | N=80 | 15.2 | 20.30 |  | 0% (baseline#2) |
| Scheme 3: Dynamic PDCCH monitoring periodicity switching | {N1,N2}={1,80} | 15.2 | 20.30 | 57% | 0% |
|  | {N1,N2}={1,120} | 11.1 | 30.64 | 68% | 27% |
|  | {N1,N2}={1,160} | 9.18 | 40.40 | 74% | 40% |
| Scheme 4: Explicit PDCCH monitoring occasion indication | N3=80 | 15.2 | 20.81 | 57% | 0% |

Simulation results for FTP model 3

The simulation results for FTP model 3 are shown in Table 7~ Table 9, for UEs with high SINR (32.4dB), UEs with medium SINR (15.2dB) and UEs with low SINR (2.4dB), respectively. Higher SINR leads to less slots needed to deliver one packet, thus lower power consumption is expected.

Four baseline cases are selected, one is CDRX and the other three cases are fixed PDCCH monitoring periodicities with 100, and 320 slots. Compared with baseline #1 (CDRX with fixed 1-slot PDCCH monitoring periodicity), Scheme 3 and Scheme 4 can achieve large power saving benefit with some tolerable average packet delay loss. Compared with baseline #2 (fixed 100-slot PDCCH monitoring periodicity without C-DRX), or baseline#3 (fixed 320-slot PDCCH monitoring periodicity without C-DRX), power saving benefit from Scheme 3 and Scheme 4 is smaller than the former cases, but the delay improvement is large. For example, Dynamic PDCCH monitoring periodicity switching between 1 slot and 100 slot can achieve 27.26 ms average packet delay in Table 6.  It can also be seen that in most cases the power saving benefit reduces for low SINR UEs compared to high SINR UEs.

Table 7: Simulation results for FTP model 3 traffic (for UE of 95 percentile SINR: one packet is transmitted in 5 slots).

| Power saving schemes | Parameter values (slot) | Average Relative Power [unit] | average packet delay (ms) | power saving gain over CDRX | Power gain over fixed PDCCH monitoring periodicity {periodicity,duration}={100,3} | Power gain over fixed PDCCH monitoring periodicity {periodicity,duration}={320,5} |
| --- | --- | --- | --- | --- | --- | --- |
| Scheme 1: CDRX | {DRX cycle, ondurationtimer, inactivitytimer} = {320,16,200} | 41.56 | 46.24 | 0% (baseline#1) |  |  |
| Scheme 2: Fixed PDCCH monitoring periodicity | periodicity =100 duration =3 | 15.48 | 107.74 |  | 0% (baseline#2) |  |
|  | Periodicity = 320 duration = 5 | 7.86 | 362.92 |  |  | 0% (baseline#3) |
| Scheme 3: Dynamic PDCCH monitoring periodicity switching | Periodicity {N1,N2}={1,100} duration =1 | 14.52 | 27.26 | 65% | 6% | - |
|  | Periodicity {N1,N2}={1,320} duration =1 | 7.7 | 81.01 | 81% | 50% | 2% |
|  | Periodicity {N1,N2}={1,400} duration =1 | 7.08 | 100.88 | 83% | 54% | 10% |
| Scheme 4: Explicit PDCCH monitoring occasion indication | Periodicity = 100 | 14.52 | 28.23 | 65% | 6% | - |
|  | Periodicity = 320 | 7.7 | 83.27 | 81% | 50% | 2% |
|  | Periodicity = 400 | 7.08 | 103.65 | 83% | 54% | 10% |

Table 8: Simulation results for FTP model 3 traffic (for UE of 50 percentile SINR: one packet is transmitted in 7 slots).

| Power saving schemes | Parameter values (slot) | Average Relative Power [unit] | average packet delay (ms) | power saving gain over CDRX | Power gain over fixed PDCCH monitoring periodicity {periodicity,duration}={100,4} | Power gain over fixed PDCCH monitoring periodicity {periodicity,duration}={320,7} |
| --- | --- | --- | --- | --- | --- | --- |
| Scheme 1: CDRX | {DRX cycle, ondurationtimer, inactivitytimer} = {320,16,200} | 42.9 | 47.31 | 0% (baseline#1) |  |  |
| Scheme 2: Fixed PDCCH monitoring periodicity | periodicity =100 duration =4 | 17.46 | 114.66 |  | 0% (baseline#2) |  |
|  | Periodicity = 320 Duration = 7 | 9.48 | 363.90 |  |  | 0% (baseline#3) |
| Scheme 3: Dynamic PDCCH monitoring periodicity switching | Periodicity {N1,N2}={1,100} duration =1 | 16 | 28.13 | 63% | 8% | - |
|  | Periodicity {N1,N2}={1,320} duration =1 | 9.2 | 81.93 | 79% | 47% | 3% |
|  | Periodicity {N1,N2}={1,400} duration =1 | 8.58 | 101.53 | 80% | 51% | 10% |
| Scheme 4: Explicit PDCCH monitoring occasion indication | Periodicity = 100 | 16 | 29.37 | 63% | 8% | - |
|  | Periodicity = 320 | 9.18 | 84.68 | 79% | 47% | 3% |
|  | Periodicity = 400 | 8.58 | 105.25 | 80% | 51% | 10% |

Table 9: Simulation results for ftp modle 3 traffic (for UE of 5 percentile SINR: one packet is transmitted in 26 slots).

| Power saving schemes | Parameter values (slot) | Average Relative Power [unit] | average packet delay (ms) | power saving gain over CDRX | Power gain over fixed PDCCH monitoring periodicity {periodicity,duration}={100,14} | Power gain over fixed PDCCH monitoring periodicity {periodicity,duration}={320,26} |
| --- | --- | --- | --- | --- | --- | --- |
| Scheme 1: CDRX | {DRX cycle, ondurationtimer, inactivitytimer} = {320,16,200} | 55.36 | 57.86 | 0% (baseline#1) |  |  |
| Scheme 2: Fixed PDCCH monitoring periodicity | periodicity =100 duration =14 | 36.88 | 124.12 |  | 0% (baseline#2) |  |
|  | Periodicity = 320 Duration = 26 | 24.86 | 372.98 |  |  | 0% (baseline#3) |
| Scheme 3: Dynamic PDCCH monitoring periodicity switching | Periodicity {N1,N2}={1,100} duration =1 | 30.22 | 37.41 | 45% | 18% | - |
|  | Periodicity {N1,N2}={1,320} duration =1 | 23.42 | 89.05 | 58% | 36% | 6% |
|  | Periodicity {N1,N2}={1,400} duration =1 | 22.8 | 106.62 | 59% | 38% | 8% |
| Scheme 4: Explicit PDCCH monitoring occasion indication | Periodicity = 100 | 30.22 | 40.11 | 45% | 18% | - |
|  | Periodicity = 320 | 23.4 | 98.03 | 58% | 37% | 6% |
|  | Periodicity = 400 | 22.8 | 119.45 | 59% | 38% | 8% |

The above results are summarized in Table 10 below, which followed the table format agreed in last meeting.

Based on the results,

Observation 1: For gaming traffic, enhanced PDCCH monitoring adaptation schemes can provide large power saving benefit with similar or tolerable packet delay loss.

Observation 2: For FTP traffic, enhanced PDCCH monitoring adaptation schemes can provide large power saving benefit with tolerable packet delay loss. Larger power saving gain for higher SINR UEs compared to low SINR UEs.

Proposal 1: At least one of the following enhanced PDCCH monitoring adaptation schemes are adopted for Rel-16 UE power saving

- Dynamic switching of PDCCH monitoring periodicities by DCI indication

- DCI explicit indication of PDCCH monitoring occasions within a periodicity

Proposal 2: Capture Table 10 in TR 38.840.

Table 10: Simulation results for Adaptive PDCCH monitoring

| Company | Power saving scheme | Power saving gain | Power saving gain for each configuration | UPT/Latency | Estimated Overhead | Evaluation methodology/baseline assumption | Note (include UE throughput) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| vivo | Dynamic switching of PDCCH monitoring periodicities by DCI | 0%-83% | 0%-74% (for gaming, and for all UE) | Average packet Latency: 20.30-40.40 ms; Latency increase: 0%-140% | No overhead (use the additional bits in scheduling DCI) | Numerical simulation; Baseline assumption: 1)Fixed PDCCH monitoring periodicity without CDRX; 2)CDRX; Traffic: gaming and ftp model 3 |  |
|  |  |  | 6%-59% (for FTP3, and for UE of 5 percentile SINR) | Average packet Latency: 37.41-106.62 ms; Latency increase: -90% -84% |  |  |  |
|  |  |  | 3%-80% (for FTP3, and for UE of 50 percentile SINR) | Average packet Latency: 28.13-101.53 ms; Latency increase: -92% -115% |  |  |  |
|  |  |  | 2%-83% (for FTP3, and for UE of 95 percentile SINR) | Average packet Latency: 27.26-100.88 ms; Latency increase: -92% -118% |  |  |  |
| vivo | DCI explicit indication of PDCCH monitoring occasions within a periodicity | 0%-83% | 0%-57% (for gaming and for all UE) | Average packet Latency: 20.81 ms; Latency increase: 3%-24% | One PDCCH per 40 ms | Numerical simulation; Baseline assumption: 1)Fixed PDCCH monitoring periodicity without CDRX; 2)CDRX; Traffic: gaming and ftp model 3 |  |
|  |  |  | 6%-59% (for FTP3, and for UE of 5 percentile SINR) | Average packet Latency: 40.11-119.45 ms; Latency increase: -89%-106% | One PDCCH per 50, 160 and 200 ms |  |  |
|  |  |  | 3%-80% (for FTP3, and for UE of 50 percentile SINR) | Average packet Latency: 29.37-105.25 ms; Latency increase: -92% -122% | One PDCCH per 50, 160 and 200 ms |  |  |
|  |  |  | 2%-83% (for FTP3, and for UE of 95 percentile SINR) | Average packet Latency: 28.23-103.65 ms; Latency increase: -92% -124% | One PDCCH per 50, 160 and 200 ms |  |  |

  - Simulation for cross-slot scheduling

Cross-slot scheduling has been identified as one potential power saving scheme since it reduces the UE data buffering before valid DL grant is detected. Cross-slot scheduling can be enabled in Rel-15 by configuring all the values in the RRC configured k0 set to be larger than 0 to a UE.

In this paper, K0 is set to be 1 slot. Power state and relative power for cross-slot scheduling and same-slot scheduling are in Table 11 below. For cross-slot scheduling (K0=1), the power for PDCCH only is 70. For same-slot scheduling, the power for PDCCH only is 100.

Table 11: Power state and relative power for cross-slot scheduling and same-slot scheduling

| Power State | Relative Power |
| --- | --- |
| PDCCH only-same slot scheduling | 100 |
| PDCCH only-cross slot scheduling | 70 |
| PDSCH only (cross-slot scheduling) | 280 |
| PDCCH+PDSCH | 300 |

The power saving of cross-slot scheduling over same-slot scheduling is evaluated in two different configurations. Configuration 1 is DRX with parameters {DRX Period, Ondurationtimer, Inactivitytimer} is (160,8,100) ms, respectively. Configuration 2 is PDCCH monitoring in every DL slots and no CDRX.  The Average Relative Power and average packet delay for ftp model 3 traffic for the two configurations are in Table 12. The results are summarized in Table 13.

Observation 3: For ftp model 3, cross-slot scheduling can provide 18%-29% power saving benefit with similar or tolerable packet delay loss, compared with same-slot scheduling.

Proposal 3: Capture Table 13 in TR 38.840.

Table 12: Average Relative Power and average packet delay for gaming traffic (Cross-slot scheduling and same-slot scheduling, ftp model 3)

| Configurations | Cross-slot /same-slot scheduling | No. of transmitted slots per packet | Average Relative Power | Average packet delay (ms) | Power saving gain |
| --- | --- | --- | --- | --- | --- |
| Configuration 1: DRX {DRX Period, Ondurationtimer, Inactivitytimer} =(160,8,100) ms | Cross-slot scheduling | 5 (95% SINR) | 31.17 | 46.74 | 25% |
|  |  | 7 (50% SINR) | 32.55 | 47.80 | 24% |
|  |  | 26 (5% SINR) | 45.52 | 58.36 | 18% |
|  | same-slot scheduling | 5 | 41.56 | 46.24 | baseline |
|  |  | 7 | 42.89 | 47.30 | baseline |
|  |  | 26 | 55.37 | 57.86 | baseline |
| Configuration 2: PDCCH monitoring in every DL slots, no CDRX | Cross-slot scheduling | 5 | 72.88 | 3.01 | 29% |
|  |  | 7 | 74.03 | 4.03 | 28% |
|  |  | 26 | 84.97 | 13.94 | 25% |
|  | same-slot scheduling | 5 | 102.50 | 2.51 | baseline |
|  |  | 7 | 103.51 | 3.53 | baseline |
|  |  | 26 | 113.02 | 13.44 | baseline |

Table 13: Simulation results for cross-slot scheduling

| Company | Power saving scheme | Power saving gain | Power saving gain for each configuration | UPT/Latency | Estimated Overhead | Evaluation methodology/baseline assumption | Note (include UE throughput) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| vivo | cross-slot scheduling | 18%-29% | 18%-25% for UE of 5 percentile SINR | Average packet Latency: 13.94- 58.36 ms; Latency increase: 1%-4% | Marginal | Numerical simulation; Baseline assumption: non-UE preferred CDRX configuration; Traffic: ftp model 3 |  |
|  |  |  | 24%-28% for UE of 50 percentile SINR | Average packet Latency: 4.03- 47.80 ms; Latency increase: 1%-14% |  |  |  |
|  |  |  | 25%-29% for UE of 95 percentile SINR | Average packet Latency: 3.01- 46.74 ms; Latency increase: 1%-20% |  |  |  |

  - Simulation for adaptation of CORSET/search space configuration –HARQ-ACK based indication

In RAN1 #95 meeting, it is agreed that [2]:

Agreements: The power saving schemes to reduce PDCCH monitoring and blind decoding for further studies are as follows, - Multiple CORESET/search space configurations - Configuration of different PDCCH periodicities with dynamic signaling - Adaptation of CORSET/search space configuration – DCI/timer/HARQ-ACK based indication

In this section, results of Adaptation of CORSET/search space configuration –HARQ-ACK/NACK based indication are given.

For services with fixed or almost fixed inter-packet arrival, e.g. VoIP or real time video, the packet arrives with fixed periodicity, e.g. 20ms. Semi-persistent scheduling (SPS) can be configure to schedule PDSCH for the new transmission of packets, to save power of PDCCH monitoring. Assuming a 10% retransmission probability it is difficult to forecast the potential retransmissions. It is better to configure some PDCCH monitoring occasions for potential retransmission next to the semi-persistent scheduling occasions and such additional PDCCH occasions are monitored only when the latest semi-persistent scheduling is not decoded successfully (as in Figure 2). By this method, UE can skip the unnecessary monitoring occasions for retransmissions and therefore save power. However, such “event triggered” PDCCH monitoring occasions are not supported in Rel-15 and can be enhanced in Rel-16.

Figure 2: PDCCH monitoring triggered by decoding failure of latest SPS PDSCH

We evaluated the above enhanced scheme (Figure 2) compared with Rel-15 baseline (shown in Figure 3), in which all the PDCCH monitoring occasions are activated by RRC configuration without additional triggering

Figure 3: Rel-15 baseline, i.e. PDCCH is always monitored according to RRC configuration

Only voice packet is modelled and SID packet is not modelled. DL only is assumed. SPS is used to transmit new transmission of voice packets. So PDSCH only power model is used for new transmission of voice packets. The RTT (round trip time) between new transmission and the relevant 1st retransmission is 10ms. 10% BLER is assumed for initial SPS PDSCH transmission. 0% BLER is assumed for the 1st retransmission, i.e., no 2nd retransmission. The used power model is listed in Table 14 below, which is the same as the agreed power model in the RAN1 #94bis and #95 meetings.

According to the simulation results, about 20% power is saved by the enhanced PDCCH monitoring scheme, compared with the existing scheme, due to the reduced PDCCH monitoring for potential retransmissions.

Observation 4: For VoIP, power saving gain of adaptation of CORSET/search space configuration –HARQ-ACK based indication (UE monitoring of PDCCH search space for retransmission triggered by decoding failure of SPS PDSCH) is about 20%, with no packet delay loss.

Proposal 4: Capture Table 15 in TR 38.840.

Table 14. The power model used for simulation

| Power State | Relative Power [unit] |
| --- | --- |
| Light Sleep | 20 |
| PDCCH-only | 100 |
| PDCCH + PDSCH | 300 |
| PDSCH only | 280 |
| Additional transition energy for light sleep | 100 X 1ms |

Table 15: Simulation results for UE monitoring of PDCCH search space for retransmission triggered by decoding failure of SPS PDSCH

| Company | Power saving scheme | Power saving gain | Power saving gain for each configuration | UPT/Latency | Estimated Overhead | Evaluation methodology/baseline assumption | Note (include UE throughput) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| vivo | Adaptation of CORSET/search space configuration – HARQ-ACK based indication (UE monitoring of PDCCH search space for retransmission is triggered by decoding failure of SPS PDSCH) | 20% | 20% | Average packet Latency: 1 ms; Latency increase: 0% | No overhead | Numerical simulation; Baseline assumption: UE monitoring of PDCCH search space for retransmission is configured by RRC; Traffic: VoIP |  |

  - Simulation for Adaptation between DRX ONduration timer and inactivitytimer

In RAN1 #95 meeting, it is agreed that [2]:

Agreements: The power saving schemes to reduce PDCCH monitoring and blind decoding for further studies are as follows, - Multiple CORESET/search space configurations - Adaptation between DRX ONduration timer and inactivitytimer

In this section, the simulation results of Adaptation between DRX ONduration timer and inactivitytimer are given.

In this scheme, as shown in Figure 4, when onDurationTimer is running, the first set of PDCCH monitoring occasions with a longer periodicity (e.g., 10 slots) configured by e.g., RRC signaling, can be used. Once PDCCH indicating a new transmission is received (the red box in Figure 4), i.e., InactivityTimer is started, the second set of PDCCH monitoring occasions with a shorter periodicity (e.g., 1 slot) are activated automatically. The second set of PDCCH monitoring occasions with shorter periodicity is configured in advance, by e.g. RRC signaling, and is automatically activated/triggered when InactivityTimer is started.

- Figure 4: PDCCH monitoring adaptation between DRX ONduration timer and inactivitytimer

The Average Relative Power and average packet delay for gaming traffic for different configurations are in Table 16 and summarized in Table 17 below.

Observation 5: For ftp model 3, power saving gain of PDCCH monitoring adaptation between DRX ONduration timer and inactivitytimer is 5%-50%, with tolerable packet delay loss.

Proposal 5: Capture Table 17 in TR 38.840.

Table 16: Average Relative Power and average packet delay for ftp model 3 traffic

| Configuration No. | Different schemes | No. of transmitted slots per packet | Average Relative Power | Average packet delay (ms) |
| --- | --- | --- | --- | --- |
| 1 | Baseline: CDRX {DRX Period, Ondurationtimer, Inactivitytimer} =(160,8,100) ms PDCCH monitoring peridicity is 0.5 ms | 5 (95% SINR) | 41.56 | 46.24 |
|  |  | 7 (50% SINR) | 42.89 | 47.30 |
|  |  | 26 (5% SINR) | 55.37 | 57.86 |
| 2 | Adaptation between DRX ONduration timer and inactivitytimer CDRX (160,8,100) ms Before InactivityTimer is started, PDCCH monitoring peridicity is 0.5 ms; After InactivityTimer is started, PDCCH monitoring peridicity is 8 ms; | 5 | 38.55 | 50.87 |
|  |  | 7 | 39.88 | 51.96 |
|  |  | 26 | 52.41 | 62.72 |
| 3 | Baseline: CDRX (160,8,0.5) ms PDCCH monitoring peridicity is 0.5 ms | 5 | 11.29 | 75.65 |
|  |  | 7 | 12.40 | 76.99 |
|  |  | 26 | 25.78 | 84.20 |
| 4 | Adaptation between DRX ONduration timer and inactivitytimer Before InactivityTimer is started, PDCCH monitoring peridicity is 0.5 ms; After InactivityTimer is started, PDCCH monitoring peridicity is 8 ms; | 5 | 7.86 | 80.85 |
|  |  | 7 | 9.36 | 81.83 |
|  |  | 26 | 23.57 | 88.10 |
| 5 | Baseline: CDRX (40,8,0.5) ms PDCCH monitoring peridicity is 0.5 ms | 5 | 34.60 | 15.57 |
|  |  | 7 | 35.66 | 16.63 |
|  |  | 26 | 48.47 | 26.22 |
| 6 | Adaptation between DRX ONduration timer and inactivitytimer Before InactivityTimer is started, PDCCH monitoring peridicity is 0.5 ms; After InactivityTimer is started, PDCCH monitoring peridicity is 8 ms; | 5 | 17.23 | 22.10 |
|  |  | 7 | 18.73 | 23.01 |
|  |  | 26 | 32.85 | 32.41 |

Table 17: Simulation results for Adaptation to DRX operation

| Company | Power saving scheme | Power saving gain | Power saving gain for each configuration | UPT/Latency | Estimated Overhead | Evaluation methodology/baseline assumption | Note (include UE throughput) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| vivo | PDCCH monitoring adaptation between DRX ONduration timer and inactivitytimer | 5%-50% | 5%-32% for UE of 5 percentile SINR | Average packet Latency: 32-88 ms; Latency increase: 5%-24% | Marginal | Numerical simulation; Baseline assumption: same PDCCH monitoring configurations between DRX ONduration timer and inactivitytimer; Traffic model: ftp model 3 |  |
|  |  |  | 7%-47% for UE of 50 percentile SINR | Average packet Latency: 23-82 ms; Latency increase: 6%-38% |  |  |  |
|  |  |  | 7%-50% for UE of 95 percentile SINR | Average packet Latency: 22-81 ms; Latency increase: 7%-42% |  |  |  |

- Simulation results for Adaptation to DRX operation

In RAN1 #95 meeting, it is agreed that UE is configured with multiple DRX configuration and Dynamic selection of DRX configuration by gNB from multiple DRX configurations (e.g., traffic, mobility). UE assistance information may be considered. FTP model 3 is assumed.

In this section, the simulation results of UE preferred CDRX configuration reporting are given.

The procedures of UE preferred CDRX configuration reporting are as follows:

- UE reports its preferred CDRX configurations to gNB

  - UE can estimate its preferred CDRX configurations according to the traffic characteristic and/or the channel conditions, and then report it to gNB. For example, for gaming traffic, UE estimates that {DRX Period, Ondurationtimer, Inactivitytimer} with (160,1,1) ms values can meet the service QoS while achieving power saving. It reports to gNB that its preferred CDRX is {DRX Period, Ondurationtimer, Inactivitytimer} with (160,1,1) ms values.

- gNB configures the CDRX to UE, e.g., by RRC signaling, based on the UE report

  - The configuration is in a semi-static fashion, so the signaling overhead is marginal

- UE adapts the CDRX accordingly

The Average Relative Power and average packet delay for gaming traffic for different configurations are in Table 18 and summarized in Table 19 below.

Observation 6: For ftp model 3, power saving gain of UE reporting preferred CDRX configuration is 4%-80%, with tolerable packet delay loss.

Proposal 6: Capture Table 19 in TR 38.840.

Table 18: Average Relative Power and average packet delay for ftp model 3 traffic

| Configuration No. | CDRX parameters {DRX Period, Ondurationtimer, Inactivitytimer} | No. of transmitted slots per packet | Average Relative Power | Average packet delay (ms) |
| --- | --- | --- | --- | --- |
| 1 | (160,8,100) ms | 5 (95% SINR) | 41.56 | 46.24 |
|  |  | 7 (50% SINR) | 42.89 | 47.30 |
|  |  | 26 (5% SINR) | 55.37 | 57.86 |
| 2 | (160,8,1) ms | 5 | 11.31 | 75.97 |
|  |  | 7 | 12.42 | 76.92 |
|  |  | 26 | 25.95 | 84.11 |
| 3 | (160,4,1) ms | 5 | 9.13 | 78.99 |
|  |  | 7 | 10.52 | 79.90 |
|  |  | 26 | 24.77 | 86.14 |
| 4 | (160,1,1) ms UE preferred CDRX configuration | 5 | 8.17 | 80.43 |
|  |  | 7 | 9.67 | 81.19 |
|  |  | 26 | 23.88 | 87.61 |
| 5 | (80,1,1) ms | 5 | 11.61 | 41.27 |
|  |  | 7 | 13.10 | 41.85 |
|  |  | 26 | 27.32 | 50.23 |

Table 19: Simulation results for Adaptation to DRX operation

| Company | Power saving scheme | Power saving gain | Power saving gain for each configuration | UPT/Latency | Estimated Overhead | Evaluation methodology/baseline assumption | Note (include UE throughput) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| vivo | UE report preferred CDRX configuration | 4%-80% | 4%-57% for UE of 5 percentile SINR | Average packet Latency: 87.61 ms; Latency increase: 2%-74% | Marginal | Numerical simulation; Baseline assumption: non-UE preferred CDRX configuration; Traffic model: ftp model 3 |  |
|  |  |  | 8%-77% for UE of 50 percentile SINR | Average packet Latency: 81.19 ms; Latency increase: 2%-94% |  |  |  |
|  |  |  | 11%-80% for UE of 95 percentile SINR | Average packet Latency: 80.43 ms; Latency increase: 2%-95% |  |  |  |

- Simulation results for Adaptation to number of antenna

In RAN1 #95 meeting, it is agreed that the UE power saving schemes with UE adaptation to the traffic is to reduce the number of antenna/panels or MIMO layers indicated by the network semi-statically or dynamically to achieve the UE power saving.

In Table 17, the average Relative Power and average packet delay of adaptation to number of Rx antenna are given. The modeled traffic is online gaming. The Baseline scheme is the Rx antenna number is always 4Rx. The power saving scheme is adaptation to number of antenna. The procedures of adaptation to number of antenna are as follows:

- UE reports its preferred Rx antenna number to gNB

  - UE can estimate its preferred number of Rx antenna according to the traffic characteristic and/or the channel conditions, and then report it to gNB. For example, for gaming traffic, UE estimates that 2Rx antenna is enough to receive the gaming packets. It reports to gNB that its preferred number of Rx antenna is 2.

- gNB configures the Rx antenna number to UE, e.g., by RRC signaling, based on the UE report

  - The configuration is in a semi-static fashion, so the signaling overhead is marginal

- UE adapts the Rx antenna number accordingly

Different configuration and parameters are assumed for simulation.

- CDRX, {DRX cycle, ondurationtimer, inactivitytimer} = {80,8,20} slots

- Fixed PDCCH monitoring periodicity(N) , N=1 slot

- Fixed PDCCH monitoring periodicity(N) , N=80 slot

The Average Relative Power and average packet delay for gaming traffic for different configurations are in Table 20 and summarized in Table 21 below.

Observation 7: For gaming traffic, power saving gain of Adaptation to number of antenna is 6%-30%, with no packet delay loss.

Proposal 7: Capture Table 21 in TR 38.840.

Table 20: Average Relative Power and average packet delay for gaming traffic (Adaptation to number of antenna, gaming traffic)

| Power saving schemes | Configuration and parameters (ms) | Average Relative Power | average packet delay (ms) | Power saving gain |
| --- | --- | --- | --- | --- |
| Baseline: The Rx antenna number is always 4Rx | CDRX {DRX cycle, ondurationtimer, inactivitytimer} = {40,4,10} | 35.18 | 16.83 | 0% (baseline) |
| Adaptation to number of antenna | CDRX {DRX cycle, ondurationtimer, inactivitytimer} = {40,4,10} | 28.24 | 16.83 | 20% |
| Baseline: The Rx antenna number is always 4Rx | Fixed PDCCH monitoring periodicity (0.5ms), no CDRX | 101.72 | 0.5 | 0% (baseline) |
|  | Fixed PDCCH monitoring periodicity (40ms), no CDRX | 15.2 | 20.30 | 0% (baseline) |
| Adaptation to number of antenna | Fixed PDCCH monitoring periodicity(0.5ms), no CDRX | 71.20 | 0.5 | 30% |
|  | Fixed PDCCH monitoring periodicity (40ms), no CDRX | 14.31 | 20.30 | 6% |

Simulation results for gaming traffic (Adaptation to number of antenna) is in Table 18.

Table 21: Simulation results for gaming traffic (Adaptation to number of antenna)

| Company | Power saving scheme | Power saving gain | Power saving gain for each configuration | UPT/Latency | Estimated Overhead | Evaluation methodology/baseline assumption | Note (include UE throughput) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| vivo | Adaptation to number of antenna (UE reports its preferred Rx antenna number) | 6%-30% | 20% for CDRX | Average packet latency: 16.83ms; Latency increase: 0%* | Marginal** | Numerical simulation; Baseline assumption: The Rx number is always 4Rx; Traffic model: gaming | {DRX Period, Ondurationtimer, Inactivitytimer}={40,4,10} ms |
|  |  |  | 30% for Fixed PDCCH monitoring periodicity (0.5ms) | Average packet latency: 0.5ms; Latency increase: 0%* |  |  |  |
|  |  |  | 6% for Fixed PDCCH monitoring periodicity (40ms) | Average packet latency: 20.30ms; Latency increase: 0%* |  |  |  |

Note*: one gaming packet can be transmitted in one slot for both 2Rx and 4Rx, so the average packet delay is the same between 2Rx and 4Rx.

Note**: The configuration of antenna number adaptation is in a semi-static fashion, so the signaling overhead is marginal

- Simulation results for Adaptation in frequency domain

In the latest TR, it is agreed that The UE BWP adaptation is to dynamically switch the BWP by gNB based on, e.g., the traffic to support efficient operation of BWP switching in reducing the UE power consumption.

In Table 22 below, the average Relative Power and average packet delay of adaptation to BWP are given. The modeled traffic is online gaming. The Baseline scheme is the BWP is always 100MHz. The power saving scheme is adaptation to BWP. The procedures of adaptation to BWP are as follows:

- UE reports its preferred BWP to gNB

  - UE can estimate its preferred BWP according to the traffic characteristic and/or the channel conditions, and then report it to gNB. For example, for gaming traffic, UE estimates that BWP of 40MHz is enough to receive the gaming packets. It reports to gNB that its preferred BWP is 40MHz.

- gNB configures the corresponding BWP to UE, e.g., by RRC signaling, based on the UE report

  - The configuration is in a semi-static fashion, so the signaling overhead is marginal

- UE adapts the BWP accordingly according to gNB configuration

The following power scaling model is used in simulation.

•	Scaling of X MHz = 0.4 + 0.6 * (X - 20) / 80. Linear interpolation for intermediate bandwidths. Valid only for X = 10, 20, 40, 80, and 100.

•	X=100MHz，Scaling factor=1；

•	X=40MHz，Scaling factor=0.55；

Different configuration and parameters are assumed for simulation.

- CDRX, {DRX cycle, ondurationtimer, inactivitytimer} = {80,8,20} slots

- Fixed PDCCH monitoring periodicity(N) , N=1 slot

- Fixed PDCCH monitoring periodicity(N) , N=80 slot

The Average Relative Power and average packet delay for gaming traffic for different configurations are in Table 22 and summarized in Table 23 below.

Observation 8: For gaming traffic, power saving gain of Adaptation in frequency domain is 9%-45%, with no packet delay loss.

Proposal 8: Capture Table 23 in TR 38.840.

Table 22: Average Relative Power and average packet delay for gaming traffic (Adaptation in frequency domain, gaming traffic)

| Power saving schemes | Configuration and parameters (ms) | Average Relative Power | average packet delay (ms) | Power saving gain |
| --- | --- | --- | --- | --- |
| Baseline: The BWP is always 100MHz | CDRX {DRX cycle, ondurationtimer, inactivitytimer} = {40,4,10} | 35.18 | 16.83 | 0% (baseline) |
| Adaptation to BWP | CDRX {DRX cycle, ondurationtimer, inactivitytimer} = {40,4,10} | 24.76 | 16.83 | 30% |
| Baseline: The BWP is always 100MHz | Fixed PDCCH monitoring periodicity (0.5ms), no CDRX | 101.72 | 0.5 | 0% (baseline) |
|  | Fixed PDCCH monitoring periodicity (40ms), no CDRX | 15.2 | 20.30 | 0% (baseline) |
| Baseline: The BWP is always 100MHz | Fixed PDCCH monitoring periodicity(0.5ms), no CDRX | 55.94 | 0.5 | 45% |
|  | Fixed PDCCH monitoring periodicity (40ms), no CDRX | 13.87 | 20.30 | 9% |

Simulation results for gaming traffic (Adaptation in frequency domain) is in Table 23.

Table 23: Simulation results for gaming traffic (Adaptation in frequency domain)

| Company | Power saving scheme | Power saving gain | Power saving gain for each configuration | UPT/Latency | Estimated Overhead | Evaluation methodology/baseline assumption | Note (include UE throughput) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| vivo | Adaptation in frequency domain (UE reports its preferred BWP) | 9%-45% | 30% for CDRX | Average packet latency: 16.83ms; Latency increase: 0%* | Marginal** | Numerical simulation; Baseline assumption: The BWP is always 100MHz; Traffic model: gaming | CDRX configuration: {DRX Period, Ondurationtimer, Inactivitytimer}={40,4,10} ms |
|  |  |  | 45% for Fixed PDCCH monitoring periodicity (0.5ms) | Average packet latency: 0.5ms; Latency increase: 0%* |  |  |  |
|  |  |  | 9% for Fixed PDCCH monitoring periodicity (40ms) | Average packet latency: 20.30ms; Latency increase: 0%* |  |  |  |

Note*: one gaming packet can be transmitted in one slot for both 40MHz BWP and 100MHz BWP, so the average packet delay is the same between 40MHz BWP and 100MHz BWP.

Note**: The configuration of BWP adaptation is in a semi-static fashion, so the signaling overhead is marginal

- Simulation results for UE report preferred PDCCH monitoring parameters

In the latest TR, it is agreed that UE preferred PDCCH monitoring parameters can be included in the UE assistance information.

The UE assistance information for the power saving schemes for further studies are as follows, UE assistance information/feedback to assist network in configurations for UE adaptation - UE preferred PDCCH monitoring parameters/search space configuration/maximum number of blind decoding

The procedures of UE report preferred PDCCH monitoring parameters are as follows:

- UE reports its preferred PDCCH monitoring parameters to gNB

  - UE can estimate its preferred PDCCH monitoring parameters according to the traffic characteristic and/or the channel conditions, and then report it to gNB. For example, for gaming traffic, because the packet interval is about 55ms and the delay requirement is about 60ms, UE estimates that PDCCH monitoring periodicity of 40ms is a good choice to meet the delay requirement of gaming. So it reports to gNB that its preferred PDCCH monitoring parameters is 40ms periodicity and 1 slot duration.

- gNB configures the corresponding PDCCH monitoring parameters to UE, e.g., by RRC signaling, based on the UE report

  - The configuration is in a semi-static fashion, for example, for one traffic type, the PDCCH monitoring parameters is fixed; so the signaling overhead is marginal

  - gNB can take into account other factors, e.g., the system load, etc.

- UE adapts the PDCCH monitoring parameters accordingly according to gNB configuration

For comparison with UE preferred PDCCH monitoring periodicity (N) of 40ms, other PDCCH monitoring periodicity of N= 0.5ms, 60ms and 80ms (UE non-preferred parameters) are also modelled. The Average Relative Power and average packet delay for gaming traffic for different configurations are in Table 24 and summarized in Table 25 below. Detailed results can be find in [5].

In Table 24, when N=0.5ms, i.e., UE monitors PDCCH every slot of 0.5ms, there is no packet delay because packets can be transmitted immediately after the packet is generated. However, the consumed power caused by huge number of “PDCCH only” is large. When N=40ms, i.e., UE monitors PDCCH every 80 slot, the consumed power decreases greatly because the “PDCCH only” is greatly reduced. But the average packet delay is larger than N=0.5ms but it still meets the delay requirements of 120 slots (i.e., 60ms). When N =60ms and N=80ms, the power consumed can be further saved than N=40ms. But their packet delay (>60 ms) is beyond the delay requirements of 120 slots. This is because the average packet interval is less than 120 slots, it is about 55ms. The PDCCH monitoring periodicity (60 or 80ms) is larger than the average packet interval. No packet bundling is assumed, so the packet cannot be delivered in time and many packets will wait in queue.

Observation 9: For gaming traffic, UE reporting preferred PDCCH monitoring parameters can achieve up to 85% power saving gain while the delay requirement of gaming (60ms) can still be fulfilled.

Proposal 9: Capture Table 25 in TR 38.840.

Table 24: Average Relative Power and average packet delay for gaming traffic (UE reporting preferred PDCCH monitoring parameters, gaming traffic)

| Power saving schemes | PDCCH monitoring periodicity N (ms) | average power | average packet delay (ms) |
| --- | --- | --- | --- |
| Fixed PDCCH monitoring periodicity (N) | N=0.5 | 101.72 | 0.50 |
|  | N=40 (UE preferred parameters) | 15.2 | 20.30 |
|  | N=60 | 11 | 117.14 |
|  | N=80 | 8.5 | 144.12 |

Table 25: Simulation results for gaming traffic (UE report preferred PDCCH monitoring parameters)

| Company | Power saving scheme | Power saving gain | Power saving gain for each configuration | UPT/Latency | Estimated Overhead | Evaluation methodology/baseline assumption | Note (include UE throughput) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| vivo | UE report preferred PDCCH monitoring parameters | 85% | 85% | Average packet latency: 20.30ms; Latency increase: The delay requirement of gaming (60ms) can be fulfilled.* | Marginal** | Numerical simulation; Baseline assumption: Fixed PDCCH monitoring periodicity of 0.5ms; Traffic model: gaming |  |

Note*: The delay requirement of gaming (60ms) can be fulfilled.

Note**: The configuration of PDCCH monitoring parameters adaptation is in a semi-static fashion, so the signaling overhead is marginal

- Conclusion

In this contribution, the UE power saving gain for different power saving schemes are simulated and compared with Rel-15 existing schemes. The following proposal and observations are given:

Observation 1: For gaming traffic, enhanced PDCCH monitoring adaptation schemes can provide large power saving benefit with similar or tolerable packet delay loss.

Observation 2: For FTP traffic, enhanced PDCCH monitoring adaptation schemes can provide large power saving benefit with tolerable packet delay loss. Larger power saving gain for higher SINR UEs compared to low SINR UEs.

Proposal 1: At least one of the following enhanced PDCCH monitoring adaptation schemes are adopted for Rel-16 UE power saving

- Dynamic switching of PDCCH monitoring periodicities by DCI indication

- DCI explicit indication of PDCCH monitoring occasions within a periodicity

Proposal 2: Capture Table 10 in TR 38.840.

Observation 3: For ftp model 3, cross-slot scheduling can provide 18%-29% power saving benefit with similar or tolerable packet delay loss, compared with same-slot scheduling.

Proposal 3: Capture Table 13 in TR 38.840.

Observation 4: For VoIP, power saving gain of adaptation of CORSET/search space configuration –HARQ-ACK based indication (UE monitoring of PDCCH search space for retransmission triggered by decoding failure of SPS PDSCH) is about 20%, with no packet delay loss.

Proposal 4: Capture Table 15 in TR 38.840.

Observation 5: For ftp model 3, power saving gain of PDCCH monitoring adaptation between DRX ONduration timer and inactivitytimer is 5%-50%, with tolerable packet delay loss.

Proposal 5: Capture Table 17 in TR 38.840.

Observation 6: For ftp model 3, power saving gain of UE reporting preferred CDRX configuration is 4%-80%, with tolerable packet delay loss.

Proposal 6: Capture Table 19 in TR 38.840.

Observation 7: For gaming traffic, power saving gain of Adaptation to number of antenna is 6%-30%, with no packet delay loss.

Proposal 7: Capture Table 21 in TR 38.840.

Observation 8: For gaming traffic, power saving gain of Adaptation in frequency domain is 9%-45%, with no packet delay loss.

Proposal 8: Capture Table 23 in TR 38.840.

Observation 9: For gaming traffic, UE reporting preferred PDCCH monitoring parameters can achieve up to 85% power saving gain while the delay requirement of gaming (60ms) can still be fulfilled.

Proposal 9: Capture Table 25 in TR 38.840.

References

- Chairman's Notes, RAN1 #94bis meeting

- Chairman's Notes, RAN1 #95 meeting

- R1-1901710, “UE adaptation to traffic and power consumption characteristics”, 3GPP TSG RAN WG1 #96

- R1-070674, “LTE physical layer framework for performance verification”, Orange, China Mobile, KPN, NTT DoCoMo et al.

- R1-1901712, “Modeling and evaluation results for gaming”, vivo

Annex 1-Genaral simulation assumptions

Table A1: Reference Configuration and power models for FR1

| Reference Configuration | Power State | Characteristics | Relative Power |
| --- | --- | --- | --- |
| Downlink: TDD, FR1, 30 kHz SCS, 1CC, 100 MHz BW, PDCCH region of 2 symbol at beginning of a slot, k0 = 0, max. #CCE = 56, 36 PDCCH blind decoding, PDSCH of max data rate with 256QAM 4x4 MIMO, #RB for TRS = 52, 4RX, Capability 1 Uplink: TDD, FR1, 30 kHz SCS, 1CC, 100MHz BW, 1TX, 2 power levels 0dBm and 23dBm Power values are averaged over the operations within a slot. | Deep Sleep | Time interval for the sleep should be larger than the total transition time entering and leaving this state. Accurate timing may not be maintained. | 1 |
|  | Light Sleep | Time interval for the sleep should be larger than the total transition time entering and leaving this state. | 20 |
|  | Micro sleep | Immediate transition is assumed for power saving study purpose from or to a non-sleep state | 45 |
|  | PDCCH-only | No PDSCH and same-slot scheduling; this includes time for PDCCH decoding and any micro-sleep within the slot. | 100 |
|  | SSB or CSI-RS proc. | SSB can be used for fine time-frequency sync. and RSRP measurement of the serving/camping cell. FFS the power scaling for RRM of neighbor cells. TRS is the considered CSI-RS for sync. FFS the power scaling for processing other configurations of CSI-RS. | 100 |
|  | PDCCH + PDSCH | PDCCH + PDSCH. ACK/NACK in long PUCCH is modeled by UL power state. FFS the power scaling for PDSCH-only slot. | 300 |
|  | UL | Long PUCCH or PUSCH. FFS the power scaling for short PUCCH and SRS. | 250 (0 dBm) 700 (23 dBm) |

Table A2: MCS index, spectral efficiency and SNR for PDSCH

| MCS Index IMCS | Modulation Order Qm | Target code Rate R x [1024] | Spectral efficiency | SNR(dB) |
| --- | --- | --- | --- | --- |
| 0 | 2 | 120 | 0.2344 | -6.3875 |
| 1 | 2 | 193 | 0.3770 | -4.3670 |
| 2 | 2 | 308 | 0.6016 | -1.9720 |
| 3 | 2 | 449 | 0.8770 | 0.0389 |
| 4 | 2 | 602 | 1.1758 | 1.9436 |
| 5 | 4 | 378 | 1.4766 | 4.0346 |
| 6 | 4 | 434 | 1.6953 | 4.9933 |
| 7 | 4 | 490 | 1.9141 | 5.8226 |
| 8 | 4 | 553 | 2.1602 | 6.7614 |
| 9 | 4 | 616 | 2.4063 | 7.7246 |
| 10 | 4 | 658 | 2.5703 | 8.3331 |
| 11 | 6 | 466 | 2.7305 | 9.6872 |
| 12 | 6 | 517 | 3.0293 | 10.5948 |
| 13 | 6 | 567 | 3.3223 | 11.5450 |
| 14 | 6 | 616 | 3.6094 | 12.4509 |
| 15 | 6 | 666 | 3.9023 | 13.3469 |
| 16 | 6 | 719 | 4.2129 | 14.4346 |
| 17 | 6 | 772 | 4.5234 | 15.4004 |
| 18 | 6 | 822 | 4.8164 | 16.3262 |
| 19 | 6 | 873 | 5.1152 | 17.3044 |
| 20 | 8 | 682.5 | 5.3320 | 18.6130 |
| 21 | 8 | 711 | 5.5547 | 19.4207 |
| 22 | 8 | 754 | 5.8906 | 20.2124 |
| 23 | 8 | 797 | 6.2266 | 21.0512 |
| 24 | 8 | 841 | 6.5703 | 22.0117 |
| 25 | 8 | 885 | 6.9141 | 23.0826 |
| 26 | 8 | 916.5 | 7.1602 | 23.9406 |
| 27 | 8 | 948 | 7.4063 | 24.9249 |
| 28 | 2 | reserved |  |  |
| 29 | 4 | reserved |  |  |
| 30 | 6 | reserved |  |  |
| 31 | 8 | reserved |  |  |

Annex 2-Traffic and delay model for gaming

Traffic modeling:

Interactive real-time gaming is a traffic with small and random packet arrivals. 3GPP proposes some guidance for gaming modeling in [4]. In this paper, the gaming model is based on Table A3, where the average packet size (parameter a in Table A3) is updated from 120 Bytes to 220 Bytes.

Table A3: Downlink Gaming Network Traffic Parameters

| Parameter | Statistical Characterization |
| --- | --- |
| Initial packet arrival | Uniform Distribution PDF: , |
| Packet arrival | Largest Extreme Value Distribution (also known as Fisher-Tippett distribution) PDF: , Values for this distribution can be generated by the following procedure: , where y is drawn from a uniform distribution in the range |
| Packet size | Largest Extreme Value Distribution (also known as Fisher-Tippett distribution) a = 120 220 Bytes, b = 36 |
| UDP header | Deterministic (2 Bytes). This is added to the packet size accounting for the UDP header after header compression. |

Delay modeling and requirements:

To model the impact of packet delay, in R1-070674 [4], the following delay modeling and requirements are defined.

“A maximum delay of 160 ms is applied to all uplink packets, i.e. a packet is dropped by the mobile station if any part of the packet has not started physical layer transmission, including HARQ operation, 160 ms after entering the mobile station buffer. The packet delay of a dropped packet is counted as 180 ms. A mobile network gaming user is in outage if the average packet delay is greater than 60 ms. The average delay is the average of the delays of all packets, including the delay of packets delivered and the delay of packets dropped.”

In this paper, the delay for each packet is considered according to the above mode. In Section 2, we compare the power consumption performance of different schemes given that the delay requirement (60ms) of gaming can be met.

Annex 3-Traffic model for VoIP

Table A4. Parameter for VoIP

| Parameter | Characterization |
| --- | --- |
| Codec | RTP AMR 12.2, Source rate 12.2 kbps |
| Encoder frame length | 20 ms |
| Voice activity factor (VAF) | 50% (c=0.01, d=0.99) |
| SID payload | Modelled 15 bytes (5Bytes + header) SID packet every 160ms during silence |
| Total voice payload on air interface | 40bytes (AMR 12.2) |
| One way delay budget | 50ms |

Figure A1: 2-state voice activity model

In the model, as shown in Figure A1, the probability of transitioning from state 1 (the active speech state) to state 0 (the inactive or silent state) while in state 1 is equal to , while the probability of transitioning from state 0 to state 1 while in state 0 is . The model is assumed updated at the speech encoder frame rate , where  is the encoder frame duration (typically, 20ms). Other details of VoIP modeling can be find in R1-070674 [4].

Annex 4-Traffic model for FTP molde 3

Table A5. Parameter for ftp model 3

| Model | FTP model 3 |
| --- | --- |
| Packet size | 0.5 Mbytes |
| Mean inter-arrival time | 200 ms |
| DRX setting | Period = 160 ms Inactivity timer = 100 ms On duration: 8 msec |