3GPP TSG RAN WG1 Meeting #96 R1-1901758 Athens, Greece, 25th February – 1st March, 2019 CR-Form-v11.2. CHANGE REQUEST. 38.214; CR; DRAFT; rev; -; Current version:; 15.4.0. For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests.. Proposed change affects:; UICC apps; ME; X; Radio Access Network; X; Core Network. Title:; Draft CR on priority rules for simultaneous Tx. Source to WG:; ZTE. Source to TSG:. Work item code:; NR_newRAT-Core; Date:; 2019-2-15. Category:; F; Release:; Rel-15. Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900.; Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16). Reason for change:; Since PDSCH with scheduling offset < K can exist in any DL OFDM symbols as one potential transmission, PDSCH with scheduling offset < K and any other DL signal can overlap in at least one symbol (for the case of intra-band CA) and they are associated with different QCL Type D assumptions. The other DL signal refers to other PDSCH with scheduling offset >=K, AP-CSI-RS with scheduling offset >=K, and sp/p-CSI-RS. In TS 38.214, the UE behaviour of this case is still open. Taking into account that the other DL signals should have a higher priority than the potential PDSCH in order to guarantee the already-scheduled/determined transmission of the other DL signals, the UE shall apply the QCL assumption of the other DL signal when receiving the PDSCH with scheduling offset <K.. Summary of change:; Adding sentences to support the case of simultaneous transmission of PDSCH and other DL signals.. Consequences if not approved:; Unclear description of QCL determination for the reception of PDSCH with scheduling offset <K in the case of simultaneous transmission of PDSCH and other DL signals.. Clauses affected:; 5.1.5. Y; N. Other specs; X; Other core specifications; TS/TR ... CR .... affected:; X; Test specifications; TS/TR ... CR .... (show related CRs); X; O&M Specifications; TS/TR ... CR .... Other comments:.

### 5.1.5	Antenna ports quasi co-location

<Unchanged part omitted>

For both the cases when tci-PresentInDCI is set to 'enabled' and tci-PresentInDCI is not configured in RRC connected mode, if the offset between the reception of the DL DCI and the corresponding PDSCH is less than the threshold timeDurationForQCL, as defined in [13, TS 38.306], the UE may assume that the DM-RS ports of PDSCH of a serving cell are quasi co-located with the RS(s) in the TCI state with respect to the QCL parameter(s) used for PDCCH quasi co-location indication of the CORESET associated with a monitored search space with the lowest CORESET-ID in the latest slot in which one or more CORESETs within the active BWP of the serving cell are monitored by the UE.

- In this case, if the 'QCL-TypeD' of the PDSCH DM-RS is different from that of the PDCCH DM-RS with which they overlap in at least one symbol, the UE is expected to prioritize the reception of PDCCH associated with that CORESET.

- In this case, if the 'QCL-TypeD' of the PDSCH DM-RS is different from that of other DL signal with which they overlap in at least one symbol, UE applies the 'QCL-TypeD' of the other DL signal when receiving the PDSCH. The other DL signal refers to other PDSCH scheduled with offset larger than or equal to the threshold timeDurationForQCL, aperiodic CSI-RS scheduled with offset larger than or equal to the minimum value between 48 and the UE reported threshold beamSwitchTiming, periodic CSI-RS and semi-persistent CSI-RS.

- This also applies to the intra-band CA case (when PDSCH and other signals are in different component carriers).

If none of configured TCI states contains 'QCL-TypeD', the UE shall obtain the other QCL assumptions from the indicated TCI states for its scheduled PDSCH irrespective of the time offset between the reception of the DL DCI and the corresponding PDSCH.

<Unchanged part omitted>