3GPP TSG RAN WG1 #96	     		     R1-1901801

Athens, Greece, 25th February – 1st March, 2019

Agenda Item:	7.2.2.2.4

Source: 	MediaTek Inc.

Title:	Discussion on NR-U configured grant

Document for: 	Discussion

Introduction

In RAN1#AH1901 meeting, the following agreements and conclusion were made [1]:

Agreement: For configured grant resource configuration in time domain, the following alternatives are to be studied with more detailed proposal and analysis, strive to down-select in RAN1#96: - Alt. 1: Bitmap based approach as baseline with potential enhancement - Companies are encouraged to provide detailed design in next meeting - Alt. 2: NR Rel-15 based time domain resource allocation approach as baseline with potential enhancement - Companies are encouraged to provide detailed design in next meeting Agreement: - Support multiple UE starting time offsets with sub-symbol granularity with FeLAA AUL approach as the baseline - FFS: Enhancements specific to NRU - Companies are encouraged to provide views and analysis on the following issues: - Whether to support allowing the UE to start transmission later than the starting symbol as indicated in configured grant based on LBT outcome - If yes, multiple starting positions within a slot for a configured grant configuration; - Alt. 1: subset of symbols - Alt. 2: any symbol - FFS: gNB knowledge of starting symbol, whether UE indicates to gNB - FFS signaling details - FFS: whether similar design for scheduled grant and configured grant - Whether the ending symbol can be punctured - Whether the position of the ending symbol can be shifted depending on the starting position due to LBT procedures Conclusion: The following aspects should be discussed further as part of the channel access discussions - Contention window adjustment - Details of COT sharing related to NRU configured grant including details and limitations on UE-initiated COT sharing with gNB and configured grant UL transmissions within gNB acquired COT Agreement: CG-UCI should at least include the following information: - HARQ ID - NDI - RV - COT sharing information, FFS details - FFS: other information including UE ID

In this contribution, we will discuss the following issues for NR-U configured grant:

- Flexible starting positions for grant-free PUSCH

- CBG-level retransmission for grant-free PUSCH

Discussion

Flexible starting positions for grant-free PUSCH transmission

For grant-free PUSCH in unlicensed band, it would be beneficial to allow multiple starting positions at least for grant-free PUSCH(s) transmitted in the beginning of an UL transmission burst to mitigate UL resource dropping due to LBT failure to maximize the resource utilization. Moreover, multiple starting positions also enable more LBT attempts to increase the channel access probability for a UE. Two options can be identified for enabling multiple flexible positions:

Option 1. Multiple mini-slot-based transmission occasions: To effectively improve the resource utilization, multiple transmission occasions could be configured with a finer granularity, e.g., mini-slot with 2 OS, at the beginning of UL transmission burst. NR Rel-15 has already supported mini-slot-based PUSCH, i.e., Type B PUSCH mapping, with length 1 OS to 14 OS. Meanwhile, to effectively increase the channel access probability, the number of mini-slots should be sufficiently large to provide time domain diversity. As the example shown in Figure 1(a), 7 mini-slot-based transmission occasions with 2 OS and 1 slot-based transmission occasion are configured for a UE, and each PUSCH has one candidate starting position at its transmission beginning. Before the beginning of the pre-configured UL transmission burst, the UE performs a LBT procedure towards accessing the first transmission occasion. If the LBT is successful, the UE can start

gNB LBT

Feedback Transmission

UL Transmission Using Remaining CG Parameters

DL Transmission

transmitting PUSCHs up to the end of the pre-configured UL transmission burst. If the LBT fails, the UE does not stop the LBT procedure. Rather, it can continue its LBT attempts towards accessing the second transmission occasion, and so on and so forth. However, we does not prefer Option 1 due to the following potential issues/impacts:

  - The granularity would be limited by UE capability since a UE may not support 7 TDMed PUSCHs per slot for different TBs. In NR Rel-15, a UE may only support up 2 or 4 TDMed PUSCHs per slot for different TBs.

  - Multiple mini-slot-based PUSCHs may increase the total number of required HARQ processes. In NR-U configured grant, since HARQ process ID for each PUSCH is selected by the UE instead of indicating by the gNB, the HARQ processes will not be wasted on the mini-slot-based transmission occasions blocked by LBT failures. However, after a successful channel access, the remaining mini-slot-based PUSCHs still occupy a number of HARQ processes. In general, increasing number of HARQ processes has difficult software/hardware implementations.

  - Dividing a slot-based PUSCH into multiple mini-slot-based PUSCHs leads to underutilized resources because there will be more DMRS resources for these mini-slot-based PUSCHs, especially for DFT-s-OFDM waveform for which data and DMRS is TDMed.

Figure 1. Two options for enabling flexible starting positions for grant-free PUSCH (a) multiple mini-slot-based transmission occasions (b) multiple candidate starting positions within a transmission occasion

- Option 2. Multiple candidate starting positions within a transmission occasion: Allowing multiple candidate starting positions within a pre-configured transmission occasion is another way to increase both resource utilization and channel access probability. Since multiple candidate starting positions could be allowed for all transmission occasions, the time domain diversity is not limited only at the beginning of a transmission burst. As the example shown in Figure 1(b), 2 slot-based transmission occasions are configured for a UE, and each PUSCH has 14 candidate starting positions. The UE performs a LBT procedure before the beginning of the pre-configured UL transmission burst until the LBT succeeds, then it can start transmitting a PUSCH on the remaining symbol(s) of a transmission occasion. In FeLAA, two candidate starting positions in a sub-frame for UL transmission are supported, where a UE can start at symbol #0 or symbol #7 depending on the outcome of LBT. Since NR Rel-15 has already supported mini-slot-based PUSCH with any length, it is reasonable to support PUSCH transmission starting at any symbol in NR-U. Whereas, the gNB may detect the PUSCH transmission on the multiple candidate starting positions blindly. To facilitate the gNB to identify the actual starting position, DMRS could be always located in the first available symbol of transmitted PUSCH for channel estimation and starting position detection. Meanwhile, indication of the starting position of a PUSCH in CG-UCI carried by this PUSCH may be helpful for the gNB to determine the actual starting position.

To avoid the TBS changed according to the varying starting position, the UE can rate-match the TB pre-determined as for a complete PUSCH transmission on remaining symbols, and the TB still can be delivered if the effective code rate is acceptable. Compared to puncturing, rate-matching the pre-determined TB on remaining symbols could avoid unnecessary retransmission. However, if there are only a few available symbols, rate-matching the pre-determined TB will make the effective code rate too high that the UE may need to retransmit the entire TB. In this case, puncturing the pre-determined TB would be a better choice because the UE may not need to retransmit the entire TB if CBG-level retransmission is adopted. Therefore, both rate-matching and puncturing should be considered to avoid changing a TBS depending on the LBT outcome if multiple candidate starting positions within a grant-free PUSCH transmission are supported in NR-U.

Proposal 1: Multiple candidate starting positions within a grant-free PUSCH transmission occasion for improving the resource utilization and increasing the channel access probability should be supported in NR-U.

Proposal 2: PUSCH DMRS could be always located in the starting symbol of a transmitted PUSCH for channel estimation and starting position detection.

Proposal 3: Indication of the starting position of a transmitted PUSCH carried by CG-UCI is beneficial for the gNB to determine the actual starting position.

Observation 1: In a PUSCH transmission with multiple candidate starting positions, if the number of remaining symbols is sufficient, rate-matching the TB on the remaining symbols could avoid unnecessary retransmission.

Observation 2: In a PUSCH transmission with multiple candidate starting positions, if only a few symbols are available, puncturing the TB could avoid retransmitting the entire TB by exploiting CBG-level retransmission.

Proposal 4: Both rate-matching and puncturing should be considered to avoid changing TBS depending on the LBT outcome if multiple candidate starting positions within a grant-free PUSCH transmission are supported in NR-U.

| Potential Issues/Impacts | Option 1 | Option 2 | Memo |
| --- | --- | --- | --- |
| Granularity is limited by UE capability | Yes | No | Up to 2/4/7 TDMed PUSCHs per slot for different TBs |
| Increase number of required HARQ processes | Yes | No |  |
| More DMRS overhead | Yes | No |  |
| Blind transmission detection | No | Yes |  |
| Increase UE complexity for a PUSCH transmission | No | Yes | Transmission depending on channel access outcome |

Table 1. Comparison between two options for enabling flexible starting positions

CBG-level retransmission for grant-free PUSCH

In NR, CBG-level retransmission has been introduced for grant-based PUSCH, where an UL grant could indicate the UE only retransmitting some CBGs rather than the entire TB using CBGTI (CBG transmission indication). It is useful for large TBS scenario especially when some CBGs preempted by URLLC transmission or corrupted by time varying interference. In NR-U, it would be beneficial to adopt CBG-level retransmission not only for grant-based PUSCH but also for grant-free PUSCH in that it is robust against interference caused by hidden nodes and significantly increases the retransmission efficiency. Moreover, if multiple candidate starting positions within a grant-free PUSCH transmission are supported in NR-U, since some symbols shall be dropped according to when the LBT succeeds, it would be favorable to retransmit the not-transmitted or partially transmitted CBG(s) instead of the entire TB.

NR-U supports the retransmission of a HARQ process initially transmitted on a grant-free PUSCH via a separate grant-based PUSCH or the same grant-free PUSCH in a separate period [2], and we considers CBG-level retransmission for the two cases:

- CBG-level retransmission on a separate grant-based PUSCH: If a DCI scheduling a PUSCH transmission includes a CBGTI field, the gNB can always use it to request retransmission for the failed CBGs, and allocate resources fewer than the initial transmission on the grant-free PUSCH to increase the resource efficiency, as shown in Figure 2. Since NR DCI Format 0_1 already includes CBGTI field, this case can be naturally supported without signaling enhancements.

Figure 2. Example CBG-level retransmission on a separate grant-based PUSCH

- CBG-level retransmission on the same grant-free PUSCH in a separate period: If a UE autonomously initiates retransmission on the same grant-free PUSCH in a separate period, the resource allocation would be the same. Therefore, no resource can be saved by CBG-level retransmission. However, the redundant resources still make the retransmission more robust due to lower effective code rate. In addition, since both initial transmission and retransmission on the grant-free PUSCH(s) at the beginning of a transmission burst may usually suffer from dropped symbols if multiple candidate starting positions within a grant-free PUSCH transmission are adopted, it may lead to a lot of retransmissions for the same TB without CBG-level retransmission, as illustrated in Figure 3. Thus, CBG operation is also necessary for retransmission on grant-free PUSCH.

Figure 3. Example of only TB-level retransmission on the same grant-free PUSCH resources

To enable CBG-level retransmission on grant-free PUSCH, as shown in Figure 4, on top of indicating RV, NDI, and HARQ ID, a bitmap like CBGTI for indicating the transmitted CBG(s) should be included in CG-UCI in every grant-free PUSCH. Also, the gNB should provide individual A/N for each CBG to the UE via CG-DFI. In FeLAA, a codebook in AUL-DFI is adopted for indicating TB-level A/N for all HARQ processes, where the codebook size is only scaled with the number of HARQ processes. When CBG-level A/N is introduced in CG-DFI, the codebook size is further scaled with the number of CBGs per TB. For example, when a UE is configured with 8 HARQ processes and 8 CBGs per TB for configured grant, it would lead to a codebook with 64 bits in CG-DFI. To reduce the large payload size for CG-DFI, one potential solution is that the gNB can hybrid provide TB-level A/N and CBG-level A/N in a codebook. This is because CBG-level A/N is not necessary for some cases, e.g., HARQ process without detected CBGs, HARQ process without correctly decoded CBGs, and HARQ process with all CBGs decoded correctly. Meanwhile, the gNB can also flexibly decide the number of HARQ processes provided with CBG-level A/N depending on maximal payload size. As shown in Figure 5, to indicate whether TB-level A/N or CBG-level A/N is adopted for each HARQ process, a bitmap can be included in CG-DFI.

Proposal 5: NR-U should support CBG-level retransmission of a HARQ process initially transmitted on a grant-free PUSCH using a separate grant-based PUSCH or the same grant-free PUSCH in a separate period.

Observation 3: CBG-level A/N is not necessary for some cases, e.g., HARQ process without detected CBGs, HARQ process without correctly decoded CBGs, and HARQ process with all CBGs decoded correctly.

Proposal 6: To reduce codebook size resulted from CBG-level HARQ A/N, the gNB could hybrid provide TB-level A/N or CBG-level A/N for HARQ processes in a codebook.

Figure 4. Example of CBG-level retransmission on the same grant-free PUSCH in a separate period

Figure 5. Example of providing hybrid TB-level A/N and CBG-level A/N in a codebook

Conclusion

The proposals and observations made in this contribution are summarized below:

Proposal 1: Multiple candidate starting positions within a grant-free PUSCH transmission occasion for improving the resource utilization and increasing the channel access probability should be allowed in NR-U.

Proposal 2: PUSCH DMRS could be always located in the starting symbol of a transmitted PUSCH for channel estimation and starting position detection.

Proposal 3: Indication of the starting position of a transmitted PUSCH carried by CG-UCI is beneficial for the gNB to determine the actual starting position.

Observation 1: In a PUSCH transmission with multiple candidate starting positions, if the number of remaining symbols is sufficient, rate-matching the TB on the remaining symbols could avoid unnecessary retransmission.

Observation 2: In a PUSCH transmission with multiple candidate starting positions, if only a few symbols are available, puncturing the TB could avoid retransmitting the entire TB by exploiting CBG-level retransmission.

Proposal 4: Both rate-matching and puncturing should be considered to avoid changing TBS depending on the LBT outcome if multiple candidate starting positions within a grant-free PUSCH transmission are supported in NR-U.

Proposal 5: NR-U should support CBG-level retransmission of a HARQ process initially transmitted on a grant-free PUSCH using a separate grant-based PUSCH or the same grant-free PUSCH in a separate period.

Observation 3: CBG-level A/N is not necessary for some cases, e.g., HARQ process without detected CBGs, HARQ process without correctly decoded CBGs, and HARQ process with all CBGs decoded correctly.

Proposal 6: To reduce codebook size resulted from CBG-level HARQ A/N, the gNB could hybrid provide TB-level A/N or CBG-level A/N for HARQ processes in a codebook.

- References

- Chairman’s Notes, 3GPP TSG RAN WG1 Meeting #AH1901, Taipei, Taiwan

- TR 38.889, Study on NR-based access to unlicensed spectrum