3GPP TSG RAN WG1 #96		R1-1902154

Athens, Greece, 25th February – 1st March, 2019

Agenda Item:	7.2.3.2

Source:	Lenovo, Motorola Mobility

Title:	Discussion on discovery and measurement for IAB network

Document for:	Discussion

- Introduction

This contribution is an update of R1-1900393.

In RAN1 NR AH#1901, corresponding agreements related to SSB based inter-IAB node discovery/measurement are as following:

Agreements:

- SSB transmission configuration information (i.e. the necessary information for the DU to determine when to transmit SSB) and SSB reception configuration information (i.e. the necessary information for the MT to determine when to search for SSB) for IAB inter-node measurements are determined in a centralized manner, i.e. not locally by the IAB node.

- An IAB node is provided with configuration of SSB transmission and reception information for IAB inter-node measurements.

  - FFS details

In this contribution, we focus on SSB related enhancement for inter-IAB node discovery and measurement.

- Discussion

2.1 SSB pattern for different IAB nodes

Due to half-duplex constraint, an IAB node can’t receive and transmit SSB at the same time. So there should be some mechanisms to allocate different time domain resources for different IAB nodes to enable inter-IAB node discovery/measurement.

In NR R15 specification, a SSB set containing multiple SSB indices is restricted to a 5ms time window. Different slot/symbols are used for different SSB indices in the 5ms window, and it is predetermined in specification depending on the frequency range and SSB subcarrier spacing. SSB set is transmitted periodically. The possible periodicity can be 5ms, 10ms, 20ms, 40ms, 80ms, etc. Whether all or part of the SSB indices within a SSB set is actually transmitted is configurable by RRC signaling. Regarding the time domain position of the 5ms time window, it is decided by the gNB itself and indicated to its serving UEs by radio frame index.

Regarding the SSB pattern for inter-IAB node discovery/measurement, different time domain positions of the 5ms time window can be used. This can be achieved by different offset values with the same periodicity. For example, both IAB node 1 and IAB node 2 have the SSB periodicity of 40ms, and IAB node 1 use offset 0, then the starting of the 5ms time window for SSB transmission will be 0ms, 40ms, 80ms, etc. For IAB node 2, the offset value is 20ms, then the starting of the 5ms time window for SSB transmission will be 20ms, 60ms, 100ms, etc.

If the number of offsets supported by a specific periodicity is not enough, time domain randomization of SSB offsets can be considered. For example as shown in Fig 1 (the green block), if the average periodicity of a SSB set is 80ms, in the time window between 0 to 79ms, the SSB set is located at 0-4ms, and in the time window between 80ms and 159ms, the SSB set is located at 110ms-114ms. In this way, the time domain offsets of a SSB set among different periodicities are changed. After the time domain randomization of the time domain offset, the SSB set doesn’t occur periodically, however, the average of the periodicity or even the periodicity to determine the randomized change of the offset of the SSB set is still same as the SSB set periodicity before randomization.

Fig 1 Example of time domain randomization of time domain offset of SSB set

In another alternative, to support more measurement opportunities for inter-IAB node discovery/measurement, there can be multiple SSB transmissions for an IAB node within a periodicity, and the multiple SSB transmissions can change in time randomly. For example as shown in Fig 2, taking the yellow blocks as an example, IAB node 4 transmits at 0-4ms and 30-34ms in the time window between 0 to 79ms, and it transmits at 80-84ms and 90-94ms in the time window between 80ms to 159ms. The time domain offset in the first time window (0-79ms) is 0 and 30ms, and the time domain offset in the second time window (80-159ms) is 0 and 10ms. In this way, multiple SSB set transmissions of IAB node 4 in a periodicity creates more detection/measurement opportunities for other IAB nodes. Even if an IAB node (e.g. IAB node 1) can’t detect IAB node 4 in the first transmission, it may have chance to do detection and measurement in the second SSB set transmission of IAB node 4. Meanwhile, time domain randomization of the offsets of SSB set also create more opportunities for detection and measurement. Even the detection can’t be performed in the first periodicity, it can be fulfilled in the second periodicity. For example, IAB node 3 can’t detect the existence of IAB node 4 in the time window between 0 and 79ms, however, in the time window from 80ms to 159ms, it can detect IAB node 4 by the SSB set transmitted with time domain offset 10ms. The time domain randomization can be seen as a regrouping of the IAB nodes to provide detection/measurement opportunities for the IAB node within the same group in previous time window.

Fig 2 Example of time domain randomization of the multiple time domain offsets of a SSB set

Proposal 1: Support the multiple SSB set transmissions in a periodicity and the time domain randomization of SSB (set) offsets for inter-IAB node discovery/measurement.

2.2 SSB coordination

Regarding how to determine the time domain offset(s) of SSB for a specific link, it can be based on explicit signaling or determined implicitly. For the explicit signaling, a central unit inform the IAB nodes on the offset(s). For the implicit way, an IAB node calculates its time domain offset(s) by a formula with some parameters. We propose to study both the explicit and implicit SSB coordination mechanism.

Proposal 2: Support both explicit and implicit SSB coordination mechanisms.

In R15, there is only one SMTC for a UE. It determines the periodicity/offset/duration for SSB measurement. For inter-IAB node measurement, SMTC can also be configured to an IAB node for measurement of other IAB nodes’ SSBs. As there is TDM restrict of the IAB node, different IAB nodes may transmit at different time domain positions to enable inter-IAB measurement more efficiently. In this case, multiple SMTC configurations are necessary to measure all the possible IAB nodes.

Proposal 3: Support multiple SMTC configurations for inter-IAB node measurement.

For PDSCH reception at the UE side, when a time-frequency resource is occupied by SSB, the resource is considered as not available for PDSCH reception. The time-frequency domain position of legacy SSB for access UE can be detected by the UE with blind decoding. The off-raster SSB for neighboring cell measurement in NR R15 can be configured to the UE. However, for the off-raster SSB for inter-IAB node discovery/measurement, the access UE can’t know its existence and position without additional signaling.

Scheduling restriction can be applied to PDSCH when there is overlapping between PDSCH and SSB for inter-IAB node discovery/measurement with loss of scheduling efficiency. There are also rate matching pattern introduced in NR R15 to solve collision between PDSCH and other channel/RS with maximum number restrict on number of the rate matching patterns. Explicit signaling can also be considered to solve the randomization pattern of SSB/SSB set for inter-IAB node discovery/measurement.

Proposal 4: Study how to address the collision between SSB for inter-IAB node discovery/measurement and PDSCH reception at UE side.

- Conclusion

In this contribution, we discussed the IAB node discovery and measurement related issues, and our proposals are as following:

Proposal 1: Support the multiple SSB set transmissions in a periodicity and the time domain randomization of SSB set offsets for inter-IAB node discovery/measurement.

Proposal 2: Support both explicit and implicit SSB coordination mechanisms.

Proposal 3: Support multiple SMTC configurations for inter-IAB node measurement.

Proposal 4: Study how to address the collision between SSB for inter-IAB node discovery/measurement and PDSCH reception at UE side.

- References

3GPP RAN1 NR AH#1901 chairman notes

RANP-182882, “New WID: Integrated Access and Backhaul for NR”, 3GPP RANP#82