3GPP TSG RAN WG1 Meeting #96 R1-1902240 Athens, Greece, February 25th – March 1st, 2019 CR-Form-v11.2. [DRAFT] CHANGE REQUEST. 38.212; CR; rev; Current version:; 15.4.0. For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests.. Proposed change affects:; UICC apps; ME; X; Radio Access Network; X; Core Network. Title:; Draft CR on PDCCH order for cross-carrier scheduling. Source to WG:; Samsung. Source to TSG:. Work item code:; NR_newRAT-Core; Date:; 2019-02-25. Category:; F; Release:; Rel-15. Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900.; Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16). Reason for change:; 1) Cross-carrier signaling for PDCCH order - If the CRC of the DCI format 1_0 is scrambled by C-RNTI and the "Frequency domain resource assignment" field are of all ones, the DCI format 1_0 is for random access procedure initiated by a PDCCH order. - Since DCI format 1_0 does not include carrier indicator field (CIF), it needs to be further clarified that how to detect the PDCCH order by the UE when cross-carrier scheduling is configured.. Summary of change:; 1) Cross-carrier signaling for PDCCH order - If cross-carrier scheduling is configured, 3 MSB bits of the reserved bits in DCI format 1_0 are used for CIF.. Consequences if not approved:; Incomplete specifications for PDCCH order with cross-carrier scheduling - If CR is not approved, the UE cannot monitor DCI format 0_0/1_0 for PDCCH order in SCell when cross-carrier scheduling is configured and Type3 CSS is not configured. - If the UE is implemented according to the change but the NW is not, the NW cannot transmit DCI format for PDCCH order via the SCell, and the UE may understand the command of DCI format for PDCCH order in different way. As a consequence, the system will be broken. - If the NW is implemented according to the change but the UE is not, the NW may transmit DCI format for PDCCH order for the SCell via the scheduling cell, but the UE may handle it error case when the reserved bits in the detected DCI format for PDCCH order are not zeros. As a consequence, the UE cannot perform rach triggered by PDCCH order properly.. Clauses affected:; 7.3.1.2.1. Y; N. Other specs; X; Other core specifications; TS/TR ... CR .... affected:; X; Test specifications; TS/TR ... CR .... (show related CRs); X; O&M Specifications; TS/TR ... CR .... Other comments:.

7.3.1.2.1 Format 1_0

*** Unchanged text is omitted ***

DCI format 1_0 is used for the scheduling of PDSCH in one DL cell.

The following information is transmitted by means of the DCI format 1_0 with CRC scrambled by C-RNTI or CS-RNTI or MCS-C-RNTI:

-	Identifier for DCI formats – 1 bits

-	The value of this bit field is always set to 1, indicating a DL DCI format

-	Frequency domain resource assignment –  bits where  is given by subclause 7.3.1.0

If the CRC of the DCI format 1_0 is scrambled by C-RNTI and the "Frequency domain resource assignment" field are of all ones, the DCI format 1_0 is for random access procedure initiated by a PDCCH order, with all remaining fields set as follows:

-	Random Access Preamble index – 6 bits according to ra-PreambleIndex in Subclause 5.1.2 of [8, TS38.321]

-	UL/SUL indicator – 1 bit. If the value of the "Random Access Preamble index" is not all zeros and if the UE is configured with SUL in the cell, this field indicates which UL carrier in the cell to transmit the PRACH according to Table 7.3.1.1.1-1; otherwise, this field is reserved

-	SS/PBCH index – 6 bits. If the value of the "Random Access Preamble index" is not all zeros, this field indicates the SS/PBCH that shall be used to determine the RACH occasion for the PRACH transmission; otherwise, this field is reserved.

-	PRACH Mask index – 4 bits. If the value of the "Random Access Preamble index" is not all zeros, this field indicates the RACH occasion associated with the SS/PBCH indicated by "SS/PBCH index" for the PRACH transmission, according to Subclause 5.1.1 of [8, TS38.321]; otherwise, this field is reserved

-	Reserved bits – 10 bits. If the UE is configured to monitor PDCCH candidates with carrier indicator field as described in Subclause 10.1 of [5, TS38.213], the 3 MSB bits correspond to the carrier indicator field.

*** Unchanged text is omitted ***