3GPP TSG RAN WG1 #96															R1-1902334

Athens, Greece, 25th February – 1st March, 2019

Source:	CMCC

Title:	Further discussion on configuration of RIM RS

Agenda Item:	7.2.5.3

Document for:	Discussion and Decision

Introduction

At RAN#82 meeting, a study item of regarding Remote Interference Management for NR (NR-RIM) was closed [1], and new work item regarding Cross Link Interference (CLI) handling and Remote Interference Management (RIM) for NR was agreed [2].

In the RIM SI, the frameworks for mechanisms for gNBs to start and terminate the transmission/ detection of reference signal(s), the functionalities and requirements of the corresponding RS(s) as well as the design of the RS(s), and the backhaul-based coordination mechanisms among gNBs have been studied, where the outcome and conclusions of the SI was captured in TR 38.866 [3].

The detailed objectives of this WI for RIM are:

- Specify RIM RS resource and configurations, including [RAN1]

  - A basic RIM-RS resource

  - Configuration of RIM-RS and distinguishable RIM RS-1/2 resources, including sequence type, time and frequency transmission pattern

  - Determine gNB set identification information through detection of RIM-RS(s) by implicit or explicit indication. Determine further information that can be carried by the RIM-RS, such as “Ducting phenomenon exists”, “Enough mitigation” & “Not enough mitigation”,   [RAN1, SA5]

- Specify the inter-set RIM backhaul signalling via the core network to convey the messages of “RIM-RS detected” and “RIM-RS disappeared [RAN3]

- Identify corresponding OAM functions to support RIM operation [RAN1, RAN3].

Impact on SA5

- Support all necessary bidirectional information exchange with the OAM and/or unidirectional information indication/configuation by the OAM for RIM operation as identified by RAN1/RAN3  [SA5]

- Support of multiple configurations for basic RIM-RS transmission, including resource and/or functionality, if not specified in RAN1 [SA5]

Impact on SA 2

- If needed, coordinate with SA2 to support core-network based inter-gNB coordination solutions[SA2]

At RAN1 Adhoc 1901 meeting, many agreements were achieved [4],

Regarding general consideration on mapping rule between the RIM-RS indices and Set ID

Agreement

- Specification defines unique RIM-RS indices for indication of RIM-RS resources (in time-domain, frequency-domain, and sequence)

- Specification provides configurable mapping rule between the RIM-RS indices and Set ID;

- FFS: Details on how to handle repetition

- Note that the configurable mapping rule should be available and common to all gNBs

Agreement

A gNB can be configured with multiple set IDs

- For each set ID, multiple RIM-RS configurations can be configured

- From RX gNB perspective, there is no additional handling required compared to multiple gNBs with each gNB configured with a single set ID

Regarding functionalities of RIM-RS as well as differentiation mechanisms

Agreement

- OAM configures RIM-RS configurations and provide necessary additional configuration information for one or more of functionalities of RIM-RS.

  - RIM-RS configuration refers to the parameter set that includes sequence, time-frequency resource, periodicity, etc

- FFS: detailed signalling for RIM-RS configuration for repetition and near/far functionality indication,

  - FFS: How to support repetition by configuration, as well as if explicit indication of near/far functionality is needed

Agreement

From the same transmitting gNB perspective, RS-1 and RS-2 differentiation is done in time domain only

Agreement

For different gNB in a network, RS-1 and RS-2 differentiation is done in time domain only.

Agreement

For indication of either “enough migitation” or “not enough mitigation”, two basic RIM-RS resources are used

- The two resources use the same time/frequency resource but different sequences

Agreement

For repetition/near-far-functionality, multiple basic RIM-RS resources for a gNB transmission can be configured only in consecutive TDD DL/UL switching period, with one basic RIM-RS resource per TDD DL/UL switching period

- The maximum number of consecutive TDD DL/UL switching periods for repetition/near-far-functionality is 8 (the number can be either 2, 4, or 8) with near-far functionality and with repetition

  - The first half of consecutive TDD DL/UL switching periods is for repetition

  - The second half of consecutive TDD DL/UL switching periods is for repetition

  - Between the first and the second half of consecutive TDD DL/UL switching periods is for near-far-functionality

- The maximum number of consecutive TDD DL/UL switching periods for repetition is 4 (the number can be either 1, 2, or 4) without near-far functionality and with repetition only

- The maximum number of consecutive TDD DL/UL switching periods is 2 with near-far functionality only

Agreement

For each basic RIM-RS resource, the OFDM symbol position within the TDD DL/UL switching period can be individually configured

Regarding frequency-domain related configuration

Agreement

RIM-RS sequence is mapped in the frequency with respect to a common reference point for all gNBs. The reference point is configured by OAM.

Agreement

The RIM RS bandwidth is defined as the number of PRBs.

- For carrier bandwidth smaller than 20MHz, the RIM RS bandwidth is configured as

  - For 30kHz SCS: min{48 RB, carrier bandwidth}

  - For 15kHz SCS: min{96 RB, carrier bandwidth}

- For carrier bandwidth larger than 20MHz, the maximum RIM RS bandwidth can be configured from a set of SCS and PRB combination, including

  - {15kHz, 96 RB} = 17.28MHz

  - {30kHz, 48 RB} = 17.28MHz

  - {30kHz, 96 RB} = 34.56MHz

  - FFS: other combinations for larger RS bandwidth and/or better frequency utilization

Agreement

The maximum number of candidate frequency resource (Y) for RIM-RS detection is

- 1, for bandwidth < 40 MHz.

- 2, for bandwidth ≥ 40 MHz and < 80 MHz.

- 4, for bandwidth ≥ 80 MHz and ≤ 100 MHz.

Agreement

All configured basic RIM-RS resources have the same bandwidth and the same subcarrier spacing

Regarding sequence related configuration

Agreement

The initialization seed for RIM-RS is configured by OAM and the subsequent seed will be changing over time

Agreement

- The number of RIM RS sequence (CDM) in each DL/UL switching points is configurable and common for all the DL/UL switching points transmitting RIM RS-1

- The number of RIM RS sequence (CDM) in each DL/UL switching points is configurable and common for all the DL/UL switching points transmitting RIM RS-2

- The maximum number of total RIM RS sequence within 10ms is 32 regardless how many DL/UL switching points are configured in the 10ms.

Regarding other issues

Agreement

Maximum 22 bit gNB set ID is supported

- Actual number of gNB set ID is configurable

For further study

Global periodicity is configured based on granularity of FS periodicity containing single DL/UL switching points or FS periodicity containing dual DL/UL switching points

- when RIM RS-1 reources and RIM RS-2 resources are configured in different DL/UL switching point for each gNB, the RIM RS-1 and RIM RS-2 for each gNB bear different set ID, global periodicity is based on both all required RIM RS-1 resources, and all required RIM RS-2 resources

In this contribution, we first discuss on the detailed configuration on RIM-RS resources, as well as its mapping rule related to set ID. And then discuss on other remaining issues on RIM-RS configurations, including detailed signalling for RIM-RS configuration for repetition and near/far functionality indication, generation formula on initialization seed for RIM-RS, etc.

Discussion on RIM-RS resources configuration

The configuration for RIM RS resources can be summarized in the following steps:

- Step 1: Determine a global RIM RS transmission periodicity based on the total number of set ID(s) and the resource(s) configured for each set.

- Step 2: Determine the RIM RS resource indices within the transmission periodicity.

- Step 3: Determine the corresponding set ID for transmission/reception according to the RIM RS resource indices based on the mapping rule.

Step 1: Determine global RIM RS transmission periodicity

The global RIM RS transmission periodicity is determined based on both total required time-domain RIM RS-1 resources, and total required time-domain RIM RS-2 resources, if configured. Furthermore, Total required RIM RS-1 (RS-2) time-domain resource is determined based on the number of consecutive TDD DL/UL switching periods configured for each set ID and the number of set IDs for RS-1 (RS-2) to be differentiated via time-domain.

Proposal 1: The global RIM RS transmission periodicity is determined based on both total required time-domain RIM RS-1 resources, and total required time-domain RIM RS-2 resources, if configured.

- Total required RIM RS-1 (RS-2) time-domain resource is determined based on the number of consecutive TDD DL/UL switching periods configured for each set ID and the number of set IDs for RS-1 (RS-2) to be differentiated via time-domain.

To be more specific, the following parameters will be configured to determine the global RIM RS transmission periodicity, as well as the layout of RIM-RS resources for different functionalities:

- Number of candidate frequency resources in the whole network: NF

- Total number of set IDs for RS-1: NSetIdRS1

  - The number of RIM RS sequences (CDM) in one DL/UL switching point: S1

  - Number of consecutive TDD DL/UL switching periods for repetition/near-far: R1

  - Indication of whether differentiating “enough mitigation”/ “not enough mitigation”: ON/OFF

- Total number of set IDs for RS-2: NSetIdRS2, if needed

  - The number of RIM RS sequences (CDM) in one DL/UL switching points: S2

  - Number of consecutive TDD DL/UL switching periods for repetition/near-far: R2

For RS-1, NF∙S1 set IDs can be supported within the same DL/UL switching period, if the indication for “enough mitigation” or “not enough mitigation” is OFF; otherwise, NF∙S12 set IDs can be supported within the same DL/UL switching period. Then, the number of set IDs for RS-1 to be differentiated via time-domain can be determined as NT, 1=NSetIdRS1NF∙S1，or NT, 1=2∙NSetIdRS1NF∙S1.

Similarly, for RS-2, NF∙S2 set IDs can be supported within the same DL/UL switching period, then the number of set IDs for RS-2 to be differentiated via time-domain can be determined as NT, 2=NSetIdRS2NF∙S2.

The global RIM RS transmission periodicity (Pt) is then determined as:

Pt=NSetIdRS1NF∙S1∙R1+ NSetIdRS2NF∙S2∙R2if the indication for “enough” or “not enough” is OFF2∙NSetIdRS1NF∙S1∙R1+ NSetIdRS2NF∙S2∙R2if the indication for “enough” or “not enough” is ON

Proposal 2: The following parameters, denotes as parameter set for RIM-RS resources configuration, will be configured by OAM to determine the global RIM RS transmission periodicity, as well as the layout of RIM-RS resources for different functionalities,

- Number of candidate frequency resources in the whole network: NF

- Total number of set IDs for RS-1: NSetIdRS1

  - The number of RIM RS sequences (CDM) in one DL/UL switching point: S1

  - Number of consecutive TDD DL/UL switching periods for repetition/near-far: R1

  - Indication of whether differentiating “enough mitigation”/ “not enough mitigation”: ON/OFF

- Total number of set IDs for RS-2: NSetIdRS2, if needed

  - The number of RIM RS sequences (CDM) in one DL/UL switching points: S2

  - Number of consecutive TDD DL/UL switching periods for repetition/near-far: R2

Figure 1. Illustration of global RIM RS transmission periodicity.

Following are some examples determining global RIM RS transmission periodicity under different configurations.

- Example 1: when only RS-1 is configured, without repetition, near/far detectability, enough/not enough indication functionalities, i.e., R1=1, R2=0, we have,

Pt=NSetIdRS1NF∙S1.

When the total number of set ID for RS-1 is 22 bit, i.e. NSetIdRS1=222, the number of frequency resource is configured as NF=4, the number of RIM RS sequence (CDM) in one DL/UL switching points is configured as S1=8. It can be obtained that Pt=2224×8=217. Considering a TDD DL/UL switching period of 5ms, the global RIM RS transmission periodicity is 217×5ms=655.36s=10.92min.

- Example 2: when only RS-1 is configured, without enough/not enough indication, but with repetition, near/far detectability functionalities, where the number of consecutive TDD DL/UL switching periods is configured as R1=8, R2=0, we have

Pt=NSetIdRS1NF∙S1∙R1

To support a total of 22bit set ID, the obtained Pt=2224×8×8=220, which equals 43.68min considering a TDD DL/UL switching period of 2.5ms.

Step 2: Determine RIM RS resource indices

After determining the global RIM RS transmission periodicity, the RIM RS resources (in time-domain, frequency-domain, and sequence) should be uniquely indexed, according to the agreement in last meeting.

The RIM RS resources can be uniquely indexed via global resource tuple it,if,is, where, it is time-domain resource index, if is frequency-domain resource index, is is sequence index.

- it∈{0,1, …, Pt-1}, where Pt is the global RIM-RS periodicity in unit of TDD DL/UL switching period;

- if∈0,1,⋯,NF-1, where NF is the number of candidate frequency resources in the whole network;

- is∈0,1,⋯,NS-1, where NS is the number of RIM-RS sequences in one TDD DL/UL switching period configured for RS-1 or RS-2.

Proposal 3: The RIM RS resources can be uniquely indexed via global resource tuple it,if,is, where, it is time-domain resource index, if is frequency-domain resource index, is is sequence index.

- it∈{0,1, …, Pt-1}, where Pt is the global RIM-RS periodicity in unit of TDD DL/UL switching period;

- if∈0,1,⋯,NF-1, where NF is the number of candidate frequency resources in the whole network;

- is∈0,1,⋯,NS-1, where NS is the number of RIM-RS sequences in one TDD DL/UL switching period configured for RS-1 or RS-2.

Specifically, for RS-1,

- it∈{0,1, …, NT, 1R1-1}, where NT, 1 is the number of set IDs for RS-1 to be differentiated via time-domain determined in section 2.1; and R1 is the number of consecutive TDD DL/UL switching periods configured for repetition/near-far for one set ID;

- if∈0,1,⋯,NF-1, where NF is the number of candidate frequency resources;

- is∈0,1,⋯,S1-1, where S1 is the number of RIM-RS sequences configured for RS-1.

  - For RS-1 indicating “Not enough mitigation”, sequence index is∈0,1,⋯,S12-1

  - For RS-1 indicating “Enough mitigation”, sequence index is∈S12,S12+1,⋯,S1-1

For RS-2, we have

- it∈{NT, 1R1, NT, 1R1+1, …, Pt-1};

- if∈0,1,⋯,NF-1;

- is∈0,1,⋯,S2-1, where S2 is the number of RIM-RS sequences configured for RS-2.

In general, the RIM-RS resources pool for different functionalities (e.g., RS-1 for not enough indication, RS-1 for enough indication, and/or RS-2) can be described by the following parameters, denotes as parameter set for RIM-RS resources pool.

- Start time-domain index offset Toffset, where, Toffset∈0,1,⋯,Pt-1;

- Number of set IDs to be differentiated via time-domain NT, where Toffset+NT≤Pt;

- Number of consecutive TDD DL/UL switching periods R, where R∈1,2,⋯,8;

- Frequency-domain indexes range, with size nf;

- Sequence indexes range, with size ns and start sequence index offset Soffset.

It is clear that the parameter set for RIM-RS resources pool can be derived from the parameter set for RIM-RS resources configuration, as shown in Table 1.

Table 1. Parameter set for RIM-RS resource pools

| Parameters | Indication for “enough mitigation” or “not enough mitigation” = ON |  |  | Indication for “enough mitigation” or “not enough mitigation” = OFF |  |
| --- | --- | --- | --- | --- | --- |
| Functionality | RS-1 for not enough indication | RS-1 for enough indication | RS-2 | RS-1 | RS-2 |
| Start time-domain index offset Toffset | 0 | 0 | 2NSetIdRS1NF∙S1R1 | 0 | NSetIdRS1NF∙S1R1 |
| Number of set IDs to be differentiated via time-domain NT | 2NSetIdRS1NF∙S1 | 2NSetIdRS1NF∙S1 | NSetIdRS2NF∙S2 | NSetIdRS1NF∙S1 | NSetIdRS2NF∙S2 |
| Number of consecutive TDD DL/UL switching periods R | R1 | R1 | R2 | R1 | R2 |
| Frequency-domain indexes range, with size nf | 0,1,⋯,NF-1, nf=NF | 0,1,⋯,NF-1, nf=NF | 0,1,⋯,NF-1, nf=NF | 0,1,⋯,NF-1, nf=NF | 0,1,⋯,NF-1, nf=NF |
| Sequence indexes range, with size ns | 0,1,⋯,S12-1, ns=S12, Soffset=0 | S12,S12+1,⋯,S1-1, ns=S12, Soffset=S12 | 0,1,⋯,S2-1, ns=S2, Soffset=0 | 0,1,⋯,S1-1, ns=S1, Soffset=0 | 0,1,⋯,S2-1, ns=S2, Soffset=0 |

where, NT, 1R1+NT, 2R2=Pt.

Proposal 4: RIM-RS resources pool for different functionalities (e.g., RS-1 for not enough indication, RS-1 for enough indication, and/or RS-2) can be described by the following parameters, denotes as parameter set for RIM-RS resources pool, which can be derived from the parameter set for RIM-RS resources configuration.

- Start time-domain index offset Toffset, where, Toffset∈0,1,⋯,Pt-1;

- Number of set IDs to be differentiated via time-domain NT, where Toffset+NT≤Pt;

- Number of consecutive TDD DL/UL switching periods R, where R∈1,2,⋯,8;

- Frequency-domain indexes range, with size nf;

- Sequence indexes range, with size ns and start sequence index offset Soffset.

Step 3: Determine mapping rule between set ID and RIM RS resource indices

Regarding that there are several RIM-RS functionalities to be supported, each configured with dependent/independent set ID range and orthogonal resource pools, then it is preferred to determine common mapping rule between set ID and RIM RS resource indices based on parameter set for RIM-RS resources pool, which can be further derived from parameter set for RIM-RS resources configuration.

Proposal 5: Determine common mapping rule between set ID and RIM RS resource indices based on parameter set for RIM-RS resources pool, which can be further derived from parameter set for RIM-RS resources configuration.

First, it needs to be determined of the mapping order of set ID to RIM RS resources in the three dimensions, i.e., time, frequency and sequence. Nevertheless, frequency resources are preferred to be mapped at last step, since it provides the potential that with the expansion of 5G network size, new deployed gNB can be mapped to new refarmed 5G frequency bands, while keeping the mapping rule between legacy set ID and RIM RS resource indices consistency. Therefore, it is preferred that sequence first, time second and frequency third.

Proposal 6: Regarding the mapping order of set ID to RIM RS resources, it is preferred that sequence first, time second and frequency third.

Then, according to the contributions in last meeting, there are mainly two approaches raised to define the mapping rule between set ID and global RIM RS resource indices, which are:

- Alt 1: Modulo-based operation, where the set IDs are mapped to one or several global resource indexes according to the modulo value of set ID over time/frequency/sequence sizes.

- Alt 2: Bitmapping. The basic idea is to divide the set ID into three parts, where each part corresponds to one or several global resource indexes in one domain.

Alt 1: Modulo-based operation

Figure 2. Illustration of mapping set ID to global RIM RS resource indices.

An example is given as follows. Consider for a dedicated RIM RS functionalities (e.g., RS-1 for not enough indication, RS-1 for enough indication, and/or RS-2), a total number of NSetId set IDs is configured, then for a particular ID SetID, the corresponding global time/frequency/sequence resource tuple it,if,is used for transmitting the RS are determined as following:

is=modSetID, ns+Soffset

it=itR+Toffset,itR+1+Toffset,⋯,itR+R-1+Toffset, with it=modSetIDns, NT

if=modSetIDNTns, nf

Vice versa, we can obtain the set ID according to the RIM RS resource index as follows,

SetID=is-Soffset+nsit-ToffsetR+NTnsif

where the above parameters of Toffset, NT, R, nf, ns, Soffset are defined by parameter set for RIM-RS resources pool, which can be further derived from the parameter set for RIM-RS resources configuration, as shown in Table 1.

Alt 2: Bitmapping

For example, as indicated in the following figure, the set ID is divided into three parts, frequencytimesequence (depending on the mapping order), the two MSB of the set ID represents the frequency resource index, the two LSB of the set ID represents the sequence index, and the bits in the middle represents the time resource index, where one set ID may be mapped to one or several resources if repetition/near-far functionalities are supported.

Figure 3. Illustration of bitmapping of set ID to global RIM RS resource index.

Compared with Alt 2, Alt 1 is preferred, since it is more general and flexible. For example, it can support arbitrary number of repetitions, as well as sequence number.

Proposal 7: Modulo-based operation is supported, where the set IDs are mapped to one or several global resource indexes it,if,is according to the modulo value of set ID over time/frequency/sequence sizes, with the following formulas,

is=modSetID, ns+Soffset

it=itR+Toffset,itR+1+Toffset,⋯,itR+R-1+Toffset, with it=modSetIDns, NT

if=modSetIDNTns, nf

Vice versa,

SetID=is-Soffset+nsit-ToffsetR+NTnsif

where Toffset, NT, R, nf, ns, Soffset are defined by parameter set for RIM-RS resources pool, which can be further derived from the parameter set for RIM-RS resources configuration.

Discussion on other remaining issues

On unused TDD DL/UL pattern

It is agreed in last meeting that “for repetition/near-far-functionality, multiple basic RIM-RS resources for a gNB transmission can be configured only in consecutive TDD DL/UL switching period, with one basic RIM-RS resource per TDD DL/UL switching period”.

However, regarding the case that when dual TDD DL/UL patterns are configured, while in one TDD DL/UL pattern, very short UL is configured, then it may not be preferred to configure basic RIM-RS resource within such TDD DL/UL pattern.

So, it is suggested that one TDD DL/UL pattern can be configured not to transmit basic RIM-RS resource, if dual TDD DL/UL patterns are supported.

Proposal 8: One TDD DL/UL pattern can be configured not to transmit basic RIM-RS resource, if dual TDD DL/UL patterns are supported.

The following question is how to configure that, to keep consistence with last meeting’s agreements that “for repetition/near-far-functionality, multiple basic RIM-RS resources for a gNB transmission can be configured only in consecutive TDD DL/UL switching period, with one basic RIM-RS resource per TDD DL/UL switching period”.

There are two alternativities,

- Alt 1: TDD DL/UL switching period can be configured to contain one or two TDD DL/UL patterns, e.g.,

  - Case A: if only one TDD DL/UL pattern is configured, a TDD DL/UL switching period contains one TDD DL/UL pattern, with one RIM-RS resource configured in such TDD DL/UL pattern;

  - Case B: if dual TDD DL/UL patterns are configured, and both TDD DL/UL pattern has enough uplink OFDM symbols, then one TDD DL/UL switching period is configured to contain one TDD DL/UL pattern, with one RIM-RS resource configured in each TDD DL/UL pattern;

  - Case C: if dual TDD DL/UL patterns are supported, and only TDD DL/UL pattern has enough uplink OFDM symbols, then one TDD DL/UL switching period is configured to contain two TDD DL/UL patterns, with only one RIM-RS resource configured in the two TDD DL/UL patterns.

- Alt 2: Muting pattern is supported, where one TDD DL/UL switching period consists of one TDD DL/UL pattern. Associating with configuring the OFDM symbols offset of basic RIM-RS resource in each consecutive TDD DL/UL switching period, a muting status is supported (e.g., using OFDM symbols offset -1 represents muting status). When setting muting status, the corresponding TDD DL/UL pattern cannot be used to transmit basic RIM-RS resource.

Alt 1 is slightly preferred, since Alt 2 may halve the maximum repetition numbers if half TDD DL/UL patterns are muted.

Proposal 9: TDD DL/UL switching period can be configured to contain one or two TDD DL/UL patterns, e.g.,

- Case A: if only one TDD DL/UL pattern is configured, a TDD DL/UL switching period contains one TDD DL/UL pattern, with one RIM-RS resource configured in such TDD DL/UL pattern;

- Case B: if dual TDD DL/UL patterns are configured, and both TDD DL/UL pattern has enough uplink OFDM symbols, then one TDD DL/UL switching period is configured to contain one TDD DL/UL pattern, with one RIM-RS resource configured in each TDD DL/UL pattern;

- Case C: if dual TDD DL/UL patterns are supported, and only TDD DL/UL pattern has enough uplink OFDM symbols, then one TDD DL/UL switching period is configured to contain two TDD DL/UL patterns, with only one RIM-RS resource configured in the two TDD DL/UL patterns.

On signalling for RIM-RS configuration for repetition and near/far functionality indication

There is a remaining issue in last RAN1 meeting that:

- FFS: detailed signalling for RIM-RS configuration for repetition and near/far functionality indication,

  - FFS: How to support repetition by configuration, as well as if explicit indication of near/far functionality is needed

There are two alternativities to indicate near/far functionality,

- Alt 1: explicit indication via additional indication of whether differentiating near/far functionality. If set to ON, then the first R2 TDD DL/UL switching periods are used for repetition to enable near detectability, and the second R2 ones are used for repetition to enable far detectability, where R is number of consecutive TDD DL/UL switching periods for repetition/near-far;

- Alt 2: implicit indication.

Repetition and near/far functionality can be implicitly indicated by investigating that whether adjacent basic RIM-RS resources within consecutive R TDD DL/UL switching periods use same or different OFDM symbol offset, which is counted from a configured reference point within a TDD DL/UL switching period.

- Reference point = TDD DL/UL pattern boundary, with single TDD DL/UL pattern

- Reference point = TDD DL/UL pattern boundary, with dual TDD DL/UL patterns

- Reference point = DL transmission boundary, with single TDD DL/UL pattern

- Reference point = DL transmission boundary, with dual TDD DL/UL patterns

Figure 4. Illustration of reference point.

As shown in Figure 4 (a), when reference point is configured as the TDD DL/UL pattern boundary, then for single TDD DL/UL pattern case, it is easy to determine whether adjacent basic RIM-RS resources are used for repetition or near/far detectability, by investigating whether they use same or different OFDM symbol offset. However, for dual TDD DL/UL patterns case, it may be hard to make such decision since adjacent basic RIM-RS resources may use different OFDM symbol offsets even though they are repetition of each other.

On the contrary, when reference point is configured as the DL transmission boundary (i.e., the 1st reference point as captured in TR 38.866 [3]), no matter for single or dual TDD DL/UL patterns case, adjacent basic RIM-RS resources which are used for repetition always use same OFDM offsets, therefore it is easy to determine whether adjacent basic RIM-RS resources are used for repetition or near/far detectability, by investigating whether they use same or different OFDM symbol offset.

Based on above discussion, it is obvious that repetition and near/far functionality can be implicitly indicated by investigating that whether adjacent basic RIM-RS resources within consecutive R TDD DL/UL switching periods use same or different OFDM symbol offset, which is counted from DL transmission boundary of each TDD DL/UL switching period.

Observation 1: Repetition and near/far functionality can be explicitly indicated via additional indication of whether differentiating near/far functionality.

Observation 2: Repetition and near/far functionality can be implicitly indicated by investigating that whether adjacent basic RIM-RS resources within consecutive R TDD DL/UL switching periods use same or different OFDM offset, which is counted from DL transmission boundary of each TDD DL/UL switching period.

Proposal 10: Explicit or implicit indication of repetition and/or near/far functionality is to be decided.

On initialization seed for RIM-RS

In last meeting, it was determined that the initialization seed will be configured by the OAM and changing over time for security. However, the configuration details and how the seed should be changing has not been determined.

It is suggested that the initialization seed is determined by a counter of RIM-RS transmission periodicity, and a scramble ID (nSCID) selected from a scramble ID group, where the scramble ID group is configured by OAM, and the scramble ID is selected by gNB itself based on its set ID in sequence. i.e.,

Similar to CSI-RS,

cinit=2M∙nt∙2nSCID+1+nSCID mod 231

where,

- nt=modnperiodicity+δ,2Q, where, nperiodicity is a counter of RIM-RS transmission periodicity (Pt),

- nperiodicity counts the number of RIM-RS transmission periodicities since 00:00:00 on Gregorian calendar date 1 January, 1900 (midnight between Sunday, December 31, 1899 and Monday, January 1, 1900),

- δ is a time offset configured by OAM for scrambling nperiodicity , in order to increase the randomness of the counter to reduce the possibility of being detected/mimicked by an attacker.

- Q is the bit width of nt.

- nSCID is scramble ID, which is selected from a scramble ID group via global sequence index is, where the scramble ID group is configured by OAM,

- M is the bit width of nSCID.

Note that the value range of nperiodicity and δ are both 0.. 2Q-1, the value range of nSCID is 0.. 2M-1, and M+Q≤31.

Furthermore, scramble ID group can be periodically updated, e.g., in granularity of daily or weekly, by OAM implementation.

For instance, assume the maximum updated period of scramble ID group is one week, and the minimum RIM-RS transmission periodicity (Pt) is 217×5ms=10.92min (where, 22 bit set ID is supported, where 3 bits are conveyed via sequences, 2 bits are conveyed via frequency-domain, and remaining 22-3-2=17 bits are conveyed via time-domain, with 5ms TDD DL/UL switching period is assumed), then there are maximin 1 week10.92min=7×24×60×60×103ms217×5ms=922.85, so 0≤nt≤210, i.e., Q, the bit width of nt, can be set as 10. Hence, M, bit width of nSCID, can be set as 31-Q=21.

Table 2. Scramble ID group, which is configured and periodically updated by OAM

| is | nSCID |
| --- | --- |
| 0 | .. |
| 1 | .. |
| … | .. |
| NS-1 | .. |

Proposal 11: Initialization seed for RIM-RS is determined by a counter of RIM-RS transmission periodicity, and a scramble ID (nSCID) selected from a scramble ID group, where the scramble ID group is configured by OAM, and the scramble ID is selected by gNB itself based on its set ID in sequence, e.g.,

cinit=2M∙nt∙2nSCID+1+nSCID mod 231

where,

- nt=modnperiodicity+δ,2Q, where, nperiodicity is a counter of RIM-RS transmission periodicity (Pt),

- nperiodicity counts the number of RIM-RS transmission periodicities since 00:00:00 on Gregorian calendar date 1 January, 1900 (midnight between Sunday, December 31, 1899 and Monday, January 1, 1900),

- δ is a time offset configured by OAM for scrambling nperiodicity , in order to increase the randomness of the counter to reduce the possibility of being detected/mimicked by an attacker.

- Q is the bit width of nt.

- nSCID is scramble ID, which is selected from a scramble ID group via global sequence index is, where the scramble ID group is configured by OAM,

- M is the bit width of nSCID,

- The value range of nperiodicity and δ are both 0.. 2Q-1, the value range of nSCID is 0.. 2M-1, and M+Q≤31.

Conclusions

In this contribution, we share our views on RIM-RS configuration. The observations and proposals are summarised as follows:

Observation 1: Repetition and near/far functionality can be explicitly indicated via additional indication of whether differentiating near/far functionality.

Observation 2: Repetition and near/far functionality can be implicitly indicated by investigating that whether adjacent basic RIM-RS resources within consecutive R TDD DL/UL switching periods use same or different OFDM offset, which is counted from DL transmission boundary of each TDD DL/UL switching period.

Proposal 1: The global RIM RS transmission periodicity is determined based on both total required time-domain RIM RS-1 resources, and total required time-domain RIM RS-2 resources, if configured.

- Total required RIM RS-1 (RS-2) time-domain resource is determined based on the number of consecutive TDD DL/UL switching periods configured for each set ID and the number of set IDs for RS-1 (RS-2) to be differentiated via time-domain.

Proposal 2: The following parameters, denotes as parameter set for RIM-RS resources configuration, will be configured by OAM to determine the global RIM RS transmission periodicity, as well as the layout of RIM-RS resources for different functionalities,

- Number of candidate frequency resources in the whole network: NF

- Total number of set IDs for RS-1: NSetIdRS1

  - The number of RIM RS sequences (CDM) in one DL/UL switching point: S1

  - Number of consecutive TDD DL/UL switching periods for repetition/near-far: R1

  - Indication of whether differentiating “enough mitigation”/ “not enough mitigation”: ON/OFF

- Total number of set IDs for RS-2: NSetIdRS2, if needed

  - The number of RIM RS sequences (CDM) in one DL/UL switching points: S2

  - Number of consecutive TDD DL/UL switching periods for repetition/near-far: R2

Proposal 3: The RIM RS resources can be uniquely indexed via global resource tuple it,if,is, where, it is time-domain resource index, if is frequency-domain resource index, is is sequence index.

- it∈0,1, …, Pt-1}, where Pt is the global RIM-RS periodicity in unit of TDD DL/UL switching period;

- if∈0,1,⋯,NF-1, where NF is the number of candidate frequency resources in the whole network;

- is∈0,1,⋯,NS-1, where NS is the number of RIM-RS sequences in one TDD DL/UL switching period configured for RS-1 or RS-2.

Proposal 4: RIM-RS resources pool for different functionalities (e.g., RS-1 for not enough indication, RS-1 for enough indication, and/or RS-2) can be described by the following parameters, denotes as parameter set for RIM-RS resources pool, which can be derived from the parameter set for RIM-RS resources configuration.

- Start time-domain index offset Toffset, where, Toffset∈0,1,⋯,Pt-1;

- Number of set IDs to be differentiated via time-domain NT, where Toffset+NT≤Pt;

- Number of consecutive TDD DL/UL switching periods R, where R∈1,2,⋯,8;

- Frequency-domain indexes range, with size nf;

- Sequence indexes range, with size ns and start sequence index offset Soffset.

Proposal 5: Determine common mapping rule between set ID and RIM RS resource indices based on parameter set for RIM-RS resources pool, which can be further derived from parameter set for RIM-RS resources configuration.

Proposal 6: Regarding the mapping order of set ID to RIM RS resources, it is preferred that sequence first, time second and frequency third.

Proposal 7: Modulo-based operation is supported, where the set IDs are mapped to one or several global resource indexes it,if,is according to the modulo value of set ID over time/frequency/sequence sizes, with the following formulas,

is=modSetID, ns+Soffset

it=itR+Toffset,   itR+1+Toffset,⋯,itR+R-1+Toffset, with it=modSetIDns, NT

if=modSetIDNTns, nf

Vice versa,

SetID=is-Soffset+nsit-ToffsetR+NTnsif

where Toffset, NT, R, nf, ns, Soffset are defined by parameter set for RIM-RS resources pool, which can be further derived from the parameter set for RIM-RS resources configuration.

Proposal 8: One TDD DL/UL pattern can be configured not to transmit basic RIM-RS resource, if dual TDD DL/UL patterns are supported.

Proposal 9: TDD DL/UL switching period can be configured to contain one or two TDD DL/UL patterns, e.g.,

- Case A: if only one TDD DL/UL pattern is configured, a TDD DL/UL switching period contains one TDD DL/UL pattern, with one RIM-RS resource configured in such TDD DL/UL pattern;

- Case B: if dual TDD DL/UL patterns are configured, and both TDD DL/UL pattern has enough uplink OFDM symbols, then one TDD DL/UL switching period is configured to contain one TDD DL/UL pattern, with one RIM-RS resource configured in each TDD DL/UL pattern;

- Case C: if dual TDD DL/UL patterns are supported, and only TDD DL/UL pattern has enough uplink OFDM symbols, then one TDD DL/UL switching period is configured to contain two TDD DL/UL patterns, with only one RIM-RS resource configured in the two TDD DL/UL patterns.

Proposal 10: Explicit or implicit indication of repetition and/or near/far functionality is to be decided.

Proposal 11: Initialization seed for RIM-RS is determined by a counter of RIM-RS transmission periodicity, and a scramble ID (nSCID) selected from a scramble ID group, where the scramble ID group is configured by OAM, and the scramble ID is selected by gNB itself based on its set ID in sequence, e.g.,

cinit=2M∙nt∙2nSCID+1+nSCID mod 231

where,

- nt=modnperiodicity+δ,2Q, where, nperiodicity is a counter of RIM-RS transmission periodicity (Pt),

- nperiodicity counts the number of RIM-RS transmission periodicities since 00:00:00 on Gregorian calendar date 1 January, 1900 (midnight between Sunday, December 31, 1899 and Monday, January 1, 1900),

- δ is a time offset configured by OAM for scrambling nperiodicity , in order to increase the randomness of the counter to reduce the possibility of being detected/mimicked by an attacker.

- Q is the bit width of nt.

- nSCID is scramble ID, which is selected from a scramble ID group via global sequence index is, where the scramble ID group is configured by OAM,

- M is the bit width of nSCID.

- The value range of nperiodicity and δ are both 0.. 2Q-1, the value range of nSCID is 0.. 2M-1, and M+Q≤31.

References

- RP-181430, “New SI proposal: Study on remote interference management for NR”, RAN#80, La Jolla, June 11 - 15, 2018.

- RP-182864, “Revised WID on Cross Link Interference (CLI) handling and Remote Interference Management (RIM) for NR”, RAN#82, Sorrento, Italy, December 10-13, 2018.

- 3GPP TR 38.866, “Study on remote interference management for NR”.

- RAN1 Chairman’s Notes, 3GPP TSG RAN WG1 Adhoc Meeting 1901, Taipei, 21 - 25 Jan 2019.

Appendix: Example of proposal framework

A proposed framework is given in Figure A-1 for reference.

Figure A-1. Proposal framework.

Specifically, if time-domain repetition of RS-1 and/or near/far detection is enabled, multiple RIM-RS corresponding to multiple configurations will be simultaneously transmitted in term of “RS-1/RS-2 transmission”. Note that regular RS-1 refers to “RS-1 for not enough mitigation”.