3GPP TSG RAN WG1 Meeting #96	R1-1903087

Athens, Greece, 25th February – 1st March, 2019

Agenda Item:	7.2.7.2

Source:	Huawei, HiSilicon

Title:	Consideration on LTE evaluation features and parameters for IMT-2020 self evaluation on eMBB spectral efficiency

Document for:	Discussion and Decision

Introduction

According to 3GPP work plan on IMT-2020 submission, the final submission will be made in June 2019. More test environments or configurations for LTE can be evaluated for the final submission. In this contribution, consideration on LTE evaluation features and parameters for eMBB technical performance requirements is provided. In addition, the new features in Rel-16 can also be considered in the evaluation.

General feature consideration

As discussed in [1], the general features of eMBB evaluation in LTE are shown in Table 1.

Table 1 General technical features for eMBB in LTE

| General features | FR1 (below 6 GHz) |
| --- | --- |
| LTE on single-band | Un-paired spectrum: TDD Paired spectrum: FDD |
| NR/LTE inter-working on multi-band | NR/LTE dual connectivity with two frequency bands, with or without SUL |

To further progress the work such that one could set up the simulator for eMBB evaluation, the detailed technical features for LTE are proposed in Section 2.1. Specifically, the technical features that could contribute to eMBB spectral efficiency and user experienced data rate are considered.

Detailed technical parameters for system level simulation

The following technical features and parameters can be considered for eMBB evaluation in LTE. Other features and parameters that are supported by Rel-15 and Rel-16 are not precluded.

DL FDD

The features for DL FDD evaluation are provided in Table 2.

Table 2 LTE parameters for DL FDD evaluation

| DL FDD – FR1 |  |  | LTE |
| --- | --- | --- | --- |
| Waveform |  |  | OFDM based |
| Multiple access |  |  | OFDMA |
| Modulation |  |  | Up to 256QAM |
| Coding on PDSCH |  |  | Turbo |
| Numerology | TTI length |  | 14 OFDM symbols |
|  | SCS |  | 15kHz |
| Number of RBs |  |  | 50 |
| Simulation bandwidth |  |  | 10 MHz |
| Frame structure |  |  | All downlink for all TTIs |
| MIMO |  | Transmission mode | Closed loop SU/MU-MIMO adaptation |
|  |  | Codebook | LTE Advanced-CSI codebook with class A CSI-RS. Detailed configuration to be reported. |
|  |  | PRB bundling | 3 PRB (It impacts DMRS channel estimation, but does not limit the scheduling unit.) |
|  |  | MU dimension | Up to 4 layer (up to 4 orthogonal DMRS ports) Or more than 4 layers by taking into account quasi-orthogonal DMRS port; channel estimation error modeled |
|  |  | SU dimension | Rank adaptation (depending on UE antenna elements) |
|  |  | Codeword (CW)-to-layer mapping | For larger than or equal to 2 layers, two CWs. |
|  |  | DMRS transmission for overhead | LTE DMRS pattern |
|  |  | CSI-RS transmission | Non-precoded CSI-RS |
|  |  | CSI feedback | PMI, CQI: every [5] TTI; RI: every [5] TTI, CRI: every [5] slot; Subband based |
|  |  | Interference measurement | ZP CSI-RS for interference measurement can be considered. |
| HARQ |  | ACK/NACK delay | N+4 |
| Overhead |  |  | PSS, SSS, PBCH CSI-RS, CSI-IM (if used) DMRS CRS MBSFN number PDCCH: 2 complete symbols (same value as that in eMBB evaluation in NR) Companies report the detailed overhead. |

DL TDD

The features for DL TDD evaluation are provided in Table 3. It is noted that the parameters sharing the same values as that in Table 2 (DL FDD table) is not shown.

Table 3 LTE parameters for DL TDD evaluation

| DL TDD – FR1 |  | LTE |
| --- | --- | --- |
| Number of RBs |  | 100 |
| Simulation bandwidth |  | 20 MHz |
| Frame structure |  | Companies report DL/UL configuration and slot format |
| MIMO | Transmission mode | Closed loop SU/MU-MIMO adaptation |
|  | PRB bundling | 2 PRB (It impacts DMRS channel estimation, but does not limit the scheduling unit.) |
|  | Downlink CSI measurement | - Option A (Non-PMI feedback): BS transmits precoded CSI-RS using N “eigen-vector”, UE measures on N CSI-RS port(s) to derive equivalent channel, and then derive rank L (L≤N) and corresponding CQI - Option B: BS transmits non-precoded CSI-RS and UE measures on CSI-RS port(s) to derive channel, and use type I/II codebook to derive precoder, and use the precoder to derive rank and CQI. - NOTE: CSI can be further determined at the BS side. |
|  | SRS transmission | - Non-precoded SRS - For normal UL subframe, one TTI is configured to have 1 OFDM symbol for SRS transmission. - For NTNR UE (N=1/2/4), one SRS resource for N port SRS. - For 1T4R/2T4R UE (SRS switch) - 1T4R: 1 SRS resource with 4-port SRS; different SRS ports occupy different TTIs for normal UL subframe; - 2T4R: 1 SRS resource with 4-port SRS; two groups of SRS ports, with each group having 2 SRS ports; different SRS group occupy different TTIs for normal UL subframe; - For 1T2R UE (SRS switch) - 1 SRS resource with 2-port SRS; different SRS ports occupy different TTIs for normal UL subframe SRS estimation error model shown in R1-145389 can be used as starting point. |
| HARQ | ACK/NACK delay | At least N+4 depending on the frame structure. |
| Overhead |  | PSS, SSS, PBCH CSI-RS, CSI-IM (if used) DMRS CRS MBSFN number PDCCH: 2 complete symbols (same value as than in eMBB evaluation in NR) GP overhead: 50% length of GP symbols is calculated in DL (same value as that in eMBB evaluation in NR) Companies report the detailed overhead. |

UL FDD

The features for UL FDD evaluation are provided in Table 4.

Table 4 LTE parameters for UL FDD evaluation

| UL FDD – FR1 |  | LTE |
| --- | --- | --- |
| Waveform |  | DFT-S-OFDM based |
| Multiple access |  | DFT-S-OFDMA Companies report power back-off model if used. |
| Modulation |  | Up to 256QAM |
| Coding on PDSCH |  | Turbo |
| Numerology | TTI length | 14 OFDM symbols |
|  | SCS | 15kHz |
| Number of RBs |  | 50 |
| Simulation bandwidth |  | 10 MHz |
| Frame structure |  | All uplink for all TTIs |
| MIMO | Transmission scheme | UL codebook based SU-MIMO |
|  | UL Codebook | - LTE 2Tx/4Tx UL codebook |
|  | MU dimension | - |
|  | SU dimension | Rank adaptation up to 4 layers depending on UE antenna number. |
|  | Codeword (CW)-to-layer mapping | For larger than or equal to 2 layers, two CWs. |
|  | DMRS transmission for overhead | LTE UL DMRS pattern (2 OFDM symbols per TTI) |
|  | SRS transmission | Non-precoded Up to one symbol in one subframe |
| HARQ | ACK/NACK delay | Companies report |
| Power control schemes |  | Companies to report |
| Overhead |  | DMRS and SRS PUCCH DL feedback Companies report the detailed overhead. |

UL TDD

The features for UL TDD evaluation are provided in as shown in Table 5. It is noted that the parameters that share the same values as that in Table 4 (UL FDD table) is not shown.

Table 5 LTE parameters for UL TDD evaluation

| UL TDD – FR1 |  | LTE |
| --- | --- | --- |
| RB number |  | 100 |
| Simulation bandwidth |  | 20 MHz |
| Frame structure |  | Companies report DL/UL configuration and slot format |
| MIMO | Transmission scheme | UL codebook based SU-MIMO |
|  | SRS transmission | - Non-precoded SRS - For normal UL subframe, one TTI is configured to have 1 OFDM symbol for SRS transmission; for special subframe, one TTI is configured to have up to 6 OFDM symbols for SRS transmission. |
| Overhead |  | DMRS and SRS PUCCH DL feedback GP overhead: 50% length of GP symbols is calculated in UL (same value as that in eMBB evaluation in NR) Companies report the detailed overhead. |

Detailed configuration parameters

The following configuration parameters in Table 6 are to be reported or used by companies.

Table 6 Other configuration parameters to be reported or used by companies

| Parameter | Value |
| --- | --- |
| Antenna configuration at BS | are the number of vertical, horizontal antenna elements within a panel, are the number of vertical, horizontal TXRUs within a panel and polarization, and is number of polarizations, is the number of panels in a column, is the number of panels in row. |
| Antenna configuration at UE |  |
| Scheduling | PF (MU-PF for MU-MIMO or SU-PF for SU-MIMO) |
| Receiver | MMSE-IRC Advance receiver is not precluded. |
| Down-tilt | Mechanical tilt (calibration parameters can be used as starting point) Electrical tilt |
| Uplink power control parameters | , P0 |
| Interference over thermal | Companies report (use the “effective IoT” model as in Section A.2.1.8 in TR36.814) |

Conclusion

In this contribution, the evaluation features and parameters for eMBB spectral efficiency evaluation in LTE are proposed. It is proposed that,

Proposal: Use the technical features and parameters as listed through Table 2 to Table 6 in Section 2 for self evaluation of LTE for eMBB technical performance requirements.

- Other features and parameters that are supported by Rel-15 and Rel-16 are not precluded.

- References

- R1-1801789, “General consideration on IMT-2020 self evaluation”, Huawei, HiSilicon, Feb 2018.