3GPP TSG-RAN WG1 Meeting #96	R1-1903182

Athens, Greece February 25th – 1st March 2019

Agenda Item:	7.2.4.6

Source:	Ericsson

Title:	Text Proposal for TR Conclusion

Document for:	Discussion, Decision

# 1	Introduction

In this paper, we present a text proposal for inclusion in the conclusions section of TR 38.885 corresponding to the SID on NR V2X [1].

# 2	Text proposal

Agree on the following TP

----- Start of the TP ----- 12 Conclusions With respect to NR sidelink, the RAN WGs conclude that: - To support advanced V2X use cases, it is feasible to specify an NR PC5 interface including sidelink unicast, groupcast, and broadcast communications. For sidelink unicast and groupcast, the RAN WGs conclude that it is beneficial to specify sidelink feedback procedures. - Coexistence of sidelink unicast, groupcast, and broadcast in a carrier is feasible as well as the coexistence between sidelink and other cellular transmissions. - NR UE autonomous resource allocation for sidelink (Mode 2) is feasible using procedures based on sensing, resource selection, and transmission with reservations. - NR Uu controlled resource allocation for sidelink (Mode 1) is feasible. The RAN WGs note that further study is required regarding the applicability in FR2 of the technical solutions for sidelink described in this TR. With respect to the NR Uu interface, the RAN WGs conclude that it is feasible for the NR Uu interface to support advanced V2X use cases. The RAN WGs have identified solutions to improve the latency, capacity, and reliability performance of the NR Uu interface. The RAN WGs recommend supporting the following solutions as identified in this TR: - Multiple simultaneously active uplink configured grants. - UE reports containing assistance information. ----- End of the TP -----

# 3	Conclusions

In this paper, we propose to agree on the TP given in Section 2.

# 4	References

RP-181480, “New SID: Study on NR V2X”, Vodafone, RAN Meeting #80, June 11th -14th, 2018.