3GPP TSG RAN WG1 #96		R1-1903604
Athens, Greece, 25th February – 1st March, 2019

Agenda item:	7.2.13.3

Source:	Apple Inc.

Title:	Summary of enhancements to single Tx switched uplink for EN-DC

Document for:	Decision

# Introduction

In this document, we summarize the open issues and proposals as discussed from the related contributions ([1]-[9]) to RAN1 #96 under agenda item 7.2.13.3.

As reference, below is the corresponding text from the endorsed WID [10] in RAN #81:

- Study and, if found beneficial over the existing single Tx switched uplink solution, specify enhancements to single Tx switched uplink solution for EN-DC, such as allowing all DL and UL subframes for data transmission for both NR and LTE. [RAN1]. - Note: UE in single Tx switched uplink mode is not expected to be scheduled simultaneous transmission on MCG and SCG

Following agreement was made in RAN1 AdHoc #1901:

Agreements: - It is concluded that it is beneficial to specify enhancements to single Tx switched uplink solution for EN-DC over the existing single Tx switched uplink solution, e.g. via introducing a new reference HARQ configuration using NR/LTE Rel-15 or LTE eIMTA solution, etc.

# LTE downlink enhancement for single Tx in EN-DC with LTE TDD PCell

The motivation for this enhancement is to fully utilize all available downlink subframes in the LTE TDD PCell while only subset of uplink subframes is available to carry the ACK/NACK bits for LTE PDSCH, due to TDM with NR UL slots. Open issues including:

- How to support LTE PDSCH transmissions in all available downlink subframes with reduced ACK/NACK feedback opportunities?

- For the reference config, should the LTE Pcell’s PDSCH-to-ACK timing reuse that of eIMTA, or reuse that of EN-DC SUO case 1 with LTE FDD-PCell?

- For the reference config, should we limit to a subset of the existing LTE TDD patterns only, e.g., pattern 2, 4, 5?

- What is the corresponding LTE PUSCH procedure with reduced uplink subframes?

Below is summary of related observations/proposals from companies:

| Company | Proposals |
| --- | --- |
| Huawei/HiSi | Observation 1: It is feasible to use case 1 HARQ timing mechanism specified in Rel-15 for intra-band EN-DC to achieve single UL transmission Observation 2: Rel-12 eIMTA framework is less flexible than the case 1 HARQ timing mechanism specified in Rel-15 in achieving single UL transmission for intra-band EN-DC. Proposal 2: Support single UL operation for intra-band EN-DC in TDD band |
| ZTE | Proposal 1: Separate UE capability of eIMTA semi-static configuration and L1 signaling to support single Tx switched uplink for EN-DC with LTE-TDD-PCell and keep them optional in Rel-16 Proposal 2: Reuse the existing eIMTA signaling as much as possible to reduce the standardization workload |
| Samsung | Proposal 1: DL-reference UL/DL configuration is supported for EN-DC between LTE TDD PCell and NR SCell |
| Nokia, NSB | Proposal 1: If DL reference HARQ timing is introduced for TDD EN-DC, only UL/DL configurations 2, 4 and 5 can be used as a reference HARQ configuration of the PCell, and the PUCCH fallback operation is not supported. |
| Intel | Proposal 1: - For single tx switched uplink mode, DL-reference UL/DL configuration is configured for TDD Pcell to semi-statically split the UL resource between LTE and NR in TDM manner. - For DL HARQ timing, the DL-reference UL/DL configuration is applied for LTE CCs. - PUCCH is limited to the UL subframes indicated by DL-reference UL/DL configuration. |
| Apple | Proposal 1: For single UL operation in EN-DC with LTE TDD PCell, where UE operates on only one carrier at a time - for LTE carrier, UE can be configured with (“case 1”) DL-reference UL/DL configuration indicated by RRC signaling “tdm-PatternConfig-r15” - For DL HARQ timing, the DL-reference UL/DL configuration, i.e. associated with “tdm-PatternConfig-r15”, is applied. RAN1 to further down-select among: - Option 1: use the PDSCH ACK timeline for LTE FDD-TDD CA with LTE TDD PCell (i.e. Table 10.1.3A-1 in 36.213) - Option 2: use the PDSCH ACK timeline for eIMTA (i.e., Table 10.1.3.1-1 in 36.213) - For UL HARQ timing, the PCell’s UL/DL configuration, i.e. associated “subframeAssignment”, is applied - UE is not expected to transmit on the MCG and SCG simultaneously |
| DCM | Proposal 2: Support DL-reference UL-DL configuration for EN-DC with TDD-PCell. |
| Qualcomm | Proposal 1: For a UE configured with EN-DC and capable of dynamic power sharing, under the case-1 single-Tx TDM solution, LTE uplink HARQ is the same as without EN-DC for both FDD and TDD LTE Pcell and all uplink subframes can be used for LTE uplink data transmission. Proposal 2: Due to the severe negative impact on UL link budget, don’t introduce case-1 singe-Tx TDM solution in TDD LTE unless data transmission on all UL subframes is enabled Proposal 3: If the need for enabling the case-1 single-Tx TDM solution for EN-DC with LTE-TDD Pcell is justified, the LTE downlink HARQ timing of the Pcell is based on the timing of the DL/UL configuration given by the RRC signaling. Similar to eIMTA, only DL/UL configurations 2, 4 and 5 can be used as a reference HARQ configuration of the Pcell. Further, the uplink HARQ procedure is similar to the case without EN-DC |

Among the 8 contributions:

- 5 companies explicitly support configuring DL-reference DL/UL configuration to utilize all available LTE DL subframes, and no company object.

- 1 company mentions that case-1 single-Tx TDM solution in TDD LTE is not supported unless data transmission on all UL subframes is enabled.

- 2 companies propose (Nokia, Qualcomm) that if DL reference HARQ timing is introduced, only allow LTE pattern 2, 4, 5 can be used as a reference HARQ configuration of the PCell.

Potential agreement:

For single tx switched UL in EN-DC with TDD PCell, the LTE PCell can be configured with DL-reference UL/DL config

- For DL HARQ timing, the DL-reference UL/DL configuration is applied

  - Only LTE TDD Pattern 2, 4, 5 can be used as DL-reference

- For UL HARQ timing, the PCell’s UL/DL configuration, i.e., associated with “subframeAssignment”, is applied

  - FFS: whether/how to support TDD pattern 0/6 for LTE PCell

- UE is not expected to transmit on the MCG and SCG simultaneously

- For type 1 UE (i.e., UE with dynamic power sharing capability),

  - The UE should not assume that LTE PUSCH is only scheduled in the UL subframes associated configured by the DL-reference configuration

  - The UE should not assume that NR PUSCH is only scheduled in the remaining UL subframes other than those configured by the DL-reference configuration

  - If there is a collision, UE is expected to drop NR PUSCH

- FFS: whether/how to support HARQ-offset (similar as in SUO case 1 in EN-DC with LTE FDD PCell)

# LTE uplink enhancements for single Tx in EN-DC

The main open questions are

- What are the assumptions (i.e. in what scenarios) to support opportunistically utilizing all available UL subframes/slots for PUSCH?

- If supported, what is the associated specification impact?

- For R16, should we support this uplink enhancements for EN-DC with LTE TDD PCell, or EN-DC with LTE FDD PCell, or both?

| Company | Proposals |
| --- | --- |
| ZTE | Observation 4: Allowing all UL subframes for LTE transmission requires tight coordination between LTE and NR. Without tight coordination, it can likely result in performance loss due to the uplink collision between LTE and NR - Observation 5: No need to further enhance Rel-15 SUO mechanism for EN-DC with LTE-FDD-PCell. Observation 6: Reusing eIMTA mechanism can allow all UL subframes for SUO UEs for EN-DC with LTE-TDD-PCell |
| Nokia/NSB | Observation 3: Utilizing all UL subframes by LTE or NR is too restrictive for eNB and gNB implementation. |
| Apple | Proposal 2: For single UL operation in EN-DC with LTE TDD PCell, where UE operates on only one carrier at a time, and UE is configured with DL-reference UL/DL configuration, - When there is tight coupling between gNB and eNB schedulers, for UE capable of dynamic power sharing, LTE UL transmission can be scheduled in all UL subframes and UpPTS configured by “subframeAssignment” - UE is not expected to transmit on the MCG and SCG simultaneously |
| DCM | Proposal 3: Consider enabling UL transmission other than offset-UL subframes when available |
| Ericsson | Proposal 2 - For Case 1 HARQ timing via UE-specific reference TDD configuration for FDD Pcell or for TDD Pcell (if supported), if PUSCH transmissions are allowed outside the subframes indicated as ‘UL’ by the reference TDD configuration, the following default UE behavior should be supported - if there are colliding LTE and NR UL transmissions in the same subframe, the UE is expected to transmit LTE and drop NR. |
| Qualcomm | Proposal 1: For a UE configured with EN-DC and capable of dynamic power sharing, under the case-1 single-Tx TDM solution, LTE uplink HARQ is the same as without EN-DC for both FDD and TDD LTE Pcell and all uplink subframes can be used for LTE uplink data transmission. Proposal 2: Due to the severe negative impact on UL link budget, don’t introduce case-1 singe-Tx TDM solution in TDD LTE unless data transmission on all UL subframes is enabled |

In case the proposal in section 2 is agreed, may further discuss whether the uplink enhancement part can be extended to EN-DC with LTE FDD-PCell.

# PUCCH formats for LTE TDD-PCell in EN-DC

Main open questions: is LTE PUCCH fallback operation supported, or only support format 3/4/5?

| Company | Proposals |
| --- | --- |
| Samsung | Proposal 2: Only PUCCH format 3/4/5 is supported for EN-DC between LTE TDD PCell and NR SCell. |
| Nokia/NSB | Proposal 1: If DL reference HARQ timing is introduced for TDD EN-DC, only UL/DL configurations 2, 4 and 5 can be used as a reference HARQ configuration of the PCell, and the PUCCH fallback operation is not supported. |
| Intel | Proposal 2 Reuse the PUCCH format reporting schemes defined for LTE FDD PCell Case 1 |
| Apple | Proposal 3: For single UL operation in EN-DC with LTE TDD PCell, where UE operates on only one carrier at a time, and UE is configured with DL-reference UL/DL configuration, - PUCCH fallback operation is not supported. |

Proposal:  If DL reference HARQ timing is introduced for TDD EN-DC, only LTE PUCCH format 3/4/5 can be used and the PUCCH fallback operation is not supported.

# Support of LTE UL CA for single Tx in EN-DC

Main open questions: in EN-DC SUO case (UL TDM between MCG and SCG), should spec support LTE UL CA? If so, should LTE SCell follow the same reference config as the LTE PCell? what is the HARQ timing for the LTE SCell?

| Company | Proposals |
| --- | --- |
| Nokia/NSB | Proposal 2: If Case 1 HARQ timing is configured, the UL subframe usage in a UL CA carrier is determinbed by whether its CA band is problematic band in EN-DC band combination. - For intra-band UL CA or inter-band CA with problematic CA band, the uplink subframes in Scell are aligned with the uplink subframe allocation in PCell For inter-band CA with non-problematic CA band, all UL subframe in the carrier of that band can be scheduled |
| DCM | Proposal 1: - Support LTE UL-CA for EN-DC with DL-reference UL-DL configuration on LTE serving cell. - The subframes available for UL transmission can be common across LTE serving cells |

Need to further discuss and more inputs on whether/how to support LTE UL CA in single Tx operation in EN-DC

# LTE PRACH resource configuration for single Tx UE in EN-DC

Open issue: should PRACH resource configuration be different between single Tx UE and other UE?

| Company | Proposals |
| --- | --- |
| Ericsson | Proposal 1 Following enhancement for PRACH should be supported - If a UE is configured with Case1 HARQ timing, UE’s PRACH transmissions are not limited to the UL subframes given by subframe-Assignment-r15 (i.e., for FDD, UE can transmit PRACH in any UL subframe). - If the UE’s LTE PRACH transmission collides with an NR UL transmission, PRACH transmission is prioritized. |

Need further discussion and more inputs.

# Allow NR SRS in LTE UL subframe

Questions:

- In SUO, should it be allowed NR SRS be sent on the last symbol of LTE slot (allocated by reference config). If so, is it always feasible for all UEs or capability signalling is needed?

| Company | Proposals |
| --- | --- |
| Huawei | Proposal 1: Allow EN-DC UE to transmit NR SRS overlapping in time with the last symbol of a LTE uplink subframe designated by subframeAssignment-r15 - The last LTE symbol thereof is reserved and not transmitted |

Need further discussion and more input

# Appendix

RAN1#90:

Agreements: - When the UE is configured with multiple UL carriers on different frequencies (where there is at least one LTE carrier and at least one NR carrier of a different carrier frequency), but the UE operates on only one of the carriers at a given time among a pair of LTE and NR carriers - For LTE carrier, UE can be configured with - Case 1: DL-reference UL/DL configuration defined for LTE-FDD-SCell in LTE-TDD-FDD CA with LTE-TDD-PCell - For scheduling/HARQ timing of LTE FDD carrier, DL-reference UL/DL configuration defined for LTE-FDD-SCell in LTE-TDD-FDD CA with LTE-TDD-PCell is applied - UE is allowed to transmit NR UL signals at least in the subframe(s) where LTE UL transmission is not allowed according to the DL-reference UL/DL configuration - FFS whether or not a UE-specific subframe offset for the DL-reference UL/DL configuration can be configured considering system resource utilization and potential spec impact - Case 2: Release 15 LTE-FDD HARQ timing - No impact on LTE RAN1 specifications - Note: it doesn’t necessarily imply that UE has to support both cases

In RAN4 #86bis: it was agreed to allow Single Tx Switched UL operation for intra-band EN-DC in band 41.

- Single Switched UL is allowed - Multiple Options for TDM – RAN4 may have specific requirements for these options. - Option 1: UL TDM pattern as per R4-1805323 - Option 2: UL TDM based on LTE eIMTA framework, to reserve subset of the available UL subframes for LTE. - Option 3: Dynamic UL TDM - Other options are not precluded. - As a per-BC UE capability, UE may signal support for SUO for a given EN-DC band combination. If dual transmission is scheduled instead of single switched uplink, UE behavior is unspecified. - No additional signaling required

# References

- R1-1901552, “Enhancements for single UL operation for EN-DC”, Huawei, HiSilicon, 3GPP RAN1 #96, Athens, Greece, February 25 – March 01, 2019.

- R1-1901778, “Discussion on single Tx switched uplink solution for EN-DC”, ZTE, 3GPP RAN1 #96, Athens, Greece, February 25 – March 01, 2019.

- R1-1902252, “On enhancements to single Tx switched uplink for EN-DC”, Samsung, 3GPP RAN1 #96, Athens, Greece, February 25 – March 01, 2019.

- R1-1902428, “Discussion on single uplink operation enhancement for EN-DC”, Nokia, Nokia Shanghai Bell, 3GPP RAN1 #96, Athens, Greece, February 25 – March 01, 2019.

- R1-1902519, “On enhanced single Tx switched uplink solution for EN-DC”, Intel, 3GPP RAN1 #96, Athens, Greece, February 25 – March 01, 2019.

- R1-1902774, “Enhancements to single Tx switched uplink for EN-DC”, Apple, 3GPP RAN1 #96, Athens, Greece, February 25 – March 01, 2019.

- R1-1902820, “Potential enhancements to single Tx switched uplink solution for EN-DC”, NTT Docomo, 3GPP RAN1 #96, Athens, Greece, February 25 – March 01, 2019

- R1-1902945, “Enhancements to single-tx operation for EN-DC”, Ericsson, 3GPP RAN1 #96, Athens, Greece, February 25 – March 01, 2019.

- R1-1903027, “Enhancements for EN-DC single-tx TDM operation”, Qualcomm, 3GPP RAN1 #96, Athens, Greece, February 25 – March 01, 2019.

- PR-182076, “WID on Multi-RAT Dual-Connectivity and Carrier Aggregation enhancements (LTE_NR_DC_CA_enh-Core)”, RAN #81, Gold Coast, Australia, 10th – 13th September, 2018.

- “Chairman Notes, RAN1 Ad-Hoc #1901”, 3GPP RAN1 Ad-Hoc #1901, Taipei, Taiwan, 21st – 25th January 2019.

- R1-1900697, “EN-DC SUO enhancements”, Nokia, Nokia Shanghai Bell, RAN1 Ad-Hoc #1901, Taipei, Taiwan, January 21st – 25th, 2019.