3GPP TSG RAN WG1 Meeting #96 R1-1903635 Athens, Greece, 25th February – 1st March, 2019 CR-Form-v11.2. CHANGE REQUEST. 38.214; CR; DRAFT; rev; -; Current version:; 15.4.0. For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests.. Proposed change affects:; UICC apps; ME; X; Radio Access Network; X; Core Network. Title:; Draft CR on AP-CSI-RS with a large latency of beam switching. Source to WG:; ZTE. Source to TSG:. Work item code:; NR_newRAT-Core; Date:; 2019-2-15. Category:; F; Release:; Rel-15. Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900.; Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16). Reason for change:; In TS38.214, the UE behaviour for ap-CSI-RS is still open, when the reported UE capability in FG2-28 is {224,336}. Only the cases of {14,28,48} are specified. Different from the cases of {14,28,48}, the candidate values of {224, 336} are related to panel activation for ap-CSI-RS measurement, rather than normal beam switching. Consequently, if one of the values of {224, 336} is reported as UE capability for ap-CSI-RS, the current UE behaviour is unknown. As a straightforward solution, the maximum value of the lowest three values in FG2-28, i.e., 48, should be applied as the threshold for beam switching for ap-CSI-RS.. Summary of change:; Adding the sentences to support all the cases including the cases of {224, 336} reported as UE capability for ap-CSI-RS in FG2-28 and the default QCL assumption for AP-CSI-RS.. Consequences if not approved:; Unclear description of default beam assumption for ap-CSI-RS if one value of {224, 336} is reported as UE capability for ap-CSI-RS in FG2-28. Clauses affected:; 5.2.1.5.1. Y; N. Other specs; X; Other core specifications; TS/TR ... CR .... affected:; X; Test specifications; TS/TR ... CR .... (show related CRs); X; O&M Specifications; TS/TR ... CR .... Other comments:; Isolated impact analysis: This CR fixes the issue of the default QCL assumption for AP-CSI-RS when one of values {224,336} is reported through UE capability signaling beamSwitchTiming, and aligns the specification to RAN1 agreements. It is expected UEs and networks are implemented in accordance to this CR and therefore no change is required on UE and network implementation.

##### 5.2.1.5.1	Aperiodic CSI Reporting/Aperiodic CSI-RS

<Unchanged part omitted>

- If the scheduling offset between the last symbol of the PDCCH carrying the triggering DCI and the first symbol of the aperiodic CSI-RS resources in a NZP-CSI-RS-ResourceSet configured without higher layer parameter trs-Info and without the higher layer parameter repetition is smaller than the minimum value between 48 and the UE reported threshold beamSwitchTiming, as defined in [13, TS 38.306].

- if there is any other DL signal with an indicated TCI state in the same symbols as the CSI-RS, the UE applies the QCL assumption of the other DL signal also when receiving the aperiodic CSI-RS. The other DL signal refers to PDSCH scheduled with offset larger than or equal to the threshold timeDurationForQCL, as defined in [13, TS 38.306], aperiodic CSI-RS scheduled with offset larger than or equal to the minimum value between 48 and the UE reported threshold beamSwitchTiming, periodic CSI-RS, semi-persistent CSI-RS.

- If the scheduling offset between the last symbol of the PDCCH carrying the triggering DCI and the first symbol of the aperiodic CSI-RS resources is equal to or greater than the minimum value between 48 and the UE reported threshold beamSwitchTiming, the UE is expected to apply the QCL assumptions in the indicated TCI states for the aperiodic CSI-RS resources in the CSI triggering state indicated by the CSI trigger field in DCI.

<Unchanged part omitted>