3GPP TSG-RAN WG1 #96	R1-1903702

Athens, Greece, February 25st – March 1st 2019

Agenda Item:	7.2.10.2

Source:	Ericsson, ESA, Qualcomm, Nokia

Title:	TP for TR 38.855, Section 8.x on RAT independent positioning evaluations

Document for:	Discussion, Decision

# 1	Text Proposal

From various offline discussions, we propose the following text proposal. It contains the scenario and simulation results provided by one company simulating GNSS+RAT dependent positioning. The GNSS models and scenarios has not been agreed in 3GPP, so therefore, a comment is provided to emphasize that.

[…]

## 8.x GNSS and Hybrid 5G-GNSS Evaluations

### 8.x.1	Hybrid GNSS-DL-TDOA evaluation (ESA) [28]

One company provided simulation results [28] for hybrid NR and GNSS positioning. No GNSS models or scenarios have been agreed in 3GPP for positioning evaluations. This section captures these results, which are based on the agreed RAT dependent simulation scenarios together with considered GNSS models and scenarios.

#### 8.x.1.1	Scenarios and Methodology

For evaluating GNSS and hybrid positioning GNSS and DL-TDOA performance, only outdoor scenarios are selected, namely UMa and UMi. In [28] the GNSS conditions (signal attenuation, multipath, etc.) were modelled accordingly ETSI TS 103 246-3 Operational Environments [28]: Urban Canyon and Asymmetric Visibility. Furthermore, in order to evaluate GNSS and RAT-dependent under same environmental conditions, in [28] the Operational Environments were aligned to 3GPP UMa and UMi based on elevation mask profiles for the GNSS satellites.

Code-based measurements have been generated for all four GNSS constellations (GPS, Galileo, BeiDou, and GLONASS) on two frequency carriers, namely 1.575 GHz and 1.176 GHz. The DL-TDOA observables are simulated according to the scenario parameters agreed by RAN1 and listed section 6.1. DL-TDOA observables are obtained from 6 gNBs with a PRS system bandwidth of 100 MHz and ideal muting pattern. The channel model as provided in the TR 38.901 is used as baseline while results with modifications to the channel model adding a NLOS bias are also presented.

Table 8.x.1.1-1: Hybrid GNSS-DL-TDOA Evaluation assumptions

| Parameter | GNSS | DL-TDOA |
| --- | --- | --- |
| Layout | A GNSS constellation simulator has been used to generate the geometry of each constellation for a specific latitude, longitude and date. (NOTE 1) | For UMa: Hexagonal grid, 19 macro sites, 3 sectors per site ISD = 500m. For UMi: Hexagonal grid, 19 micro sites, 3 sectors per site ISD = 200m |
| System Bandwidth per Carrier | L1/E1: 24.552 MHz L5/E5a: 20.460 MHz | 100 MHz |
| GNSS constellations | Galileo, GPS, GLONASS, BeiDou | N.A |
| Carrier Frequency | L1: 1.575 GHz L5: 1.176 GHz | 4 GHz |
| Number of Carriers | 2 | 1 |
| Subcarrier spacing | N.A | 30 kHz |
| HDOP range | 1.6 to 2.5 | 0.6 to 2.0 |
| Penetration | From [32]: For outdoor UEs in UMa: X1=0 dB, X2=100 dB, X3=15 dB For outdoor UEs in UMi: X1=0 dB, X2=100 dB | For outdoor UEs: 0dB |
| Satellite / BS antenna Height | GPS = 20 200 km Galileo = 23 222 km GLONASS = 19 100 km BeiDou MEO = 21 528 km (NOTE 2) | For UMa: 25m + α α ~ uniform(-5,25) m For UMi: 10m + β, where β~uniform[-5, 10 |
| Satellite masking | From [32]: For UMa: 60 degrees, one side of the street. For UMi: 60 degrees, both sides of the street | N.A |
| Antenna Configuration | N.A | For UMa: 4 GHz: (8, 8, 2, 1, 1) (dH, dV) = (0.5, 0.8)  For UMi: 4 GHz: (8, 8, 2, 1, 1) (dH, dV) = (0.5, 0.8)  |
| Network synchronization assumptions | N.A | T1: 0ns (perfectly synchronized), 50ns The network synchronization error, per UE dropping, is defined as a truncated Gaussian distribution of (T1 ns) rms values between an eNB and a timing reference source which is assumed to have perfect timing, subject to a largest timing difference of T2 ns, where T2 = 2*T1. That is, the range of timing errors is [-T2, T2] |
| UE Height | hUT = 1.5 m |  |
| UE Dropping | For outdoor UEs: uniform in cell |  |
| UE noise figure | 9 dB |  |
| UE Speed | 3 km/h |  |
| Description of positioning technique / applied positioning algorithm | GNSS standalone: Weighted Least Square Hybrid GNSS-DL-TDOA: tight integration i.e., measurement level in Weighted Least Square estimator |  |
| Note 1: The assumption is that all GNSS constellations are at Full Operational Capabilities i.e., all satellites are deployed and operational. Note 2: All GNSS satellites are placed in Medium Earth Orbit (MEO). Each GNSS constellation has a specific height at which its satellites are placed. |  |  |

#### 8.x.1.2	Simulation result

Simulation results for GNSS and hybrid GNSS-RAT-dependent were provided by one company [28]. The following section presents the evaluation results obtained for hybrid positioning between standalone GNSS and hybrid positioning between GNSS and DL-TDOA at FR1.

Table 8.x.1.2-1: UMa - GNSS horizontal positioning error [m]

| Company | 50% | 67% | 80% | 90% | 95% |
| --- | --- | --- | --- | --- | --- |
| ESA | 2.33 | 3.01 | 3.72 | 4.64 | 5.42 |

Table 8.x.1.2-2: UMa - Hybrid GNSS-DL-TDOA horizontal positioning error [m]

| Company | Conditions |  | 50% | 67% | 80% | 90% | 95% |
| --- | --- | --- | --- | --- | --- | --- | --- |
| ESA | 0ns | Baseline channel model | 0.70 | 0.96 | 1.24 | 1.59 | 1.98 |
|  |  | Modified channel model | 2.24 | 2.91 | 3.63 | 4.51 | 5.33 |
|  | 50ns | Baseline channel model | 2.29 | 2.99 | 3.71 | 4.61 | 5.43 |
|  |  | Modified channel model | 2.29 | 2.99 | 3.71 | 4.61 | 5.45 |
| NOTE: To modify the channel model, the normalization of the spread delays has been removed from equation 7.5-2 in TR 38.901 to introduce a NLOS bias. |  |  |  |  |  |  |  |

Figure 8.x.1.2-1: Horizontal positioning error in UMa for GNSS and hybrid GNSS-DL-TDOA

Table 8.x.1.2-3: UMi - GNSS horizontal positioning error [m]

| Company | 50% | 67% | 80% | 90% | 95% |
| --- | --- | --- | --- | --- | --- |
| ESA | 4.07 | 5.56 | 7.19 | 9.30 | 10.51 |

Table 8.x.1.2-4: UMi - Hybrid GNSS-DL-TDOA horizontal positioning error [m]

| Company | Conditions |  | 50% | 67% | 80% | 90% | 95% |
| --- | --- | --- | --- | --- | --- | --- | --- |
| ESA | 0ns | Baseline channel model | 0.83 | 1.14 | 1.46 | 1.85 | 2.26 |
|  |  | Modified channel model | 2.0 | 2.67 | 3.40 | 4.37 | 5.48 |
|  | 50ns | Baseline channel model | 3.84 | 5.16 | 6.61 | 8.40 | 10.02 |
|  |  | Modified channel model | 3.87 | 5.16 | 6.66 | 8.47 | 10.09 |
| NOTE: To modify the channel model, the normalization of the spread delays has been removed from equation 7.5-2 in TR 38.901 to introduce a NLOS bias. |  |  |  |  |  |  |  |

Figure 8.x.1.2-2: Horizontal positioning error in UMi for GNSS and hybrid GNSS-DL-TDOA

Below are the vertical positioning error for GNSS and hybrid GNSS-DL-TDOA.

Table 8.x.1.2-5: UMa - GNSS vertical positioning error [m]

| Company | 50% | 67% | 80% | 90% | 95% |
| --- | --- | --- | --- | --- | --- |
| ESA | 2.28 | 3.40 | 4.55 | 5.87 | 7.05 |

Table 8.x.1.2-6: UMa - Hybrid GNSS-DL-TDOA vertical positioning error [m]

| Company | Conditions |  | 50% | 67% | 80% | 90% | 95% |
| --- | --- | --- | --- | --- | --- | --- | --- |
| ESA | 0ns | Baseline channel model | 1.89 | 2.82 | 3.76 | 4.98 | 6.06 |
|  |  | Modified channel model | 2.25 | 3.36 | 4.50 | 5.86 | 6.98 |
|  | 50ns | Baseline channel model | 2.29 | 3.39 | 4.53 | 5.89 | 7.07 |
|  |  | Modified channel model | 2.28 | 3.40 | 4.55 | 5.89 | 7.04 |
| NOTE: To modify the channel model, the normalization of the spread delays has been removed from equation 7.5-2 in TR 38.901 to introduce a NLOS bias. |  |  |  |  |  |  |  |

Figure 8.x.1.2-3: Vertical positioning error in UMa for GNSS and hybrid GNSS-DL-TDOA

Table 8.x.1.2-7: UMi - GNSS vertical positioning error [m]

| Company | 50% | 67% | 80% | 90% | 95% |
| --- | --- | --- | --- | --- | --- |
| ESA | 2.74 | 4.03 | 5.48 | 7.12 | 8.64 |

Table 8.x.1.2-8: UMi - Hybrid GNSS-DL-TDOA vertical positioning error [m]

| Company | Conditions |  | 50% | 67% | 80% | 90% | 95% |
| --- | --- | --- | --- | --- | --- | --- | --- |
| ESA | 0ns | Baseline channel model | 2.18 | 3.27 | 4.41 | 5.81 | 7.02 |
|  |  | Modified channel model | 2.77 | 4.03 | 5.39 | 7.00 | 8.47 |
|  | 50ns | Baseline channel model | 2.49 | 3.72 | 4.98 | 6.37 | 7.77 |
|  |  | Modified channel model | 2.76 | 4.03 | 5.37 | 7.02 | 8.46 |
| NOTE: To modify the channel model, the normalization of the spread delays has been removed from equation 7.5-2 in TR 38.901 to introduce a NLOS bias. |  |  |  |  |  |  |  |

Figure 8.x.1.2-4: Vertical positioning error in UMi for GNSS and hybrid GNSS-DL-TDOA

#### 8.x.1.3	Summary for horizontal and vertical positioning error

Below are the summary of the simulations results. Due to usage of GNSS all UEs are considered to be outdoor.

Table 8.x.1.3-1: GNSS and Hybrid GNSS-DL-TDOA CDF percentiles for 50m, 10m, 3m and 1m horizontal positioning error [%]

| Scenario | 50m |  | 10m |  | 3m |  | 1m |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | UMa | UMi | UMa | UMi | UMa | UMi | UMa | UMi |
| GNSS | 100% | 100% | 100% | 92% | 67% | 35% | 14% | 7% |
| Hybrid GNSS – DL-TDOA perfect synchronisation | 100% | 100% | 100% | 100% | 100% | 99% | 70% | 61% |
| Hybrid GNSS – DL-TDOA 50 ns synchronisation error | 100% | 100% | 100% | 95% | 68% | 38% | 15% | 7% |
| Hybrid GNSS – DL-TDOA perfect synchronisation, modified channel model | 100% | 100% | 100% | 100% | 70% | 74% | 16% | 23% |
| Hybrid GNSS – DL-TDOA 50ns synchronisation error, modified channel model | 100% | 100% | 100% | 95% | 68% | 38% | 15% | 7% |
| NOTE: The 3m target corresponds to the horizontal positioning accuracy requirement for NR Positioning Service Level 2 while the 1m target corresponds to the horizontal positioning accuracy requirement for NR Positioning Service Levels 3 and 4 as defined in TS 22.261. |  |  |  |  |  |  |  |  |

Table 8.x.1.3-2: GNSS and Hybrid GNSS-DL-TDOA CDF percentiles for 50m, 10m, 3m and 2m vertical positioning error [%]

| Scenario | 50m |  | 10m |  | 3m |  | 2m |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | UMa | UMi | UMa | UMi | UMa | UMi | UMa | UMi |
| GNSS | 100% | 100% | 100% | 98% | 62% | 54% | 45% | 38% |
| Hybrid GNSS – DL-TDOA perfect synchronisation | 100% | 100% | 100% | 100% | 70% | 64% | 53% | 47% |
| Hybrid GNSS – DL-TDOA 50 ns synchronisation error | 100% | 100% | 100% | 98% | 62% | 54% | 46% | 38% |
| Hybrid GNSS – DL-TDOA perfect synchronisation, modified channel model | 100% | 100% | 100% | 99% | 63% | 58% | 46% | 42% |
| Hybrid GNSS – DL-TDOA 50ns synchronisation error, modified channel model | 100% | 100% | 100% | 98% | 62% | 54% | 46% | 38% |
| NOTE: The 3m target corresponds to the horizontal positioning accuracy requirement for NR Positioning Service Levels 1 and 2 while the 2m target corresponds to the horizontal positioning accuracy requirement for NR Positioning Service Levels 3 to 6 as defined in TS 22.261. |  |  |  |  |  |  |  |  |