3GPP TSG RAN WG1 #96bis			R1- 1904948

Xi’an, China, 8th – 12th April, 2019

Source:	NTT DOCOMO, INC.

Title:	UL signals and channels for NR-U

Agenda Item:	7.2.2.1.3

Document for: 	Discussion and Decision

- Introduction

At the RAN1 #96 meeting, UL signals and channels for NR-U were discussed and following agreement were made [1].

Agreement: - Support short and long PUCCH durations based on enhancements of at least Rel-15 PUCCH formats PF2 and PF3. The enhancements include at least the following aspects: - For a 20 MHz carrier bandwidth, support mapping to physical resources of at least one full interlace - Mechanism to support user multiplexing for both data and reference symbols of PUCCH - The following aspects are FFS: - Support for small payloads (1 and 2 bits) - Alt-1: Support both small payloads and larger payloads (> 2 bits) for enhanced PF2 and enhanced PF3 - Alt-2: Small payloads are supported by enhanced PF0 and/or enhanced PF1 - Whether or not to replace DFT-s-OFDM with CP-OFDM for the enhanced PF3 Agreement: Support configuration of an SRS resource in additional OFDM symbol locations other than the last 6 symbols of a slot with PUSCH and SRS time division multiplexed as in Rel-15. - FFS: which symbols locations. Agreement: Sub-PRB interlace design for PUSCH and PUCCH is not supported.

In this contribution, we discuss on the UL signals and channels for NR-U, including PUCCH formats, PUSCH interlace design for wideband transmission and PUSCH starting position.

- PUCCH design enhancement for NR-U

At the last RAN1 meeting, it was agreed to support short and long PUCCH durations based on enhancements of at least Rel-15 PUCCH formats PF2 and PF3, and how to support for small payloads (1 and 2 bits) is remained as FFS. There are following alternatives to support small payloads.

  - Alt-1: Support both small payloads and larger payloads (> 2 bits) for enhanced PF2 and enhanced PF3

  - Alt-2: Small payloads are supported by enhanced PF0 and/or enhanced PF1

Alt-2 may improve radio resource usage efficiency compared with Alt-1 where zeros are padded with information bits. On the other hand, Alt-1 would require less effort to be specified than Alt-2.  Considering limited time for NR-U WI in Rel-16 with its large scope, we prefer to deprioritize Alt-2.

Additionally, user multiplexing may be required in order to mitigate a loss in PUCCH capacity due to interlaced resource mapping for PF2 and PF3. As some companies pointed out, OCC in time domain and/or frequency domain is one of solutions to expand the capacity [2].

Proposal 1: Small payloads (1 and 2 bits) are carried on enhanced PF2 and/or enhanced PF3.

- Frequency domain resource allocation signaling for interlaced PUSCH

As we mentioned in [3], partial resource allocation in frequency domain in case of wideband operation is beneficial to use radio resource efficiently. To achieve this mechanism with interlaced UL signal, gNB needs to inform at least following information to UEs for interlaced allocation within continuous LBT sub-band(s).

  - Starting RB position

  - Number of RBs

  - Selected interlace

UL resource allocation type 1 in Rel-15 NR needs enhancement in which information for interlace mapping is added.

If interlaced transmission in discontinuous LBT sub-bands is supported, multiple set of above information need to be informed in the same DCI. However, although this mechanism may help more efficient usage of frequency resources, it requires large effort to specifying and it also needs more discussion on any issue e.g., related to OCB requirement for interlaced transmission in discontinuous LBT subbands. Therefore, we prefer to deprioritize the interlaced allocation in discontinuous LBT subbands in Rel-16 NR-U.

Proposal 2: gNB allocates frequency resources to UEs with informing following information to achieve contiguous partial allocation

  - Starting RB position

  - Number of RBs

  - Selected interlace

- Additional symbol locations for SRS

At the last RAN1 meeting, RAN1 agreed to support additional symbol location(s) for SRS other than the last 6 symbols of a slot with PUSCH and SRS time division multiplexed as in Rel-15. Additional exact positions are remained as FFS.

As we proposed in [4], it is beneficial that SRS is transmitted to inform receiver clearness state according to gNB’s order. For the purpose of this operation, we think that SRS transmission should be allowed in symbols of a slot after the symbols in which DMRS for COT detection is transmitted to response at instant to gNB’s order and to achieve flexible DL transmission. Therefore, the exact symbols in a slot should be discussed after specification of DMRS for COT detection details.

Proposal 3: SRS transmission should be allowed in symbols after the symbols in which DMRS for COT detection is transmitted.

  - Exact symbols are to be discussed after specification of DMRS for COT detection.

- PUSCH starting position

During the NR-U SI, the PUSCH transmission at the beginning of COT had been discussed and following is captured in the TR38.889 [5].

It has been identified to be beneficial for the NR-U design to not require the UE to change a granted TBS for a PUSCH transmission depending on the LBT outcome. The following options have been identified as possible candidate at least for the first PUSCH(s) transmitted in the UL transmission burst. - Option 1: PUSCH(s) as in Rel-15 NR - Option 2: Multiple starting positions in one or multiple slot(s) are allowed for PUSCH(s) scheduled by a single UL grant (i.e., not a configured grant) and one of the multiple PUSCH starting positions can be decided depending on LBT outcome. It is noted that for above options, the ending position of the PUSCH is fixed as indicated by the UL grant. It is noted that above options are not mutually exclusive.

At the last RAN1 meeting, multi-TTI scheduling for PUSCH was agreed to be supported to facilitate efficient scheduling of PUSCH in UE-initiated COT. The multi-TTI scheduling grant can provide multiple candidate PUSCH starting positions such as first symbols in multiple slots or potentially first symbols in multiple mini-slots. Then, the necessity of finer candidate starting position granularity, i.e., symbol/sub-symbol level multiple candidate starting positions within a slot or mini-slot should be discussed. We think such symbol/sub-symbol level granularity for multiple candidate PUSCH starting positions isn’t necessary at least for Rel-16 NR-U, because it needs large effort to specify while it has only limited improvement to the performance on top of slot/mini-slot level granularity for multiple candidate PUSCH starting positions supported by multi-TTI scheduling.

Proposal 4: Symbol/sub-symbol level multiple candidate PUSCH starting positions within a slot or mini-slot is not necessary for Rel-16 NR-U.

- Conclusion

In this contribution, we discussed on the UL signals and channels for NR-U, including PUCCH formats, PUSCH interlace design for wideband transmission and PUSCH starting position. Based on the discussion, we made following proposals.

Proposal 1: Small payloads (1 and 2 bits) are carried on enhanced PF2 and/or enhanced PF3.

Proposal 2: gNB allocates frequency resources to UEs with informing following information to achieve contiguous partial allocation

  - Starting RB position

  - Number of RBs

  - Selected interlace

Proposal 3: SRS transmission should be allowed in symbols after the symbols in which DMRS for COT detection is transmitted.

  - Exact symbols are to be discussed after specification of DMRS for COT detection.

Proposal 4: Symbol/sub-symbol level multiple candidate PUSCH starting positions within a slot or mini-slot is not necessary for Rel-16 NR-U.

References

- 3GPP, RAN1#96, RAN1 Chairman’s Notes, Feb. 2019.

- 3GPP, R1-1903338, Ericsson, “Feature lead summary for UL Signals and Channels,” Mar. 2019.

- 3GPP, R1-1904952, NTT DOCOMO, “Wideband operation for NR-U,” April 2019.

- 3GPP, R1-1904949, NTT DOCOMO, “Channel access procedure for NR-U,” April 2019.

- 3GPP, TR38.889 v1.0.0, Dec. 2018.