3GPP TSG RAN WG1 Meeting #97	R1-1907519

Reno, USA, 13th May – 17th May, 2019

Agenda Item:	7.2.9.3

Source:	Huawei, HiSilicon

Title:	On UE assistance information

Document for:	Discussion and Decision

# Introduction

Reporting UE assistance information is considered for saving UE power since some useful information for UE power saving may not be known by gNB or may not be maintained by gNB with an acceptable complexity. In RAN1#95, companies discussed the potential UE assistance information and the following was agreed [1], which are also captured into TR 38.840[2].

Agreements: The UE assistance information for the power saving schemes for further studies are as follows, - UE assistance information/feedback to assist network in configurations for UE adaptation - UE preferred processing timeline parameters, e.g., K0, K1, K2 values - UE preferred BWP information/configuration - UE preferred antenna configuration, including MIMO layers, antenna panel awareness information - UE assistance/feedback on the DRX configurations/parameters - UE preferred BWP provided to assist network in BWP switching - UE request on SCell/SCG activation/de-activation/configuration - UE preferred PDCCH monitoring parameters/search space configuration/maximum number of blind decoding Other UE assistance information for the power saving schemes is not precluded.

In RAN2 #105bis, UE assistance information for C-DRX configuration was discussed, and many companies think that UE assistance information for C-DRX configuration is beneficial. A RAN2 email discussion on what UE assistance information is needed is ongoing.

In this contribution, we discuss which aspects of UE assistance information are needed, and provide our views on the content of the UE assistance information and UE procedures for reporting assistance information.

# Discussion

## Aspects of UE assistance information to be reported to gNB

The motivation for UE reporting assistance information is to provide information that the gNB cannot obtain or it is difficult to obtain. In this section, we analyse which aspects of UE assistance information are useful to be reported. A summary is given in Table 1.

- For C-DRX, the aspects that influence UE power consumption include the length of DRX cycle, On Duration and Inactivity Timer. It is desirable to have suitable C-DRX configurations for UEs for power saving. A gNB may configure UE-specific C-DRX parameters based on the QoS information. However, when in a cell there is a large number of UEs with different application types and different traffic characteristics, it would be difficult for gNB to acquire service characteristics and decide which set of C-DRX parameters is optimized for each connected mode UE. Therefore in real deployments the parameters for C-DRX configured by gNB may not be optimized per UE basis.  A UE can always know its application type and traffic characteristic, based on which the UE can determine its desired C-DRX configuration, at least from its own perspective. If gNB acquires the assistance information on C-DRX configuration from the UE, it would be helpful for gNB to adjust the configuration per UE basis.

- For CA, the aspects that influence UE power consumption include the number of configured SCells and activated SCells. Of course the gNB knows the CA configuration of a UE, but in Rel-15 the operations of activation/deactivation of carriers are done by the gNB in some sense independently of the UE power status, UE characteristics or UE implementation. The overheating issue has been discussed in RAN2, and it is agreed that when the UE experiences internal overheating the UE can report the number of maximum SCells the UE prefers to be configured with, and the maximum total bandwidth to be aggregated. Similarly, for the purpose of power saving, the UE can also report such kinds of information. It can be studied whether gNB can receive some UE assistance information so that it can adapt the choices of carriers to activate/deactivate for CA.  In addition, the UE may report CC-group configuration, which is analysed in [5].

- The concept of BWP was introduced so that the UE can access part of the BW to save power while the system retains the benefit of using the large BW [3][4]. The large BW implies a higher UE power consumption for both the RF and baseband parts. Similar to CA, although the gNB can always know the BWP configuration of the UE, and the gNB is able to indicate the UE to work on a BWP that matches the UE traffic, the gNB cannot know whether UE needs to save power (e.g. the gNB doesn’t know the UE battery level). Also gNB cannot know the implementation of the UE RF frontend, the impact of which will be explained in Section 2.2. Therefore, it is desirable that the UE reports its desired BWP and/or BWP configuration in the UE assistance information.

- For MIMO, the UE power consumption mainly depends on how many RF chains and antennas are used for transmitting/receiving. From a network perspective, it is better that the UE always uses its full capability to transmit and receive signals in case of the arrival of a burst of data. On the other hand, since the gNB is not aware of the UE power status or UE characteristics (e.g., the UE temperature), the gNB doesn’t know when to allow the UE to use a lower MIMO capability to help the UE to save power. Therefore the UE can report its desired maximum number of layers and/or desired number of antennas in UE assistance information. Note that the UE desired maximum number of layers is already agreed to be reported for the purpose of overheating issue, which can easily be extended to the power saving purpose.

- For processing timeline, the timeline relaxation is helpful for reducing UE power consumption [2]. Further discussions can be found in [6]. To achieve consensus between gNB and UE on processing/scheduling time relaxation, a possible mechanism could be that, UE reports a relaxed processing timeline (other than UE capability associated timeline) to gNB, such as the UE desired values of K0, K1 and K2.

Table 1 UE assistance information that are useful to gNB

| Case | UE assistance information |
| --- | --- |
| C-DRX | - UE desired values of DRX parameters, such as - The length of DRX cycle, and/or - The length of On Duration, and/or - The length of Inactivity Timer. |
| CA | - UE desired number of carriers, and/or - UE desired total bandwidth to be aggregated, and/or - UE desired CC-group configuration. |
| BWP | - UE desired BWP, i.e. the one out of the configured BWPs, and/or - UE desired BWP configuration. |
| MIMO | - UE desired maximum number of layers, and/or - UE desired number of antennas. |
| Processing timeline | - UE desired values of K0, K1 and K2 |

Proposal 1: Consider to support UE reporting assistance information for power saving purpose at least from the following aspects:

- For C-DRX, UE desired lengths of DRX cycle, On Duration and Inactivity Timer.

- For CA, UE desired number of carriers, and/or total bandwidth to be aggregated, and/or CC-group configuration.

- For BWP, UE desired BWP and/or BWP configuration.

- For MIMO, UE desired maximum number of layers and/or number of antennas.

- For processing timeline, UE desired values of K0, K1 and K2.

## Content of UE assistance information

UE may report a single desired value for a parameter. For example, on C-DRX configuration, the UE can simply report its desired DRX cycle length is 160ms, On Duration length is 10ms, and InactivityTimer length is 80ms. This is the simplest and most straightforward way. And the gNB can take the reported values as inputs to make the final decision.

However, such kind of reported information may not be good enough for the network to make the best decision for the configuration or else. From the network’s perspective, when selecting the values to be configured to the target UE, many factors other than power consumption of the target UE are taken into consideration, such as the number of UEs under the same gNB, latency, resource utilization rate, blockage probability, scheduling priority and fairness, and so on. On the contrary, the UE’s desire may only be based on its own status. For example, the UE may report that it desires to adjust the value of C-DRX cycle length from 40ms to 320ms. After the network receiving the information, the network may only be able to configure the UE with C-DRX cycle length equalling to 160ms. In such a case, if 320ms is used, then there will be significant impact on the network’s performance. If 160ms is used, the network doesn’t know whether it is acceptable for the UE to achieve its power saving purpose, since no more information is provided. If 40ms is still used, the reporting from the UE is then meaningless.

In order to optimize the configured C-DRX parameters, the gNB can configure the UE with multiple C-DRX configurations that gNB can support, and then the UE can report the desired configuration from the configurations sent by gNB.

Proposal 2: At least for UE assistance information on C-DRX configuration, the UE can report a desired set of C-DRX configurations selected from the multiple sets of C-DRX configurations that gNB can configure.

Alternatively the UE may report that a larger or a smaller value than current configuration, or a default value is desired. For example, the UE is configured with 4 SCells, which are all activated. When the UE want to save power (maybe the UE is in a low battery level), instead of reporting the exact number, the UE may just report it desires a smaller number of activated SCells. Then the network can deactivate, for example, 2 SCells. Also, when the UE doesn’t want to save power (maybe the UE is fully charged), the UE may report it desires a smaller number of activated SCells. Or the UE may require to recover to the origin number (which is also called default value in the proposal) of activated SCells which is before it report the UE assistance information, i.e. 4 SCells. Similar method can also be used to other aspects, such as a longer or shorter C-DRX cycle, a wider or narrower BWP, and so on.

The UE may also report more than one value for a parameter, one is the “optimal” value, and the other one is the “suboptimal” value. The “optimal” value means that this value can achieve a target power saving, and the “suboptimal” value means that this value at least allows to achieve some power saving gain. For example, for the bandwidth of the active BWP, the relationship between the bandwidth of BWP and the power consumption depends on UE implementation. The RF component of the UE usually has several levels of working bandwidth, which may be for example 10MHz, 20MHz, and 50MHz. When the UE is configured with a BWP whose bandwidth is larger than 10MHz, smaller than or equal to 20MHz, the power consumption of RF component may be the same since in all the cases the working bandwidth of the RF component keeps unchanged. If the UE is working on a BWP of 20MHz, and needs to save power, then it can report to the network its desired optimal bandwidth of the active BWP is 5MHz. The network will consider many factors to make the final decision. And the network may only be able to adjust the UE active BWP with a bandwidth of 15MHz. From the network perspective, the UE reporting has been taken into account, since the bandwidth is scaled down, however, from the UE perspective, the power consumption decrease is not significant. In this example, the UE may report its suboptimal value is 10MHz, which means that if the network cannot configure a bandwidth that is equal to or smaller than 10MHz, then the network does not need to change the bandwidth at all.

Proposal 3: For UE assistance information provided to gNB,

- UE can report that a larger or smaller value than the current configured value, or a default value, is desired.

- UE can report the desired range of the value for a parameter.

## UE procedures for reporting assistance information

The feature of UE assistance information is not mandatory. The indication that the network supports or enables such function can be either broadcasted in system information or indicated by dedicated signalling. After the UE knows the UE assistance reporting is enabled by network, the UE could report the assistance information. Two possible procedures are shown in Figure 1(a) and Figure 1(b). In Figure 1(a), when the network indicates enabling of the function, the UE can report the assistance information based on UE implementation. In Figure 1(b), the UE assistance information reporting is requested by the network. For example, when the network wants to reconfigure the parameters to the UE, it may send a message to request the UE’s preference on the configuration.

(a)

(b)

Figure 1 Procedures of UE assistance information

Regarding the mechanism of reporting UE assistance information, it is understood that the UE assistance information reported by the UE for power saving is purely a UE recommendation and it is up to the network to decide whether and how to use the recommendation from UE. It is worth noting that while UE can show its preference on the aspects discussed above, it is up to gNB to decide whether or not to apply each of the configuration or indication. The trade-off between UE power saving gains and potential performance loss at system level should be evaluated by gNB before a decision is made. Thus we have the following proposals.

Proposal 4: UE reporting assistance information is not a mandatory function supported by the network.

Proposal 5: UE assistance information reported by UE for power saving is only UE recommendation and the network shall make the final decision on whether and how to use that information.

# Conclusions

In this contribution, UE assistance information is discussed. We have the following proposals:

Proposal 1: Consider to support UE reporting assistance information for power saving purpose at least from the following aspects:

- For C-DRX, UE desired lengths of DRX cycle, On Duration and Inactivity Timer.

- For CA, UE desired number of carriers, and/or total bandwidth to be aggregated, and/or CC-group configuration.

- For BWP, UE desired BWP and/or BWP configuration.

- For MIMO, UE desired maximum number of layers and/or number of antennas.

- For processing timeline, UE desired values of K0, K1 and K2.

Proposal 2: At least for UE assistance information on C-DRX configuration, the UE can report a desired set of C-DRX configurations selected from the multiple sets of C-DRX configurations that gNB can configure.

Proposal 3: For UE assistance information provided to gNB,

- UE can report that a larger or smaller value than the current configured value, or a default value, is desired.

- UE can report the desired range of the value for a parameter.

Proposal 4: UE reporting assistance information is not a mandatory function supported by the network.

Proposal 5: UE assistance information reported by UE for power saving is only UE recommendation and the network shall make the final decision on whether and how to use that information.

# References

Chairman's Notes RAN1#95

3GPP TR 38.840, v1.0.0.

Chairman’s Notes RAN1_NRAH2_final

R1-1711795, On bandwidth parts and RF requirements, Ericsson

R1-1907522, CC group operation for UE power saving, Huawei, HiSilicon

R1-1906006, Procedure of cross-slot scheduling for UE power saving, Huawei, HiSilicon